3GPP TSG-RAN WG3 Meeting #127b	R3-252010

Wuhan, China, 07th – 11th April, 2025

Agenda Item:	14.3

Source:	Huawei

Title:	(TP for TS 38.300) Support of regenerative payload - various topics

Document for:	discussion

# 1	Introduction

In RAN3#126 meeting, the following FFS is given for the regenerative payload [1]:

Whether to support NG suspend/Resume?

In this paper, we continue to discuss the NG suspend/resume in the regenerative payload.

Then we provide our views on this feeder link switch over, CHO enhancements and Mapped cell Id.

# 2	Discussion

## 2.1. NG suspend/resume operation

The NG removal procedure was already agreed and captured by the BLCRs. The previous meetings mainly discussed whether to introduce the NG suspend/resume procedure also, which was first proposed by the paper [2] for the case of hard feeder link switchover, which was discussed at RAN3#125bis meeting with the following note [3]:

- RAN CONFIGURATION UPDATE procedure could be reused to support NG suspend/Resume, by introducing suspend/resume indicators to avoid frequent TNL disconnection and re-connection?

This topic was discussed during several meetings without conclusion. From the proposals, the claimed advantage for the NG suspend/resume procedure may be a little signalling overhead reduction when establishing the NG connection with the same AMF again and not re-establishing the interface context (NG configurations). But it is very doubtful whether the same NG configurations can be actually reused when the satellite gNB accesses the same AMF again. For example, when the gNB resumes NG connection from different location, the supported TA information could be different, so the old stored supported TA is useless; and, if the gNB resumes NG connection via a different GW, the current TNL address is different from the old TNL address, so the old NG-U tunnel configuration is also outdated.

Considering the feeder link switch is generally caused by the changing of satellite coverage, and as analyzed above, if the coverage area changes and GW changes, the NG configurations cannot be reused directly.

Observation 1: The old NG configurations cannot be reused when the gNB accesses the AMF at different location or via different GW.

In addition, the NG suspend/resume operation requires the gNB stores those NG configurations related to multiple AMFs. Considering the earth rotation, it may take a long time for the gNB connects to the same AMF again, which causes much storage consumption on the gNB for retaining multiple AMFs’ NG configurations.

Observation 2: NG suspend/resume operation requires the gNB to store NG configurations relevant to multiple AMFs, which leads to much storage consumption on the regenerative payload/gNB.

Based on the observations above, we propose:

- No need to introduce the NG suspend/resume towards the AMF.

## 2.2. Feeder link switch over consideration

For the feeder link switch over, RAN3 already made the following agreements.

Feeder link switch issue:

RAN3 assumes that during the feeder link switch, AMF is not changed for the UEs. Whether there has any standard impact needs to be further checked.

The supported TAIs of an on-board gNB could be provided to AMF by OAM/pre-configuration.

The current mechanisms including, e.g. multiple TNLA association, are sufficient for TNL management during feeder link switch.

In the previous meetings, some companies had proposals on the DL NG-U TNL address updating after the hard feeder link switch over (FLSO).

First, it does not always happen that the DL NG-U TNL address is changed after each hard FLSO. This is true for soft FLSO, but not for hard FLSO. As agreed above, the multiple TNLAs are introduced for the soft FLSO, that is, the satellite gNB can establish multiple TNLAs via separate GWs at the same time, so that when the old feeder link shuts down, the new TNLA associated with the new GW can be directly used. However, for the hard FLSO, the TNL can be kept. The change of the gateway working as a TNL router, and should not influence the TNL address on the satellite gNB.

- The hard FLSO is irrelevant to the DL NG-U TNL address update.

Second, if the DL NG-U TNL address for a specific UE needs to be changed regardless of the FLSO, the existing PDU Session Resource Modify Indication procedure can be used. Also, there is no need that the NG-U TNL addresses for all served UEs need to be changed simultaneously. Therefore, introducing a non-UE associated signalling to update the DL NG-U TNL addresses is not needed.

- Current PDU Session Resource Modify Indication message is sufficient for the DL NG-U TNL address update.

With the above agreements agreed at previous meetings, it is suggested to agree the TP on the multiple SCTP associations for the feeder link switch.

- RAN3 to agree the TP on the multiple SCTP associations for the feeder link switch.

TP for TS 38.300 on top of R3-250937

<<<<<<<<<<<<<<<<<<<< Change Begins >>>>>>>>>>>>>>>>>>>>

### 16.14.4	Switchover

#### 16.14.4.1	Definitions

A feeder link switchover is the procedure where the feeder link is changed from a source NTN Gateway to a target NTN Gateway for an NTN payload. The feeder link switchover is a Transport Network Layer procedure. Service link switch refers to a change of the serving NTN payload.

Both hard and soft feeder link switchover are supported in NTN payloads.

#### 16.14.4.2	Assumptions

A feeder link switch over may result in transferring the established connection for the affected UEs between two gNBs.

For soft feeder link switch over, an NTN payload is able to connect to more than one NTN Gateway during a given period, i.e. a temporary overlap can be ensured during the transition between the feeder links.

For hard feeder link switch over, an NTN payload connects to only one NTN Gateway at any given time, i.e. a radio link interruption may occur during the transition between the feeder links.

#### 16.14.4.3	Procedures

The NTN Control function (see Annex B.4) determines the point in time when the feeder link switch is performed. The transfer of the affected UE(s)' context at feeder link switch over is performed by means of either NG based or Xn based handover or TNL function, and it depends on the gNBs' implementation and configuration information provided to the gNBs by the NTN Control function.

<<<<<<<<<<<<<<<<<<<< Change Ends >>>>>>>>>>>>>>>>>>>>

## 2.3. location-based CHO enhancement for NR NTN

There were proposals at previous meetings to introduce the location-based CHO enhancement for NR NTN. Generally, two different ways or combination of these are proposed as observed from the contributions as follows:

- Transfer the handover execution condition, i.e., the distance threshold to the reference locations of the serving cell and candidate cell;

- the UE coarse location.

The claimed motivation is that the target gNB can easily infer the UE handover execution time so that it can prepare the resource reservation accordingly.

In our understanding, the resource reservation is possible for the time-based CHO, since the handover execution time is deterministic in this case. However, for the location-based CHO, it is very hard for the target gNB to determine the UE handover execution time accurately based on the UE coarse location and/or the handover execution conditions.

First, according to TS 38.331, the coarse location reported by the UE is with a granularity of approximately 2 km, copied as follows. The target gNB cannot easily determine the exact UE handover execution time based on such coarse information.

UEInformationResponse-IEs field descriptions

coarseLocationInfo Parameter type Ellipsoid-Point defined in TS 37.355 [49]. The first/leftmost bit of the first octet contains the most significant bit. The least significant bits of degreesLatitude and degreesLongitude are set to 0 to meet the accuracy requirement corresponds to a granularity of approximately 2 km. It is up to UE implementation how many LSBs are set to 0 to meet the accuracy requirement.

Second, for the location-based CHO, the HANDOVER REQUEST message may be transmitted long time before the UE HO execution time given that it is always the UE to evaluate the execution conditions. In this case, the UE location may be changed a lot during the time between the HO preparation and the real HO execution. This also lead to the situation that the target gNB is impossible to generate the correct HO execution time.

In summary, it is observed and proposed:

Observation 1: The target gNB is hard to determine the UE CHO execution time for the location-based CHO to prepare the resource reservation, considering the coarse granularity of UE location and the UE movement during the time between HO preparation and HO execution.

- Not to enhance the location-based CHO in Xn or NG.

## 2.4 Mapped cell Id

In the previous meetings, some companies had proposals that the gNB ID contained in the mapped cell ID should remain constant in a geographical area. But we see some restrictions for such proposal.

Firstly, it would be natural to pre-configure the satellite ephemeris information on the CN. When the satellite gNB ID remains constant as the regenerative payload moves, similar to the TN gNB, it is easy for the CN to know the relationship between the gNB ID, cell ID and its location, and finally to know the geographical location corresponding to the mapped cell ID. There is no need to restrict the gNB ID to an earth-static location.

Secondly, it would be very challenging to define exact the “geographic area” for the gNB ID contained in the mapped cell, considering the moving cell scenario and the different coverage range of different satellites (e.g., with different altitudes). For example, when two regenerative payloads with different ephemeris cover the different range of areas, it is very little hard to define a unified “geographic area” concept. Also, it would be also difficult when a satellite covers a space across multiple “geographic areas”.

Hence it would be better to leave the gNB ID contained in the mapped cell ID as an implementation issue.

Observation 1: The AMF is able to know the relationship between the gNB ID and its location by pre-configuration if the satellite gNB ID remains constant as it moves.

Observation 2: For the case of moving cell and different satellites with different coverage range, defining the geographic area would be very challenging.

- Not to specify that the gNB ID in the mapped cell ID remains constant in a geographical area.

# 3	Conclusion

This contribution discusses the NG suspend/resume, FSLO issues in regenerative payload for the NR NTN. It is proposed:

- No need to introduce the NG suspend/resume towards the AMF.

- The hard FLSO is irrelevant to the DL NG-U TNL address update.

- Current PDU Session Resource Modify Indication message is sufficient for the DL NG-U TNL address update.

- RAN3 to agree the TP on the multiple SCTP associations for the feeder link switch.

- Not to enhance the location-based CHO in Xn or NG.

- Not to specify that the gNB ID in the mapped cell ID remains constant in a geographical area.

# 4	References

- RAN3#126 Chair notes.

- R3-245434, (TP for TS 38.300) Discussion on RAN Signaling impacts for NR NTN Regenerative Payload, Qualcomm Incorporated, RAN3 125b.

- RAN3#125bis Chair notes.