3GPP TSG-RAN WG3 Meeting #127-bis	R3-252011

Wuhan, China, 7-11 April 2025

Title: 	Further discussion on Energy Saving Indicator

Source: 	Ericsson, Jio Platforms (JPL), BT

Agenda item:	8.3

Document Type:	Discussion and agreement

# 1. Introduction

In this contribution we further discuss aspects related to the energy saving indicator included in the subscription date for a UE and the benefits of receiving this indication at the RAN.

# 2. Discussion

At RAN3#127 meeting, we received from SA2 an LS in R3-250024 (S2-2413034) related to “KI#2: UE subscription and policy control for energy efficiency and energy saving.”. The CR 5739 to TS 23.501 attached to the LS indicates that, subscription data for a UE may include an Energy Saving Indicator. It is FFS whether AMF can forward this information to RAN. It is also FFS whether there can be multiple values of the indicator.

5.X.Y Subscription information aspects The Subscription data for a UE may include an Energy Saving indicator that the UE is subject to network energy saving operation. The AMF receives the Energy Saving indicator in the UE Subscription data. The AMF forwards the Energy Saving indicator to the NG-RAN and to the PCF. Editor’s Note: The indicator is expected to select an energy saving strategy for the UE in the system. Whether there are multiple values of the indicator is FFS. Editor’s Note: Whether the indicator is provided to the NG-RAN and how it is used in the NG-RAN is FFS. Editor’s Note: Whether the Energy Saving indicator is also stored in the UDR is FFS. 5.X.Z Policy control aspects During AM policy association establishment procedure, the AMF sends to the PCF the Energy Saving indicator as described in clause 5.x.y. Based on the Energy Saving indicator the PCF may make policy decisions reusing the existing parameters based on the operator policy. Editor’s Note: A notification for the energy related information, if any, should not be provided from the UDM/UDR. Whether the notification for the energy related information from the EIF is provided to the PCF and what are the notification details, are FFS.

The Energy Saving indicator can be part of user subscription data and when present, this information indicates that the user agrees the UE can be subject to network energy saving operation. The indication is generic, meaning that the network can decide which energy saving operation(s) to adopt. It is also generic in the sense that it is not precluded that such network energy saving operations can be decided, in CN or in RAN, or both.

Simply, the user accepts some kind of performance degradation (or that performances may not exceed certain levels) in exchange of some energy saving gain (at the network, or at the UE, or both).

Observation 1: If subscription data for a UE includes the Energy Saving indicator, the user subscribed to network energy saving operation.

Different network implementations can support different energy saving operations, and the same implementation may adopt different strategies to achieve energy savings depending on various factors (for example, depending on the number of served UEs, or which services are being used, distinguishing between users at cell edge or at cell center, etc.)

Observation 2: Different network implementations can support different energy saving operations.

Observation 3: Different network implementations can achieve the same energy saving in different ways.

At previous meeting, some companies propose that to make use of the subscription data for network energy saving operations controlled by CN, e.g., setting up different QoS parameters for a UE depending on whether its subscription data includes or not includes the energy saving indicator. This is a certainly a possibility, however we see at least two drawbacks or limitations if we restrict the use of the energy saving indicator solely in the CN.

Let’s assume to have two UEs, UE1 and UE2. UE1 subscribed to network energy saving operation, UE2 did not. The two UEs execute the same best effort service, but the QoS parameters that RAN receives are different for the two UEs.  A first problem is that RAN will try to respect the QoS parameters, but it is oblivious of the fact that such QoS parameters are different due to the different UE subscriptions. Since the service is best effort, the RAN can adopt its energy saving strategies and algorithms and treat the two UEs in the same way, which means that setting different QoS parameter will have no practical effect.

A user running best effort service can get all the resources left from other users running non-best effort services. Therefore, if UE1 subscribed to network energy saving operation, and UE2 did not, in case UE1 executes a best effort and UE2 instead a non-best effort service, the RAN does not know that it can spare some resources for UE1, and this goes against the network energy saving the RAN could have achieved.

Let’s assume now that UE1 and UE2 execute a non-best effort service. In this case, the RAN will try to treat differently the two UEs, striving to respect the QoS parameters. However, the RAN (still) does not know that the QoS parameters are different because of the different subscriptions. Also, while QoS parameters provide “long-term” (or target) indications, the RAN has a real-time view of the energy usage and user performance. Therefore, without knowing the willingness of the users, the RAN may decide to execute its network energy saving strategy and algorithms, and temporarily sacrifice the performance of UE2 (not subscribed to network energy saving operation) instead of sacrificing the performance of UE1 (subscribed to network energy saving operation). The QoS parameters will be eventually respected for both users, but there will be fluctuations in performance for UE2 instead of UE1, which goes in the opposite direction compared to what is desirable.

If the RAN knows that UE1 and UE2 have different willingness in terms of network energy treatment, in both the examples provided can try to accommodate higher resources to the correct UE, while still running its network energy strategies and algorithms.

Based on the analysis above, we think that it is beneficial for RAN to become aware of the energy saving indicator.

Observation 4: It is beneficial for RAN to be aware of user agreement to use network energy saving operation.

As a further observation, we know that various network energy saving mechanisms have been or are being standardized. In rel-18, network energy saving mechanisms have been defined for CHO (in case the source cell is using a network energy saving solution, the UE may use NES-specific CHO event for executing CHO to a candidate cell). In rel-19 Network Energy Saving Enhancement WI, RAN3 is discussing mechanisms related to network energy saving operations, such as support for on-demand SSB SCell operation, and support for on-demand SIB1 for UEs.

Observation 5: RAN WGs have already defined or are working on solutions for RAN network energy saving.

Also, as acknowledged during last RAN3 meeting, there is not relation between the Energy Saving indicator under discussion in SA2 and the other network energy saving mechanisms discussed in RAN3. It has also been acknowledged that if RAN receives the Energy Saving indicator, there should not be any direct implications on RAN behaviour, including the mechanisms already defined (or being defined) in RAN for network energy saving.

Taking the above into account, we propose that AMF provides to RAN information on whether a user subscribed to network energy saving operation as a simple value (e.g., a Boolean), and it is left to RAN implementation whether, when and how to use it.

Proposal 1: NG-RAN can receive from AMF a single value (e.g., a Boolean) for a UE, to indicate whether the user subscribed to network energy saving operation.

Based on the presented discussion, we propose to reply to the incoming LS R3-250024 as described in R3-252015.

Proposal 2: Agree to the reply LS in R3-252015.

# 3. Conclusions

Based on the discussion in this paper, the following observations and proposals are derived:

Observation 1: If subscription data for a UE includes the Energy Saving indicator, the user subscribed to network energy saving operation.

Observation 2: Different network implementations can support different energy saving operations.

Observation 3: Different network implementations can achieve the same energy saving in different ways.

Observation 4: It is beneficial for RAN to be aware of user agreement to use network energy saving operation.

Observation 5: RAN WGs have already defined or are working on solutions for RAN network energy saving.

Proposal 1: NG-RAN can receive from AMF a single value (e.g., a Boolean) for a UE, to indicate whether the user subscribed to network energy saving operation.

Proposal 2: Agree to the reply LS in R3-252015.