3GPP TSG-RAN WG3 Meeting #127-bis	R3-252020

Wuhan, CN, 7 - 11 April, 2025

Agenda Item:	8.1

Source:	Ericsson

Title:	Discussion on SA2 LS regarding Emergency PDU Session

Document for:	Response paper

# 1	Introduction

This paper provides a view on RAN3’s action regarding the LS reply from SA2 to RAN2 on usage of eDRX in RRC_INACTVE during an emergency PDU session in [1], as previous discussion in [2].

# 2	Discussion

Background:

RAN2 asked SA2 [3], cc-ing RAN3, if there may be cases where the UE has an emergency PDU session, but the gNB is not aware of this, i.e. the gNB may release the UE to RRC_INACTIVE with eDRX configured. This topic was discussed by SA2 for two meetings and a LS reply from SA2 was sent to RAN2. Below their feedback:

SA2 analysed the identified issue above and would like to provide the following feedback: - Existing specifications have specified that each PLMN/operator reserves a specific ARP value for emergency PDU session and provide to RAN node. See related text in TS 23.501 clause 5.16.4.3, 5.16.4.7, 5.16.4.9 and TS 23.502 clause 6.1.3.10. Therefore the RAN node can be aware of an emergency PDU session based on the reserved ARP value when the PDU session context exist in the RAN node. - In addition there is inactivity monitoring for the presence of user plane resources for the emergency PDU session in SMF as defined in TS 23.502 clause 4.4.2.2 and TS 23.503 clause 6.1.3.10. Based on the above, some companies in SA2 believe that the RAN node is capable through configuration to not release the emergency PDU session user plane resources based on existing implementation, and to not release the UE to RRC_INACTIVE state/RRC_IDLE state when the UE has an emergency PDU session. Therefore the issue highlighted in RAN2 LS may not exist. However there is no consensus in SA2 whether the UP of emergency PDU session can be deactivated, or whether to introduce new signalling to NG-RAN.

Based on the above SA2 LS, we can deduce the following:

- The NG-RAN knows when there is an emergency PDU session based on the ARP value. Hence, there is no need to introduce any new NGAP signaling from CN to NG-RAN to indicate whether presence of emergency PDU session.

- The call back issue can be handled by NW implementation:

- Note that the call back is handled as a normal call see extract from TS 23.167 below). The UE is kept in RRC_CONNECTED when UE has emergency PDU session based on existing implementation (e.g. no paging is needed during call back and CN is responsible for the emergency PDU session release based on specific resource monitoring timer for emergency).

Extract from TS 23.167 cl. 4.1: 12. The architecture shall enable emergency centres and PSAPs to request a PSAP call back to a UE with which the Emergency centres or PSAPs had an emergency session. The serving network of the UE shall use the appropriate call termination procedures e.g. IMS if the UE is available for voice over PS, or ICS if the user is available over CS. PSAP call back is subject to local regulation. NOTE 3: PSAP call back sessions are treated as normal calls. NOTE 4: Subject to local regulation, any supported media can be used during a call back attempt from a PSAP. Extract from TS 22.101 cl. 10.1.3: ### 10.1.3 Call-Back Requirements Subject to local/regional regulations the network shall support a call-back from a PSAP. It shall be possible to supply the user's Directory Number/MSISDN/SIP URI as the CLI to the PSAP to facilitate call-back. The CLI used on call-back shall allow the PSAP to contact the same terminal that originated the emergency call. If the incoming call can be identified by the core network as a call-back to an emergency call (i.e. coming from a PSAP) then supplementary services at the terminating party shall be handled as described in TS 22.173 [40] for Multimedia Telephony (e.g. Communication Diversion, Communication Hold, Communication Barring). NOTE: There is no specific callback requirement for CS supplementary services. A call-back may be attempted for a period of time defined by local regulations after the emergency call release. In case of a UE in limited service state, call-back is not required.

- The RAN2 issue may not exist. A UE configured with emergency PDU session is not expected to be released to RRC_INACTIVE state with eDRX for RAN paging

Observation 1: The RAN can know that the UE is in emergency PDU session because of the operator assigned ARP value, hence no new NGAP signalling is needed.

Observation 2: Any identified issue related to handling of call back during emergency PDU session can be solved by NW implementation (e.g., no paging is needed during call back and CN is responsible for the emergency PDU session release based on specific resource monitoring timer for emergency).

Observation 3: UE configured with emergency PDU session is not expected to be released to RRC_INACTIVE state with eDRX for RAN paging

Therefore, RAN3 can note the SA2 solution: a UE with emergency PDU Session stays in RRC_CONNECTED state (e.g. there is no scenario that paging will be triggered when UE has emergency PDU session)

Proposal 1: RAN3 can note the SA2 solution: a UE with emergency PDU Session stays in RRC_CONNECTED state i.e., there is no scenario that paging will be triggered when UE has emergency PDU session).

A reply LS to confirm SA2 assumption is available in [3]

# 3	Conclusion

Observation 1: The RAN can know that the UE is in emergency PDU session because of the operator assigned ARP value, hence no new NGAP signalling is needed.

Observation 2: Any identified issue related to handling of call back during emergency PDU session can be solved by NW implementation (e.g., no paging is needed during call back and CN is responsible for the emergency PDU session release based on specific resource monitoring timer for emergency).

Observation 3: UE configured with emergency PDU session is not expected to be released to RRC_INACTIVE state with eDRX for RAN paging

Proposal 1: RAN3 can note the SA2 solution: a UE with emergency PDU Session stays in RRC_CONNECTED state, i.e., there is no scenario that paging will be triggered when UE has emergency PDU session.

# 4	References

- R3-251521, Reply LS on emergency call back and paging, ZTE

- R3-250006, LS on Emergency Services and eDRX, RAN2

- R3-252021, [Draft] Reply LS on emergency call back and paging