3GPP TSG-RAN WG3 Meeting #127bis	R3-252030

Wuhan, China, 07 - 11 April, 2025

Agenda Item:	21.3

Source:	Ericsson

Title:	(TP for XR BL CR for TS38.413) Support of PDU Set QoS enhancement

Document for:	Other

# 1	Introduction

The WID of R19 XR has been updated in [1] to add the following objective:

- Support of PDU set based QoS handling enhancement [RAN3]: - Support of DL PDU Set marking without PDU Set QoS - Support of Alternative PDU Set QoS, which may contain UL and/or DL PDU Set QoS Parameters (i.e. UL PSDB, DL PSDB, UL PSER and/or DL PSER). - QoS Handling when Traffic Characteristics Change Dynamically [RAN3] - TTNB and burst size to be provided over GTP-U - Support of exposure of available data rate [RAN3] Note: Coordination with SA2, as needed - Specify Core requirements related to the above objectives as necessary [RAN4].

This paper provides our views on the above enhancements.

# 2	Discussion

## 2.1 DL PDU Set Marking

SA2’s Release-19 work on PDU Set QoS enhancement added the option that no PDU Set QoS Parameters (PSQPs) are provided by the AF, thus the SMF does not include any PDU Set QoS information (PSDB, PSER and PSIHI) in the QoS parameters within the NGAP PDU Session message to RAN. SA2 has agreed instead that a new PDU Set Information Marking Support Indicator is sent to RAN to indicate that CN supports PDU Set marking.

Below the relevant extract from TS 23.501 latest spec:

### 5.37.5 PDU Set based Handling #### 5.37.5.1 General [skipped irrelevant SA2 text] If no DL PDU Set QoS parameters are included in the PCC rule, but a DL Protocol Description is included in the PCC rule received by the SMF and the PSA UPF supports PDU Set Handling (i.e. supports to perform PDU Set Information marking), the SMF may, based on the local operator policy, send to the NG-RAN, a DL PDU Set Information Marking Support Indication indicating that the DL PDU Set Information marking is supported. [skipped irrelevant SA2 text] If the DL PDU Set Information Marking Support Indication has been sent to the 5G-AN (as described in clause 5.37.5.1) and the 5G-AN has provided a PDU Set based handling support Indication indicating that PDU Set handling is supported, the SMF instructs PSA UPF to perform PDU Set marking and provides the PSA UPF the DL Protocol Description used by the service data flow. [skipped irrelevant SA2 text] #### 5.37.5.3 Non-homogenous support of PDU set based handling in NG-RAN The SMF, by sending PDU Set QoS parameters (as described in clause 5.7.7.1) or a DL PDU Set Information Marking Support Indication (as described in clause 5.37.5.1) to the NG-RAN, requests the NG-RAN to activate PDU Set handling for a given QoS Flow and the NG-RAN provides the SMF with a PDU Set Based Handling Support Indication if the PDU Set based handling is supported. Based on this indication, SMF may activate the PDU Set identification and marking in the PSA UPF as described in clause 5.37.5.2. During mobility procedures that result in the change of NG-RAN, the target NG-RAN provides to the SMF a PDU Set Based Handling Support Indication if it supports PDU Set based handling, as specified in TS 38.413 [34]. Based on the target NG-RAN indication, the SMF may, upon completion of the mobility procedure, initiate the PDU Session modification procedure to provide PDU Set QoS parameters or the DL PDU Set Information Marking Support Indication to NG-RAN and may configure the PSA UPF to activate the PDU Set identification and marking. If the PDU Set Based Handling Support Indication is not received from the target NG-RAN and PDU Set identification and marking is active in the PSA UPF, the SMF may deactivate it. In the case where the PSA UPF identifies and marks PDUs with PDU Set information in GTP-U header, it shall start doing so from a complete PDU Set. NOTE: Based on the PDU Set Based Handling Support Indication from the target NG-RAN, the SMF can update the QoS profile and, if applicable, Alternative QoS Profiles as defined in clauses 5.7.1.2 and 5.7.1.2a to include the PDU Set QoS parameters respectively.

Observation 1: In Release-19, a new DL PDU Set Information Marking Support Indicator is sent from SMF to NG-RAN when no PSQP are present. The new indication refers to CN’s support of DL PDU Set marking.

Form the SA2 text, the NG-RAN when it receives the DL PDU Set Information Marking Support Indication IE feedbacks to CN its support of PDU Set handling in this case by re-using the existing PDU Set Based Handling Support Indication IE, similar as to when receiving PSQP in Rel-18.

Observation 2: The existing PDU Set Based Handling Support Indication IE is re-used when the NG-RAN receives the new DL PDU Set Information Marking Support Indication IE from the CN to indicate its support of PDU Set handling.

It is possible therefore to support the enhancement related to this objective by adding the following two changes:

- Introducing the new DL PDU Set Information Marking Support Indication IE in the QoS parameters per QoS flow level, similar to PDU Set QoS Parameters.

- Extending the procedural text in TS 38.413 related to RAN signalling of PDU Set Based Handling Indicator IE, to support the case of receiving the new PDU Set Information Marking Support Indicator from SMF.

Proposal 1: Add the new DL PDU Set Information Marking Support Indication IE in the QoS parameters per QoS flow level, similar to PDU Set QoS Parameters

Proposal 2: Extend the procedural text in TS 38.413 related to RAN signalling of PDU Set Based Handling Indicator IE, to support the case of receiving the new PDU Set Information Marking Support Indicator from SMF

An encoding example of TS 38.413 is presented in the annexed TP of this paper.

We note that XnAP needs also to be impacted due to the presence of PDU Set Based Handling Indicator IE in other procedures.

Proposal 3: same changes as proposals 1 and 2 are needed in XnAP spec.

## 2.2 Alternative QoS Parameters in PDU Set

SA2 have also added impact to the concept of alternative QoS Profile, long debated topic in RAN3 in Rel-16, to add indication of PSQP in the Alternative QoS profiles when the PDU Set QoS are present together with AQP in a specific direction:

#### 5.7.1.2a Alternative QoS Profile The Alternative QoS Profile(s) can be optionally provided for a GBR QoS Flow with Notification control enabled. If the corresponding PCC rule contains the related information (as described in TS 23.503 [45]), the SMF shall provide, in addition to the QoS profile, a prioritized list of Alternative QoS Profile(s) to the NG-RAN. If the SMF provides a new prioritized list of Alternative QoS Profile(s) to the NG-RAN (if the corresponding PCC rule information changes), the NG-RAN shall replace any previously stored list with it. An Alternative QoS Profile represents a combination of QoS parameters PDB, PER, Averaging Window and GFBR to which the application traffic is able to adapt. The Alternative QoS Profile(s) shall also contain the PDU Set QoS parameters PSDB and PSER in the respective direction(s) when the QoS Profile contains PDU Set QoS parameters PSDB and PSER in UL and/or DL as described in clause 5.7.7; otherwise, the corresponding Alternative QoS Profile(s) shall not contain the PDU Set QoS parameters PSDB and PSER. NOTE 1: There is no requirement that the GFBR monotonically decreases, nor that the PDB or PER (or PSDB or PSER) monotonically increase as the Alternative QoS Profiles become less preferred. For a GBR QoS Flow using the Delay-critical resource type, an Alternative QoS Profile may also include the QoS parameter MDBV. For the NG-RAN determination whether the GFBR, the PDB and the PER can be fulfilled (i.e. for all Notification control scenarios described in clause 5.7.2.4), the NG-RAN shall also apply the value of the optional QoS parameter present in an Alternative QoS Profile instead of the corresponding QoS parameter's value in the QoS Profile (which is either provided explicitly or implicitly via the standardized 5QI QoS characteristics defined in clause 5.7.4) for every optional QoS parameter present in an Alternative QoS Profile (e.g. MDBV). NOTE 2: For the NG-RAN behaviour related to the mandatory QoS parameters in the Alternative QoS Profile (i.e. GFBR, PDB, PER), see clause 5.7.2.4. When the NG-RAN sends a notification to the SMF that the QoS profile is not fulfilled, the NG-RAN shall, if the currently fulfilled values match an Alternative QoS Profile, include also the reference to the Alternative QoS Profile to indicate the QoS that the NG-RAN currently fulfils (see clause 5.7.2.4). The NG-RAN shall enable the SMF to determine when an NG-RAN node supports the Alternative QoS Profile feature but cannot fulfil even the least preferred Alternative QoS Profile. NOTE 3: To reduce the risk that GBR QoS Flows are released in case of RAN resource limitations (and then experience difficulties in being re-established), Application Functions can set the least preferred Alternative Service Requirement to an undemanding level.

We remark that when PDU Set QoS is present, the monitoring is done on the PDU Set QoS instead of the QoS profile, i.e., the PSQP override the QoS parameters in notification control:

#### 5.7.2.4 Notification control ##### 5.7.2.4.1 General The QoS Parameter Notification control indicates to the NG-RAN that notifications of "GFBR can no longer (or can again) be guaranteed" are requested when the NG-RAN determines that the GFBR, the PDB or the PER of the QoS profile cannot be fulfilled (or can be fulfilled again) for a QoS Flow (during the lifetime of the QoS Flow) and that the QoS Flow should be kept while the NG-RAN is not fulfilling the requested QoS profile. Notification control may be used for a GBR QoS Flow if the application traffic is able to adapt to the change in the QoS (e.g. if the AF is capable to trigger rate adaptation). The NG-RAN determination whether the GFBR, the PDB and the PER can be fulfilled or not, is done under consideration of the QoS parameters Averaging Window and MDBV (for a GBR QoS Flow using the Delay-critical resource type), which are either provided explicitly as part of the QoS profile or implicitly via the standardized 5QI QoS characteristics defined in clause 5.7.4. The SMF shall only enable Notification control when the QoS Notification Control parameter is set in the PCC rule (received from the PCF) that is bound to the QoS Flow. The Notification control parameter is signalled to the NG-RAN as part of the QoS profile. If the NG-RAN has received the PDU Set QoS parameters PSDB and PSER for UL and/or DL (as described in clause 5.7.7) and PDU Set QoS handling is applied in the respective direction(s), the NG-RAN uses the PSDB and PSER instead of the PDB and PER in the respective direction(s) for the actions described in all (sub) clauses of clause 5.7.2.4. If the PDU Set QoS handling, PSER and PSDB are applied in one direction only, the NG-RAN still uses the PDB and PER in the other direction for the actions described in all (sub) clauses of clause 5.7.2.4.

It can be understood, that SMF provides one or more Alternative QoS Profile(s) with UL and/or DL PDU Set QoS parameters to NG-RAN, and this only when the QoS profile includes UL and/or DL PDU Set QoS parameter. Otherwise, the Alternative QoS profile does not include the corresponding PSQP.

Observation 3: The SMF provides to RAN the AQP containing the UL and/or DL PSQP, only when UL and/or DL PSQP are present in the QoS flow parameters.

The legacy AQP in RAN3 specs: NGAP, XnAP, F1AP and E1AP need to be extended to include the PSQP per direction. An abnormal condition text is also needed to capture that if the NG-RAN receives the PSQP in AQP but the PSQP are absent in the QoS flow parameters, the procedure should fail.

Proposal 4: Extend the legacy AQP in RAN3 specs: NGAP, XnAP, F1AP and E1AP by including the PSQP per direction. An abnormal condition text is needed to capture that if the NG-RAN receives the PSQP in AQP but the PSQP are absent in the QoS flow parameters, the procedure should fail.

The PSQP need also to be included in the PDU Session Resource Notify Transfer IE in the notification control procedure.

Proposal 5: Add the PSQP in the PDU Session Resource Notify Transfer IE

A TP capturing the changes for NGAP is present in annex of this document.

## 2.3 Available bitrate reporting

The SA2 text on available bitrate reported is described below:

### 5.45.6 Available bitrate monitoring The QoS Monitoring for available bitrate allows the measurement and report of the UL and/or DL available bitrate for a GBR QoS Flow without Alternative QoS Profiles at the NG-RAN. The available bitrate refers to the bitrate that NG-RAN is able to provide for a QoS Flow within a duration. The duration is based on RAN implementation. NOTE 1: How the NG-RAN measures the available bitrate is NG-RAN implementation specific. NOTE 2: The available bitrate can be between the GFBR and the MFBR. The available bitrate can also be below the GFBR for a QoS Flow with QoS Notification Control enabled without Alternative QoS Profiles. For the reporting of the available bitrate, periodic reporting is not applied and only threshold-based reporting applies. Based on the PCC rule containing available bitrate as QoS monitoring parameter and Reporting threshold(s) per direction in the QoS Monitoring policy (as described in clause 6.1.3.27.1 of TS 23.503 [45]), the SMF configures the NG-RAN to measure and report the available bitrate by sending QoS monitoring request including the threshold(s). Based on the QoS monitoring request, if the bitrate that NG-RAN is able to provide matches or exceeds indicated Reporting threshold(s), the NG-RAN reports the UL and/or DL available bitrate information of the highest fulfilled threshold to the PSA UPF via the GTP-U header of UL packets. NOTE 3: The NG-RAN is expected to immediately report the UL and/or DL available bitrate when it matches or exceeds a lower threshold value and report the UL and/or DL available bitrate when it matches or exceeds a higher threshold value with some hysteresis. The SMF may configure the PSA UPF to perform QoS monitoring for available bitrate as described in clause 5.8.2.18. For the reporting of the available bitrate via the PSA UPF, only event-based reporting applies in the PSA UPF. The Reporting threshold(s) in the PCC rule are not sent to UPF. In UPF, the event for the reporting is the reception of available bitrate information from NG-RAN. According to the QoS Monitoring request for the available bitrate from the SMF, the PSA UPF monitors and exposes the available bitrate information reported from NG-RAN to the target NF as instructed. During handover, the QoS Monitoring configuration is provided to the Target NG-RAN per QoS Flow in the QoS Monitoring configuration.

In the SA2 description of available bitrate reporting, when the NG-RAN receives the thresholds from the SMF, the NG-RAN is expected to perform reporting of the available Bitrate for a GBR QoS Flow when the available bitrate value crosses one of the indicated thresholds. However, if the NG-RAN does not support such threshold-based QoS Monitoring, or the threshold values themselves, the CN will not receive any available bitrate and failure of the PDU Session will happen.

Observation 4: If the NG-RAN does not support threshold-based QoS Monitoring for available bitrate reporting, or the threshold values themselves, the CN will not receive any available bitrate and failure of the PDU Session will happen.

Therefore, two options need to be considered:

- The SMF queries first of the RAN support of threshold-based monitoring for available bit rate reporting. This can be done in similar fashion as the PSQP with PDU Set based handling indicator, where the NG-RAN receives the thresholds from SMF, and replies, in case of support, with a specific support indication. Alternatively, the RAN replies with the immediate available bitrate to the SMF which acts as implicit support indication.

- Assume that the NG-RAN supports available bit rate reporting in homogenous scenario.

However, in XR work we have already been tasked since Rel-18 to consider non-homogenous support, and specific section considering handling in non-homogenous scenarios are specified in TS 38.300 and TS 23.501. Therefore option 2 cannot be assumed.

Observation 5: XR works consider non-homogenous support, therefore the available bitrate reporting system needs to be considered also for non-homogenous scenarios.

Proposal 6: RAN3 to agree that for non-homogenous deployment, the SMF needs to know that gNB supports threshold-based reporting at the setup of the PDU session for available bitrate reporting.

Proposal 7: The NG-RAN indicates via dedicated support indication or by indicating the current available bitrate to SMF that reporting of available bitrate reporting is supported.

Furthermore, in case of handover, when the QoS monitoring configuration is provided for a QoS Flow to the target node, the CN needs to know if the new NG-RAN node supports the available bitrate reporting. The new NG-RAN node should indicate to the CN its available bitrate reporting supporting indication IE (or immediate available bitrate as implicit indication ) in the NGAP PATH SWITCH REQUEST message, so that the CN is aware of new node being a supporting node, similar to PDU Set handling. The CN can hence take action e.g., to configure new nodes with different thresholds for the reporting of bitrate.

Observation 6: After a successful Xn-based handover the CN needs to know of the new NG-RAN node can support available bitrate reporting based on the QoS configuration received from previous source NG-RAN node.

Proposal 8: Add a new support indication of available bitrate reporting, or current available bitrate, in the NGAP PATH SWITCH REQUEST message, so that CN is made ware of new node’s support.

In NG-RAN configuration, the gNB-CU-UP that reports the bitrate to UPF needs to be configured by gNB-CUP-CP with the QoS-monitoring threshold-based configuration, hence E1 needs to be impacted.

Proposal 9: The gNB-CU-UP receives the QoS monitoring threshold-based configuration from the gNB-CU-CP.

# 3	Conclusion

In this paper, we have made the following observations and proposals:

Observation 1: In Release-19, a new DL PDU Set Information Marking Support Indicator is sent from SMF to NG-RAN when no PSQP are present. The new indication refers to CN’s support of DL PDU Set marking.

Observation 2: The existing PDU Set Based Handling Support Indication IE is re-used when the NG-RAN receives the new DL PDU Set Information Marking Support Indication IE from the CN to indicate its support of PDU Set handling.

Proposal 1: Add the new DL PDU Set Information Marking Support Indication IE in the QoS parameters per QoS flow level, similar to PDU Set QoS Parameters

Proposal 2: Extend the procedural text in TS 38.413 related to RAN signalling of PDU Set Based Handling Indicator IE, to support the case of receiving the new PDU Set Information Marking Support Indicator from SMF

Proposal 3: same changes as proposals 1 and 2 are needed in XnAP spec.

Observation 3: The SMF provides to RAN the AQP containing the UL and/or DL PSQP, only when UL and/or DL PSQP are present in the QoS flow parameters.

Proposal 4: Extend the legacy AQP in RAN3 specs: NGAP, XnAP, F1AP and E1AP by including the PSQP per direction. An abnormal condition text is needed to capture that if the NG-RAN receives the PSQP in AQP but the PSQP are absent in the QoS flow parameters, the procedure should fail.

Proposal 5: Add the PSQP in the PDU Session Resource Notify Transfer IE

Observation 5: If the NG-RAN does not support threshold-based QoS Monitoring for available bitrate reporting, or the threshold values themselves, the CN will not receive any available bitrate and failure of the PDU Session will happen.

Observation 6: XR works consider non-homogenous support, therefore the available bitrate reporting system needs to be considered also for non-homogenous scenarios.

Proposal 6: RAN3 to agree that for non-homogenous deployment, the SMF needs to know that gNB supports threshold-based reporting at the setup of the PDU session for available bitrate reporting.

Proposal 7: The NG-RAN indicates via dedicated support indication or by indicating the current available bitrate to SMF that reporting of available bitrate reporting is supported.

Observation 6: After a successful Xn-based handover the CN needs to know of the new NG-RAN node can support available bitrate reporting based on the QoS configuration received from previous source NG-RAN node.

Proposal 8: Add a new support indication of available bitrate reporting, or current available bitrate, in the NGAP PATH SWITCH REQUEST message, so that CN is made ware of new node’s support.

Proposal 9: The gNB-CU-UP receives the QoS monitoring threshold-based configuration from the gNB-CU-CP.

# 4	References

[1] RP-250107, Revised WID on XR (eXtended Reality) for NR Phase 3, Nokia (Rapporteur), 3GPP TSG RAN Meeting #107.

# 5	TP to NGAP BL CR

Start of first change

### 8.2.1	PDU Session Resource Setup

#### 8.2.1.1	General

The purpose of the PDU Session Resource Setup procedure is to assign resources on Uu and NG-U for one or several PDU sessions and the corresponding QoS flows, and to setup corresponding DRBs for a given UE. The procedure uses UE-associated signalling.

#### 8.2.1.2	Successful Operation

Figure 8.2.1.2-1: PDU session resource setup: successful operation

The AMF initiates the procedure by sending a PDU SESSION RESOURCE SETUP REQUEST message to the NG-RAN node.

Skipped text unchanged

If the PDU Set QoS Parameters IE or DL PDU Set Information Marking Support Indication IE is included in the PDU SESSION RESOURCE SETUP REQUEST message, the NG-RAN node shall, if supported, report in the PDU SESSION RESOURCE SETUP RESPONSE message the PDU Set based Handling Indicator IE in the PDU Session Resource Setup Response Transfer IE. If the PDU Set based Handling Indicator IE is included in the PDU Session Resource Setup Response Transfer IE in the PDU SESSION RESOURCE SETUP RESPONSE message, the SMF shall, if supported, handle this information as specified in TS 23.501 [9].

Skipped text unchanged

#### 8.2.1.4	Abnormal Conditions

If the NG-RAN node receives a PDU SESSION RESOURCE SETUP REQUEST message containing several PDU Session ID IEs (in the PDU Session Resource Setup Request List IE) set to the same value, the NG-RAN node shall report the establishment of the corresponding PDU sessions as failed in the PDU SESSION RESOURCE SETUP RESPONSE message with an appropriate cause value.

If the NG-RAN node receives a PDU SESSION RESOURCE SETUP REQUEST message containing a PDU Session ID IE (in the PDU Session Resource Setup Request List IE) set to a value that identifies an active PDU session (established before the PDU SESSION RESOURCE SETUP REQUEST message was received), the NG-RAN node shall report the establishment of the new PDU session as failed in the PDU SESSION RESOURCE SETUP RESPONSE message with an appropriate cause value.

If the NG-RAN node receives a PDU SESSION RESOURCE SETUP REQUEST message containing a QoS Flow Setup Request List IE in the PDU Session Resource Setup Request Transfer IE including at least one Non-GBR QoS flow but the PDU Session Aggregate Maximum Bit Rate IE is not present, the NG-RAN node shall report the establishment of the corresponding PDU session as failed in the PDU SESSION RESOURCE SETUP RESPONSE message with an appropriate cause value.

If the NG-RAN node receives a PDU SESSION RESOURCE SETUP REQUEST message containing a QoS Flow Level QoS Parameters IE in the PDU Session Resource Setup Request Transfer IE for a GBR QoS flow but the GBR QoS Flow Information IE is not present, the NG-RAN node shall report the establishment of the corresponding QoS flow as failed in the PDU Session Resource Setup Response Transfer IE of the PDU SESSION RESOURCE SETUP RESPONSE message with an appropriate cause value.If the NG-RAN node receives a PDU SESSION RESOURCE SETUP REQUEST message containing the Delay Critical IE in the Dynamic 5QI Descriptor IE of the QoS Flow Level QoS Parameters IE of the PDU Session Resource Setup Request Transfer IE set to the value “delay critical” but the Maximum Data Burst Volume IE is not present, the NG-RAN node shall report the establishment of the corresponding QoS flow as failed in the PDU Session Resource Setup Response Transfer IE of the PDU SESSION RESOURCE SETUP RESPONSE message with an appropriate cause value.

If the NG-RAN receives the Alternative QoS Parameters Set List IE containing the PDU Set QoS Parameters IE, but no PDU Set QoS Parameters are included in the QoS Flow Level QoS Parameters IE in the PDU Session Resource Setup Request Transfer IE of the PDU SESSION RESOURCE SETUP REQUEST message, the NG-RAN node shall report the establishment of the corresponding PDU session as failed in the PDU SESSION RESOURCE SETUP RESPONSE message with an appropriate cause value.

NEXT CHANGE

### 8.2.3	PDU Session Resource Modify

#### 8.2.3.1	General

The purpose of the PDU Session Resource Modify procedure is to enable configuration modifications of already established PDU session(s) for a given UE. It is also to enable the setup, modification and release of the QoS flow for already established PDU session(s). The procedure uses UE-associated signalling.

#### 8.2.3.2	Successful Operation

Figure 8.2.3.2-1: PDU session resource modify: successful operation

The AMF initiates the procedure by sending a PDU SESSION RESOURCE MODIFY REQUEST message to the NG-RAN node.

Skipped text unchanged

If the PDU Set QoS Parameters IE or the DL PDU Set Information Marking Support Indication IE is included in the PDU SESSION RESOURCE MODIFY REQUEST message, the NG-RAN node shall, if supported, report in the PDU SESSION RESOURCE MODIFY RESPONSE message the PDU Set based Handling Indicator IE in the PDU Session Resource Modify Response Transfer IE. If the PDU Set based Handling Indicator IE is included in the PDU Session Resource Modify Response Transfer IE in the PDU SESSION RESOURCE MODIFY RESPONSE message, the SMF shall, if supported, handle this information as specified in TS 23.501 [9].

Skipped text unchanged

#### 8.2.3.4	Abnormal Conditions

If the NG-RAN node receives a PDU SESSION RESOURCE MODIFY REQUEST message containing several PDU Session ID IEs (in the PDU Session Resource Modify Request List IE) set to the same value, the NG-RAN node shall report the modification of the corresponding PDU sessions as failed in the PDU SESSION RESOURCE MODIFY RESPONSE message with an appropriate cause value.

If the NG-RAN node receives a PDU SESSION RESOURCE MODIFY REQUEST message containing some PDU Session ID IEs (in the PDU Session Resource Modify Request List IE) that the NG-RAN node does not recognize, the NG-RAN node shall report the corresponding invalid PDU sessions as failed in the PDU SESSION RESOURCE MODIFY RESPONSE message with an appropriate cause value.

If the NG-RAN node receives a PDU SESSION RESOURCE MODIFY REQUEST message containing a QoS Flow Level QoS Parameters IE in the PDU Session Resource Modify Request Transfer IE for a GBR QoS flow but the GBR QoS Flow Information IE is not present, the NG-RAN node shall report the addition or modification of the corresponding QoS flow as failed in the PDU Session Resource Modify Response Transfer IE of the PDU SESSION RESOURCE MODIFY RESPONSE message with an appropriate cause value.

If the NG-RAN node receives a PDU SESSION RESOURCE MODIFY REQUEST message containing the Delay Critical IE in the Dynamic 5QI Descriptor IE of the QoS Flow Level QoS Parameters IE of the PDU Session Resource Modify Request Transfer IE set to the value “delay critical” but the Maximum Data Burst Volume IE is not present, the NG-RAN node shall report the addition or modification of the corresponding QoS flow as failed in the PDU Session Resource Modify Response Transfer IE of the PDU SESSION RESOURCE MODIFY RESPONSE message with an appropriate cause value.

If the NG-RAN node receives a PDU SESSION RESOURCE MODIFY REQUEST message containing a PDU session in the PDU Session Resource Modify Request List IE with the same QoS flow included both in the QoS Flow Add or Modify Request List IE and the QoS Flow to Release List IE, the NG-RAN node shall report the corresponding QoS flow as failed in the QoS Flow Failed to Add or Modify List IE in the PDU Session Resource Modify Response Transfer IE of the PDU SESSION RESOURCE MODIFY RESPONSE message with an appropriate cause value if the PDU session is modified successfully. The NG-RAN node shall not release the QoS flow when the corresponding QoS flow already exists.

If the NG-RAN receives the Alternative QoS Parameters Set List IE containing the PDU Set QoS Parameters IE, but no PDU Set QoS Parameters are included in the QoS Flow Level QoS Parameters IE in the PDU Session Resource Setup Modify Transfer IE of the PDU SESSION RESOURCE MODIFY REQUEST message, , the NG-RAN node shall report the addition or modification of the corresponding PDU session as failed in the PDU SESSION RESOURCE MODIFY RESPONSE message with an appropriate cause value.

NEXT CHANGE

### 8.4.2	Handover Resource Allocation

#### 8.4.2.1	General

The purpose of the Handover Resource Allocation procedure is to reserve resources at the target NG-RAN node for the handover of a UE. The procedure uses UE-associated signalling.

#### 8.4.2.2	Successful Operation

Figure 8.4.2.2-1: Handover resource allocation: successful operation

The AMF initiates the procedure by sending the HANDOVER REQUEST message to the target NG-RAN node.

If the Masked IMEISV IE is contained in the HANDOVER REQUEST message the target NG-RAN node shall, if supported, use it to determine the characteristics of the UE for subsequent handling.

Upon receipt of the HANDOVER REQUEST message the target NG-RAN node shall

-	attempt to execute the requested PDU session configuration and associated security;

-	store the received UE Aggregate Maximum Bit Rate in the UE context, and use the received UE Aggregate Maximum Bit Rate for all Non-GBR QoS flows for the concerned UE as specified in TS 23.501 [9];

-	store the received Mobility Restriction List in the UE context;

-	store the received UE Security Capabilities in the UE context;

-	store the received Security Context in the UE context and take it into use as defined in TS 33.501 [13];

-	if supported, store the received UE Slice Maximum Bit Rate List in the UE context and use the received UE Slice Maximum Bit Rate List for each S-NSSAI for the concerned UE as specified in TS 23.501 [9].

-	if supported, store the received PDU Set QoS parameters in the UE context and use it as specified in TS 23.501 [9].

Upon reception of the UE History Information IE, which is included within the Source to Target Transparent Container IE of the HANDOVER REQUEST message, the target NG-RAN node shall collect the information defined as mandatory in the UE History Information IE and shall, if supported, collect the information defined as optional in the UE History Information IE, for as long as the UE stays in one of its cells, and store the collected information to be used for future handover preparations.

Upon receiving the PDU Session Resource Setup List IE contained in the HANDOVER REQUEST message and the HANDOVER REQUEST message does not contain the No PDU Session Indication IE, the target NG-RAN node shall behave the same as defined in the PDU Session Resource Setup procedure. The target NG-RAN node shall report to the AMF in the HANDOVER REQUEST ACKNOWLEDGE message the result for each PDU session resource requested to be setup. In particular, for each PDU session resource successfully setup, it shall include the Handover Request Acknowledge Transfer IE containing the following information:

-	The list of QoS flows which have been successfully established in the QoS Flow Setup Response List IE.

-	The Data Forwarding Accepted IE if the data forwarding for the QoS flow is accepted.

-	The list of QoS flows which have failed to be established, if any, in the QoS Flow Failed to Setup List IE.

-	The UP transport layer information to be used for the PDU session.

-	The security result associated to the PDU session.

-	The redundant UP transport layer information to be used for the redundant transmission for the PDU session.

-	The PDU Set based Handling Indicator if the HANDOVER REQUEST message includes the PDU Set QoS Parameters IE or the DL PDU Set Information Marking Support Indication IE.

-	The ECN Marking or Congestion Information Reporting Status if the HANDOVER REQUEST message includes the ECN Marking or Congestion Information Reporting Request IE.

NEXT CHANGE

#### 9.3.1.12	QoS Flow Level QoS Parameters

This IE defines the QoS parameters to be applied to a QoS flow.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| CHOICE QoS Characteristics | M |  |  |  | - |  |
| >Non-dynamic 5QI |  |  |  |  |  |  |
| >>Non Dynamic 5QI Descriptor | M |  | 9.3.1.28 |  | - |  |
| >Dynamic 5QI |  |  |  |  |  |  |
| >>Dynamic 5QI Descriptor | M |  | 9.3.1.18 |  | - |  |
| Allocation and Retention Priority | M |  | 9.3.1.19 |  | - |  |
| GBR QoS Flow Information | O |  | 9.3.1.10 | This IE shall be present for GBR QoS flows and is ignored otherwise. | - |  |
| Reflective QoS Attribute | O |  | ENUMERATED (subject to, …) | Details in TS 23.501 [9]. This IE may be present in case of Non-GBR QoS flows and is ignored otherwise. | - |  |
| Additional QoS Flow Information | O |  | ENUMERATED (more likely, …) | This IE indicates that traffic for this QoS flow is likely to appear more often than traffic for other flows established for the PDU session. This IE may be present in case of Non-GBR QoS flows and is ignored otherwise. | - |  |
| QoS Monitoring Request | O |  | ENUMERATED (UL, DL, Both, …, stop) | Indicates to measure UL, or DL, or both UL/DL delays for the associated QoS flow or stop the corresponding QoS monitoring. | YES | ignore |
| QoS Monitoring Reporting Frequency | O |  | INTEGER (1.. 1800, …) | Indicates the reporting frequency for RAN part delay for QoS monitoring. Units: second | YES | ignore |
| PDU Set QoS Parameters |  | 0..1 |  |  | YES | ignore |
| >UL PDU Set QoS Information | O |  | PDU Set QoS Information 9.3.1.264 |  | - |  |
| >DL PDU Set QoS Information | O |  | PDU Set QoS Information 9.3.1.264 |  | - |  |
| DL PDU Set Information Marking Support Indication | O |  | ENUMERATED (true, …) |  | YES | ignore |

NEXT CHANGE

#### 9.3.1.151	Alternative QoS Parameters Set List

This IE contains alternative sets of QoS parameters which the NG-RAN node can indicate to be fulfilled when notification control is enabled and it cannot fulfil the requested list of QoS parameters.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Alternative QoS Parameters Set Item |  | 1..<maxnoofQoSparaSets> |  |  |  |  |
| >Alternative QoS Parameters Set Index | M |  | 9.3.1.152 |  | - |  |
| >Guaranteed Flow Bit Rate Downlink | O |  | Bit Rate 9.3.1.4 |  | - |  |
| >Guaranteed Flow Bit Rate Uplink | O |  | Bit Rate 9.3.1.4 |  | - |  |
| >Packet Delay Budget | O |  | 9.3.1.80 |  | - |  |
| >Packet Error Rate | O |  | 9.3.1.81 |  | - |  |
| >Maximum Data Burst Volume | O |  | 9.3.1.83 | Maximum Data Burst Volume is specified in TS 23.501 [9]. This IE is included if the Delay Critical IE is set to "delay critical" and is ignored otherwise. | YES | ignore |
| >PDU Set QoS Parameters |  | 0..1 |  |  | YES | ignore |
| >>UL PDU Set QoS Information | O |  | PDU Set QoS Information 9.3.1.264 |  | - |  |
| >>DL PDU Set QoS Information | O |  | PDU Set QoS Information 9.3.1.264 |  | - |  |

| Range bound | Explanation |
| --- | --- |
| maxnoofQoSparaSets | Maximum no. of alternative sets of QoS Parameters allowed for the QoS profile. Value is 8. |

NEXT CHANGE

#### 9.3.4.5	PDU Session Resource Notify Transfer

This IE is transparent to the AMF.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| QoS Flow Notify List |  | 0..1 |  |  | - |  |
| >QoS Flow Notify Item |  | 1..<maxnoofQoSFlows> |  |  | - |  |
| >>QoS Flow Identifier | M |  | 9.3.1.51 |  | - |  |
| >>Notification Cause | M |  | ENUMERATED (fullfilled, not fulfilled, …) |  | - |  |
| >>Current QoS Parameters Set Index | O |  | Alternative QoS Parameters Set Notify Index 9.3.1.153 | Index to the currently fulfilled alternative QoS parameters set. Value 0 indicates that NG-RAN cannot even fulfil the lowest alternative parameters set. | YES | Ignore |
| >>TSC Traffic Characteristics Feedback | O |  | 9.3.1.257 |  | YES | ignore |
| QoS Flow Released List | O |  | QoS Flow List with Cause 9.3.1.13 |  | - |  |
| Secondary RAT Usage Information | O |  | 9.3.1.114 |  | YES | ignore |
| QoS Flow Feedback List |  | 0..1 |  |  | YES | ignore |
| >QoS Flow Feedback Item |  | 1..<maxnoofQoSFlows> |  |  | - |  |
| >>QoS Flow Identifier | M |  | 9.3.1.51 |  | - |  |
| >>Update Feedback | O |  | BIT STRING { CN PDB DL(0), CN PDB UL(1)} (SIZE(8, …)) | Each position in the bitmap represents a QoS parameter. If a bit is set to "1", the respective parameter was not updated. If a bit is set to "0", the respective parameter was successfully updated. Bits 2-7 reserved for future use. | - |  |
| >>CN Packet Delay Budget Downlink | O |  | Extended Packet Delay Budget 9.3.1.135 | Indicates when the packet delay budget downlink was not updated in path switch that NG-RAN can offer this value | - |  |
| >>CN Packet Delay Budget Uplink | O |  | Extended Packet Delay Budget 9.3.1.135 | Indicates when the packet delay budget uplink was not updated in path switch that NG-RAN can offer this value | - |  |
| >>PDU Set QoS Parameters |  | 0..1 |  |  | YES | ignore |
| >>>UL PDU Set QoS Information | O |  | PDU Set QoS Information 9.3.1.264 |  | - |  |
| >>>DL PDU Set QoS Information | O |  | PDU Set QoS Information 9.3.1.264 |  | - |  |

| Range bound | Explanation |
| --- | --- |
| maxnoofQoSFlows | Maximum no. of QoS flows allowed within one PDU session. Value is 64. |