3GPP TSG-RAN WG3 Meeting #127 Bis	             R3-252036

Wuhan, China, 7 – 11 April 2025

Agenda Item:	21.3

Source:	CMCC

Title:	(TPs to BL CRs) Support of QoS Handling Enhancement and Exposure of Available Data Rate

Document for:	Discussions & Approval

# Introduction

The following revised WID on XR (eXtended Reality) for NR Phase 3 [1] was approved at RAN Plenary meeting #107:

The Rel-19 XR Phase 3 objectives are as follows: - Support and specify multi-modality awareness for QoS flows in both DL and UL RAN [RAN3]: - Specify enhancements to enable transmission/reception in gaps/restrictions that are caused by RRM measurements (from inter-frequency RRM measurement gaps, or intra-frequency measurements, or other scheduling restrictions etc). [RAN1, RAN2, RAN4] - Specify the corresponding measurement gap and scheduling restriction to enable the identified enhancements with RRM performance impact taken into consideration, work being triggered by LS. [RAN4] - Specify Enhancements for support of UL scheduling to enable high XR capacity while meeting delay requirements/avoiding too late PDUs, as follows [RAN2]: - Specify additional Logical Channel priority handling using delay/deadline information of packets; - Specify enhanced DSR (Delay Status Report) reporting with multiple pairs of remaining time and buffer size for an LCG. - Specify the following user plane enhancements [RAN2]: - RLC re-transmission related enhancements for operation of RLC Acknowledged Mode (AM) with small packet delay budget. - Extend Release 18 standalone mechanism to support NR-NR dual connectivity as follows [RAN3]: - PDU set based handling; - ECN marking; - Burst Arrival Time reporting, if needed; - PSI Discard coordination, if needed; NOTE: No RAN2 impact from above items. - Specify uplink congestion signalling [RAN2, RAN3]: - Specify in MAC layer XR rate control signalling over downlink per QoS flow/per DRB and the associated F1 signalling, if any, to enable faster source rate adaption to uplink congestion. - Support of PDU set based QoS handling enhancement [RAN3]: - Support of DL PDU Set marking without PDU Set QoS - Support of Alternative PDU Set QoS, which may contain UL and/or DL PDU Set QoS Parameters (i.e. UL PSDB, DL PSDB, UL PSER and/or DL PSER). - QoS Handling when Traffic Characteristics Change Dynamically [RAN3] - TTNB and burst size to be provided over GTP-U - Support of exposure of available data rate [RAN3] Note: Coordination with SA2, as needed - Specify Core requirements related to the above objectives as necessary [RAN4].

In this document, we focus on the following topics:

-	QoS Handling when Traffic Characteristics Change Dynamically [RAN3]

-	TTNB and burst size to be provided over GTP-U

-	Support of exposure of available data rate [RAN3]

# Discussions

## QoS Handling When Traffic Characteristics Change Dynamically

In Section 5.37.10.1 and Section 5.37.10.2 of TS 23.501, it describes the TTNB and Burst Size as below:

#### 5.37.10.1 DL Data Burst Size marking The Data Burst Size may be provided to the NG-RAN in the downlink GTP-U header by the UPF in order to assist radio resource management. As described in clause 6.1.3.27.5 of TS 23.503 [45] the PCF may include a Data Burst Size Marking Indication within a PCC Rule to request the UPF to identify and mark the Data Burst Size of the data burst for the corresponding QoS Flow. The PCF also includes the DL Protocol Description received from AF. Based on the Data Burst Size Marking Indication in a PCC Rule, the SMF instructs the UPF to identify and mark the Data Burst Size of the data burst in downlink for the corresponding PDR. The SMF may provide the UPF the Protocol Description received in the PCC Rule. According to the Data Burst Size Marking Indication and DL Protocol Description from the SMF, the UPF identifies in the first DL packets of the data burst the Data Burst Size carried in an N6 RTP Header Extension/ transport protocol header as defined in TS 26.522 [179]. The UPF sends the identified Data Burst size to NG-RAN in the downlink GTP-U header of the first PDUs of the data burst. #### 5.37.10.2 Time to Next Burst marking The time to next data burst, which is the interval between the transmission of the last PDU in the current data burst and the first PDU of the next data burst, may be provided to the NG-RAN by the UPF to assist NG-RAN's behaviour in downlink. As described in clause 6.1.3.27.5 of TS 23.503 [45] the PCF may include a Time to Next Burst Marking Indication within a PCC Rule to request the UPF to identify and mark the time to next data burst for the corresponding QoS Flow. The PCF also includes the DL Protocol Description received from AF. Based on the Time to Next Burst indication in a PCC Rule, the SMF instructs the UPF to identify and mark the time interval to next data burst in downlink. The SMF may provide the UPF the Protocol Description used by the service data flow received in the PCC Rule. According to the Time to Next Burst indication and DL Protocol Description from the SMF, the UPF identifies the time interval to next data burst carried in an N6 RTP Header Extension/transport protocol header as defined in TS 26.522 [179] and sends the identified time interval to next data burst to NG-RAN in a downlink GTP-U header. Time to next data burst marking is only used when the deployment is such that the time to next data burst is sufficiently accurate i.e. the jitter (e.g. the N6 jitter) is sufficiently limited without impact to the accuracy of the time to next data burst.

The UPF could identify in the first DL packets of the data burst the Data Burst Size carried in an N6 RTP Header Extension/ transport protocol header and sends the identified Data Burst size to NG-RAN in the downlink GTP-U header of the first PDUs of the data burst.

The UPF could identify the time interval to the next data burst carried in an N6 RTP Header Extension/transport protocol header and sends the identified time interval to next data burst to NG-RAN in a downlink GTP-U header.

The CN sends NG-RAN the Data Burst size in a downlink GTP-U header, could be included in the DL PDU SESSION INFORMATION, or in the DL PDU SET Information if PDU set is marked.

The CN sends NG-RAN the time interval to the next data burst in a downlink GTP-U header, could be included in the DL PDU SESSION INFORMATION, or in the DL PDU SET Information if PDU set is marked.

## Support of Exposure of Available Data Rate

SA2 has agreed the following updates regarding exposure of available data rate. For TS 23.503, the following revisions are agreed as below [2]:

For TS 23.501, the following revisions are agreed as below [3]:

For TS 23.502, the following revisions are agreed as below [4]:

The CN provides NG-RAN with predefined threshold values that define specific available data rate ranges, which NG-RAN uses to trigger reporting through the user plane (UP). The threshold configuration can be independently managed for uplink (UL) and downlink (DL), allowing for flexibility in adapting to network conditions. When the available data rate crosses a predefined threshold, NG-RAN generates a notification to indicate the change. Instead of reporting exact data rate values, NG-RAN uses the reporting thresholds that correspond to specific data rate ranges, reducing signaling overhead. The available data rate within a range is reported as the lower bound of the range, or by referencing to the identifier of the lower threshold.

The CN provides NG-RAN with predefined reporting thresholds that define specific available data rate ranges.  The threshold configuration can be independently managed for DL and UL.

MFBR and GFBR can be included in the reporting threshold list.

NG-RAN enables user-plane notifications of available data rate to the UPF, triggered by predefined reporting thresholds, by using the GTP-U header of UL link packets.

For CU-DU split architecture, gNB-CU informs the gNB-DU the available data rate reporting thresholds for both uplink and downlink.

gNB-CU-CP needs to inform the gNB-CU-UP the available data rate reporting thresholds for both uplink and downlink.

To further optimize the reporting mechanism, hysteresis is applied to prevent excessive notifications when the available data rate increases. When the data rate crosses a higher threshold, NG-RAN introduces a delay before reporting the change, ensuring stability and preventing frequent toggling between thresholds. This approach minimizes unnecessary signaling and avoids fluctuations in resource allocation. In contrast, when the available data rate decreases and crosses a lower threshold, NG-RAN must immediately report the change to prevent service degradation. In these cases, a prompt uplink notification is crucial to maintaining service continuity and allowing the application to adjust its data usage accordingly.

NG-RAN shall wait for the data rate to stabilize before sending a notification when the available data rate increases beyond a higher threshold.

NG-RAN shall immediately trigger an uplink report of the available data rate, when it decreases and crosses a lower threshold.

# Conclusions

In this submission, we discussed the support of he QoS Handling when Traffic Characteristics Change Dynamically with TTNB and burst size to be provided over GTP-U, and support of exposure of available data rate in RAN3. The following proposals were made:

- The CN sends NG-RAN the Data Burst size in a downlink GTP-U header, could be included in the DL PDU SESSION INFORMATION, or in the DL PDU SET Information if PDU set is marked.

The CN sends NG-RAN the time interval to the next data burst in a downlink GTP-U header, could be included in the DL PDU SESSION INFORMATION, or in the DL PDU SET Information if PDU set is marked.

The CN provides NG-RAN with predefined reporting thresholds that define specific available data rate ranges.  The threshold configuration can be independently managed for DL and UL.

MFBR and GFBR can be included in the reporting threshold list.

NG-RAN enables user-plane notifications of available data rate to the UPF, triggered by predefined reporting thresholds, by using the GTP-U header of UL link packets.

For CU-DU split architecture, gNB-CU informs the gNB-DU the available data rate reporting thresholds for both uplink and downlink.

gNB-CU-CP needs to inform the gNB-CU-UP the available data rate reporting thresholds for both uplink and downlink.

NG-RAN shall wait for the data rate to stabilize before sending a notification when the available data rate increases beyond a higher threshold.

NG-RAN shall immediately trigger an uplink report of the available data rate, when it decreases and crosses a lower threshold.

# References

- RP-240107, Revised WID on XR (eXtended Reality) for NR Phase 3

- S2-2502471 was2429 was2215 was1919 was0906 was11917 was10486 23503 support of available data rate exposure-v6

- S2-2502675 revision of S2-2502470, S2-2502284, S2-2502213 and S2-2501426_XRM r19_KI#9 501_avalable bitrate

- S2-2502676

# Annex - TP for TS 38.415 For QoS Handling When Traffic Characteristics Change Dynamically

<<<<<<<<<<<<<<<<<<<<<<<<<<<<<<<<<<start of change>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>

#### 5.5.2.1	DL PDU SESSION INFORMATION (PDU Type 0)

This frame format is defined to allow the NG-RAN to receive some control information elements which are associated with the transfer of a packet over the interface.

The following shows the respective DL PDU SESSION INFORMATION frame.

| Bits |  |  |  |  |  |  |  |  | Number of Octets |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 7 | 6 | 5 | 4 | 3 | 2 | 1 |  | 0 |  |
| PDU Type (=0) |  |  |  | QMP | SNP | MSNP |  | TTNBI | 1 |
| PPP | RQI | QoS Flow Identifier |  |  |  |  |  |  | 1 |
| PPI |  |  | Spare |  |  |  | DBSI |  | 0 or 1 |
| DL Sending Time Stamp |  |  |  |  |  |  |  |  | 0 or 8 |
| DL QFI Sequence Number |  |  |  |  |  |  |  |  | 0 or 3 |
| DL MBS QFI Sequence Number |  |  |  |  |  |  |  |  | 0 or 4 |
| DL Time To Next Burst |  |  |  |  |  |  |  |  | 0 or 4 |
| DL Data Burst Size |  |  |  |  |  |  |  |  | 0 or 3 |
| Padding |  |  |  |  |  |  |  |  | 0-3 |

Figure 5.5.2.1-1: DL PDU SESSION INFORMATION (PDU Type 0) Format

<<<<<<<<<<<<<<<<<<<<<<<<<<<<<<<<<<next change >>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>

#### 5.5.3.a	TTNBI (Time To Next Burst Indicator)

Description: This parameter indicates the presence of Time to Next Burst (TTNB).

Value range: {0= TTNB not present, 1= TTNB present}.

Field length: 1 bit.

#### 5.5.3.b	DBSI (Data Burst Size Indicator)

Description: This parameter indicates the presence of Data Burst Size (DBS).

Value range: {0= DBS not present, 1= DBS present}.

Field length: 1 bit.

#### 5.5.3.c	DL Time To Next Burst

Description: This parameter indicates the time interval to the next data burst.

Value range: {0..232-1}.

Field length: 4 octets.

#### 5.5.3.d	 DL Data Burst Size

Description: This parameter indicates the total size of the data burst.

Value range: {0..224-1}.

Field length: 3 octets.

<<<<<<<<<<<<<<<<<<<<<<<<<<<<<<<<<<end of change>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>

# Annex - TP for TS 38.415 For Support of Exposure of Available Data Rate

<<<<<<<<<<<<<<<<<<<<<<<<<<<<<<<<<<start of change>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>

#### 5.5.2.2	UL PDU SESSION INFORMATION (PDU Type 1)

This frame format is defined to allow the UPF to receive some control information elements which are associated with the transfer of a packet over the interface.

The following shows the respective UL PDU SESSION INFORMATION frame.

| Bits |  |  |  |  |  |  |  | Number of Octets |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 7 | 6 | 5 | 4 | 3 | 2 | 1 | 0 |  |
| PDU Type (=1) |  |  |  | QMP | DL Delay Ind. | UL Delay Ind. | SNP | 1 |
| N3/N9 Delay Ind. | New IE Flag | QoS Flow Identifier |  |  |  |  |  | 1 |
| DL Sending Time Stamp Repeated |  |  |  |  |  |  |  | 0 or 8 |
| DL Received Time Stamp |  |  |  |  |  |  |  | 0 or 8 |
| UL Sending Time Stamp |  |  |  |  |  |  |  | 0 or 8 |
| DL Delay Result |  |  |  |  |  |  |  | 0 or 4 |
| UL Delay Result |  |  |  |  |  |  |  | 0 or 4 |
| UL QFI Sequence Number |  |  |  |  |  |  |  | 0 or 3 |
| N3/N9 Delay Result |  |  |  |  |  |  |  | 0 or 4 |
| New IE flag 7(E) | New IE Flag 6 | New IE Flag 5 | New IE Flag 4 | New IE Flag 3 | New IE Flag 2 | New IE Flag 1 | New IE Flag 0 | 0 or 1 New IE Flags Octet |
| Spare |  |  |  |  |  |  | D1 UL PDCPDelay Result Ind | 0 or 1 |
| UL Congestion Information |  |  |  |  |  |  |  | 0 or 2 |
| DL Congestion Information |  |  |  |  |  |  |  | 0 or 2 |
| UL Available Data Rate Information |  |  |  |  |  |  |  | 0 or 6 |
| DL Available Data Rate Information |  |  |  |  |  |  |  | 0 or 6 |
| Padding |  |  |  |  |  |  |  | 0-3 |

Figure 5.5.2.2-1: UL PDU SESSION INFORMATION (PDU Type 1) Format

The New IE Flag in bit 6 of 2nd octet in UL PDU SESSION INFORMATION (PDU Type 1) indicates if the first octet of New IE Flags Octet is present or not.

Bit 0 of New IE Flags Octet in UL PDU SESSION INFORMATION (PDU Type 1) indicates if the D1 UL PDCP Delay Result Ind is present (1) or not (0).

Bit 1 of New IE Flags Octet in UL PDU SESSION INFORMATION (PDU Type 1) indicates if the UL Congestion Information is present (1) or not (0).

Bit 2 of New IE Flags Octet in UL PDU SESSION INFORMATION (PDU Type 1) indicates if the DL Congestion Information is present (1) or not (0).

Bit 3 of New IE Flags Octet in UL PDU SESSION INFORMATION (PDU Type 1) indicates if the UL Available Data Rate Information is present (1) or not (0).

Bit 4 of New IE Flags Octet in UL PDU SESSION INFORMATION (PDU Type 1) indicates if the DL Available Data Rate Information is present (1) or not (0).

<<<<<<<<<<<<<<<<<<<<<<<<<<<<<<<<<<next change >>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>

#### 5.5.3.e	UL Available Data Rate Information

Description: This parameter indicates the UL available data rate [7]. When present, it means the UL available data rate falls within a predefined range, i.e., it is greater than or equal to a lower threshold and strictly less than the next higher threshold, if any. The reported value corresponds to the lower threshold of the range.

Value range: {0.. ﻿4,000,000,000,000}.

Field length: 6 octet.

#### 5.5.3.f	DL Available Data Rate Information

Description: This parameter indicates the DL available data rate [7]. When present, it means the DL available data rate falls within a predefined range, i.e., it is greater than or equal to a lower threshold and strictly less than the next higher threshold, if any. The reported value corresponds to the lower threshold of the range.

Value range: {0.. ﻿4,000,000,000,000}.

Field length: 6 octet.

<<<<<<<<<<<<<<<<<<<<<<<<<<<<<<<<<<end of change>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>