3GPP TSG-RAN WG3 Meeting #127bis	R3-252056

Wuhan, PR China, 7th – 11th April 2025

Agenda Item:	16.2

Source:	Ericsson

Title:	[TP for BL CR 38.401 and 38.300] Multiplexing several A-IoT service operations concerning multiple A-IoT devices on NG-C and other architectural topics

Document for:	Discussion and Agreement

# 1	Introduction

Last meeting we started to discuss details for the A-Iot specific control signalling on NG-C, still skipping a lot of details:

-	how to handle – at the gNB – multiple concurrent A-IoT service operations, potentially stemming from different AIOTFs,

-	how to correlate the “inventory part” and “command part” of an A-IoT service operations

-	how to associate inventory report and command signalling, if the A-IoT transaction concerns multiple A-IoT devices

-	and an architectural topic dealing with cardinality in case of indirect connectivity

# 2	Discussion

## 2.1	Assumptions on AMF’s and eNB’s functions concerning connectivity options

We had some discussions at RAN3#127 on connectivity options and the assumptions to make in order to define proper protocol support.

If we start with direct connectivity, we observe that the association between the AIOTF and gNBs is rather straight forward. In case of multiple RAN and CN nodes deployed, both, CN and RAN are able to – directly – discern the node from which certain information is communicated by means of NGAP signalling.

In case of indirect connectivity, having multiple AMFs deployed in between deployed gNBs and AIOTFs, two aspects were raised at the last meeting discussions:

a)	Shall the gNB notice any difference as to whether it is directly or indirectly connected to AIOTFs?

b)	To which extent should the AMF be involved in AIOTF specific procedural logic?

Ad a)	Discussions at RAN3#127 have investigated the possibility to simplify gNB logic to the extent of specifying a single gNB behaviour that is applied for both connectivity options, well knowing that some of the protocol information included in NGAP A-IoT messages may not apply for both options.
It is easy to guess that for the direct connectivity option NGAP would not need to carry any routing information, while the indirect connectivity option would.

Ad b)	At the last meeting it appeared as if the majority of companies were considering a stateless AMF: i.e. the AMF should “just” route NGAP A-IoT messages from the gNB/AITOF to the AIOTF/gNB, w/o applying any procedure logic, such procedure-state logic would only reside in the AIOTF.
Therefore, the NGAP A-IoT messages would need to carry proper gNB/AIOTF identifiers. In order to be able to route AIOTF communication, the AMF would only need to process the “routing part” of each AIOTF message issued by either the AIOTF of the gNB.

Proposal 1:	In order to enable a uniform gNB behaviour w.r.t. connectivity option and a stateless AMF it is proposed to introduce AIOTF and NG-RAN node identifiers in each AIOTF NGAP message. Those identifiers are examined in case of indirect connectivity by the AMF, in case of direct connectivity, these identifiers can be ignored.

## 2.2	Correlating “Inventory” and “Command” parts of an A-IoT service operations

A-IoT enabled gNBs are expected to serve a multitude of “readers”, providing A-IoT radio resources in a specific geographical area/space where a certain A-IoT service operation shall be executed. Multiple A-IoT service operations may be ongoing concurrently. In order to differentiate these concurrent A-IoT service operations, they have to be labelled by an identifier, at least unique across the currently ongoing A-IoT service operations.

As in Rel-19 an A-IoT service operation is always triggered by the CN, more specifically by the AIOTF, this identifier is contained in the Inventory Request message. During the inventory, the request/response/report messages part of the same inventory operation instance are correlated by the same A-IoT Correlation ID. In case a command operation follows the inventory, this identifier indicates the correlation of both parts of the “inventory&command” service operation.

Along current status of SA2 discussions (see TPs for the new stage-2 TS in [x1][x2]) this AIOTF generates a Correlation ID (see SA2 current status of discussions), which allows to correlate A-IoT NGAP messages belonging to that service operation, i.e. also to correlate NGAP signalling for the inventory part to that for the command part of the A-IoT service operation. The Correlation ID also allows an AIOTF to issue several service operations in parallel (restrictions as of the reader operation are discussed in RAN2) and the gNB to accept service requests from several AIOTFs.

Observation 2:	Along SA2 agreements, the AIOTF generates a Correlation ID and provides it to the A-IoT enabled gNB in order associate NGAP signalling with a specific A-IoT service operation. This also allows the A-IoT enabled gNB to accept parallel A-IoT service operations. On A-IoT radio, RAN2 discussed restrictions on how many parallel A-IoT service operations could be served in parallel by a reader at a certain point in time would have to be taken into account. The Correlation ID associated to an “inventory&command” operation is the same for the “inventory” part and the “command” part.

Proposal 3:	Introduce a “Correlation ID” in the respective NGAP inventory and command transfer containers. In Rel-19 the A-IoT Correlation Identifier is allocated by the AIOTF to uniquely identify A-IoT service operations controlled by the AIOTF.

## 2.3	Multiple concurrent A-IoT service operations stemming from different AIOTFs

There are no architectural restrictions as to how many AIOTFs are communicating with an A-IoT enabled gNB (indirectly via AMFs or directly).

For indirect connectivity, an A-IoT enabled gNB would need to distinguish the A-IoT Correlation IDs stemming from different AIOTFs, therefore, the A-IoT enabled gNB needs to know the AIOTF which has assigned the ID.

In order to make the gNB act in the same way irrespective the connectivity options chosen, it is proposed to always include identifiers identifying the AIOTF and the A-IoT enabled gNB in the respective messages.

Observation 4:	Support of multiple concurrent A-IoT service operations stemming from different AIOTFs is supported by communication means introduced in Proposal 1.

## 2.4	Correlating device specific information of A-IoT operations consisting of inventory and command

For Rel-19, “command-only” A-IoT operations were ruled out – “command” is only possible as “inventory-and-command”, i.e. in order to perform the “command” part of the operation an A-IoT device needs to be inventoried first.

The “inventory” part of the operation preceding the “command” part not only provides the network with information about the “whereabouts” of A-IoT devices, it also has to allow the network to keep communication with the A-IoT device established in a way so that subsequent communication for e.g. read/write operations are possible.

One aspect of the Access-Stratum part of communication between the network and the A-IoT device is the introduction of an “Access Stratum” Identifier (AS ID), which has been agreed by RAN2 to identify information exchanged between the A-IoT device and the A-IoT reader. (See details of RAN2 agreements regarding that AS ID (e.g. allocation, scope, etc. in [draft meeting report RAN2#129]). Apart from some details, RAN2 has recognised and agreed that an AS ID is needed for “inventory-and-command”, regardless the contention resolution scheme used for random access.

Observation 5:	RAN2 agreed to introduce an “Access Stratum Identifier” (AS-ID) identifying communication of a particular A-IoT device with a particular A-IoT reader, and always allocate and use it for an AIoT device for “inventory and command” operations.

The command operation on NG-C is separated from the inventory operation – along the current agreements – i.e. the AIOTF would first issue an inventory operation and only after successful response from an A-IoT device, it would issue a command operation. Therefore, the A-IoT enabled gNB and the A-IoT device would need to keep their communication means established, implying that the passive A-IoT device would need to be kept energized and that the AS ID would need to be kept associated for communication with that particular A-IoT device - as discussed/designed by RAN2.

In order to enable the A-IoT RAN and the A-IoT device to stay prepared for the follow-on command operation, they would need to be informed at the initiation of the inventory operation that a follow-on command operation will be issued.

Observation 6:	For “inventory&command” operations, the A-IoT RAN and the A-IoT device would need to stay in a “prepared” state after the successful inventory operation to be able to execute the follow-on command operation.

Proposal 7:	Agree that for “inventory&command” operations the AIOTF informs the A-IoT enabled gNB at inventory request for a certain A-IoT device/group of A-IoT devices, that a “command” operation follows the inventory.

An A-IoT service operation may concern multiple devices. Either a group identification(s) and/or individual A-IoT device identifications are provided at A-IoT inventory request by the AIOTF. In order to associate information on NG-C for a particular device at “inventory & command” operations, each device-specific information needs to be associated with a particular device-specific identifier – on the one hand side to associate the device-specific Inventory Report information with subsequent device-specific information contained in the Command Request – at the A-IoT enabled gNB and the AIOTF, and on the other hand side to associate the device specific Command Response with the Command Request at the AIOTF.

Proposal 8:	Acknowledge the necessity to introduce in NGAP an A-IoT device specific identifier, allocated by the A-IoT enabled gNB at Inventory Report and included to associate device-specific information in Inventory Report, Command Request and Command Response.

On the A-IoT radio, the identification of that Access Stratum communication association between the A-IoT device and the network is performed by means of the “AS ID” allocated during the “inventory” operation. One may regard this “AS ID” for A-IoT radio signalling as an equivalent to the RNTI for UE identification on Uu. The reason why on Uu AS layer signalling does not use NAS layer UE identification is many-fold: layer separation (as far as possible), avoidance of NAS UE identifiers (even though it is mostly a temporary one) to be sent on AS and size of that NAS UE identifier (which needs to be unique at least within the serving AMF) and NAS layer encryption where NAS UE identifiers are exchanged.

Observation 9:	The scope of the AS ID (single reader) does not allow to use it as NGAP device identification.

On NG-C, we are faced with the fact that A-IoT device identifiers used for paging may be either a group identification (which doesn’t allow to use this identifier for matching the response), or, in case of initial registration, paging and paging response IDs do not match, only in case of using temporary A-IoT device identification (see latest SA3 agreements [see latest draft SA3 meeting minutes and the SA3 WID in SP-250359]). However, also in the latter case, the temporary device identification might be changed during the A-IoT operation (in principle each A-IoT inventory operation will have to always contain a command part, if the temporary id is changed at each A-IoT operation), which also shows that it is not convenient to use such temporary device identifier.

Observation 10:	In order to keep security, system and protocol design principles intact, in analogy to UEs, NAS-level A-IoT device identifiers should be exchanged on NG-C/AS only as rarely as possible.

So, we suggest introducing at least one NG-C specific A-IoT device identifier, which is allocated by the A-IoT enabled gNB, associated with the AS ID and used to identify NG-C information signalled on NG-C for a specific A-IoT device during a certain A-IoT service operation (identified by a Correlation ID).

Proposal 11:	Introduce an “RAN A-IoT Device NGAP ID”, to be allocated by the A-IoT enabled gNB at Inventory Report and used as reference for A-IoT device specific communication within an A-IoT operation. The A-IoT enabled gNB associates the “RAN A-IoT Device NGAP ID” with the “AS ID”.

While the Correlation ID is allocated by the AIOTF and can be seen as the equivalent to AMF UE NGAP ID for UE-associated signalling connections, and the proposed “RAN A-IoT Device NGAP ID” can be seen as the equivalent to the RAN UE NGAP ID, their nature is different:  the Correlation ID is associated with an A-IoT service operation while the proposed “RAN A-IoT Device NGAP ID” is associated to A-IoT device specific information exchanged during the course of an A-IoT service operation.

Observation 11: One has to bear in mind that the Correlation ID, allocated by the AIOTF, is associated with an A-IoT service operation while the proposed “RAN A-IoT Device NGAP ID”, allocated by the A-IoT enabled gNB, is associated to A-IoT device specific information exchanged during the course of an A-IoT service operation.

Having underlined that there exists an analogy between A-IoT service operation level signalling connection (denoted by a Correlation ID) and within it, A-IoT device related signalling connection (denoted by a proposed “RAN A-IoT Device NGAP ID”) with UE-associated signalling connections (denoted by an AMF UE NGAP ID and a RAN UE NGAP ID), might create the demand to discuss whether the Correlation ID and the proposed “RAN A-IoT Device NGAP ID” should be accompanied by a RAN / AIOTF allocated ID. At least RAN3 should make a conscious decisions for that topic.

Proposal 12:	Discuss whether the Correlation ID (allocated by the AIOTF) the proposed “RAN A-IoT Device NGAP ID” (allocated by the gNB) would need a RAN/AIOTF counterpart, in analogy to e.g. UE-associated signalling connections on NGAP denoted by an AMF allocated and RAN allocated NGAP UE ID.

## 2.5	Necessary size of “Correlation ID” and “RAN A-IoT Device NGAP ID”

The remaining question is to discuss the size of the Correlation ID and the proposed “RAN A-IoT Device NGAP ID”.

We would like to provide the following aspects:

Correlation ID:

Whether many or only a few A-IoT service operations can be performed by an A-IoT enabled gNB concurrently depends on the serving area of the gNB, the device distribution within this area, the likelihood of different service operations to run in parallel (diversity of possible service requests from an Application Functions) and many others.

It might also depend on for how long a Correlation ID shall be kept associated by the same A-IoT service operation.

NGAP A-IoT Device ID:

The proposed “NGAP A-IoT Device ID“ needs to be kept unique only within the scope of a single A-IoT service operation, i.e. it is associated with a specific Correlation ID only.

Still, given a theoretically large service area and high device density, a single service operation could affect a significant number of devices. Given a rather limiting maximum number of devices able to address with a single operation on NGAP would require the AIOTF to split an application-level service request into several NGAP operations, which could affect the timely response expected by the application. If the distribution of devices is known, a single application-level service request could be split into several, each concerning a different area (which in turn could have an effect on the necessary range of the Correlation ID).

Proposal 13:	Discuss the size of the Correlation ID and the proposed “RAN A-IoT Device NGAP ID”. A starting point could be 1 byte for the Correlation ID and 3 bytes for the proposed “RAN A-IoT Device NGAP ID”.

Concluding Proposal:	Agree on the Text Proposals for TS 38.401 and TS 38.300 in the Annexes.

## 2.6	Cardinality aspects in case of indirect connectivity via AMF(s)

In case of indirect communication, it should be possible to apply the same connectivity cardinality as for NG-RAN node/AMF/SMF cardinality, i.e. to assume that an AIOTF is reachable by an A-IoT enabled gNB via all A-IoT enabled AMF the gNB is connected to. Such approach would have implications on TS 38.401.

Proposal 14:	Specify in TS 38.401 that each A-IoT enabled NG-RAN node is connected to all A-IoT enabled AMFs of AMF Sets within an AMF Region.

The cardinality aspect in case of indirect connectivity is proposed to be marked as “FFS” for now. It seems appropriate to inform SA2 on these assumptions.

Proposal 15:	Inform SA2 about the cardinality assumptions in proposal 4 (see draft LS out in R3-250446).

# 3	Conclusion and Proposals

We have discussed how to multiplex several concurrent A-IoT service operations concerning multiple A-IoT devices on NG-C. The observations and concluding proposals were discussed along the following lines:

Proposal 1:	In order to enable a uniform gNB behaviour w.r.t. connectivity option and a stateless AMF it is proposed to introduce AIOTF and NG-RAN node identifiers in each AIOTF NGAP message. Those identifiers are examined in case of indirect connectivity by the AMF, in case of direct connectivity, these identifiers can be ignored.

Observation 2:	Along SA2 agreements, the AIOTF generates a Correlation ID and provides it to the A-IoT enabled gNB in order associate NGAP signalling with a specific A-IoT service operation. This also allows the A-IoT enabled gNB to accept parallel A-IoT service operations. On A-IoT radio, RAN2 discussed restrictions on how many parallel A-IoT service operations could be served in parallel by a reader at a certain point in time would have to be taken into account. The Correlation ID associated to an “inventory&command” operation is the same for the “inventory” part and the “command” part.

Proposal 3:	Introduce a “Correlation ID” in the respective NGAP inventory and command transfer containers. In Rel-19 the A-IoT Correlation Identifier is allocated by the AIOTF to uniquely identify A-IoT service operations controlled by the AIOTF.

Observation 4:	Support of multiple concurrent A-IoT service operations stemming from different AIOTFs is supported by communication means introduced in Proposal 1.

Observation 5:	RAN2 agreed to introduce an “Access Stratum Identifier” (AS-ID) identifying communication of a particular A-IoT device with a particular A-IoT reader, and always allocate and use it for an AIoT device for “inventory and command” operations.

Observation 6:	For “inventory&command” operations, the A-IoT RAN and the A-IoT device would need to stay in a “prepared” state after the successful inventory operation to be able to execute the follow-on command operation.

Proposal 7:	Agree that for “inventory&command” operations the AIOTF informs the A-IoT enabled gNB at inventory request for a certain A-IoT device/group of A-IoT devices, that a “command” operation follows the inventory.

Proposal 8:	Acknowledge the necessity to introduce in NGAP an A-IoT device specific identifier, allocated by the A-IoT enabled gNB at Inventory Report and included to associate device-specific information in Inventory Report, Command Request and Command Response.

Observation 9:	The scope of the AS ID (single reader) does not allow to use it as NGAP device identification.

Observation 10:	In order to keep security, system and protocol design principles intact, in analogy to UEs, NAS-level A-IoT device identifiers should be exchanged on NG-C/AS only as rarely as possible.

Proposal 11:	Introduce an “RAN A-IoT Device NGAP ID”, to be allocated by the A-IoT enabled gNB at Inventory Report and used as reference for A-IoT device specific communication within an A-IoT operation. The A-IoT enabled gNB associates the “RAN A-IoT Device NGAP ID” with the “AS ID”.

Proposal 12:	Discuss whether the Correlation ID (allocated by the AIOTF) the proposed “RAN A-IoT Device NGAP ID” (allocated by the gNB) would need a RAN/AIOTF counterpart, in analogy to e.g. UE-associated signalling connections on NGAP denoted by an AMF allocated and RAN allocated NGAP UE ID.

Proposal 13:	Discuss the size of the Correlation ID and the proposed “RAN A-IoT Device NGAP ID”. A starting point could be 1 byte for the Correlation ID and 3 bytes for the proposed “RAN A-IoT Device NGAP ID”.

Proposal 14:	Specify in TS 38.401 that each A-IoT enabled NG-RAN node is connected to all A-IoT enabled AMFs of AMF Sets within an AMF Region.

Proposal 15:	Inform SA2 about the cardinality assumptions in proposal 4 (see draft LS out in R3-250446).

Final Proposal:	Agree on the Text Proposals in the Annexes below.

# 4	References

[x1]	S2-2502588 "Command procedure", approved at SA2#167

[x2]	S2-2502590 "PCR to 23.xyz: Reader selection for Ambient IoT service request", approved at SA2#167

# Annex A:	Text Proposals for TS 38.401

<<<<<<<<<<<<<<<<<<<< First Change >>>>>>>>>>>>>>>>>>>>

### 6.1.1	Overall Architecture of NG-RAN

Figure 6.1-1: Overall architecture

The NG-RAN consists of a set of gNBs connected to the 5GC through the NG interface.

NOTE 1:	As specified in TS 38.300 [2], NG-RAN could also consists of a set of ng-eNBs, an ng-eNB may consist of an ng-eNB-CU and one or more ng-eNB-DU(s). An ng-eNB-CU and an ng-eNB-DU is connected via W1 interface. The general principle described in this clause also applies to ng-eNB and W1 interface, if not explicitly specified otherwise.

An gNB can support FDD mode, TDD mode or dual mode operation.

gNBs can be interconnected through the Xn interface.

A gNB may consist of a gNB-CU and one or more gNB-DU(s). A gNB-CU and a gNB-DU is connected via F1 interface.

One gNB-DU is connected to only one gNB-CU.

NOTE 2:	In case of network sharing with multiple cell ID broadcast, each Cell Identity associated with a subset of PLMNs corresponds to a gNB-DU and the gNB-CU it is connected to, i.e. the corresponding gNB-DUs share the same physical layer cell resources.

NOTE 3:	For resiliency, a gNB-DU may be connected to multiple gNB-CUs by appropriate implementation.

NG, Xn and F1 are logical interfaces.

For NG-RAN, the NG and Xn-C interfaces for a gNB consisting of a gNB-CU and gNB-DUs, terminate in the gNB-CU. For EN-DC, the S1-U and X2-C interfaces for a gNB consisting of a gNB-CU and gNB-DUs, terminate in the gNB-CU. The gNB-CU and connected gNB-DUs are only visible to other gNBs and the 5GC as a gNB. A possible deployment scenario is described in Annex A.

The node hosting user plane part of NR PDCP (e.g. gNB-CU, gNB-CU-UP, and for EN-DC, MeNB or SgNB depending on the bearer split) shall perform user inactivity monitoring and further informs its inactivity or (re)activation to the node having C-plane connection towards the core network (e.g. over E1, X2). The node hosting NR RLC (e.g. gNB-DU) may perform user inactivity monitoring and further inform its inactivity or (re)activation to the node hosting control plane, e.g. gNB-CU or gNB-CU-CP.

UL PDCP configuration (i.e. how the UE uses the UL at the assisting node) is indicated via X2-C (for EN-DC), Xn-C (for NG-RAN) and F1-C. Radio Link Outage/Resume for DL and/or UL is indicated via X2-U (for EN-DC), Xn-U (for NG-RAN) and F1-U.

The NG-RAN is layered into a Radio Network Layer (RNL) and a Transport Network Layer (TNL).

The NG-RAN architecture, i.e. the NG-RAN logical nodes and interfaces between them, is defined as part of the RNL.

For each NG-RAN interface (NG, Xn, F1) the related TNL protocol and the functionality are specified. The TNL provides services for user plane transport, signalling transport.

In NG-Flex configuration, each NG-RAN node is connected to all AMFs of AMF Sets within an AMF Region supporting at least one slice also supported by the NG-RAN node. The AMF Set and the AMF Region are defined in TS 23.501 [3]. Each A-IoT enabled NG-RAN node is connected to all A-IoT enabled AMFs of AMF Sets within an AMF Region.

Editor’s Note:	This cardinality related aspect may need further discussions, for now it follows non-A-IoT architecture principles.

If security protection for control plane and user plane data on TNL of NG-RAN interfaces has to be supported, NDS/IP TS 33.501 [13] shall be applied.

<<<<<<<<<<<<<<<<<<<< Next Change >>>>>>>>>>>>>>>>>>>>

## 6.2	NG-RAN identifiers

### 6.2.1	Principle of handling Application Protocol Identities

An Application Protocol Identity (AP ID) is allocated when a new UE-associated logical connection is created in either an NG-RAN node or an AMF. An AP ID shall uniquely identify a logical connection associated to a UE over the NG interface or Xn interface within a node (NG-RAN node or AMF) or over the F1 interface or over the E1 interface or over the W1 interface. Upon receipt of a message that has a new AP ID from the sending node, the receiving node shall store the AP ID of the sending node for the duration of the logical connection. The receiving node shall assign the AP ID to be used to identify the logical connection associated to the UE and include it as well as the previously received new AP ID from the sending node, in the first returned message to the sending node. In all subsequent messages to and from sending node, both AP IDs of sending node and receiving node shall be included. For MBS-associated logical connections of the E1 interface and the F1 interface the same principles for AP IDs apply as for UE-associated logical connections.

The definitions of AP IDs as used on NG interface or Xn interface or F1 interface or E1 interface are shown below:

RAN UE NGAP ID:

A RAN UE NGAP ID shall be allocated so as to uniquely identify the UE over the NG interface within an gNB. When an AMF receives an RAN UE NGAP ID it shall store it for the duration of the UE-associated logical NG-connection for this UE. Once known to an AMF this is included in all UE associated NGAP signalling.

The RAN UE NGAP ID shall be unique within the logical NG-RAN node.

AMF UE NGAP ID:

An AMF UE NGAP ID shall be allocated so as to uniquely identify the UE over the NG interface within the AMF. When a NG-RAN node receives an AMF UE NGAP ID it shall store it for the duration of the UE-associated logical NG-connection for this UE. Once known to a NG-RAN node this ID is included in all UE associated NGAP signalling.

The AMF UE NGAP ID shall be unique within an AMF Set as specified in TS 23.501 [3].

Old NG-RAN node UE XnAP ID:

An Old NG-RAN node UE XnAP ID shall be allocated so as to uniquely identify the UE over the Xn interface within an old NG-RAN node. When a new NG-RAN node receives an Old NG-RAN node UE XnAP ID it shall store it for the duration of the UE-associated logical Xn-connection for this UE. Once known to a new NG-RAN node this ID is included in all UE associated XnAP signalling. The Old NG-RAN node UE XnAP ID shall be unique within the logical NG-RAN node.

New NG-RAN node UE XnAP ID:

A New NG-RAN node UE XnAP ID shall be allocated so as to uniquely identify the UE over the Xn interface within a new NG-RAN node. When an old NG-RAN node receives a New NG-RAN node UE XnAP ID it shall store it for the duration of the UE-associated logical Xn-connection for this UE. Once known to an old NG-RAN node this ID is included in all UE associated XnAP signalling. The New NG-RAN node UE XnAP ID shall be unique within the logical NG-RAN node.

Source NG-RAN node UE XnAP ID:

A Source NG-RAN node UE XnAP ID shall be allocated so as to uniquely identify the UE over the Xn interface within a source NG-RAN node. When a target NG-RAN node receives a Source NG-RAN node UE XnAP ID it shall store it for the duration of the UE-associated logical Xn-connection for this UE. Once known to a target NG-RAN node this ID is included in all UE associated XnAP signalling. The Source NG-RAN node UE XnAP ID shall be unique within the logical NG-RAN node.

Target NG-RAN node UE XnAP ID:

A Target NG-RAN node UE XnAP ID shall be allocated so as to uniquely identify the UE over the Xn interface within a target NG-RAN node. When a source NG-RAN node receives a Target NG-RAN node UE XnAP ID it shall store it for the duration of the UE-associated logical Xn-connection for this UE. Once known to a source NG-RAN node this ID is included in all UE associated XnAP signalling. The Target NG-RAN node UE XnAP ID shall be unique within the logical NG-RAN node.

M-NG-RAN node UE XnAP ID:

An M-NG-RAN node UE XnAP ID shall be allocated so as to uniquely identify the UE over the Xn interface within an M-NG-RAN node for dual connectivity. When an S-NG-RAN node receives an M-NG-RAN node UE XnAP ID it shall store it for the duration of the UE-associated logical Xn-connection for this UE. Once known to an S-NG-RAN node this ID is included in all UE associated XnAP signalling. The M-NG-RAN node UE XnAP ID shall be unique within the logical NG-RAN node.

S-NG-RAN node UE XnAP ID:

A S-NG-RAN node UE XnAP ID shall be allocated so as to uniquely identify the UE over the Xn interface within an S-NG-RAN node for dual connectivity. When an M-NG-RAN node receives a S-NG-RAN node UE XnAP ID it shall store it for the duration of the UE-associated logical Xn-connection for this UE. Once known to an M-NG-RAN node this ID is included in all UE associated XnAP signalling. The S-NG-RAN node UE XnAP ID shall be unique within the logical NG-RAN node.

gNB-CU UE F1AP ID:

A gNB-CU UE F1AP ID shall be allocated so as to uniquely identify the UE over the F1 interface within a gNB-CU. When a gNB-DU receives a gNB-CU UE F1AP ID it shall store it for the duration of the UE-associated logical F1-connection for this UE. The gNB-CU UE F1AP ID shall be unique within the gNB-CU logical node.

gNB-DU UE F1AP ID:

A gNB-DU UE F1AP ID shall be allocated so as to uniquely identify the UE over the F1 interface within a gNB-DU. When a gNB-CU receives a gNB-DU UE F1AP ID it shall store it for the duration of the UE-associated logical F1-connection for this UE. The gNB-DU UE F1AP ID shall be unique within the gNB-DU logical node.

gNB-CU-CP UE E1AP ID:

A gNB-CU-CP UE E1AP ID shall be allocated so as to uniquely identify the UE over the E1 interface within a gNB-CU-CP (respectively an ng-eNB-CU-CP, or an eNB-CP as defined in TS 36.401[28]). When a gNB-CU-UP (respectively an ng-eNB-CU-UP, or an eNB-UP as defined in TS 36.401[28]) receives a gNB-CU-CP UE E1AP ID it shall store it for the duration of the UE-associated logical E1-connection for this UE. The gNB-CU-CP UE E1AP ID shall be unique within the gNB-CU-CP (respectively the ng-eNB-CU-CP, or the eNB-CP as defined in TS 36.401[28]) logical node.

gNB-CU-UP UE E1AP ID:

A gNB-CU-UP UE E1AP ID shall be allocated so as to uniquely identify the UE over the E1 interface within a gNB-CU-UP (respectively an ng-eNB-CU-UP, or an eNB-UP as defined in TS 36.401[28]). When a gNB-CU-CP (respectively an ng-eNB-CU-CP, or an eNB-CP as defined in TS 36.401[28]) receives a gNB-CU-UP UE E1AP ID it shall store it for the duration of the UE-associated logical E1-connection for this UE. The gNB-CU-UP UE E1AP ID shall be unique within the gNB-CU-UP (respectively the ng-eNB-CU-UP, or the eNB-UP as defined in TS 36.401[28]) logical node.

ng-eNB-CU UE W1AP ID:

An ng-eNB-CU UE W1AP ID shall be allocated so as to uniquely identify the UE over the W1 interface within an ng-eNB-CU. When an ng-eNB-DU receives an ng-eNB-CU UE W1AP ID it shall store it for the duration of the UE-associated logical W1-connection for this UE. The ng-eNB-CU UE W1AP ID shall be unique within the ng-eNB-CU logical node.

ng-eNB-DU UE W1AP ID:

An ng-eNB-DU UE W1AP ID shall be allocated so as to uniquely identify the UE over the W1 interface within an ng-eNB-DU. When an ng-eNB-CU receives an ng-eNB-DU UE W1AP ID it shall store it for the duration of the UE-associated logical W1-connection for this UE. The ng-eNB-DU UE W1AP ID shall be unique within the ng-eNB-DU logical node.

gNB-CU MBS F1AP ID:

A gNB-CU MBS F1AP ID shall be allocated so as to uniquely identify the MBS Session Context over the F1 interface within a gNB-CU. When a gNB-DU receives a gNB-CU MBS F1AP ID it shall store it for the duration of the MBS-associated logical F1-connection for that MBS Session. The gNB-CU MBS F1AP ID shall be unique within the gNB-CU logical node.

gNB-DU MBS F1AP ID:

A gNB-DU MBS F1AP ID shall be allocated so as to uniquely identify the MBS Session Context over the F1 interface within a gNB-DU. When a gNB-CU receives a gNB-DU MBS F1AP ID it shall store it for the duration of the MBS-associated logical F1-connection for this MBS Session. The gNB-DU MBS F1AP ID shall be unique within the gNB-DU logical node.

gNB-CU-CP MBS E1AP ID:

A gNB-CU-CP MBS E1AP ID shall be allocated so as to uniquely identify the MBS Session Context over the E1 interface within a gNB-CU-CP. When a gNB-CU-UP receives a gNB-CU-CP MBS E1AP ID it shall store it for the duration of the MBS-associated logical E1-connection for that MBS Session. The gNB-CU-CP MBS E1AP ID shall be unique within the gNB-CU-CP logical node.

gNB-CU-UP MBS E1AP ID:

A gNB-CU-UP MBS E1AP ID shall be allocated so as to uniquely identify the MBS Session Context over the E1 interface within a gNB-CU-UP. When a gNB-CU-CP receives a gNB-CU-UP MBS E1AP ID it shall store it for the duration of the MBS-associated logical E1-connection for this MBS Session. The gNB-CU-UP MBS E1AP ID shall be unique within the gNB-CU-UP logical node.

A-IoT Correlation ID:

An A-IoT Correlation ID shall be allocated so as to uniquely identify an A-IoT service operation over the NG-C interface. When a gNB receives an A-IoT Correlation ID it shall store it for the duration of the A-IoT service operation. The A-IoT Correlation ID shall be unique within an AIOTF as specified in <reference to SA2 TS to be added>.

RAN A-IoT Device ID:

A RAN A-IoT Device ID shall be allocated so as to uniquely identify A-IoT device specific information within an A-IoT service operation over the NG-C interface. When the AIOTF receives a RAN A-IoT Device ID it shall store it for the duration of the A-IoT service operation. The RAN A-IoT Device ID shall be unique within an A-IoT service operation over the NG-C interface.

<<<<<<<<<<<<<<<<<<<< End of Changes >>>>>>>>>>>>>>>>>>>>

# Annex B: Text Proposal for TS 38.300 (against R3-251563)

Note:	Changes with respect to R3-251563 highlighted in yellow.

<<<<<<<<<<<<<<<<<<<< Begin of Changes >>>>>>>>>>>>>>>>>>>>

### 16.xx.x3	Inventory procedure

Figure 16.xx.x3-1 depicts the basic communication between the gNB and the A-IoT CN node for the Inventory procedure.

Editor’s Note 3:	Entity names, message names and question whether to depict direct and indirect connectivity somehow in a different way are subject to further discussions.

Figure 16.xx.x3-1: Inventory procedure

In order to support indirect connectivity, as specified in <reference to SA2 TS to be added>, each NGAP message carries the Global gNB ID and the Global AIOTF ID to denote the gNB and AIOTF involved in the Inventory procedure.

1.	The A-IoT CN node initiates the Inventory procedure over NG-C by sending the Inventory Request message to the gNB and assigns a Correlation ID which is contained in all NGAP messages of the Inventory procedure. The Inventory Request may include information concerning one or multiple A-IoT devices.

Editor’s Note 4:	Specification of further parameterisation of the Inventory Request message (e.g., assistance information from 5GC) needs further work.

2.	The gNB allocates and co-ordinates the usage of A-IoT radio resources.

3.	The gNB confirms the request from the A-IoT CN node by replying with the Inventory Response message.

NOTE 2:	If the gNB is not able to perform the Inventory procedure, it rejects the request and sends an Inventory Failure message to the A-IoT CN node.

4.	The gNB performs the Inventory procedure towards the A-IoT device(s) over the A-IoT radio interface.

5./6.	Upon receiving the inventory result from the A-IoT device(s), the gNB sends the Inventory Report message to the A-IoT CN node. If the Inventory procedure concerns multiple A-IoT devices, multiple Inventory Reports may be sent to the A-IoT CN node.

NOTE 3:	If the Inventory procedure concerns multiple A-IoT devices, an Inventory Report may include reports from multiple A-IoT devices,

### 16.xx.x4	Command procedure

Figure 16.xx.x4-1 depicts the basic communication between the gNB and the A-IoT CN node for the Command procedure.

Editor’s Note 5:	Entity names, message names and question whether to depict direct and indirect connectivity somehow in a different way are subject to further discussions.

Figure 16.xx.x4-1: Command procedure

In order to support indirect connectivity, as specified in <reference to SA2 TS to be added>, each NGAP message carries the Global gNB ID and the Global AIOTF ID to denote the gNB and AIOTF involved in theCommand procedure.

1.	Prior to the Command procedure, the Inventory procedure is performed for the concerned devices. The Correlation ID assigned during the Inventory procedure is is contained in all NGAP messages of the Command procedure. The Inventory Request message in the Inventory procedure carries an indication that the Inventory procedure is followed by a Command procedure. The Inventory Report message in the Inventory procedure carries a per-A-IoT device RAN A-IoT Device ID allocated by the gNB and associated to the AS ID assigned during the Inventory procedure over the A-IoT radio interface.

Editor’s Note 6:	Further details on the relation and correlation between the Inventory procedure and the Command procedure needs further discussion may need input from SA2/SA3/RAN2.

2.	The A-IoT CN node initiates the Command procedure over NG-C by sending the Command Request message to the gNB containing the RAN A-IoT Device ID associated to per-A-IoT device information.

3.	The gNB allocates and co-ordinates the usage of A-IoT radio resources and performs the command procedure over the A-IoT radio interface.

4.	The gNB replies to the Command Request with the Command Response message containing the RAN A-IoT Device ID associated to per-A-IoT device information.

NOTE 4:	If the gNB is not able to perform the Command procedure, it rejects the request and sends a Command Failure message to the A-IoT CN node.

<<<<<<<<<<<<<<<<<<<< End of Changes >>>>>>>>>>>>>>>>>>>>