3GPP TSG-RAN WG3 Meeting #127bis	R3-252081

Wuhan, China, 7 – 11 April, 2025

Agenda item:	11.4

Source:	Nokia

Title:	Procedures for AI/ML Mobility Optimization in NR-DC

Document for:	Discussion and Decision

# 1	Introduction

In RAN3 #126 we made the following agreement regarding the procedures to trigger the collection of UE Performance measurements for NR-DC:

Using the Data Collection Reporting Initiation procedure to configure the collection of UE performance at SN side.

Introduce the Data Collection ID IE into the SN Addition Request message to identify a Data Collection Reporting context.

In addition, there are two open issues related to assistance information from MN to SN, namely:

Introduce predicted PSCell ID in the DC procedure (e.g., SN Addition, MN-initiated SN change) as one of assistance information?

Other information, e.g., predicted SCG bearer?

In this contribution, we provide further views on the introduced procedures, including starting and stopping conditions for the collection  of UE Performance measurements in NR-DC. We also address the open issues from the previous meeting.

# 2	Discussion

## 2.1 UE Data Collection Triggers in NR-DC

In Dual Connectivity, Master Node and the Secondary Node are interconnected via Xn-C. There are different U-plane connectivity options of the MN and SN involved in MR-DC for a certain UE, as shown in Figure 1. According to TS 37.340, the U-plane connectivity depends on the bearer option configured, namely:

“

-	For MN terminated bearers, the user plane connection to the CN entity is terminated in the MN;

-	For SN terminated bearers, the user plane connection to the CN entity is terminated in the SN;

-	The transport of user plane data over the Uu either involves MCG or SCG radio resources or both:

-	For MCG bearers, only MCG radio resources are involved;

-	For SCG bearers, only SCG radio resources are involved;

-	For split bearers, both MCG and SCG radio resources are involved.

-	For split bearers, MN terminated SCG bearers and SN terminated MCG bearers, PDCP data is transferred between the MN and the SN via the MN-SN user plane interface”

Figure 1 NR-DC network side protocol termination options for MCG, SCG and split bearers (TS 37.340).

Regarding Data Collection initiation and reporting between Master Node and Secondary Node, the following agreement was made in RAN3#126: Using the Data Collection Reporting Initiation procedure to configure the collection of UE performance at SN side. Introduce the Data Collection ID IE into the SN Addition Request message to identify a Data Collection Reporting context.

In the rest of the paper, we discuss triggers to initiate and terminate the data collection in Dual Connectivity.

### 2.1.1. Conditions to start the data collection in NR-DC

In case of handover triggered UE Performance feedback data collection from target NG-RAN Node, TS 38.423 Section 8.2.1.2 indicates the start of the data collection “after successful handover” as follows:

Interaction with the Data Collection Reporting and the Data Collection Reporting Initiation procedures:

If the Data Collection ID IE is contained in the HANDOVER REQUEST message, the target NG-RAN node shall, if supported, report to the source NG-RAN node after successful handover, via the Data Collection Reporting procedure, the requested information configured via the previous Data Collection Reporting Initiation procedure corresponding to the NG-RAN node1 Measurement ID IE, allocated by the source NG-RAN node, and the NG-RAN node2 Measurement ID IE, allocated by the target NG-RAN node.

However, even though handover is associated with coverage, Dual Connectivity is associated with capacity. While there is only a single procedure controlling the handover preparation (Handover Preparation Request), in Dual Connectivity there exist two independent processes, namely S-Node Addition and SCG Activation/Deactivation (see TS 37.340). For SN-terminated bearers, this allows to establish an endpoint for data transfer in the SN (or the CU-UP of the SN) immediately as part of QoS flow setup or incoming handover without necessarily requiring the UE to actively monitor the SCG (e.g., PDCCH decoding) in periods without data. Therefore, it is possible to indicate whether the SCG resources are required to be activated or deactivated when S-Node Addition procedure is sent from Master Node to Secondary Node. SCG can also be independently activated or deactivated by the SN. Only when the SCG resources are activated, data may actually be transferred over the SCG  and corresponding UE Performance measurements can be collected. We therefore believe that UE Performance measurements for bearers handled over the SCG leg should only be collected after SCG resources are activated.

In TS 38.423 we have captured the following interaction between the Data collection Reporting and Data Collection Reporting Initiation procedures:

Subsequently, based on the above discussion we think that the measurement collection should be initiated not after S-Node Addition, but after SCG activation in case of SCG bearers.

Observation 1: Starting Measurement Collection in Secondary Node based on S-Node Addition trigger can result in useless measurements and exchange since, in case of SCG bearers, there is no data flow until SCG is activated.

Proposal 1: Measurement Collection for UE Performance in Dual Connectivity is started after SCG activation instead of S-Node Addition Request.

Proposal 2: Update the text in TS 38.423 as follows:

Interaction with the Data Collection Reporting Initiation and the Data Collection Reporting procedures:

If the Data Collection ID IE is contained in the S-NODE ADDITION REQUEST message, the S-NG-RAN node shall, if supported, report to the M-NG-RAN node after successful S-NG-RAN node addition and SCG Activation, via the Data Collection Reporting procedure, the requested information configured via the previous Data Collection Reporting Initiation procedure corresponding to the NG-RAN node1 Measurement ID IE, allocated by the M-NG-RAN node, and the NG-RAN node2 Measurement ID IE, allocated by the S-NG-RAN node.

TS 37.340 Clause 7.13 Activation and Deactivation of SCG defines the activation and deactivation mechanism of SCG. SCG activation can be requested by the MN, by the SN, and by the UE. SCG deactivation can be requested by the MN and by the SN.

SCG Activation/deactivation status impacts the UE performance feedback collection and reporting from SN to MN. We further discuss the impact of UE performance feedback reporting between Master Node and Secondary Node when SCG is activated/deactivated within the measurement collection duration. When SCG is deactivated, UE Performance measurements are not relevant because there is no UP traffic. If UE performance is still measured and reported during this period, the corresponding measurements will be zero and when used for averaging together with other samples in a reporting period it will impact the accuracy of the reported measurements. Hence, reporting should be suspended when SCG is deactivated and resumed when SCG is activated. Figure 2 below depicts the reporting suspension and resumption due to SCG deactivation and activation respectively. In particular, in this figure, periodic reporting is configured at the SN at time T1, T2, …T5, …. Reporting takes place at T1 and T2 but right before time T3 SCG is deactivated. This will suspend the reporting at T3. The SN will report again at T4, after SCG activation and will continue at times T5,….

Figure 2 Reporting Suspension and Resumption during SCG deactivation and SCG activation.

Proposal 3: Suspend the reporting of UE Performance measurements during SCG deactivation.

### 2.1.2. Conditions to terminate the data collection in NR-DC

As captured in TS 38.423, in Rel-18 when a UE is handed over from source NG-RAN node to target NG-RAN Node, target NG-RAN node shall terminate the collection when at least one of the following conditions is fulfilled:

- the time since UE was successfully handed over to NG-RAN node2 is equal to the value of the Collection Time Duration for UE Performance IE;

-  UE moves to RRC_INACTIVE or RRC_IDLE state;

-  UE is handed over to another cell.

We think that similar conditions can be defined in case of Dual Connectivity. Specifically, also as in Rel-18 UE Performance case, in Dual Connectivity UE Performance can be terminated if UE moves to RRC_INACTIVE or RRC_IDLE state.

In addition, in Dual Connectivity a Collection Time Duration can be defined to terminate UE Performance Collection. The time to start measuring this time duration is after successful S-Node Addition and SCG Activation.

Another condition to terminate UE Performance Measurement Collection in Dual Connectivity is when S-NG-RAN node is released by the MN or the SN. This is because there is no more an SN node to serve the UE.

Finally, a change of SN node will obviously need to trigger the termination of UE Performance measurements, and it would be up to the MN to trigger again such measurements in the new SN. As we discuss in section 2.3 the MN plays only a limited role in the selection of the PSCell, and it therefore seems that a change in PSCell would not need to terminate the UE Performance measurements.

Proposal 4: UE Performance measurement collection terminating conditions in Dual Connectivity comprise:

- A time equal to Collection Time Duration is passed after successful S-Node Addition and SCG Activation;

- UE moving to RRC_INACTIVE or RRC_IDLE state;

- S-NG-RAN node Release by the MN or the SN;

- S-NG-RAN node initiated S-NG-RAN node Change.

## 2.2 Handover Scenario of Reporting UE performance in Dual Connectivity

In Rel-18 UE Performance measurements are configured from a source node to a target node and the actual triggering of UE Performance measurement collection is performed through a Handover event when a UE is being handed over. UE Performance feedback from target node to source node is introduced to evaluate UE performance after a successful Handover. In case of Dual Connectivity, UE Performance information is also useful to evaluate the performance of UEs handled via MCG or SCG. Consider, as an example, a scenario where a target gNB configured with UE Performance measurement collection subsequently initiates Dual Connectivity with a target secondary NG-RAN node. This scenario is illustrated in Figure 4 (TS 37.340, clause 10.7.2).

Figure 4 Inter-MN handover with/without MN initiated SN change procedure (TS 37.340, Fig 10.7.2-1)

When the source node requests UE Performance from the target node, the target node should be able to report the UE Performance both at the Master and Secondary nodes back to the source node in case Dual Connectivity is activated. This means that UE Performance configuration needs to propagate to the Target Secondary NG-RAN node. Therefore, the Rel-18 procedures on configuring UE Performance measurements are not immediately applicable for this Dual Connectivity scenario. Since there is no Handover event between the Target Master NG-RAN node and the Target Secondary NG-RAN node, a different measurement collection trigger is needed for the collection of UE Performance from the Target Secondary NG-RAN node.

Proposal 5: Enable the UE Performance measurement collection in NR-DC for inter-MN handover scenario.

## 2.3. Assistance Information from MN to SN

Two topics that were discussed in the previous meeting comprise sending the predicted (or assumed) PSCell ID from MN to SN during Dual Connectivity and an assumed SCG bearer. Next we highlight the procedure with which PSCell is selected. The SN selects the target PSCell among the reported cells by the UE, considering the radio quality, load of the reported cells, UE Capability, etc. The SN consequently further checks based on the band combination, the feature set combination to select one combination that maximizes the throughput. Thus, the PSCell selection is up to target SN and its internal logic.

Observation 2: PSCell selection is up to target SN and its internal logic.

MN can get a list of visited PSCells for a UE from SCG UE History Information IE. However, it cannot know which were the reasons that led SN to choose those PSCells. For example, when establishing NR-DC for two different UE(s), SN can choose two different PSCell(s) because the actual selection algorithm in SN considers their bearer characteristics and UE Capabilities (e.g., band combination) amongst other considerations. Thus, MN cannot accurately predict a PSCell since it is missing input on the internal logic that led SN into its PSCell decisions at each time.

Observation 3: An MN attempt to predict the PSCell ID chosen by the SN without considering the SN internal logic that resulted in the PSCell selection will result in inaccurate PSCell predictions.

Any attempt to predict by the MN the SN PSCell selection process would mean that that MN needs to be provided along with SCG UE History Information, all the associated criteria taken into consideration by the SN including load and deployment aspects or other implementation specific details. This, makes it very complex if not impossible for the MN to predict or even assume the PSCell choices by the SN.

Observation 4: It is very complex if not impossible for the MN to predict (or assume) the PSCell choices by the SN.

Another type of assistance information for further discussion pertains to transferring from MN to SN a list of QoS flows that are “assumed” to be served by the SN. In our view, the choice of SCG bearers is also fully in control of the SN, making it impossible for MN to predict or assume this information. In current legacy behavior, the MN indicates to the SN a list of bearers to be handled in SCG but the SN may decide that only some of those bearers will be handled by SCG and the rest are up to MCG to serve. Of course over time, MN may be able to derive some information about the bearers that SN decides to handle in SCG by implementation. However, there isn’t enough justification to change legacy behavior where the choice of bearers to be handled by SCG is fully in control of the SN, independently on what MN may assume or not.

Observation 5: There is no reason to change the legacy behavior where the choice of bearers to be handled by SCG is fully in control of the SN.

Proposal 6: Do not transfer from MN to SN a Predicted PSCell ID or list of QoS flows assumed to be served by the SN as SCG bearers.

# 3	Conclusion

In this paper we provide the following observations and proposals:

Observation 1: Starting Measurement Collection in Secondary Node based on S-Node Addition trigger can result in useless measurements and exchange since, in case of SCG bearers, there is no data flow until SCG is activated.

Proposal 1: Measurement Collection for UE Performance in Dual Connectivity is started after SCG activation instead of S-Node Addition Request.

Proposal 2: Update the text in TS 38.423 as follows:

Interaction with the Data Collection Reporting Initiation and the Data Collection Reporting procedures:

If the Data Collection ID IE is contained in the S-NODE ADDITION REQUEST message, the S-NG-RAN node shall, if supported, report to the M-NG-RAN node after successful S-NG-RAN node addition and SCG Activation, via the Data Collection Reporting procedure, the requested information configured via the previous Data Collection Reporting Initiation procedure corresponding to the NG-RAN node1 Measurement ID IE, allocated by the M-NG-RAN node, and the NG-RAN node2 Measurement ID IE, allocated by the S-NG-RAN node.

Proposal 3: Suspend the reporting of UE Performance measurements during SCG deactivation.

Proposal 4: UE Performance measurement collection terminating conditions in Dual Connectivity comprise:

- A time equal to Collection Time Duration is passed after successful S-Node Addition and SCG Activation;

- UE moves to RRC_INACTIVE or RRC_IDLE state;

- S-NG-RAN node Release by the MN or the SN;

- S-NG-RAN node initiated S-NG-RAN node Change.

Proposal 5: Enable the UE Performance measurement collection in NR-DC for inter-MN handover scenario.

Observation 2: PSCell selection is up to target SN and its internal logic.

Observation 3: An MN attempt to predict the PSCell ID chosen by the SN without considering the SN internal logic that resulted in the PSCell selection will result in inaccurate PSCell predictions.

Observation 4: It is very complex if not impossible for the MN to predict (or assume) the PSCell choices by the SN.

Observation 5: There is no reason to change the legacy behavior where the choice of bearers to be handled by SCG is fully in control of the SN.

Proposal 6: Do not transfer from MN to SN a Predicted PSCell ID or list of QoS flows assumed to be served by the SN as SCG bearers.