3GPP TSG-RAN WG3 Meeting #127bis	R3-252085

Wuhan, China, 7 – 11 April, 2025

Agenda item:	20.2

Source:	Nokia

Title:	(TP to TS 38.300) Intermediate feature reporting and general principles for case 3a

Document for:	Decision

# 1	Introduction

In this contribution, we first provide our views with respect to general principles that we see applicable to case 3a. We also provide our views with respect to reporting of intermediate features from the gNB to the LMF. Finally, we provide a Stage 2 TP to TS 38.300 in the Annex of this paper.

# 2	Discussion

## 2.1	General Principles

RAN3 has agreed that for Case 3a AI/ML Model Training and AI/ML Model Inference functions are located at the gNB. Another possible option for the AI/ML Model Training is OAM, if OAM receives training data (measurements, ground truth label and related data) so that an AI/ML Model can also be trained offline. This would be in line with the Rel-18 framework of NG-RAN AI/ML described in TS 38.300 where AI/ML Model Training may be located in OAM or in the gNB. In addition, this would be in line with the TR 38.843 where the following is mentioned: “For gNB-side model, training data can be generated by the gNB, while the termination point for training data may include the gNB, or OAM.”. This means that OAM may be a termination point receiving training data and hence may also train the AI/ML model of a gNB.

Proposal 1: For case 3a, besides the gNB, AI/ML Model Training can also be located in OAM.

Note that if AI/ML Model Training is in OAM we can also reuse the Model Deployment/Update functionality that SA5 has introduced to support the deployment of the AI/ML Model from OAM to RAN.

Several aspects remain open for AI/ML assisted positioning (case 3a) that we address next.

Case 3a assumes the existence of an AI/ML Model in the gNB. We, therefore, see useful to agree for this case some high-level principles from TR 37.817 and from the AI/ML for NG-RAN work, that were considered valid during Rel-18 AI/ML for NG-RAN normative phase and are also used for the Rel-19 work that is ongoing.

Specifically, even though in Case 3a we provide support so that a gNB may train an AI/ML Model we should follow the same assumption as in NG-RAN AI/ML that AI/ML Models and Algorithms are not in scope of 3GPP specifications.

Proposal 2: Details of AI/ML Models and Algorithms for AI/ML positioning case 3a are implementation specific and out of scope of 3GPP specifications.

Another principle that we followed in Rel-18 NG-RAN AI/ML work is that an AI/ML Model used for Model Inference needs to have completed all the steps of Life Cycle Management, meaning AI/ML Model Training, AI/ML Model Validation and AI/ML Testing.  Since in Case 3a, a gNB can provide the LMF with inferred measurements we think that the same principle should be valid for the AI/ML Air Interface work.

Proposal 3: An AI/ML model for AI/ML Positioning case 3a used for Model Inference, has to be initially trained, validated and tested before deployment.

Finally, another fundamental principle for Data Collection is related to respecting user data privacy and anonymization and it shall be valid for this work item also.

Proposal 4: User data privacy and anonymization need to be respected during AI/ML operation.

In RAN3 #126 we confirmed a working assumption that NG-RAN node performs monitoring metric calculation for its own model. Model Monitoring in TR 38.843 corresponds to Model Performance monitoring in RAN3 discussions, which is the process enabling the monitoring of the performance of an AI/ML model. The outcome of Model Performance Monitoring is provided by the Model Inference to the Model Training entity through Model Performance Feedback.

Proposal 5: Use the term Model Performance monitoring to refer to Model Monitoring for better alignment with RAN3 terminology.

Similarly to the RAN3 NG-RAN AI/ML framework, the process of Model Performance Monitoring and Model Performance Feedback is implementation specific and out of RAN3 scope.

Proposal 6: Model Performance Feedback is implementation-specific and out of RAN3 scope.

We think that case 3a falls under the NG-RAN AI/ML framework and we therefore think it should be captured in TS 38.300, together with the other NG-RAN AI/ML use cases. This is reflected in the TP to TS 38.300 in the Annex of this paper.

## 2.2	Reporting of Intermediate Features to LMF

RAN1 made the following agreements with respect to the intermediate features:

RAN1 #116

Agreement

For AI/ML assisted positioning Case 3a, at least LOS/NLOS indicator and/or timing information are supported for reporting.

- If LOS/NLOS indicator is reported, the indicator can be reported as soft indicator or hard indicator as defined in 38.214.

- If timing information is reported, the timing information at least can be reported via UL RTOA or gNB Rx-Tx time difference as defined in 38.215.

- Note: details of the report are pending further discussion.

RAN1 #118bis

Agreement

From RAN1 perspective, when timing information is reported for Rel-19 AI/ML positioning Case 3a, at least the following are mandatorily or optionally supported in a measurement report from gNB to LMF:

- (Mandatory) timing information;

- (Optional) Quality of the timing information;

  - Existing IE “Timing Measurement Quality” can be reused.

- (Mandatory) Time stamp.

FFS: LOS/NLOS indicator.

Note: The final decision of “mandatory” or “optional” presence of each field is up to RAN3.

RAN1 #119

Conclusion

For measurement report of AI/ML assisted positioning Case 3a, regarding the report of LOS/NLOS indicator,

- LOS/NLOS indicator can’t be reported independently from other measurements

According to legacy positioning methods, an LMF requests measurements from a gNB and uses those measurements for the calculation of a UE positioning estimate. The LMF’s positioning algorithm does not depend on the method that gNB used to determine those measurements, while at the same time it is up to gNB implementation whether and how to perform those measurements.

Observation 1: For case 3a, the gNB, by its implementation, will use its best available method to determine the timing information measurements requested by the LMF.

Looking at the TRP Measurement Result IE in TS 38.455, we observe that the timing information that a gNB can provide through AI/ML corresponds to measurements that can be reported already by a gNB to the LMF. From LMF point of view, there is no difference between measurements that are derived by AI/ML Model Inference or by legacy measurements, measured by the gNB/TRP. Thus, it should be transparent to the LMF what methods (AI/ML or legacy) the gNB uses to provide the requested measurements to the LMF. The gNB should respond to the LMF request by choosing the best measurement it has available by its implementation, either by deploying AI/ML or legacy procedures.

Proposal 7: gNB decides whether the measurements reported to LMF are outcomes of legacy measurements or produced by AI/ML operations.

Therefore, existing methods can be used to configure reporting by a gNB in Case 3a.

Proposal 8: Reuse existing NRPPa methods to configure gNB to report LoS/NLoS indication and timing information, such as UL RTOA and gNB Rx-Tx Time Difference.

# 3	Conclusion

In this paper we make the following observation and proposals:

Proposal 1: For case 3a, besides the gNB, AI/ML Model Training can also be located in OAM.

Proposal 2: Details of AI/ML Models and Algorithms for AI/ML positioning case 3a are implementation specific and out of scope of 3GPP specifications.

Proposal 3: An AI/ML model for AI/ML Positioning case 3a used for Model Inference, has to be initially trained, validated and tested before deployment.

Proposal 4: User data privacy and anonymization need to be respected during AI/ML operation.

Proposal 5: Use the term Model Performance monitoring to refer to Model Monitoring for better alignment with RAN3 terminology.

Proposal 6: Model Performance Feedback is implementation-specific and out of RAN3 scope.

Observation 1: For case 3a, the gNB, by its implementation, will use its best available method to determine the timing information measurements requested by the LMF.

Observation 1: For case 3a, the gNB, by its implementation, will use its best available method to determine the timing information measurements requested by the LMF.

Proposal 7: gNB decides whether the measurements reported to LMF are outcomes of legacy measurements or produced by AI/ML operations.

Proposal 8: Reuse existing NRPPa methods to configure gNB to report LoS/NLoS indication and timing information, such as UL RTOA and gNB Rx-Tx Time Difference.

# Annex A -TP to TS 38.300

## <<< start of changes >>>

## 16.20	Support of AI/ML for NG-RAN

### 16.20.1	General

Support of AI/ML for NG-RAN, as a RAN function, is used to facilitate Artificial Intelligence (AI) and Machine Learning (ML) techniques in NG-RAN.

The objective of AI/ML for NG-RAN is to improve network performance and user experience, through analysing the data collected and autonomously processed by the NG-RAN, which can yield further insights, e.g., for Network Energy Saving, Load Balancing, Mobility Optimization, AI/ML assisted positioning with gNB-side AI/ML model.

### 16.20.2	Principles

Support of AI/ML for NG-RAN requires inputs from neighbour NG-RAN nodes (e.g., predicted information, feedback information, measurements) and/or UEs (e.g., measurement results).

Signalling procedures used for the exchange of information to support AI/ML for NG-RAN, are use case and data type agnostic, which means that the intended usage (e.g., input, output, feedback) of the data exchanged via these procedures is not indicated.

AI/ML algorithms and models are out of 3GPP scope. Model-specific performance information, e.g. model performance indicators specified in clause 6 of TS 28.105 [64], is not exchanged over NG-RAN interfaces in TS 38.401 [4].

Support of AI/ML for NG-RAN does not apply to ng-eNB.

For the deployment of AI/ML for NG-RAN the following scenarios may be supported:

-	AI/ML Model Training is located in the OAM and AI/ML Model Inference is located in the NG-RAN node;

-	AI/ML Model Training and AI/ML Model Inference are both located in the NG-RAN node.

AI/ML Model Training follows the definition of the "ML model training" as specified in clause 3.1 of TS 28.105 [64]. An AI/ML Model needs to be trained, validated and tested before deployment for AI/ML Model Inference.

AI/ML Model Inference follows the definition of the "AI/ML inference" as defined in clause 3.1 of TS 28.105 [64].

### 16.20.3	Data Collection and Reporting

The following information can be configured to be reported by an NG-RAN node:

-	Predicted resource status information;

-	UE performance feedback;

-	Measured UE trajectory;

-	Energy Cost (EC).

The collection and reporting are configured through the Data Collection Reporting Initiation procedure, while the actual reporting is performed through the Data Collection Reporting procedure.

The collection of measured UE trajectory and UE performance feedback is triggered at successful Handover.

Cell-based UE trajectory prediction, which can be used, e.g., for the Mobility Optimization use case, is transferred to the target NG-RAN node via the Handover Preparation procedure to provide information for, e.g., subsequent mobility decisions. Cell-based UE trajectory prediction is limited to the first-hop target NG-RAN node.

Procedures for Data Collection and Reporting for AI/ML assisted positioning with gNB-side AI/ML model are described in TS 38.305 [42].

## <<< end of changes >>>