3GPP TSG-RAN WG3 Meeting #127bis	R3-252102

Wuhan, China, 7 – 11 April 2025

Agenda Item:	12.3

Source: 				LG Electronics Inc.

Title:		 (TP to TS 38.300) Further discussion on access control in 5G Femto

Document for:		Discussion and Decision

1.	Introduction

SA2 sent a LS [1] to RAN3 to clarify the access control for the NR Femto cell shared by PLMN and PNI-NPN. In this contribution, we will discuss further clarification on access control in a cell shared by PLMN and PNI-NPN, and provide our view on it.

2.	Discussion

In RAN3#126 meeting, it was agreed to capture the following TP for access control in NR Femto in [2].

In R3-247829 [2], … ### 4.x.4 Access Control - Cells served by an NR Femto node may be deployed as part of a PNI-NPN (see clause 4.8) in order to restrict access to UEs according to the respective subscription. NOTE: The NR Femto node may use the CAG mechanism for PNI NPN (see clause 16.7.4) as follows: - The NR Femto node may activate a PLMN cell, which can be accessed by legacy UE without access control of CAG. - The NR Femto may activate a cell shared by both PLMN and CAG, through broadcasting both the plmn-IdentityInfoList and the npn-IdentityInfoList-r16 in the SIB1, but without the cellReservedForOtherUse. Then, this cell is accessible to UEs which have the allowed CAG list including a CAG-ID broadcasted by the cell. For the legacy UE not supporting CAG, this cell is viewed as a normal PLMN cell. - The NR Femto node may activate an NPN-only cell by broadcasting the cellReservedForOtherUse IE with value “true”, then this cell can only be accessed by the UEs whose allowed CAG list includes a CAG-ID broadcasted by the cell. …

In LTE, the hybrid cell is defined as the cell which is accessible as a CSG cell by UEs which are members of the CSG and as a normal cell by all other UEs. The hybrid cell in LTE broadcasts a CSG indicator set to false and a specific CSG identity. As highlighted one above, the cell shared by PLMN and PNI-NPN is introduced for the NR Femto, which is similar with the hybrid cell in 4G Femto. That is, the cell broadcasting the npn-IdentityInfoList-r16 IE and the cellReservedForOtherUse IE set to false is used to support both the CAG supporting UE and CAG non-supporting UE.

Observation 1: The NR Femto cell shared by PLMN and PNI-NPN is introduced for the NR Femto, which is similar with the hybrid cell in 4G Femto.

In SA2#167 meeting, SA2 discussed the access control in this NR Femto cell shared by PLMN and PNI-NPN from AMF point of view, and decided to send a LS to RAN3 to clarify whether this shared cell has any contradictions with the existing definitions in SA2 specifications [1] as follows:

LS from SA2 [1], … Among above three options, SA2 seeks some clarifications regarding the second option. As described in TS 23.501 clause 5.30.3.4 for PNI-NPN, the AMF and NG-RAN would reject the UE if none of the CAG ID of the cell are part of the UE's Allowed CAG list. As described in TS 23.501 clause 5.18, the MOCN sharing between the PLMN and PNI-NPN is achieved by using different cell IDs with separate configuration. Therefore, SA2 would like to understand whether the second option in the RAN3 BL CR for NR Femto contradicts with the existing SA2 specifications. …

As mentioned in SA2 LS, we also think that further clarification on how to support the access control in the NR Femto cell shared by PLMN and PNI-NPN is needed. In current RAN3 BL CR to TS 38.300 [3], it is unclear whether this NR Femto cell shared by PLMN and PNI-NPN is supported by using a single cell ID (i.e., Approach 1) or two different cell IDs (i.e., Approach 2).

Suppose that the shared cell is supported by a single cell ID (i.e., Approach 1). As described in [4], when the UE accesses to the shared cell, the NR Femto cannot differentiate a CAG supporting UE with a CAG non-supporting UE because it does not have knowledge on UE CAG capability. Therefore, during the UE’s initial access, the NR Femto always provides the supported CAG ID(s) of the shared cell to the AMF irrespective of the UE CAG capability. The AMF performs the access control for the CAG supporting UE as described in TS 23.501. However, we think that since the AMF does not know whether the CAG non-supporting UE accesses to AMF via the cell shared by PLMN and PNI-NPN, the AMF rejects the Registration Request from the CAG non-supporting UE based on UE subscription and the supported CAG ID(s) of the cell as follows:

From TS 24.501, … ##### 5.5.1.2.8 Abnormal cases on the network side The following abnormal cases can be identified: … j) Based on operator policy, if the initial registration request from a UE not supporting CAG is rejected due to CAG restrictions, the network shall reject the initial registration with a 5GMM cause value other than the 5GMM cause #76 (Not authorized for this CAG or authorized for CAG cells only). …

SA2 also mentioned in [1] that the AMF rejects the UE because none of the CAG ID of the cell are part of the UE's Allowed CAG list.

However, from the CAG non-supporting UE’s perspective, this shared cell is viewed as the PLMN cell. In this case, the AMF should not reject the Registration Request from this UE due to CAG restrictions (e.g., Not authorized for this CAG). To avoid this problem, the AMF needs to know whether the UE’s serving cell is shared by the PLMN and PNI-NPN.

Observation 2: According to the current specification, the AMF rejects the Registration Request from the CAG non-supporting UE which accesses to the cell shared by PLMN and PNI-NPN.

Observation 3: For Approach 1, the AMF needs to know whether the UE’s serving cell is shared by the PLMN and PNI-NPN for access control.

If the access of the CAG non-supporting UE via the shared cell in NR Femto is always rejected by the AMF, there is no need to introduce the cell shared by the PLMN and PNI-NPN in NR Femto. Therefore, the NR Femto needs to provide to the AMF whether the UE’s serving cell is shared by the PLMN and PNI-NPN. That is, a new indication needs to be defined into the INITIAL UE MESSAGE message.

Observation 4: If Approach 1 is adopted, the NR Femto provides to the AMF whether the UE’s serving cell is shared by the PLMN and PNI-NPN in the INITIAL UE MESSAGE message.

Another remaining issue for Approach 1 is to clarify whether a cell ID can be used for PLMN and PNI-NPN based on the existing specification. As mentioned in SA2 LS, the existing specifications describe that a cell ID is only associated with one network type among PLMN, PNI-NPN or SNPN as follows:

From TS 23.501, … ### 5.18.1 General concepts A network sharing architecture shall allow multiple participating operators to share resources of a single shared network according to agreed allocation schemes. The shared network includes a radio access network. The shared resources include radio resources. … In all non-public network sharing scenarios, each Cell Identity as specified in TS 38.331 [28] is associated with one of the following configuration options: - one or multiple SNPNs; - one or multiple PNI-NPNs (with CAG); or - one or multiple PLMNs only. …

From TS 38.300, … ## 4.6 Radio Access Network Sharing NG-RAN supports radio access network sharing as defined in TS 23.501 [3]. If NR access is shared, system information broadcast in a shared cell indicates a TAC and a Cell Identity for each subset of PLMNs, PNI-NPNs and SNPNs. NR access provides only one TAC and one Cell Identity per cell per PLMN, SNPN or PNI-NPN. In this version of the specification, a Cell Identity can only belong to one network type among PLMN, PNI-NPN or SNPN as defined in TS 23.501 [3]. Each Cell Identity associated with a subset of PLMNs, SNPNs or PNI-NPNs identifies its serving NG-RAN node. …

Therefore, Approach 1 causes the contradiction with the existing specifications. During the study phase for NR Femto, there is no detailed discussion on how to configure the cell ID for the NR Femto cell shared by PLMN and PNI-NPN. If Approach 1 is adopted, RAN3 needs to further discuss how to resolve this problem.

Observation 5: Approach 1 contradicts with the existing specifications.

In Approach 2, the plmn-IdentityInfoList IE and the npn-IdentityInfoList-r16 IE contain different cell ID values. Hence, the CAG supporting UE and the CAG non-supporting UE select different cell IDs for the NR Femto cell shared by PLMN and PNI-NPN. Then, for the initial access of the CAG supporting UE, the NR Femto provides the supported CAG ID(s) of the shared cell to the AMF, whereas this information is not provided to the AMF for the CAG non-supporting UE’s access. In this case, the AMF performs the CAG access control only for the CAG supporting UE. As a result, the NR Femto cell shared by PLMN and PNI-NPN in Approach 2 can be supported without the impact of other WG.

Observation 6: The NR Femto cell shared by PLMN and PNI-NPN in Approach 2 can be supported without the impact of other WG.

Considering the above observations and proposals, Approach 2 should be supported instead of Approach 1. However, we think that the description related to the difference cell ID configuration is still missing in current BL CR to TS 38.401. Therefore, we also propose to capture the cell ID related description into the BL CR to TS 38.401 as follows:

- The NR Femto may activate a cell shared by both PLMN and CAG, through broadcasting both the plmn-IdentityInfoList and the npn-IdentityInfoList-r16 in the SIB1, but without the cellReservedForOtherUse. In this case, the plmn-IdentityInfoList IE and the npn-IdentityInfoList-r16 IE shall contain different Cell Identities as described in clause 4.6. Then, this cell is accessible to UEs which have the allowed CAG list including a CAG-ID broadcasted by the cell. For the legacy UE not supporting CAG, this cell is viewed as a normal PLMN cell.

Proposal 1: The NR Femto cell shared by PLMN and PNI-NPN should broadcast different cell IDs for PLMN and PNI-NPN.

Proposal 2: The BL CR to TS 38.401 should be updated to support Approach 2 as follows:

- The NR Femto may activate a cell shared by both PLMN and CAG, through broadcasting both the plmn-IdentityInfoList and the npn-IdentityInfoList-r16 in the SIB1, but without the cellReservedForOtherUse. In this case, the plmn-IdentityInfoList IE and the npn-IdentityInfoList-r16 IE shall contain different Cell Identities as described in clause 4.6. Then, this cell is accessible to UEs which have the allowed CAG list including a CAG-ID broadcasted by the cell. For the legacy UE not supporting CAG, this cell is viewed as a normal PLMN cell.

In addition, we propose to agree the corresponding TP in Appendix and the Relay LS to SA2 in [5].

Proposal 3: It is proposed to agree the corresponding TP in Appendix and Relay LS to SA2 in [5].

3.	Conclusion

In this contribution, we focused on further clarification on access control in a cell shared by PLMN and PNI-NPN, and then provided our view on it. The following proposals are kindly suggested to RAN3:

Observation 1: The NR Femto cell shared by PLMN and PNI-NPN is introduced for the NR Femto, which is similar with the hybrid cell in 4G Femto.

Observation 2: According to the current specification, the AMF rejects the Registration Request from the CAG non-supporting UE which accesses to the cell shared by PLMN and PNI-NPN.

Observation 3: For Approach 1, the AMF needs to know whether the UE’s serving cell is shared by the PLMN and PNI-NPN for access control.

Observation 4: If Approach 1 is adopted, the NR Femto provides to the AMF whether the UE’s serving cell is shared by the PLMN and PNI-NPN in the INITIAL UE MESSAGE message.

Observation 5: Approach 1 contradicts with the existing specifications.

Observation 6: The NR Femto cell shared by PLMN and PNI-NPN in Approach 2 can be supported without the impact of other WG.

Proposal 1: The NR Femto cell shared by PLMN and PNI-NPN should broadcast different cell IDs for PLMN and PNI-NPN.

Proposal 2: The BL CR to TS 38.401 should be updated to support Approach 2 as follows:

- The NR Femto may activate a cell shared by both PLMN and CAG, through broadcasting both the plmn-IdentityInfoList and the npn-IdentityInfoList-r16 in the SIB1, but without the cellReservedForOtherUse. In this case, the plmn-IdentityInfoList IE and the npn-IdentityInfoList-r16 IE shall contain different Cell Identities as described in clause 4.6. Then, this cell is accessible to UEs which have the allowed CAG list including a CAG-ID broadcasted by the cell. For the legacy UE not supporting CAG, this cell is viewed as a normal PLMN cell.

Proposal 3: It is proposed to agree the corresponding TP in Appendix and Relay LS to SA2 in [5].

4.	Reference

- R3-251524/S2-2502787, “LS on NR Femto node shared by PLMN and PNI-NPN”, LG Electronics, February, 2025.

- R3-247829, “Access Control with NR Femto – Stage 2 TP,” Ericsson, Huawei, Nokia, China Telecom, Lenovo, Samsung, NTT DOCOMO, ZTE, Verizon, BT, November, 2024.

- R3-51548, “(BL CR to 38.300) Introduction of NR Femto Architecture and Protocol Aspects”, Ericsson, Nokia, TMO US, AT&T, Verizon Wireless, BT, Charter, Huawei, LG Electronics, Samsung, Lenovo, Baicells, ZTE, NEC, April, 2025.

- R3-250209, “Further discussion on access control in 5G Femto,” LG Electronics, February, 2025.

- R3-252103, “[Draft] Reply LS on NR Femto node shared by PLMN and PNI-NPN”, LG Electronics, April, 2025.

5.	Appendix: TP for TS 38.300

This appendix provides the Text proposal for TS 38.300 based on the proposals of this contribution.

----------------Start of the First Change---------------

### 4.x.4	Access Control

Cells served by an NR Femto node may be deployed as part of a PNI-NPN (see clause 4.8) in order to restrict access to UEs according to the respective subscription.

NOTE:	The NR Femto node may use the CAG mechanism for PNI-NPN (see clause 16.7.4) as follows:

-	The NR Femto node may activate a PLMN cell, which can be accessed by legacy UE without access control of CAG.

-	The NR Femto may activate a cell shared by both PLMN and PNI-NPN, through broadcasting both the plmn-IdentityInfoList and the npn-IdentityInfoList-r16 in the SIB1, but without the cellReservedForOtherUse. In this case, the plmn-IdentityInfoList IE and the npn-IdentityInfoList-r16 IE shall contain different Cell Identities as described in clause 4.6. Then, this cell is accessible to UEs which have the allowed CAG list including a CAG-ID broadcasted by the cell. For the legacy UE not supporting CAG, this cell is viewed as a normal PLMN cell.

-	The NR Femto node may activate an NPN-only cell by broadcasting the cellReservedForOtherUse IE with value “true”, then this cell can only be accessed by the UEs whose allowed CAG list includes a CAG-ID broadcasted by the cell.

-----------------End of the First Change---------------