3GPP TSG-RAN WG3 #127bis	R3-252103

Wuhan, China, 7th – 11th April 2025

Title:	[Draft] Reply LS on NR Femto node shared by PLMN and PNI-NPN

Response to:	R3-251524/S2-2502787

Release:	Rel-19

Work Item:	5G_Femto, NR_WAB_5GFemto-Core, eNPN

Source:	RAN3

To:	SA2

Cc:	RAN2

Contact Person:

Name:	Seokjung Kim

E-mail Address:	seokjung.kim@lge.com

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

Attachments:	R3-252102

1	Overall description

RAN3 thanks SA2 for the LS on NR Femto node shared by PLMN and PNI-NPN. RAN3 has discussed this issue and concluded that when the NR Femto activates a cell shared by both PLMN and CAG, the PLMN cell and CAG cell should have different Cell Identities as described in TS 23.501 clause 5.18.1 and TS 38.300 clause 4.6. Therefore, it is agreed in R3-252102 to capture the following description for different Cell Identities in RAN3 BL CR to TS 38.300.

### 4.x.4 Access Control Cells served by an NR Femto node may be deployed as part of a PNI-NPN (see clause 4.8) in order to restrict access to UEs according to the respective subscription. NOTE: The NR Femto node may use the CAG mechanism for PNI-NPN (see clause 16.7.4) as follows: - The NR Femto node may activate a PLMN cell, which can be accessed by legacy UE without access control of CAG. - The NR Femto may activate a cell shared by both PLMN and PNI-NPN, through broadcasting both the plmn-IdentityInfoList and the npn-IdentityInfoList-r16 in the SIB1, but without the cellReservedForOtherUse. In this case, the plmn-IdentityInfoList IE and the npn-IdentityInfoList-r16 IE shall contain different Cell Identities as described in clause 4.6. Then, this cell is accessible to UEs which have the allowed CAG list including a CAG-ID broadcasted by the cell. For the legacy UE not supporting CAG, this cell is viewed as a normal PLMN cell. - The NR Femto node may activate an NPN-only cell by broadcasting the cellReservedForOtherUse IE with value “true”, then this cell can only be accessed by the UEs whose allowed CAG list includes a CAG-ID broadcasted by the cell.

2. Actions:

To SA2:

ACTION: 	RAN3 kindly asks SA2 to take the above into account.

3. Date of Next RAN3 Meetings:

3GPP TSG RAN3#128		19 – 23 May 2025		St Julian’s, Malta

3GPP TSG RAN3#129		25 – 29 August 2025		Bangalore, India