3GPP TSG-RAN WG3 #127bis	                                        R3-252105

Wuhan, China, 7th – 11th April 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.413 | CR | 1272 | rev | - | Current version: | 18.5.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Support for Broadcast Service Area Signaling in NGAP |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Jio Platforms Ltd (JPL) |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R3 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_NTN_Ph3-Core |  |  |  |  |  | Date: |  |  | 2025-3-27 |
| Category: | C |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | In the current version of the NGAP specification (TS 38.413), there is no explicit support to signal a broadcast service area from the 5G Core (5GC) to the NG-RAN for 5G multicast-broadcast services (MBS) in broadcast mode (e.g. for NR Non-Terrestrial Networks, NTN). According to the Stage-2 architecture in TS 23.247 (Rel-17) for 5G MBS, a Broadcast MBS session is characterized not only by its content but also by the geographical area where the content is to be distributed​. TS 23.247 further specifies that the 5GC (via AMF/MB-SMF) should provide the intended broadcast service area to NG-RAN as part of the N2 signaling (NGAP) when initiating a broadcast session​. This ensures the NG-RAN is aware of the exact area within which the broadcast data should be transmitted​. For NTN scenarios, a gNB (e.g. satellite-based NG-RAN) can cover extremely large regions, potentially spanning multiple countries or regions. It is often necessary to restrict or define the service area for a broadcast (e.g. due to content licensing or regulatory reasons) so that broadcast content is delivered only within a specific intended area. Without a mechanism to signal the intended area to the RAN, the gNB might either broadcast everywhere (inefficient and possibly unauthorized) or have no guidance on where the broadcast should be confined. This gap can lead to inefficient use of radio resources and misalignment with the core network’s expectations of service delivery area in NTN use cases. In Rel-17/18, NGAP introduced basic support for MBS (Multicast/Broadcast) sessions, including procedures to start and stop broadcast sessions, but it lacks an IE to convey a geographic service area. A placeholder concept of an “MBS Service Area” exists (intended for location-dependent MBS, typically as a list of cells or area identifiers), but this does not cover arbitrary geographic shapes needed for large-area broadcast in NTN. Thus, there is a need to add a new information element to NGAP to explicitly carry the coordinates of the broadcast service area as defined at the 5GC side. This change will align NGAP with Stage-2 requirements in TS 23.247 and ongoing RAN3 discussions on NR NTN broadcast support, completing the end-to-end signaling needed for broadcast MBS sessions in NTN. |  |  |  |  |  |  |  |  |
| Summary of change: |  | This CR introduces a new Information Element (IE) called BroadcastServiceAreaCoordinates to the NGAP protocol. This IE is an OCTET STRING encoding of the intended broadcast service area’s geographic coordinates/shape, using the established location encoding format from TS 23.032 (Universal Geographical Area Description, GAD). Supported shapes (e.g., polygon, circular area, etc.) will follow TS 23.032 conventions, where the first octet indicates the shape type and subsequent octets carry the shape parameters​. This IE allows the 5GC to inform the NG-RAN of the exact geographic area in which the broadcast content should be delivered. The new IE will be included in relevant NGAP procedures for MBS broadcast session initiation (and updates), specifically in the MBS Session Setup (or Session Start) request message sent from AMF (on behalf of MB-SMF) to NG-RAN. The presence of this IE will be conditional – it shall be included when the MBS session is a broadcast session (as opposed to a multicast session). NGAP message definitions and ASN.1 will be updated accordingly: A new IE definition for BroadcastServiceAreaCoordinates (OCTET STRING) will be added in the NGAP IE specifications (with a new IE identifier to be assigned). The MBS Session Setup Request (and any similar message used to initiate a broadcast MBS session, e.g. an MBS Session Start Request) will be modified to include this IE. The IE’s criticality is proposed as “ignore” (since the information is additional and its absence or unrecognized presence in older RAN should not cause session setup failure), and it is optional, present only for broadcast sessions. If there is a procedure for updating an ongoing broadcast session (e.g. to change the area), the corresponding NGAP message (MBS Session Update) will also be able to carry this IE, allowing dynamic adjustment of the broadcast area if needed. ASN.1 definitions in TS 38.413 will be updated to reflect the above: defining the new OCTET STRING type and adding it to the MBSSessionSetupRequest message structure (and any relevant sequence types). The changes maintain backward compatibility by using the ASN.1 extension mechanism for new IEs. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Without this change, NG-RAN nodes will not receive any explicit indication of the intended broadcast area for MBS sessions. In NTN deployments or any scenario where broadcast coverage needs to be controlled, the gNB would have to either broadcast indiscriminately to its entire coverage or rely on proprietary configuration, leading to inefficiencies and potential regulatory breaches. This misalignment between 5GC and NG-RAN could result in unnecessary load (broadcasting where not needed) or coverage gaps (if the RAN guesses incorrectly or is manually configured with static areas). Moreover, not approving this change leaves NGAP non-compliant with the Stage-2 design for broadcast MBS​, hindering the completion of the NR NTN Phase 3 work. Approving this CR is essential to ensure full support of 5G broadcast services, especially for NTN, in a standardized and interoperable manner. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 8.17.1.2, 9.2.16.1, 9.4.5 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | - |  |  |  |  |  |  |  |  |

| This CR's revision history: |  |
| --- | --- |

-------------------------------------------Start of changes-------------------------------------------

## 8.17	Broadcast Session Management Procedures

### 8.17.1	Broadcast Session Setup

#### 8.17.1.1	General

The purpose of the Broadcast Session Setup procedure is to request the NG-RAN node to setup MBS session resources for a broadcast MBS session. The procedure uses non-UE associated signalling.

#### 8.17.1.2	Successful Operation

Figure 8.17.1.2-1: Broadcast Session Setup, successful operation.

The AMF initiates the procedure by sending a BROADCAST SESSION SETUP REQUEST message to the NG-RAN node. If the NG-RAN node accepts all the MBS QoS flows in the MBS session at least in one of its cells, the NG-RAN node responds with the BROADCAST SESSION SETUP RESPONSE message.

If the MBS Service Area IE is included in the BROADCAST SESSION SETUP REQUEST message, the NG-RAN node shall take it into account as specified in TS 23.247 [44].

If the MBS Session FSA ID List IE is included in the BROADCAST SESSION SETUP REQUEST message, the NG-RAN node shall take it into account to determine cells/frequencies within the MBS service area to broadcast MBS session data as specified in TS 23.247 [44].

If the Broadcast Service Area Coordinates IE is included in the BROADCAST SESSION SETUP REQUEST message, the NG-RAN node shall take it into account to determine the geographical boundaries of the MBS broadcast service area, as specified in TS 23.247 [44].

Editorial Note: By inserting this after the FSA ID List IE description, we maintain a coherent sequence: the AMF may provide an MBS Service Area (overall area context), optionally a list of frequency-specific area IDs within that service area, and now optionally the geographic coordinates of the service area. The NG-RAN node will use all provided area parameters in concert when setting up the broadcast session. This ordering is consistent with how the spec already lists area-related IEs in the Broadcast Session Setup procedure

If the Supported UE Type List IE is included in the BROADCAST SESSION SETUP REQUEST message, the NG-RAN node shall, if supported, take it into account for configuring MBS session resources.

If the Associated Session ID IE is included in the BROADCAST SESSION SETUP REQUEST message the NG-RAN node shall, if supported, take this information into account to determine whether MBS session resource sharing is possible, as specified in TS 38.300 [8].

-------------------------------------------Next changes-------------------------------------------

### 9.2.16		Broadcast Session Management Messages

#### 9.2.16.1	BROADCAST SESSION SETUP REQUEST

This message is sent by the AMF to establish MBS session resources for a broadcast MBS session.

Direction: AMF  NG-RAN node

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.3.1.1 |  | YES | reject |
| MBS Session ID | M |  | 9.3.1.206 |  | YES | reject |
| S-NSSAI | M |  | 9.3.1.24 |  | YES | reject |
| MBS Service Area | M |  | 9.3.1.208 |  | YES | reject |
| Broadcast Service Area Coordinates | O |  | OCTET STRING | Contains the geographic coordinates for the broadcast | ignore | ignore |
| MBS Session Setup Request Transfer | M |  | OCTET STRING | Containing the MBS Session Setup or Modification Request Transfer IE specified in subclause 9.3.5.3 | YES | reject |
| Associated Session ID | O |  | 9.3.1.262 |  | YES | ignore |

-------------------------------------------Next changes-------------------------------------------

### 9.4.5	Information Element Definitions

-- ASN1START

-- **************************************************************

--

-- Information Element Definitions

--

-- **************************************************************

NGAP-IEs {

itu-t (0) identified-organization (4) etsi (0) mobileDomain (0)

ngran-Access (22) modules (3) ngap (1) version1 (1) ngap-IEs (2) }

DEFINITIONS AUTOMATIC TAGS ::=

BEGIN

IMPORTS

id-AdditionalDLForwardingUPTNLInformation,

id-AdditionalULForwardingUPTNLInformation,

id-AdditionalDLQosFlowPerTNLInformation,

*****Same as befor***

id-MBS-AreaSessionID,

id-MBS-QoSFlowsToBeSetupList,

id-MBS-QoSFlowsToBeSetupModList,

id-MBS-QoSFlowToReleaseList,

id-MBS-ServiceArea,

id-BroadcastServiceAreaCoordinates

id-MBS-SessionFSAIDList,

id-MBS-SessionID,

id-MBS-ActiveSessionInformation-SourcetoTargetList,

id-MBS-ActiveSessionInformation-TargettoSourceList,

id-MBS-AssistanceInformation,

id-MBS-SessionTNLInfo5GC,

id-MBS-SupportIndicator,

id-MBSSessionFailedtoSetupList,

id-MBSSessionFailedtoSetuporModifyList,

id-MBSSessionSetupResponseList,

id-MBSSessionSetuporModifyResponseList,

id-MBSSessionToReleaseList,

id-MBSSessionSetupRequestList,

id-MBSSessionSetuporModifyRequestList,

***Same as befor***

FROM NGAP-Constants

Criticality,

ProcedureCode,

ProtocolIE-ID,

TriggeringMessage

FROM NGAP-CommonDataTypes

ProtocolExtensionContainer{},

ProtocolIE-Container{},

NGAP-PROTOCOL-EXTENSION,

ProtocolIE-SingleContainer{},

NGAP-PROTOCOL-IES

FROM NGAP-Containers;

MBS-ServiceArea ::= CHOICE {

locationindependent		MBS-ServiceAreaInformation,

locationdependent		MBS-ServiceAreaInformationList,

choice-Extensions		ProtocolIE-SingleContainer { {MBS-ServiceArea-ExtIEs} }

}

MBS-ServiceArea-ExtIEs NGAP-PROTOCOL-IES ::= {

...

}

MBS-ServiceAreaInformationList ::= SEQUENCE (SIZE(1..maxnoofMBSServiceAreaInformation)) OF MBS-ServiceAreaInformationItem

MBS-ServiceAreaInformationItem ::= SEQUENCE {

mBS-AreaSessionID				MBS-AreaSessionID,

mBS-ServiceAreaInformation		MBS-ServiceAreaInformation,

iE-Extensions					ProtocolExtensionContainer { {MBS-ServiceAreaInformationItem-ExtIEs} }	OPTIONAL,

...

}

MBS-ServiceAreaInformationItem-ExtIEs NGAP-PROTOCOL-EXTENSION ::= {

...

}

MBS-ServiceAreaInformation ::= SEQUENCE {

mBS-ServiceAreaCellList		MBS-ServiceAreaCellList																OPTIONAL,

mBS-ServiceAreaTAIList		MBS-ServiceAreaTAIList																OPTIONAL,

iE-Extensions					ProtocolExtensionContainer { {MBS-ServiceAreaInformation-ExtIEs} }	OPTIONAL,

...

}

MBS-ServiceAreaInformation-ExtIEs NGAP-PROTOCOL-EXTENSION ::= {

...

}

MBS-ServiceAreaCellList ::= SEQUENCE (SIZE(1.. maxnoofCellsforMBS)) OF NR-CGI

MBS-ServiceAreaTAIList ::= SEQUENCE (SIZE(1.. maxnoofTAIforMBS)) OF TAI

MBS-SessionID ::= SEQUENCE {

tMGI								TMGI,

nID								NID																			OPTIONAL,

iE-Extensions					ProtocolExtensionContainer { {MBS-SessionID-ExtIEs} } 		OPTIONAL,

...

}

MBS-SessionID-ExtIEs NGAP-PROTOCOL-EXTENSION ::= {

...

}

MBSSessionFailedtoSetupList ::= SEQUENCE (SIZE(1.. maxnoofMBSSessions)) OF MBSSessionFailedtoSetupItem

MBSSessionFailedtoSetupItem ::= SEQUENCE {

mBS-SessionID			MBS-SessionID,

mBS-AreaSessionID		MBS-AreaSessionID															OPTIONAL,

cause					Cause,

iE-Extensions			ProtocolExtensionContainer { { MBSSessionFailedtoSetupItem-ExtIEs} }		OPTIONAL,

...

}

MBSSessionFailedtoSetupItem-ExtIEs NGAP-PROTOCOL-EXTENSION ::= {

...

}

MBS-ActiveSessionInformation-SourcetoTargetList ::= SEQUENCE (SIZE(1..maxnoofMBSSessionsofUE)) OF MBS-ActiveSessionInformation-SourcetoTargetItem

MBS-ActiveSessionInformation-SourcetoTargetItem ::= SEQUENCE {

mBS-SessionID 								MBS-SessionID,

mBS-AreaSessionID							MBS-AreaSessionID							OPTIONAL,

mBS-ServiceArea								MBS-ServiceArea								OPTIONAL,

mBS-QoSFlowsToBeSetupList 					MBS-QoSFlowsToBeSetupList,

mBS-MappingandDataForwardingRequestList 	MBS-MappingandDataForwardingRequestList		OPTIONAL,

iE-Extensions		ProtocolExtensionContainer { { MBS-ActiveSessionInformation-SourcetoTargetItem-ExtIEs} }	OPTIONAL,

...

}

BroadcastServiceAreaCoordinates-IE NGAP-PROTOCOL-IES ::= {

{ ID id-BroadcastServiceAreaCoordinates

CRITICALITY ignore

TYPE BroadcastServiceAreaCoordinates

PRESENCE optional

},

-------------------------------------------End of changes-------------------------------------------