3GPP TSG-RAN WG3 Meeting #127bis	R3-252118

Wuhan, China, 7 - 11 April, 2025

Title:	(TP for LTM BLCR for TS38.473, TS38.300, TS38.470): Further discussion on inter-CU LTM procedure

Agenda Item:	13.2

Source:	Huawei

Document for:	Other

1	Introduction

In this contribution, we continue to discuss the basic procedures of inter-CU LTM and focus on the standalone case.

2	Discussion

2.1	Security key update

In subsequent inter-CU LTM, how to update the security key is one issue to be considered when UE switches among candidate cells. After several meetings discussion in both RAN2 and SA3, SA3 replied a LS in [1] to inform RAN2 and RAN3 that the NCC is transmitted via MAC CE. The details on key distribution between the source gNB and the target gNB are left to RAN3 to decide.

SA3 thanks RAN2 for the LS on the security handling for inter-CU LTM in non-DC cases. Considering the RAN2’s request on avoiding RRC configuration between cell switches for subsequent inter-CU LTM in solution selection, SA3 reached a compromise that the NCC can be transmitted to the UE in plaintext via MAC CE message, even though the unprotected MAC leads to increased attack scenarios and probability of handover failure. In addition, SA3 will not consider the protection of MAC CE messages in this release, but may consider studying MAC CE protection in the future. The details on key distribution between the source gNB and the target gNB are left to RAN3 to decide and inform SA3. RAN3 should consider the scenario where the serving gNB utilizes the unused NH (if any is received from a path switch procedure at execution phase) to derive the KNG-RAN* for LTM handover.

In the existing Xn handover procedure, the key used at the target gNB is derived by the source gNB. In inter-CU LTM, this principle should be re-used, i.e., the source gNB delivers the new key (KNG-RAN Node*) for the target gNB. The question to be discussed is when the source gNB sends the new key. There are two options:

- Option 1: the source gNB sends the new key only to the target gNB when LTM is triggered, i.e., the source gNB can send the new key in CELL SWITCH NOTIFICATION message

- Option 2: the source gNB sends the new key to multiple candidate gNBs during LTM preparation or before next LTM execution. I.e., the source gNB sends the new key to the candidate gNBs in Handover Request message as legacy for the initial LTM preparation, and the new serving gNB sends the new key to the candidate gNBs in LTM configuration update procedure before next LTM execution.

IThe drawback of option 1 is if the Xn delay is longer than the Uu handover execution time, the target gNB cannot decrypt the RRCReconfigurationComplete message from the UE before receiving the new key.

Therefore, we think that it is better to utilize Option 2 and allow the flexibility to send a new key to multiple candidate gNBs before LTM is triggered.

Proposal 1: The source gNB sends the new key to the candidate gNBs in Handover Request message as legacy for the initial LTM preparation.

Proposal 2: The new serving gNB sends the new key to the candidate gNBs in LTM configuration update procedure before next LTM execution.

In the existing Xn handover  procedure, the new serving gNB obtains a new NCC from the AMF during path switch procedure. The procedure of NCC delivery between the AMF and the gNB can be re-used for inter-CU LTM. Then the serving gNB sends the obtained NCC to the UE via MAC CE for next inter-CU LTM cell switch. Since it is the gNB-CU that receives a NCC from the AMF, and it is the serving gNB-DU that sends the MAC CE to the UE, Therefore, in CU-DU split case, a new class 2 F1AP procedure is needed from the gNB-CU to the gNB-DU to transfer the new NCC to the UE.

Proposal 3a: After path switch, the new serving gNB-CU sends the new NCC to the serving gNB-DU via a new class 2 F1AP message.

An example flow chart of the security key update procedure is shown in the figure below, and some of the existing LTM procedures are omitted in the flow chart. In this example, we assume a new MAC CE is introduced for NCC delivery to the UE.

Figure 1. An example of key change for subsequent inter-CU LTM

In addition, how to update the mismatched UE security context is another issue to be considered. In the LTM preparation phase, the source gNB-CU sends the UE security context (e.g. UE’s 5G security capabilities and/or UE’s UP security policy) to all candidate gNBs. SA3 already agrees that serving gNB-CU updates the UE security context to all candidate gNBs if UE security context mismatch is identified in LTM execution phase. The agreement in SA3 can be found in S3-250765. The details on security context update from serving gNB-CU to all candidate gNBs are left to RAN3 to decide.

For the case where CU is acting as MN and DC is not configured, it is agreed that the normative work is based on the following principles: - For the parameters in AS security context, which are not changing per inter-CU LTM handover (including UE’s 5G security capabilities, selected AS security algorithms, UE’s UP security policy, UE’s UP security activation status): - If the gNB receives mismatched UE security context (e.g. UE’s 5G security capabilities and/or UE’s UP security policy) in the Path-Switch Acknowledge message after LTM switch, - it initiates an intra-cell handover procedure which includes RRC Connection Reconfiguration procedure indicating the change of selected algorithms (as defined in clause 6.7.3.1) and/or UE’s UP activation status (as defined in clause 6.6.1) to the UE. - it updates the UE’s 5G security capabilities and/or UE’s UP security policy for the active PDU session to all candidate gNBs

In Rel-19, LTM CONFIGURATION UPDATE and LTM CONFIGURATION UPDATE ACKNOWLEDGE messages are introduced to transfer UE configurations, set up UE associations, etc. This LTM configuration update procedure can be re-used for the new serving gNB to deliver mismatched UE security context (e.g. UE’s 5G security capabilities and/or UE’s UP security policy) to other candidate gNBs in subsequent LTM.

Proposal 3b: To reuse LTM configuration update procedure to deliver mismatched UE security context (e.g. UE’s 5G security capabilities and/or UE’s UP security policy) from the new serving gNB to the other candidate gNBs in subsequent LTM.

2.2	Semi-persistent CSI-RS measurement

In previous RAN3 meeting, RAN3 had the following agreements to support semi-persistent (SP) CSI-RS measurement.

RAN3 #127 For both inter-CU and intra-CU cases: For the network Semi-Persistent CSI-RS coordination, source gNB-DU/source gNB triggers the activation/deactivation of the CSI-RS transmission in the candidate cell. For the activation/deactivation procedure, a class 1 procedure is needed from the source gNB-DU/source gNB. FFS for reusing existing one or a new one.

It is FFS on whether to re-use the existing signalling or introduce new signalling to achieve the SP CSI-RS activation/deactivation. The existing signalling in F1, e.g., UE CONTEXT MODIFICATION REQUIRED message triggered by the gNB-DU is used to update some UE context/configuration, which seems not suitable for activation of SP CSI-RS transmission at the candidate cell. New F1AP signalling should be introduced in F1AP. In Xn, the current LTM CONFIGURATION UPDATE message can be re-used for the SP CSI-RS activation/deactivation transfer.

According to the agreements, it is the source gNB-DU that determines to trigger SP CSI-RS transmission for the candidate gNB/gNB-DU, and determines to activate SP CSI-RS measurement for the UE. Therefore, in the new F1/Xn message, the SP CSI-RS resource set ID for each candidate cell should be included.

Proposal 4a: Introduce new F1 class 1 signalling to support SP CSI-RS activation/deactivation transfer. The CSI-RS resource set ID selected by the source gNB-DU for each candidate cell is included in the new signalling.

Proposal 4b: In XnAP, reuse the LTM configuration update procedure to support SP CSI-RS activation/deactivation transfer.

As for how the source gNB-DU can select a suitable SP CSI-RS resource set ID, the straightforward method is to let the source gNB-DU know the SSB and CSI-RS association of the candidate cell, and then it determines the suitable CSI-RS resources by the received SSB measurement results. The source gNB-DU can obtain the SSB and CSI-RS association in the LTM preparation phase.

The reason is that in current RRC spec, the TCI-StateId which can be used to deduce the related SSB in the NZP-CSI-RS Resource IE will be present only for periodic CSI-RS.

Proposal 5: In LTM preparation phase, the source gNB-CU obtains the candidate cell SP CSI-RS and SSB mapping in HANDOVER REQUEST procedure, then the source gNB-CU sends the mapping information to the source gNB-DU via UE Context Modification procedure.

RAN3 agreed that the source gNB is able to request the candidate gNB to feedback the CSI-RS resource configuration during LTM preparation. This will impact F1AP, as usually, the CSI-RS resource configuration is generated by the gNB-DU.

Proposal 6: To update UE Context Setup and UE Context Modification procedures to enable the gNB-CU request CSI-RS resource from the gNB-DU.

2.3	Reference configuration

RAN3 had the following agreements for the reference configuration generation:

RAN3 #127 The source gNB can generate reference configuration and provide a reference configuration for LTM in a Handover Request and LTM configuration update message. The candidate gNB indicates whether a LTM candidate configuration is a complete candidate configuration in the Handover Request Acknowledge and LTM configuration update acknowledge message. FFS on whether the source gNB can request a candidate gNB to provide a reference configuration.

It has been agreed that the source gNB can generate reference configuration by itself. Some companies proposed to also enable the candidate gNB provide reference configuration in Handover Request Acknowledge as per the source gNB request. This could be beneficial for RRC signalling overhead reduction when this candidate gNB are configured with many candidate cells.

Proposal 7: The source gNB may request a candidate gNB to provide a reference configuration in either Handover Request or LTM Configuration Update.

2.4	Early UL synchronization

RAN3 had the following WA in RAN3 #127 meeting:

RAN3 #127 WA: Introduce a new non-UE associated class 2 procedure on Xn, namely TA information Transfer message, to transfer the TA information from the candidate gNB-CU to the source gNB-CU.

According to the RAN2 progress on PDCCH ordered early RACH, there is no intention in RAN2 to further enhance the Rel-18 mechanism. In this case, when a candidate cell receives a preamble for early TA acquisition, it does not know the UE identity because the early RACH resources are shared by multiple UEs in the source cell. Therefore, the TA information transfer procedure should be a new non-UE associated class 2 procedure also on Xn interface.

The WA can be turned to agreement.

Proposal 8: Turn the WA to agreement: Introduce a new non-UE associated class 2 procedure on Xn, namely TA information Transfer message, to transfer the TA information from the candidate gNB-CU to the source gNB-CU.

The parameters (e.g. candidate Cell ID, TA value, Preamble index, and RA-RNTI) included in F1AP TA Information Transfer messages can be reused by the new Xn procedure for inter-CU LTM.

Proposal 9: The parameters (e.g. candidate Cell ID, TA value, Preamble index, and RA-RNTI) in F1AP TA Information Transfer messages are reused by the Xn TA Information Transfer message for inter-CU LTM.

2.5	UE association setup

The following agreements were made at previous RAN3 meetings.

RAN3 #125bis Allow UE association in-between candidate CUs (in case the Xn connectivity existed) for subsequent LTM, when and how to establish the UE association is FFS. RAN3 #126 To support subsequent LTM, the LTM Configuration Update procedure is reused to establish UE association between the new source gNB and the other candidate gNB(s) after each inter-CU LTM Cell Switch.

After each cell switch, the new serving gNB signals the new allocated source gNB UE XnAP ID(s) to other candidate gNBs in the LTM Configuration Update message. Then the candidate gNB replies with the target gNB UE XnAP ID in LTM Configuration Update Acknowledgement message.

One issue that RAN3 needs to discuss is how does the candidate gNB identify the UE context. The candidate gNB needs to know some kinds of UE identity to associate the new allocated UE XnAP ID with the UE context. To solve this issue, in the LTM Configuration Update message, the UE XnAP ID that the candidate gNB allocated for the first source gNB should be included in the LTM Configuration Update message.

Proposal 10: The new serving gNB includes the UE XnAP ID allocated by the candidate gNB for the initial source gNB in the LTM Configuration Update message.

2.6	Early data forwarding data discard

RAN3 agreed to support early data forwarding for subsequent inter-CU LTM. There is one proposal to discuss how to deal with the data forwarded by early data forwarding procedure.

In legacy handover, the data forwarded to candidate gNB in early data forwarding will be released after handover procedure is completed, e.g. by reception of HANDCOVER CANCEL message. But for subsequent LTM, the data forwarded to the candidate gNB except target gNB in early data forwarding will be useless after the precedent LTM procedure is completed. And in the subsequent LTM procedure, the new source gNB may perform early data forwarding to the candidate too. For the candidate gNB, it is unclear whether and when it should flush the data received in the precedent LTM procedure. If the data received in precedent LTM procedure is not flushed, they will bring additional buffer requirement to the candidate gNB. Proposal 5: RAN3 to discuss whether and when a candidate gNB flush UE data that is received in early data forwarding after a LTM procedure.

The candidate gNB should discard the data after it determines that the UE is switched to another candidate gNB. One straightforward method is to use the LTM CONFIGURATION UPDATE message to let the candidate gNB know that the serving gNB is changed. After receiving the LTM CONFIGURATION UPDATE message, the candidate gNB determines that the UE has switched to another gNB, and it may discard the buffered data received from the original source gNB.

Proposal 11: After receiving the LTM CONFIGURATION UPDATE message, the candidate gNB discards the buffered data received from the original source gNB.

2.7 Updates to XnAP BLCR

The following highlight procedural text is misleading and the referred LTM-CandidateConfig IE is not a XnAP defined IE, therefore, should be removed.

If the CSI Resource Configuration IE is contained in the LTM Information Request IE included in the HANDOVER REQUEST message, the target NG-RAN node shall, if supported, use it to generate the LTM CSI reporting configuration included in the LTM-CandidateConfig IE for the serving cell.

Secondly, there is no request for CSI Resource Configuration function agreed, but request for CSI-RS Resource Configuration. And the reference in the tabular is incorrect. The SSB information is mandate to be included in the Handover Request Ack message to align with F1AP. It currently misplaced.

Therefore, the following procedural text and tabular should be updated.

If the Request for CSI Resource Configuration IE is contained in the LTM Information Request IE included in the HANDOVER REQUEST message, the target NG-RAN node shall, if supported, include the CSI Resource Configuration IE in the LTM Information Response IE in the HANDOVER REQUEST ACKNOWLEDGE message. 9.2.1.xx2 LTM Information Response This IE contains LTM related information to be provided in response to information provided in the LTM Information Request IE. Editor’s note: Details on configurations need to be continued. IE/Group Name; Presence; Range; IE type and reference; Semantics description. CSI Resource Configuration; O; 9.2.1.xx8. LTM Candidate Configuration; M; OCTET STRING; Contains ltm-CandidateConfig as defined in TS 38.331 [10].. Reference Configuration (FFS); O; OCTET STRING; Contains the ltm-ReferenceConfiguration as defined in TS 38.331 [10].. Complete Candidate Configuration Indicator; O; ENUMERATED (complete, ...). 9.2.1.xx8 CSI Resource Configuration This IE contains the CSI resource configuration used for LTM. IE/Group Name; Presence; Range; IE type and reference; Semantics description. CSI Resource Configuration To AddModList; O; OCTET STRING; Contains the ltm-CSI-ResourceConfigToAddModList as defined in TS 38.331 [10].. CSI Resource Configuration To Release List; O; OCTET STRING; Includes the ltm-CSI-ResourceConfigToReleaseList contained in the LTM-Config IE as defined in TS 38.331 [10].. SSB Information; O; 9.2.1.x10.

Thirdly, the LTM indicator in Handover Request message is missing.

Proposal 12: To capture the above changes proposed above into XnAP BLCR.

3	Conclusion

This document proposes:

Security key update:

Proposal 1: The source gNB sends the new key to the candidate gNBs in Handover Request message as legacy for the initial LTM preparation.

Proposal 2: The new serving gNB sends the new key to the candidate gNBs in LTM configuration update procedure before next LTM execution.

Proposal 3a: After path switch completion, the serving gNB-CU sends the new NCC to the serving gNB-DU via a new class 2 F1 message.

Proposal 3b: To reuse LTM configuration update procedure to deliver mismatched UE security context (e.g. UE’s 5G security capabilities and/or UE’s UP security policy) from the new serving gNB to the other candidate gNBs in subsequent LTM.

Semi-persistent CSI-RS measurement:

Proposal 4a: Introduce new F1 class 1 signalling to support SP CSI-RS activation/deactivation transfer. The CSI-RS resource set ID selected by the source gNB-DU for each candidate cell is included in the new signalling.

Proposal 4b: In XnAP, reuse the LTM configuration update procedure to support SP CSI-RS activation/deactivation transfer.

Proposal 5: In LTM preparation phase, the source gNB-CU obtains the candidate cell SP CSI-RS and SSB mapping in HANDOVER REQUEST procedure, then the source gNB-CU sends the mapping information to the source gNB-DU via UE Context Modification procedure.

Proposal 6: To update UE Context Setup and UE Context Modification procedures to enable the gNB-CU request CSI-RS resource from the gNB-DU.

Reference configuration:

Proposal 7: The source gNB may request a candidate gNB to provide a reference configuration in either Handover Request or LTM Configuration Update.

Early UL synchronization:

Proposal 8: Turn the WA to agreement: Introduce a new non-UE associated class 2 procedure on Xn, namely TA information Transfer message, to transfer the TA information from the candidate gNB-CU to the source gNB-CU.

Proposal 9: The parameters (e.g. candidate Cell ID, TA value, Preamble index, and RA-RNTI) in F1AP TA Information Transfer messages are reused by the Xn TA Information Transfer message for inter-CU LTM.

UE association setup:

Proposal 10: The new serving gNB includes the UE XnAP ID allocated by the candidate gNB for the initial source gNB in the LTM Configuration Update message.

Early data forwarding data discard:

Proposal 11: After receiving the LTM CONFIGURATION UPDATE message, the candidate gNB discards the buffered data received from the original source gNB.

Updates to XnAP BLCR:

Proposal 12: To capture the changes proposed in section 2.7 into XnAP BLCR.

The corresponding TPs are provided in the Annex.

4	Reference

- S3-251124, Reply LS on security handling for inter-CU LTM in non-DC cases, SA3.

Annex – Text Proposal for LTM BLCR for TS 38.473

/*********************Start of changes***********************/

8.3.x	DU-CU CSI-RS Activation

8.3.x.1	General

The purpose of the DU-CU CSI-RS Activation procedure is to enable the gNB-DU to request the gNB-CU to activate/deactivate the SP CSI-RS transmission in neighbour cells. The procedure uses UE-associated signalling.

8.3.x.2	Successful Operation

Figure 8.3.x.2-1: DU-CU CSI-RS Activation procedure. Successful operation.

The gNB-DU initiates the procedure by sending a DU-CU CSI-RS ACTIVATION REQUEST message.

8.3.x.3	Unsuccessful Operation

8.3.x.4	Abnormal Conditions

Not applicable.

8.3.y	CU-DU CSI-RS Activation

8.3.y.1	General

The purpose of the CU-DU CSI-RS Activation procedure is to enable the gNB-CU to to request the gNB-DU to activate/deactivate the SP CSI-RS transmission in some cells. The procedure uses UE-associated signalling.

8.3.y.2	Successful Operation

Figure 8.3.y.2-1: CU-DU CSI-RS ACTIVATION procedure. Successful operation.

The gNB-CU initiates the procedure by sending a CU-DU CSI-RS ACTIVATION message.

8.3.y.3	Unsuccessful Operation

.

8.3.y.4	Abnormal Conditions

Not applicable

8.3.z	Security Parameter Transfer

8.3.z.1	General

The purpose of the Security Parameter Transfer procedure is to enable the gNB-CU to transfer the security parameters to the gNB-DU for the UE. The procedure uses UE-associated signalling.

8.3.z.2	Successful Operation

Figure 8.3.z.2-1: Security Parameter Transfer procedure. Successful operation.

The gNB-CU initiates the procedure by sending a Security Parameter Transfer message.

8.3.10.3	Unsuccessful Operation

Not applicable.

8.3.10.4	Abnormal Conditions

Not applicable.

/*********************Next change***********************/

8.3.1.1	General

The purpose of the UE Context Setup procedure is to establish the UE Context including, among others, SRB, DRB, BH RLC channel, Uu Relay RLC channel, PC5 Relay RLC channel, and SL DRB configuration. The procedure uses UE-associated signalling.

8.3.1.2	Successful Operation

Figure 8.3.1.2-1: UE Context Setup Request procedure: Successful Operation

<skip unchanged part>

If the DL LBT Failure Information Request IE is included in the UE CONTEXT SETUP REQUEST message, the gNB-DU shall, if supported, consider that the gNB- CU requests collection of DL LBT failure information for the analysis of the MRO events of the UE specified in TS 38.300 [6], and act as specified in TS 38.401 [4].

If the Ranging and Sidelink Positioning Service Information IE is contained in the UE CONTEXT SETUP REQUEST message, the gNB-DU shall, if supported, take it into account for the UE’s Ranging and Sidelink Positioning service.

If the Request for CSI-RS Resource Configuration IE is contained in the UE CONTEXT SETUP REQUEST message, the gNB-DU shall, if supported, include the CSI-RS Resource Configuration IE in the UE CONTEXT SETUP RESPONSE message.

Interaction with UE Inactivity Notification procedure

If the SDT Volume Threshold IE is contained in the UE CONTEXT SETUP REQUEST message, the gNB-DU shall, if supported, use the information during an SDT transaction to inform the gNB-CU via the UE INACTIVITY NOTIFICATION message as specified in TS 38.401 [4].

/*********************Next change***********************/

8.3.4	UE Context Modification (gNB-CU initiated)

8.3.4.1	General

The purpose of the UE Context Modification procedure is to modify the established UE Context, e.g., establishing, modifying and releasing radio resources or sidelink resources. This procedure is also used to command the gNB-DU to stop data transmission for the UE for mobility (see TS 38.401 [4]). The procedure uses UE-associated signalling.

8.3.4.2	Successful Operation

Figure 8.3.4.2-1: UE Context Modification procedure. Successful operation

The UE CONTEXT MODIFICATION REQUEST message is initiated by the gNB-CU.

<skip unchanged part>

If the Ranging and Sidelink Positioning Service Information IE is contained in the UE CONTEXT MODIFICATION REQUEST message, the gNB-DU shall, if supported, update its service information for the UE accordingly. If the Ranging and Sidelink Positioning Authorized IE within the Ranging and Sidelink Positioning Service Information IE is set to "not authorized", the gNB-DU shall, if supported, initiate actions to ensure that the UE is no longer accessing the Ranging and Sidelink Positioning service.

If the Request for CSI-RS Resource Configuration IE is contained in the UE CONTEXT MODIFICATION message, the gNB-DU shall, if supported, include the CSI-RS Resource Configuration IE in the UE CONTEXT MODIFICATION RESPONSE message.

Interaction with UE Inactivity Notification procedure

If the SDT Volume Threshold IE is contained in the UE CONTEXT MODIFICATION REQUEST message, the gNB-DU shall, if supported, use the information during an SDT transaction to inform the gNB-CU via the UE INACTIVITY NOTIFICATION message as specified in TS 38.401 [4].

Interaction with UE Context Setup or UE Context Modification (gNB-CU initiated) procedures

If the UE CONTEXT MODIFICATION REQUEST message is sent for a UE context set up for S-CPAC and contains the Transmission Action Indicator IE set to "stop", the gNB-DU shall, if supported, reset the UE context for the included SpCell ID IE, prepare for subsequent CPAC. The gNB-DU shall include the SpCell ID IE as the Requested Target Cell ID IE in the UE CONTEXT MODIFICATION RESPONSE message.

/*********************Next change***********************/

9.2.2.1	UE CONTEXT SETUP REQUEST

This message is sent by the gNB-CU to request the setup of a UE context.

Direction: gNB-CU  gNB-DU.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.3.1.1 |  | YES | reject |
| gNB-CU UE F1AP ID | M |  | 9.3.1.4 |  | YES | reject |
| gNB-DU UE F1AP ID | O |  | 9.3.1.5 |  | YES | ignore |
| SpCell ID | M |  | NR CGI 9.3.1.12 | Special Cell as defined in TS 38.321 [16]. For handover case, this IE is considered as target cell. | YES | reject |
| ServCellIndex | M |  | INTEGER (0..31,...) |  | YES | reject |
| <skip unchanged part> |  |  |  |  |  |  |
| LTM InformationSetup |  | 0..1 |  |  | YES | reject |
| >LTM Indicator | M |  | ENUMERATED (true, …) |  | - |  |
| >LTM Configuration ID | M |  | INTEGER (1..8) | Corresponds to the LTM-CandidateId IE, as defined in TS 38.331 [8]. | - |  |
| >Reference Configuration | O |  | 9.3.1.292 |  | - |  |
| >CSI Resource Configuration | O |  | 9.3.1.330 |  | - |  |
| Request for CSI-RS Resource Configuration | O |  | ENUMERATED (true, …) |  | YES | ignore |
| LTM Configuration ID Ma pping List | O |  | 9.3.1.294 |  | YES | reject |
| Early Sync Information Request |  | 0..1 |  |  | YES | ignore |
| >Request for RACH Configuration | O |  | ENUMERATED (true, …) |  | - |  |
| >LTM gNB-DUs List |  | 0..1 |  | This IE contains the IDs of the source gNB-DU and candidate gNB-DU(s). | YES | reject |
| >>LTM gNB-DUs Item IEs |  | 1..< maxnoofLTMgNBDUs> |  |  |  |  |
| >>>LTM gNB-DU ID | M |  | gNB-DU ID 9.3.1.9 |  |  |  |
| >>>LTM gNB ID | O |  | Global gNB ID 9.3.1.305 |  | - |  |
| Path Addition Information | O |  | 9.3.1.296 | This IE contains either the Indirect Path Addition IE or the N3C Indirect Path Addition IE. | YES | reject |
| NR A2X Services Authorized | O |  | 9.3.1.323 |  | YES | ignore |
| LTE A2X Services Authorized | O |  | 9.3.1.324 |  | YES | ignore |
| NR UE Sidelink Aggregate Maximum Bit Rate for A2X | O |  | NR UE Sidelink Aggregate Maximum Bit Rate 9.3.1.119 | This IE applies only if the UE is authorized for NR A2X services. | YES | ignore |
| LTE UE Sidelink Aggregate Maximum Bit Rate for A2X | O |  | LTE UE Sidelink Aggregate Maximum Bit Rate 9.3.1.118 | This IE applies only if the UE is authorized for LTE A2X services. | YES | ignore |
| DL LBT Failure Information Request | O |  | ENUMERATED (inquiry, …) |  | YES | ignore |
| Ranging and Sidelink Positioning Service Information | O |  | 9.3.1.331 | This IE applies only if the UE is authorized for NR V2X services and/or 5G ProSe services. | YES | ignore |

/*********************Next change***********************/

9.2.2.2	UE CONTEXT SETUP RESPONSE

This message is sent by the gNB-DU to confirm the setup of a UE context.

Direction: gNB-DU  gNB-CU.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.3.1.1 |  | YES | reject |
| gNB-CU UE F1AP ID | M |  | 9.3.1.4 |  | YES | reject |
| gNB-DU UE F1AP ID | M |  | 9.3.1.5 |  | YES | reject |
| DU To CU RRC Information | M |  | 9.3.1.26 |  | YES | reject |
| C-RNTI | O |  | 9.3.1.32 | C-RNTI allocated at the gNB-DU | YES | ignore |
| Resource Coordination Transfer Container | O |  | OCTET STRING | Includes the SgNB Resource Coordination Information IE as defined in subclause 9.2.117 of TS 36.423 [9] for EN-DC case or MR-DC Resource Coordination Information IE as defined in TS 38.423 [28] for NGEN-DC and NE-DC cases. | YES | ignore |
| Full Configuration | O |  | ENUMERATED (full, ...) |  | YES | reject |
| DRB Setup List |  | 0..1 |  | The List of DRBs which are successfully established. | YES | ignore |
| >DRB Setup Item Iist |  | 1 .. <maxnoofDRBs> |  |  | EACH | ignore |
| >>DRB ID | M |  | 9.3.1.8 |  | - |  |
| >>LCID | O |  | 9.3.1.35 | LCID for the primary path or for the split secondary path for fallback to split bearer if PDCP duplication is applied. | - |  |
| >>DL UP TNL Information to be setup List |  | 1 |  |  | - |  |
| >>>DL UP TNL Information to Be Setup Item IEs |  | 1 .. <maxnoofDLUPTNLInformation> |  |  | - |  |
| >>>>DL UP TNL Information | M |  | UP Transport Layer Information 9.3.2.1 | gNB-DU endpoint of the F1 transport bearer. For delivery of DL PDUs. | - |  |
| >>Additional PDCP Duplication TNL List |  | 0..1 |  |  | YES | ignore |
| >>>Additional PDCP Duplication TNL Items |  | 1 .. <maxnoofAdditionalPDCPDuplicationTNL> |  |  | EACH | ignore |
| >>>>Additional PDCP Duplication UP TNL Information | M |  | UP Transport Layer Information 9.3.2.1 | gNB-DU endpoint of the F1 transport bearer. For delivery of DL PDUs. | - |  |
| >>>>BH Information | O |  | 9.3.1.114 | This IE is not used in this version of the specification. | YES | ignore |
| >>Current QoS Parameters Set Index | O |  | Alternative QoS Parameters Set Index 9.3.1.123 | Index to the currently fulfilled alternative QoS parameters set. | YES | ignore |
| >>TSC Traffic Characteristics Feedback | O |  | 9.3.1.302 |  | YES | ignore |
| >>ECN Marking or Congestion Information Reporting Status | O |  | 9.3.1.322 |  | YES | ignore |
| SRB Failed to Setup List |  | 0..1 |  |  | YES | ignore |
| >SRB Failed to Setup Item |  | 1 .. <maxnoofSRBs> |  |  | EACH | ignore |
| >>SRB ID | M |  | 9.3.1.7 |  | - |  |
| >>Cause | O |  | 9.3.1.2 |  | - |  |
| DRB Failed to Setup List |  | 0..1 |  |  | YES | ignore |
| >DRB Failed to Setup Item |  | 1 .. <maxnoofDRBs> |  |  | EACH | ignore |
| >>DRB ID | M |  | 9.3.1.8 |  | - |  |
| >>Cause | O |  | 9.3.1.2 |  | - |  |
| SCell Failed To Setup List |  | 0..1 |  |  | YES | ignore |
| >SCell Failed to Setup Item |  | 1 .. <maxnoofSCells> |  |  | EACH | ignore |
| >>SCell ID | M |  | NR CGI 9.3.1.12 | SCell Identifier in gNB | - |  |
| >>Cause | O |  | 9.3.1.2 |  | - |  |
| Inactivity Monitoring Response | O |  | ENUMERATED (not-supported, ...) |  | YES | reject |
| Criticality Diagnostics | O |  | 9.3.1.3 |  | YES | ignore |
| SRB Setup List |  | 0..1 |  |  | YES | ignore |
| >SRB Setup Item |  | 1 .. <maxnoofSRBs> |  |  | EACH | ignore |
| >>SRB ID | M |  | 9.3.1.7 |  | - |  |
| >>LCID | M |  | 9.3.1.35 | LCID for the primary path if PDCP duplication is applied | - |  |
| BH RLC Channel Setup List |  | 0..1 |  | The list of BH RLC channels which are successfully established. | YES | ignore |
| >BH RLC Channel Setup Item |  | 1 .. <maxnoofBHRLCChannels> |  |  | EACH | ignore |
| >>BH RLC CH ID | M |  | BH RLC Channel ID 9.3.1.113 |  | - |  |
| BH RLC Channel Failed to be Setup List |  | 0..1 |  | The list of BH RLC channels whose setup has failed. | YES | ignore |
| >BH RLC Channel Failed to be Setup Item |  | 1 .. <maxnoofBHRLCChannels> |  |  | EACH | ignore |
| >>BH RLC CH ID | M |  | BH RLC Channel ID 9.3.1.113 |  | - |  |
| >>Cause | O |  | 9.3.1.2 |  | - |  |
| SL DRB Setup List |  | 0..1 |  | The List of SL DRBs which are successfully established. | YES | ignore |
| >SL DRB Setup Item IEs |  | 1 .. <maxnoofSLDRBs> |  |  | EACH | ignore |
| >>SL DRB ID | M |  | 9.3.1.120 |  | - |  |
| SL DRB Failed To Setup List |  | 0..1 |  |  | EACH | ignore |
| >SL DRB Failed To Setup Item IE |  | 1 .. <maxnoofSLDRBs> |  |  | EACH | ignore |
| >>SL DRB ID | M |  | 9.3.1.120 |  | - |  |
| >>Cause | O |  | 9.3.1.2 |  | - |  |
| Requested Target Cell ID | O |  | NR CGI 9.3.1.12 | Special Cell indicated in the UE CONTEXT SETUP REQUEST message. | YES | reject |
| SCG Activation Status | O |  | 9.3.1.234 |  | YES | ignore |
| Uu RLC Channel Setup List |  | 0..1 |  | This IE is not used in this version of the specification. | YES | ignore |
| >Uu RLC Channel Setup Item IEs |  | 1 .. <maxnoofUuRLCChannels> |  |  | - |  |
| >>Uu RLC Channel ID | M |  | 9.3.1.266 |  | - |  |
| Uu RLC Channel Failed to be Setup List |  | 0..1 |  | This IE is not used in this version of the specification. | YES | ignore |
| >Uu RLC Channel Failed to be Setup Item IEs |  | 1 .. <maxnoofUuRLCChannels> |  |  | - |  |
| >>Uu RLC Channel ID | M |  | 9.3.1.266 |  | - |  |
| >>Cause | O |  | 9.3.1.2 |  | - |  |
| PC5 RLC Channel Setup List |  | 0..1 |  |  | YES | ignore |
| >PC5 RLC Channel Setup Item IEs |  | 1 .. <maxnoofPC5RLCChannels> |  |  | - |  |
| >>PC5 RLC Channel ID | M |  | 9.3.1.265 |  | - |  |
| >>Remote UE Local ID | O |  | 9.3.1.267 | This IE is not used in this version of the specification. |  |  |
| PC5 RLC Channel Failed to be Setup List |  | 0..1 |  |  | YES | ignore |
| >PC5 RLC Channel Failed to be Setup Item IEs |  | 1 .. <maxnoofPC5RLCChannels> |  |  | - |  |
| >>PC5 RLC Channel ID | M |  | 9.3.1.265 |  | - |  |
| >>Remote UE Local ID | O |  | 9.3.1.267 | This IE is not used in this version of the specification. | - |  |
| >>Cause | O |  | 9.3.1.2 |  | - |  |
| ServingCellMO-encoded-in-CGC List |  | 0..1 |  |  | YES | ignore |
| >ServingCellMO-encoded-in-CGC Item IEs |  | 1 .. <maxNrofBWPs> |  | The servingCellMO which has been encoded in CellGroupConfig IE. | EACH | ignore |
| >>servingCellMO | M |  | INTEGER (1..64, ...) |  | - |  |
| >>BWP ID | M |  | INTEGER (0..4) |  | YES | ignore |
| UE Multicast MRB Setup List |  | 0..1 |  |  | YES | reject |
| >UE Multicast MRB Setup Item IEs |  | 1 .. <maxnoofMRBsforUE> |  |  | EACH | reject |
| >>MRB ID | M |  | 9.3.1.224 | MRB ID for the UE. | - |  |
| >>Multicast F1-U Context Reference CU | M |  | 9.3.2.13 |  | - |  |
| Dedicated SI Delivery Indication | O |  | ENUMERATED (true, ...) |  | YES | ignore |
| Configured BWP List |  | 0..1 |  | This IE is present when the gNB-DU configures at least one BWP with NCD-SSB or without SSB. | YES | ignore |
| >Configured BWP Item IEs |  | 1 .. <maxNrofBWPs> |  |  | EACH | ignore |
| >>BWP-Id | M |  | INTEGER (0..4) | The IE is used to refer to one BWP. | - |  |
| >>BWP Location And Bandwidth | M |  | INTEGER (0..37949) | The IE type range is the same as the locationAndBandwidth IE in BWP IE as specified in TS 38.331 [8]. | - |  |
| Early Sync Information |  | 0..1 |  |  | YES | ignore |
| >TCI States Configurations List | M |  | OCTET STRING | Includes the LTM-TCI-Info IE, as defined in TS 38.331 [8]. | - |  |
| >Early UL Sync Configuration | O |  | 9.3.1.328 |  | - |  |
| >Early UL Sync Configuration for SUL | O |  | Early UL Sync Configuration 9.3.1.328 | This IE applies for SUL carrier. | - |  |
| LTM Configuration |  | 0..1 |  |  | YES | ignore |
| >SSB Information | M |  | 9.3.1.202 | Includes the SSB Information for the requested target cell. | - |  |
| >Reference Configuration Information | O |  | OCTET STRING | Includes the CellGroupConfig IE, as defined in TS 38.331 [8]. | - |  |
| >Complete Candidate Configuration Indicator | O |  | ENUMERATED (complete, ...) |  | - |  |
| >LTM CFRA Resource Configuration | O |  | OCTET STRING | Includes the RACH-ConfigDedicated IE, as defined in TS 38.331 [8]. | - |  |
| >LTM CFRA Resource Configuration for SUL | O |  | OCTET STRING | Includes the RACH-ConfigDedicated IE, as defined in TS 38.331 [8]. This IE applies for SUL carrier. | - |  |
| CSI-RS Resource Configuration | O |  | FFS |  | YES | ignore |
| S-CPAC Configuration |  | 0..1 |  |  | YES | ignore |
| >Reference Configuration Information | O |  | OCTET STRING | Includes the CellGroupConfig IE, as defined in TS 38.331 [8]. | - |  |
| >Complete Candidate Configuration Indicator | O |  | ENUMERATED (complete, ...) |  | - |  |

/*********************Next change***********************/

9.2.2.7	UE CONTEXT MODIFICATION REQUEST

This message is sent by the gNB-CU to provide UE Context information changes to the gNB-DU.

Direction: gNB-CU  gNB-DU

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.3.1.1 |  | YES | reject |
| gNB-CU UE F1AP ID | M |  | 9.3.1.4 |  | YES | reject |
| gNB-DU UE F1AP ID | M |  | 9.3.1.5 |  | YES | reject |
| SpCell ID | O |  | NR CGI 9.3.1.12 | Special Cell as defined in TS 38.321 [16]. For handover case, this IE is considered as target cell. | YES | ignore |
| <skip unchanged part> |  |  |  |  |  |  |
| LTM Information Modify |  | 0..1 |  |  | YES | reject |
| >LTM Indicator | M |  | ENUMERATED (true, …) |  | - |  |
| >Reference Configuration | O |  | 9.3.1.292 |  | - |  |
| >CSI Resource Configuration | O |  | 9.3.1.330 |  | - |  |
| Request for CSI Resource Configuration | O |  | ENUMERATED (true, …) |  | YES | ignore |
| LTM CFRA Resource Config List |  | 0..1 |  |  | YES | ignore |
| >LTM CFRA Resource Config Item IEs |  | 1 .. <maxnoofLTMCells> |  |  | EACH | ignore |
| >>Cell ID | M |  | NR CGI 9.3.1.12 |  | - |  |
| >>LTM CFRA Resource Configuration | O |  | OCTET STRING | Includes the RACH-ConfigDedicated IE, as defined in TS 38.331 [8]. | - |  |
| >>LTM CFRA Resource Configuration for SUL | O |  | OCTET STRING | Includes the RACH-ConfigDedicated IE, as defined in TS 38.331 [8]. This IE applies for SUL carrier. | - |  |
| LTM Configuration ID Mapping List | O |  | 9.3.1.294 |  | YES | reject |
| Early Sync Information Request |  | 0..1 |  |  | YES | ignore |
| >Request for RACH Configuration | M |  | ENUMERATED (true, …) |  | - |  |
| >LTM gNB-DUs ID List |  | 1 |  | This IE contains the IDs of the source gNB-DU and candidate gNB-DU(s). | YES | reject |
| >>LTM gNB-DUs Item IEs |  | 1..< maxnoofLTMgNBDUs> |  |  | - |  |
| >>>LTM gNB-DU ID | M |  | gNB-DU ID 9.3.1.9 |  | - |  |
| >>>LTM gNB ID | O |  | Global gNB ID 9.3.1.305 |  | - |  |
| Early Sync Candidate Cell Information List |  | 0..1 |  |  | YES | ignore |
| >Early Sync Candidate Cell Information Item IEs |  | 1 .. <maxnoofLTMCells> |  |  | EACH | ignore |
| >>Cell ID | M |  | NR CGI 9.3.1.12 |  | - |  |
| >>TCI States Configurations List | O |  | OCTET STRING | Includes the LTM-TCI-Info IE, as defined in TS 38.331 [8]. | - |  |
| >>Early UL Sync Configuration | O |  | 9.3.1.328 |  | - |  |
| >>Early UL Sync Configuration for SUL | O |  | Early UL Sync Configuration 9.3.1.328 | This IE applies for SUL carrier. | - |  |
| >>TA Assistance Information | O |  | ENUMERATED (zero, …) | The value "zero" corresponds to TA value of the cell being equal to zero. | - |  |
| >>UE Based TA Measurement Configuration | O |  | OCTET STRING | Includes the ltm-UE-MeasuredTA-ID contained in the LTM-Candidate IE, as defined in TS 38.331 [8], for the LTM candidate cell identified by the Cell ID IE. | - |  |
| >>SSB Positions In Burst | C-ifEarlyUL |  | 9.3.1.138 | This IE applies to early TA acquisition. | YES | ignore |
| Early Sync Serving Cell Information |  | 0..1 |  |  | YES | ignore |
| >UE Based TA Measurement Configuration | O |  | OCTET STRING | Includes the ltm-ServingCellUE-MeasuredTA-ID contained in the LTM-Config IE, as defined in TS 38.331 [8], for the current serving cell. | - |  |
| LTM Cells To Be Released List | O |  | 9.3.1.291 |  | YES | reject |
| Path Addition Information | O |  | 9.3.1.296 |  | YES | reject |
| NR A2X Services Authorized | O |  | 9.3.1.323 |  | YES | ignore |
| LTE A2X Services Authorized | O |  | 9.3.1.324 |  | YES | ignore |
| NR UE Sidelink Aggregate Maximum Bit Rate for A2X | O |  | NR UE Sidelink Aggregate Maximum Bit Rate 9.3.1.119 | This IE applies only if the UE is authorized for NR A2X services. | YES | ignore |
| LTE UE Sidelink Aggregate Maximum Bit Rate for A2X | O |  | LTE UE Sidelink Aggregate Maximum Bit Rate 9.3.1.118 | This IE applies only if the UE is authorized for LTE A2X services. | YES | ignore |
| DL LBT Failure Information Request | O |  | ENUMERATED (inquiry, …) |  | YES | ignore |
| Ranging and Sidelink Positioning Service Information | O |  | 9.3.1.331 | This IE applies only if the UE is authorized for NR V2X services and/or 5G ProSe services. | YES | ignore |
| Non-Integer DRX Cycle | O |  | 9.3.1.344 |  | YES | ignore |
| LTM Reset Information | O |  | 9.3.1.346 |  | YES | ignore |

/*********************Next change***********************/

9.2.2.8	UE CONTEXT MODIFICATION RESPONSE

This message is sent by the gNB-DU to confirm the modification of a UE context.

Direction: gNB-DU  gNB-CU.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.3.1.1 |  | YES | reject |
| gNB-CU UE F1AP ID | M |  | 9.3.1.4 |  | YES | reject |
| gNB-DU UE F1AP ID | M |  | 9.3.1.5 |  | YES | reject |
| Resource Coordination Transfer Container | O |  | OCTET STRING | Includes the SgNB Resource Coordination Information IE as defined in subclause 9.2.117 of TS 36.423 [9] for EN-DC case or MR-DC Resource Coordination Information IE as defined in TS 38.423 [28] for NGEN-DC and NE-DC cases. | YES | ignore |
| DU To CU RRC Information | O |  | 9.3.1.26 |  | YES | reject |
| DRB Setup List |  | 0..1 |  | The List of DRBs which are successfully established. | YES | ignore |
| >DRB Setup Item IEs |  | 1 .. <maxnoofDRBs> |  |  | EACH | ignore |
| >>DRB ID | M |  | 9.3.1.8 |  | - |  |
| >>LCID | O |  | 9.3.1.35 | LCID for the primary path or for the split secondary path for fallback to split bearer if PDCP duplication is applied. | - |  |
| >>DL UP TNL Information to be setup List |  | 1 |  |  | - |  |
| >>>DL UP TNL Information to Be Setup Item IEs |  | 1 .. <maxnoofDLUPTNLInformation> |  |  | - |  |
| >>>>DL UP TNL Information | M |  | UP Transport Layer Information 9.3.2.1 | gNB-DU endpoint of the F1 transport bearer. For delivery of DL PDUs. | - |  |
| >>Additional PDCP Duplication TNL List |  | 0..1 |  |  | YES | ignore |
| >>>Additional PDCP Duplication TNL Items |  | 1 .. < maxnoofAdditionalPDCPDuplicationTNL> |  |  | EACH | ignore |
| >>>>Additional PDCP Duplication UP TNL Information | M |  | UP Transport Layer Information 9.3.2.1 | gNB-DU endpoint of the F1 transport bearer. For delivery of DL PDUs. | - |  |
| >>>>BH Information | O |  | 9.3.1.114 | This IE is not used in this version of the specification. | YES | ignore |
| >>Current QoS Parameters Set Index | O |  | Alternative QoS Parameters Set Index 9.3.1.123 | Index to the currently fulfilled alternative QoS parameters set. | YES | ignore |
| >>TSC Traffic Characteristics Feedback | O |  | 9.3.1.302 |  | YES | ignore |
| >>ECN Marking or Congestion Information Reporting Status | O |  | 9.3.1.322 |  | YES | ignore |
| DRB Modified List |  | 0..1 |  | The List of DRBs which are successfully modified. | YES | ignore |
| >DRB Modified Item IEs |  | 1 .. <maxnoofDRBs> |  |  | EACH | ignore |
| >>DRB ID | M |  | 9.3.1.8 |  | - |  |
| >>LCID | O |  | 9.3.1.35 | LCID for the primary path or for the split secondary path for fallback to split bearer if PDCP duplication is applied. | - |  |
| >>DL UP TNL Information to be setup List |  | 1 |  |  | - |  |
| >>>DL UP TNL Information to Be Setup Item IEs |  | 1 .. <maxnoofDLUPTNLInformation> |  |  | - |  |
| >>>>DL UP TNL Information | M |  | UP Transport Layer Information 9.3.2.1 | gNB-DU endpoint of the F1 transport bearer. For delivery of DL PDUs. | - |  |
| >>RLC Status | O |  | 9.3.1.69 | Indicates the RLC has been re-established at the gNB-DU. | YES | ignore |
| >>Additional PDCP Duplication TNL List |  | 0..1 |  |  | YES | ignore |
| >>>Additional PDCP Duplication TNL Items |  | 1 .. < maxnoofAdditionalPDCPDuplicationTNL> |  |  | EACH | ignore |
| >>>>Additional PDCP Duplication UP TNL Information | M |  | UP Transport Layer Information 9.3.2.1 | gNB-DU endpoint of the F1 transport bearer. For delivery of DL PDUs. | - |  |
| >>>>BH Information | O |  | 9.3.1.114 | This IE is not used in this version of the specification. | YES | ignore |
| >>Current QoS Parameters Set Index | O |  | Alternative QoS Parameters Set Index 9.3.1.123 | Index to the currently fulfilled alternative QoS parameters set. | YES | ignore |
| >>TSC Traffic Characteristics Feedback | O |  | 9.3.1.302 |  | YES | ignore |
| >>ECN Marking or Congestion Information Reporting Status | O |  | 9.3.1.322 |  | YES | ignore |
| SRB Failed to be Setup List |  | 0..1 |  | The List of SRBs which are failed to be established. | YES | ignore |
| >SRB Failed to be Setup Item IEs |  | 1 .. <maxnoofSRBs> |  |  | EACH | ignore |
| >>SRB ID | M |  | 9.3.1.7 |  | - |  |
| >>Cause | O |  | 9.3.1.2 |  | - |  |
| DRB Failed to be Setup List |  | 0..1 |  | The List of DRBs which are failed to be setup. | YES | ignore |
| >DRB Failed to be Setup Item IEs |  | 1 .. <maxnoofDRBs> |  |  | EACH | ignore |
| >>DRB ID | M |  | 9.3.1.8 |  | - |  |
| >>Cause | O |  | 9.3.1.2 |  | - |  |
| SCell Failed To Setup List |  | 0..1 |  |  | YES | ignore |
| >SCell Failed to Setup Item |  | 1 .. <maxnoofSCells> |  |  | EACH | ignore |
| >>SCell ID | M |  | NR CGI 9.3.1.12 | SCell Identifier in gNB | - |  |
| >>Cause | O |  | 9.3.1.2 |  | - |  |
| DRB Failed to be Modified List |  | 0..1 |  | The List of DRBs which are failed to be modified. | YES | ignore |
| >DRB Failed to be Modified Item IEs |  | 1 .. <maxnoofDRBs> |  |  | EACH | ignore |
| >>DRB ID | M |  | 9.3.1.8 |  | - |  |
| >>Cause | O |  | 9.3.1.2 |  | - |  |
| Inactivity Monitoring Response | O |  | ENUMERATED (Not-supported, ...) |  | YES | reject |
| Criticality Diagnostics | O |  | 9.3.1.3 |  | YES | ignore |
| C-RNTI | O |  | 9.3.1.32 | C-RNTI allocated at the gNB-DU | YES | ignore |
| Associated SCell List | O |  | 9.3.1.77 |  | YES | ignore |
| SRB Setup List |  | 0..1 |  |  | YES | ignore |
| >SRB Setup Item |  | 1 .. <maxnoofSRBs> |  |  | EACH | ignore |
| >>SRB ID | M |  | 9.3.1.7 |  | - |  |
| >>LCID | M |  | 9.3.1.35 | LCID for the primary path if PDCP duplication is applied | - |  |
| SRB Modified List |  | 0..1 |  |  | YES | ignore |
| >SRB Modified Item |  | 1 .. <maxnoofSRBs> |  |  | EACH | ignore |
| >>SRB ID | M |  | 9.3.1.7 |  | - |  |
| >>LCID | M |  | 9.3.1.35 | LCID for the primary path if PDCP duplication is applied | - |  |
| Full Configuration | O |  | ENUMERATED (full, ...) |  | YES | reject |
| BH RLC Channel Setup List |  | 0..1 |  | The list of BH RLC channels which are successfully established. | YES | ignore |
| >BH RLC Channel Setup Item |  | 1 .. <maxnoofBHRLCChannels> |  |  | EACH | ignore |
| >>BH RLC CH ID | M |  | BH RLC Channel ID 9.3.1.113 |  | - |  |
| BH RLC Channel Modified List |  | 0..1 |  | The list of BH RLC channels which are successfully modified. | YES | ignore |
| >BH RLC Channel Modified Item |  | 1 .. <maxnoofBHRLCChannels> |  |  | EACH | ignore |
| >>BH RLC CH ID | M |  | BH RLC Channel ID 9.3.1.113 |  | - |  |
| BH RLC Channel Failed to be Setup List |  | 0..1 |  | The list of BH RLC channels whose setup has failed. | YES | ignore |
| >BH RLC Channel Failed to be Setup Item |  | 1 .. <maxnoofBHRLCChannels> |  |  | EACH | ignore |
| >>BH RLC CH ID | M |  | BH RLC Channel ID 9.3.1.113 |  | - |  |
| >>Cause | O |  | 9.3.1.2 |  | - |  |
| BH RLC Channel Failed to be Modified List |  | 0..1 |  | The list of BH RLC channels whose modification has failed. | YES | ignore |
| >BH RLC Channel Failed to be Modified Item |  | 1 .. <maxnoofBHRLCChannels> |  |  | EACH | ignore |
| >>BH RLC CH ID | M |  | BH RLC Channel ID 9.3.1.113 |  | - |  |
| >>Cause | O |  | 9.3.1.2 |  | - |  |
| SL DRB Setup List |  | 0..1 |  | The List of SL DRBs which are successfully established. | YES | ignore |
| >SL DRB Setup Item IEs |  | 1 .. <maxnoofSLDRBs> |  |  | EACH | ignore |
| >>SL DRB ID | M |  | 9.3.1.120 |  | - |  |
| SL DRB Modified List |  | 0..1 |  | The List of SL DRBs which are successfully modified. | YES | ignore |
| >SL DRB Modified Item IEs |  | 1 .. <maxnoofSLDRBs> |  |  | EACH | ignore |
| >>SL DRB ID | M |  | 9.3.1.120 |  | - |  |
| SL DRB Failed To Setup List |  | 0..1 |  | The List of SL DRBs which are failed to be setup. | YES | ignore |
| >SL DRB Failed To Setup Item |  | 1 .. <maxnoofSLDRBs> |  |  | EACH | ignore |
| >>SL DRB ID | M |  | 9.3.1.120 |  | - |  |
| >>Cause | O |  | 9.3.1.2 |  | - |  |
| SL DRB Failed To be Modified List |  | 0..1 |  | The List of SL DRBs which are failed to be modified. | YES | ignore |
| >SL DRB Failed To be Modified Item |  | 1 .. <maxnoofSLDRBs> |  |  | EACH | ignore |
| >>SL DRB ID | M |  | 9.3.1.120 |  | - |  |
| >>cause | O |  | 9.3.1.2 |  | - |  |
| Requested Target Cell ID | O |  | NR CGI 9.3.1.12 | Special Cell or PSCell ID in the CPAC MCG Information IE indicated in the UE CONTEXT MODIFICATION REQUEST message. | YES | reject |
| SCG Activation Status | O |  | 9.3.1.234 |  | YES | ignore |
| Uu RLC Channel Setup List |  | 0..1 |  |  | YES | ignore |
| >Uu RLC Channel Setup Item IEs |  | 1 .. <maxnoofUuRLCChannels> |  |  | - |  |
| >>Uu RLC Channel ID | M |  | 9.3.1.266 |  |  |  |
| Uu RLC Channel Failed to be Setup List |  | 0..1 |  |  | YES | ignore |
| >Uu RLC Channel Failed to be Setup Item IEs |  | 1 .. <maxnoofUuRLCChannels> |  |  | - |  |
| >>Uu RLC Channel ID | M |  | 9.3.1.266 |  | - |  |
| >>Cause | O |  | 9.3.1.2 |  | - |  |
| Uu RLC Channel Modified List |  | 0..1 |  |  | YES | ignore |
| >Uu RLC Channel Modified Item IEs |  | 1 .. <maxnoofUuRLCChannels> |  |  | - |  |
| >>Uu RLC Channel ID | M |  | 9.3.1.266 |  | - |  |
| Uu RLC Channel Failed to be Modified List |  | 0..1 |  |  | YES | ignore |
| >Uu RLC Channel Failed to be Modified Item IEs |  | 1 .. <maxnoofUuRLCChannels> |  |  | - |  |
| >>Uu RLC Channel ID | M |  | 9.3.1.266 |  | - |  |
| >>Cause | O |  | 9.3.1.2 |  | - |  |
| PC5 RLC Channel Setup List |  | 0..1 |  |  | YES | ignore |
| >PC5 RLC Channel Setup Item IEs |  | 1 .. <maxnoofPC5RLCChannels> |  |  | - |  |
| >>PC5 RLC Channel ID | M |  | 9.3.1.265 |  | - |  |
| >>Remote UE Local ID | O |  | 9.3.1.267 |  | - |  |
| PC5 RLC Channel Failed to be Setup List |  | 0..1 |  |  | YES | ignore |
| >PC5 RLC Channel Failed to be Setup Item IEs |  | 1 .. <maxnoofPC5RLCChannels> |  |  | - |  |
| >>PC5 RLC Channel ID | M |  | 9.3.1.265 |  | - |  |
| >>Remote UE Local ID | O |  | 9.3.1.267 |  | - |  |
| >>Cause | O |  | 9.3.1.2 |  | - |  |
| PC5 RLC Channel Modified List |  | 0..1 |  |  | YES | ignore |
| >PC5 RLC Channel Modified Item IEs |  | 1 .. <maxnoofPC5RLCChannels> |  |  | - |  |
| >>PC5 RLC Channel ID | M |  | 9.3.1.265 |  | - |  |
| >>Remote UE Local ID | O |  | 9.3.1.267 |  | - |  |
| PC5 RLC Channel Failed to be Modified List |  | 0..1 |  |  | YES | ignore |
| >PC5 RLC Channel Failed to be Modified Item IEs |  | 1 .. <maxnoofPC5RLCChannels> |  |  | - |  |
| >>PC5 RLC Channel ID | M |  | 9.3.1.265 |  | - |  |
| >>Remote UE Local ID | O |  | 9.3.1.267 |  | - |  |
| >>Cause | O |  | 9.3.1.2 |  | - |  |
| SDT Bearer Configuration Info | O |  | 9.3.1.277 |  | YES | ignore |
| UE Multicast MRB Setup List |  | 0..1 |  |  | YES | reject |
| >UE Multicast MRB Setup Item IEs |  | 1 .. <maxnoofMRBsforUE> |  |  | EACH | reject |
| >>MRB ID | M |  | 9.3.1.224 | MRB ID for the UE. | - |  |
| >>Multicast F1-U Context Reference CU | M |  | 9.3.2.13 |  | - |  |
| ServingCellMO-encoded-in-CGC List |  | 0..1 |  |  | YES | ignore |
| >ServingCellMO-encoded-in-CGC Item IEs |  | 1 .. <maxNrofBWPs> |  | The servingCellMO which has been encoded in CellGroupConfig IE. | - |  |
| >>servingCellMO | M |  | INTEGER (1..64, ...) |  | - |  |
| >>BWP ID | M |  | INTEGER (0..4) |  | YES | ignore |
| Dedicated SI Delivery Indication | O |  | ENUMERATED (true, ...) |  | YES | ignore |
| Configured BWP List |  | 0..1 |  | This IE is present when the gNB-DU configures at least one BWP with NCD-SSB or without SSB. | YES | ignore |
| >Configured BWP Item IEs |  | 1 .. <maxNrofBWPs> |  |  | EACH | ignore |
| >>BWP-Id | M |  | INTEGER (0..4) | The IE is used to refer to one BWP. | - |  |
| >>BWP Location And Bandwidth | M |  | INTEGER (0..37949) | The IE type range is the same as the locationAndBandwidth IE in BWP IE as specified in TS 38.331 [8]. |  |  |
| Early Sync Information |  | 0..1 |  |  | YES | ignore |
| >TCI States Configurations List | M |  | OCTET STRING | Includes the LTM-TCI-Info IE, as defined in TS 38.331 [8]. | - |  |
| >Early UL Sync Configuration | O |  | 9.3.1.328 |  | - |  |
| >Early UL Sync Configuration for SUL | O |  | Early UL Sync Configuration 9.3.1.328 | This IE applies for SUL carrier. | - |  |
| LTM Configuration |  | 0..1 |  |  | YES | ignore |
| >SSB Information | M |  | 9.3.1.202 | Includes the SSB Information for the requested target cell | - |  |
| >Reference Configuration Information | O |  | OCTET STRING | Includes the CellGroupConfig IE, as defined in TS 38.331 [8]. | - |  |
| >Complete Candidate Configuration Indicator | O |  | ENUMERATED (complete, ...) |  | - |  |
| >LTM CFRA Resource Configuration | O |  | OCTET STRING | Includes the RACH-ConfigDedicated IE, as defined in TS 38.331 [8]. | - |  |
| >LTM CFRA Resource Configuration for SUL | O |  | OCTET STRING | Includes the RACH-ConfigDedicated IE, as defined in TS 38.331 [8]. This IE applies for SUL carrier. | - |  |
| CSI-RS Resource Configuration | O |  | FFS |  | YES | ignore |
| S-CPAC Configuration |  | 0..1 |  |  | YES | ignore |
| >Reference Configuration Information | O |  | OCTET STRING | Includes the CellGroupConfig IE, as defined in TS 38.331 [8]. | - |  |
| >Complete Candidate Configuration Indicator | O |  | ENUMERATED (complete, ...) |  | - |  |

/*********************Next change***********************/

9.2.2.x1	DU-CU CSI-RS ACTIVATION REQUEST

This message is sent by the gNB-DU to request the gNB-CU to activate/deactivate the SP CSI-RS transmission in neighbour cells.

Direction: gNB-DU  gNB-CU

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| Message Type | M |  | 9.3.1.1 |  |
| gNB-CU UE F1AP ID | M |  | 9.3.1.4 |  |
| gNB-DU UE F1AP ID | M |  | 9.3.1.5 |  |
| CSI-RS to be Activated List |  | 0..1 |  |  |
| > CSI-RS to be Activated Item IEs |  | 1 .. <maxnoofCellList> |  |  |
| >>Candidate Cell ID | M |  | NR CGI 9.3.1.12 |  |
| >>SP CSI-RS Resource Set ID | M |  | FFS |  |
| CSI-RS to be Deactivated List |  | 0..1 |  |  |
| > CSI-RS to be Deactivated Item IEs |  | 1 .. <maxnoofCellList> |  |  |
| >>Candidate Cell ID | M |  | NR CGI 9.3.1.12 |  |
| >>SP CSI-RS Resource Set ID | M |  | FFS |  |

| Range bound | Explanation |
| --- | --- |
| maxnoofCellList | Maximum no. of Cells in which the SP CSI-RS is to be activated or deactivated, the maximum value is 8. |

9.2.2.x2	CU-DU CSI-RS ACTIVATION RESPONSE

This message is sent by the gNB-CU to inform the gNB-DU about the SP CSI-RS activation/deactivation result.

Direction: gNB-CU  gNB-DU

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| Message Type | M |  | 9.3.1.1 |  |
| gNB-CU UE F1AP ID | M |  | 9.3.1.4 |  |
| gNB-DU UE F1AP ID | M |  | 9.3.1.5 |  |
| CSI-RS Activation Result List |  | 0..1 |  |  |
| > CSI-RS Activation Item IEs |  | 1 .. <maxnoofCellList> |  |  |
| >>Candidate Cell ID | M |  | NR CGI 9.3.1.12 |  |
| CSI-RS Deactivation List |  | 0..1 |  |  |
| > CSI-RS Deactivation Item IEs |  | 1 .. <maxnoofCellList> |  |  |
| >>Candidate Cell ID | M |  | NR CGI 9.3.1.12 |  |

| Range bound | Explanation |
| --- | --- |
| maxnoofCellList | Maximum no. of Cells in which the SP CSI-RS is to be activated or deactivated, the maximum value is 8. |

9.2.2.y1	CU-DU CSI-RS ACTIVATION REQUEST

This message is sent by the gNB-CU to request the gNB-DU to activate/deactivate the SP CSI-RS transmission in neighbour cells.

Direction: gNB-CU  gNB-DU

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| Message Type | M |  | 9.3.1.1 |  |
| gNB-CU UE F1AP ID | M |  | 9.3.1.4 |  |
| gNB-DU UE F1AP ID | M |  | 9.3.1.5 |  |
| CSI-RS to be Activated List |  | 0..1 |  |  |
| > CSI-RS to be Activatedtem IEs |  | 1 .. <maxnoofCellList> |  |  |
| >>Candidate Cell ID | M |  | NR CGI 9.3.1.12 |  |
| >>SP CSI-RS Resource Set ID | M |  | FFS |  |
| CSI-RS to be Deactivated List |  | 0..1 |  |  |
| > CSI-RS to be Deactivatedtem IEs |  | 1 .. <maxnoofCellList> |  |  |
| >>Candidate Cell ID | M |  | NR CGI 9.3.1.12 |  |
| >>SP CSI-RS Resource Set ID | M |  | FFS |  |

| Range bound | Explanation |
| --- | --- |
| maxnoofCellList | Maximum no. of Cells in which the SP CSI-RS is to be activated or deactivated, the maximum value is 8. |

9.2.2.y2	DU-CU CSI-RS ACTIVATION RESPONSE

This message is sent by the gNB-DU to inform the gNB-CU about the SP CSI-RS activation/deactivation result.

Direction: gNB-DU  gNB-CU

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| Message Type | M |  | 9.3.1.1 |  |
| gNB-CU UE F1AP ID | M |  | 9.3.1.4 |  |
| gNB-DU UE F1AP ID | M |  | 9.3.1.5 |  |
| CSI-RS Activation Result List |  | 0..1 |  |  |
| > CSI-RS Activation Item IEs |  | 1 .. <maxnoofCellList> |  |  |
| >>Candidate Cell ID | M |  | NR CGI 9.3.1.12 |  |
| CSI-RS Deactivation List |  | 0..1 |  |  |
| > CSI-RS Deactivation Item IEs |  | 1 .. <maxnoofCellList> |  |  |
| >>Candidate Cell ID | M |  | NR CGI 9.3.1.12 |  |

| Range bound | Explanation |
| --- | --- |
| maxnoofCellList | Maximum no. of Cells in which the SP CSI-RS is to be activated or deactivated, the maximum value is 8. |

9.2.2.z	Security Parameter Transfer

This message is sent by the gNB-CU to transfer the security parameter to the gNB-DU for the UE.

Direction: gNB-CU  gNB-DU

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| Message Type | M |  | 9.3.1.1 |  |
| gNB-CU UE F1AP ID | M |  | 9.3.1.4 |  |
| gNB-DU UE F1AP ID | M |  | 9.3.1.5 |  |
| Next Hop Chaining Count | M |  | INTEGER (0..7) | Next Hop Chaining Counter (NCC) defined in TS 33.501 [13]. |

/*********************End of changes***********************/

Annex – Text Proposal for LTM BLCR for TS 38.300

/*********************Start of changes***********************/

9.2.3.5.X C-Plane Handling of inter-gNB L1/L2 Triggered Mobility

This procedure is used for the case when the UE moves from one gNB to another gNB during NR operation for LTM. Figure 9.2.3.5.X-1 shows the inter-gNB LTM procedure for intra-NR.

Figure 9.2.3.5.X-1: Signalling procedure for inter-gNB LTM

The procedure for inter-gNB LTM is as follows:

1.	The UE sends a MeasurementReport message (L3 measurement result) to the source gNB containing measurements of neighbouring cells.

2.	The source gNB decides to configure inter-gNB LTM.

3.	The source gNB requests inter-gNB LTM for one or more candidate cells belonging to one or more candidate gNB(s). The source gNB initiates a HANDOVER REQUEST message per candidate cell containing one candidate cell ID and the CSI resource configuration for subsequent LTM. The source gNB may request the candidate gNB(s) to provide the CSI-RS configuration.

4.	Admission Control may be performed by the candidate gNB(s).

5.	The candidate gNB(s) prepares the LTM configuration(s) and sends inter-gNB response (HO REQUEST ACKNOWLEDGE) to the source gNB including the generated RRC configurations for the accepted candidate cell. The candidate gNB(s) may also include the CSI-RS configuration upon request.

6.	The source gNB sends an LTM CONFIGURATION UPDATE to the candidate gNB(s) to update the LTM configurations of candidate cell(s). The source gNB may include the common CSI-RS resource configuration.

7.	The candidate gNB(s) sends the LTM CONFIGURATION UPDATE ACKNOWLEDGE message to the source gNB. The candidate gNB(s) may also provide the CSI-RS report configuration.

NOTE :	Step 6 may also be triggered after step 14, or after step 17 by implementation for subsequent LTM.

8.	The source gNB sends an RRCReconfiguration message to the UE.

9.	The UE stores the LTM candidate configurations and sends an RRCReconfigurationComplete message to the source gNB.

9a	If early data forwarding is applied, the source gNB sends the EARLY STATUS TRANSFER message.

10/11. Early synchronization to some LTM candidate cell(s) belonging to the candidate gNB(s) may be performed. The candidate gNB(s) sends the TA INFORMATION TRANSFER message to the source gNB if early TA acquisition is performed to the candidate gNB(s).

12.	 The UE performs L1 measurements on the configured LTM candidate cell(s) and transmits L1 measurement reports to the source gNB. L1 measurement should be performed as long as RRC reconfiguration (step 8) is applicable.

13.	The source gNB determines to initiate inter-gNB LTM.

14.	The source gNB decides to execute cell switch to a target cell and transmits an LTM cell switch command MAC CE triggering cell switch by including a target configuration ID which indicates the index of the candidate configuration of the target cell, a beam indicated with a TCI state or beams indicated with DL and UL TCI states, and a timing advance command for the target cell, if available. The UE switches to the target cell and applies the candidate configuration indicated by the target configuration ID.

15.	The source gNB sends the CELL SWITCH NOTIFICATION message to the target gNB to indicate the initiation of Cell Switch command to the UE.

16.	The target gNB detects the UE access. The UE performs the random access procedure towards the target cell, if UE does not have valid TA of the target cell as specified in clause 5.18.35 of TS 38.321[6].

17/18. The target gNB sends the HANDOVER SUCCESS message to the source gNB to inform that the UE has successfully accessed the target cell. In return, the source gNB sends the SN STATUS TRANSFER message following the principles described in step 7 of Intra-AMF/UPF Handover in clause 9.2.3.2.1.

NOTE :	Late data forwarding may be initiated as soon as the source gNB receives the HANDOVER SUCCESS message. Up to implementation, late data forwarding and SN Status Transfer may be initiated once the source gNB triggers the inter-gNB LTM cell switch for the UE in Step 14.

19.	The UE sends an RRCReconfigurationComplete message to the target gNB.

20. The new source gNB (i.e., the target gNB) sends the LTM CONFIGURATION UPDATE message to the candidate gNBs to setup the UE associated Xn signalling connection, and update the necessary LTM related configurations.

21. The candidate gNB(s) responds the LTM CONFIGURATION UPDATE ACKNOWLEDGE message to the new source gNB.

22. The new source gNB may send the UE CONTEXT RELEASE message to inform the old source gNB to release radio and C-plane related resources associated to the UE context if no LTM candidate cell(s) exist in the old source gNB. Any ongoing data forwarding may continue.

/*********************End of changes***********************/

Annex – Text Proposal for LTM BLCR for TS 38.470

/*********************Start of changes***********************/

5.2.x	CSI-RS Activation function

The CSI-RS Activation function enables the gNB-DU to request the gNB-CU, and the gNB-CU to request the gNB-DU to activate or deactivate the semi-persistent CSI-RS transmission in specified cells .

5.2.y	Security Parameter Transfer function

The Security Parameter Transfer function enables the gNB-CU to transfer the security parameters to the gNB-DU for the UE.

/*********************Next change***********************/

6.1.x	CSI-RS Activation procedures

The following procedures are used to activate or deactivate the semi-persistent CSI-RS transmission in specified cells:

-	DU-CU CSI-RS Activation;

-	CU-DU CSI-RS Activation.

6.1.y	Security Parameter Transfer procedures

The following procedures are used to transfer the security parameters:

-	Security Parameter Transfer.

/*********************End of changes***********************/