3GPP TSG-RAN WG3 Meeting #127-bis	R3-252123

Wuhan, CN, 7-11 Apr, 2025

Title:	(TP for MDT BLCR for TS38.423): Logged MDT enhancement for NTN

Agenda Item:	10.3.1

Source:	Huawei, CMCC, China Unicom

Document for:	Other

1	Introduction

This document contains a TP for MDT BLCR for TS38.423 to implement the logged MDT enhancement for NTN.

Annex	Text Proposal

<<<<<<<<<<<<<<<<<<<< First Change >>>>>>>>>>>>>>>>>>>>

9.2.3.137	Logged Event Trigger Config

This IE configures with UE with specific events for triggering MDT configuration. Current specified event is based on out of coverage (OOC) detection.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| CHOICE Event Type Trigger | M |  |  |  | – |  |
| >Out of Coverage |  |  |  |  |  |  |
| >>Out of Coverage Indication | M |  | ENUMERATED (true, …) |  |  |  |
| >L1 Event |  |  |  |  |  |  |
| >>CHOICE L1 Event Threshold | M |  |  |  |  |  |
| >>>RSRP |  |  |  |  |  |  |
| >>>>Threshold RSRP | M |  | INTEGER (0..127) | Corresponds to information provided in the RSRP-Range IE as defined in TS 38.331 [10]. |  |  |
| >>>RSRQ |  |  |  |  |  |  |
| >>>>Threshold RSRQ | M |  | INTEGER (0..127) | Corresponds to information provided in the RSRQ-Range IE as defined in TS 38.331 [10]. |  |  |
| >>Hysteresis | M |  | INTEGER (0..30) | This parameter is used within the entry and leave condition of an event triggered reporting condition and corresponds to information provided in the Hysteresis IE as defined in TS 38.331 [10]. |  |  |
| >>Time to trigger | M |  | ENUMERATED (ms0, ms40, ms64, ms80, ms100, ms128, ms160, ms256, ms320, ms480, ms512, ms640, ms1024, ms1280, ms2560, ms5120) | Time during which specific criteria for the event needs to be met in order to trigger a measurement report. Corresponds to information provided in the TimeToTrigger IE as defined in TS 38.331 [10] |  |  |
| Location Event Trigger Type |  | 0..1 |  |  | YES | ignore |
| >Threshold Distance | M |  | INTEGER(0.. 65535) | The threshold distance from the reference location of the serving cell to the UE. |  |  |

<<<<<<<<<<<<<<<<<<<< End of Changes >>>>>>>>>>>>>>>>>>>>