3GPP TSG-RAN WG3 #127bis	             		                           R3-252154

Wuhan, China, 7th – 11th April 2025

Agenda Item:	20.2

Source:	Jio Platforms Ltd (JPL)

Title:	AI/ML-Enhanced Positioning Enhancements for NRPPa

Document for:	Discussion

# Introduction

This Technical Document propose to introduce enhancements for AI/ML-assisted positioning. The proposed changes target 5G-Advanced positioning use cases where machine learning models improve accuracy, in line with the approved Work Item NR_AIML_air-Core (RP-250792). Specifically, this document details the introduction of:

An AI/ML Mode Selection Indicator in LMF–gNB positioning request messages to distinguish whether the inference model runs at the gNB or at the LMF (i.e. to differentiate Case 3a vs. Case 3b positioning architectures).

An enhanced measurement reporting structure enabling richer position-related measurements, including:

Multi-path timing and angle reports (e.g. providing the top-k propagation delays with associated power/angle information for multiple signal paths)

Line-of-Sight (LoS) / Non-Line-of-Sight (NLoS) classification of the positioning signals, along with confidence indicators for these classifications.

Reuse and clarification of existing NRPPa Information Elements (IEs) to carry AI/ML-related metadata (for example, leveraging the existing Measurement Quality IE or LoS/NLoS Information IE to convey ML-derived attributes such as prediction confidence).

The content is authored by Jio Platforms and structured per RAN3 TDoc format with sections for Background, Observations, Proposal, and Impact on Specifications. It aligns with agreements from RAN3#127 (Athens) and RAN1 inputs on AI/ML positioning, ensuring consistency with the aims of Work Item NR_AIML_air-Core​. All technical details are provided in 3GPP-standard language, making this document self-contained and ready for submission to the 3GPP RAN3 archive.

# Background AND Discussion

AI/ML-assisted positioning in 5G-Advanced: 3GPP Release-18 included a study on AI/ML for the NR air interface that identified positioning accuracy enhancement as a key use case​. Two deployment scenarios for AI/ML-based positioning were defined for NG-RAN assistance:

- Case 3a: NG-RAN node assisted positioning with a gNB-hosted ML model (gNB-side inference).

- Case 3b: NG-RAN node assisted positioning with an LMF-hosted ML model (LMF-side inference).

In Case 3a, the gNB (Next-Gen NodeB) uses local measurements and an on-board AI/ML model to directly infer the UE’s position or an intermediate statistic, and then provides the result to the Location Management Function (LMF). In Case 3b, the gNB collects detailed channel measurements (e.g. uplink signal characteristics) and forwards them to the LMF, where a centralized ML model computes the position​. Table I of a recent study outlines these cases, showing that for LMF-side inference (Case 3b) the input data (such as channel impulse responses) are generated by the gNB and communicated to the LMF​. Conversely, for gNB-side inference (Case 3a), the model runs at the gNB itself, so only the final position or related result is sent to the LMF. Need for new signaling: The introduction of AI/ML in positioning brings new kinds of data and decisions that the existing NRPPa protocol (defined in TS 38.455) must support. The current NRPPa procedures (as of Release-18) were originally designed for “classic” positioning methods (e.g. PRS based TDOA, angle measurements) and have IEs for basic measurement results and LoS indicators. However, AI/ML methods can produce richer information: for example, a neural network might infer a position using multipath patterns or classify whether the direct path is Line-of-Sight or blocked. To fully exploit AI/ML models, the RAN3 study on AI/ML for NR recommended enhancements in the data exchanged between LMF and gNB:

- Model hosting awareness: The LMF and gNB need to know which side is executing the ML inference (Case 3a vs 3b) to coordinate their actions. In absence of an explicit indicator, a gNB receiving a positioning request cannot discern whether it is supposed to run its internal model or simply collect and send raw measurements for the LMF’s model.

- Extended measurements (multi-path, channel impulse response): AI/ML positioning models, especially in NLoS-rich environments, often leverage the full Channel Impulse Response (CIR) or multiple strongest paths rather than a single timing measurement. RAN1 has proposed that for Case 3b, the gNB should report sample-based channel responses (CIR data) to the LMF​.This implies sending multiple delay/power samples (and possibly angle information) to characterize the multipath propagation. Such data can significantly improve ML model accuracy by providing more environmental context​.

- LoS/NLoS context and confidence: Traditional positioning can indicate if a signal path is LoS or NLoS (e.g., via the LoS-NLoS Information IE in NRPPa). AI/ML can enhance this by classifying the condition of the path and even providing a confidence level (probability) for the classification. For example, an ML model might determine that a reported timing measurement is likely NLoS with 90% confidence. Currently, NRPPa does not explicitly carry a “confidence” value with the LoS/NLoS flag.

RAN3#127 agreements and WID context: At RAN3 Meeting #127 in Athens, companies discussed how to adapt NRPPa for AI/ML positioning. It was agreed that the existing location services (LCS) framework and NRPPa measurement procedure can be largely reused for AI/ML use cases​. In particular, for Case 3b, RAN3 concluded that the LMF can use the standard NRPPa Measurement Request/Report mechanism to obtain the necessary channel measurements from the gNB​. This reuse simplifies design by piggybacking on known message sequences, requiring mostly new IEs or extensions to convey the additional ML-related information. The work is aligned with the ongoing Work Item NR_AIML_air-Core (Rel-19) which earmarks positioning accuracy enhancements as a normative objective​. Input contributions from RAN1 and RAN2 during the Release-18 study phase (e.g. RAN1#116-120 drafts) have provided technical basis for these enhancements, such as recommending that NRPPa support reporting of multi-path CIR data (power, time, phase) for AI/ML processing​ and that timing for those measurements align with existing timing IE definitions (e.g. relative to UL RTOA)​.

In summary, the background study and discussions show a strong consensus to introduce an AI/ML mode indicator and enriched measurement information in NRPPa. This will enable the network to seamlessly support AI/ML-assisted positioning with minimal changes to the established protocol, enhancing positioning accuracy especially in challenging NLoS scenarios​.

Observations

No indicator for AI/ML model location: TS 38.455 (NRPPa) currently has no field to indicate where an AI/ML positioning inference is to be performed. As a result, a gNB cannot differentiate whether a positioning request from the LMF expects the gNB to run an ML model (Case 3a) or to forward measurements for LMF-side computation (Case 3b). This ambiguity could lead to improper handling of the request. For instance, in Case 3b the gNB should report raw measurements, not attempt to calculate a position. A clear mode selection flag is needed so that both ends share a common understanding of the procedure for that request​. RAN3 noted this gap and the need for an explicit indicator was highlighted as part of the AI/ML positioning discussions (to avoid relying on implicit configurations or separate OAM coordination).

- Need for multi-path measurement reporting: Current NRPPa measurement reports (e.g., timing or angle measurements delivered from gNB to LMF) are generally limited to a single observed value or a summary (like the strongest path timing plus a quality metric). However, AI/ML models can utilize richer channel information. It was observed that providing multiple strongest path components (with their delays, angles, and powers) can markedly improve positioning in multipath environments. For example, an ML model might learn to identify the direct path vs. reflections by comparing the delay/power profile. RAN1 contributions specifically suggested defining a new data structure in NRPPa to convey sample-based channel impulse responses (multipath profiles) from gNB to LMF​.

- In line with this, RAN3 agreed that the positioning measurement procedure should support reporting of multiple path measurements (e.g., top-k echoes). Without this, the LMF’s ML model in Case 3b would be starved of critical information that it needs for accurate inference. Notably, even for Case 3a, if the gNB hosts the model, the LMF might benefit from receiving more than just a final location – for instance, it could receive an intermediate result or quality indicator per path for validation or logging.

- LoS/NLoS classification is valuable but incomplete in current form: TS 38.455 defines an IE for LoS/NLoS indication (to flag if the path to the UE is line-of-sight or not). This IE is currently a binary or enumerated indicator, possibly set based on geometry or network knowledge. With AI/ML, the ability to classify LoS vs NLoS can be enhanced using real-time data (e.g., using channel characteristics). Additionally, ML algorithms can output a confidence level for such classification. During the study, it was observed that just knowing “LoS” or “NLoS” is not sufficient – the network may want to know how certain that classification is (since a misclassification could mislead the positioning solution). For instance, a path might be classified as NLoS with only 60% confidence; the LMF could treat that differently than an NLoS classification with 99% confidence. Presently, NRPPa has no standardized way to convey a confidence value alongside the LoS/NLoS flag. This is seen as a gap, given that ML-based positioning will inherently produce probabilistic outputs.

- Existing IEs could carry ML metadata with minor tweaks: NRPPa already includes generic IEs such as Measurement Quality and LoS-NLoS Information, which are candidates for conveying ML-derived information without defining entirely new IEs. The Measurement Quality IE (used, for example, to indicate the quality or uncertainty of a timing measurement) could potentially be repurposed or extended to indicate the ML model’s output quality – for example, the expected error standard deviation of the ML position estimate, or the confidence level of a provided measurement. Similarly, the LoS/NLoS Information IE can be extended via its extension container to include a confidence metric for the classification. It was observed that reusing these existing structures not only minimizes the protocol overhead but also aligns with 3GPP’s design preference to extend rather than reinvent IEs when adding new capabilities. Any reuse, however, must be done with clear semantics to avoid ambiguity (e.g., defining that Measurement Quality in an ML context refers to the ML inference confidence or error bound, as opposed to, say, purely physical measurement variance). RAN3 delegates noted the importance of clarifying how these IEs apply in AI/ML scenarios, possibly via updated semantics in the specs or additional comments in the ASN.1 description.

- Alignment with RAN1/RAN2 on measurement definitions: Another observation is the need to ensure that any new measurements or indicators introduced in NRPPa have counterparts in the lower layers. For example, if a “multi-path report” IE is added, it should correspond to measurements that the gNB can obtain (likely from physical layer channel sounding). RAN1 has been developing the physical layer support (e.g., using SRS-based channel sounding to capture CIR)​.

- Such measurements (time delay, angle of arrival, power for multiple paths) are made available at the gNB. Likewise, if the ML inference is at the gNB (Case 3a), the gNB will produce a position estimate and possibly an uncertainty – which should map to existing positioning result IEs (like location coordinates and quality). It was observed that no changes are needed in the NGAP layer; the NRPPa messages are encapsulated over NGAP transparently​, so the focus is purely on NRPPa content.

In summary, the current specification needs targeted updates to handle AI/ML positioning: a mode indicator to direct the workflow, extended measurement reporting for multi-path and ML classifications, and careful reuse of IEs to carry new metadata. These observations are backed by study item outputs and agreement in RAN3 that such enhancements are both necessary and feasible within the NRPPa framework​

Proposal

To address the above observations, the following changes are proposed to 3GPP TS 38.455 (NRPPa):

1. Introduce a new “AI/ML Mode Selection Indicator” IE in the LMF→gNB Positioning Request message. This IE explicitly signals the intended inference mode:

- Enumerated values: e.g., "gNB-side-inference" (indicating Case 3a) and "LMF-side-inference" (indicating Case 3b). Only one value is present per request.

- Semantics: If set to gNB-side-inference, the gNB is expected to use its on-board AI/ML model to compute the position (or related metric) and return the result to the LMF. If set to LMF-side-inference, the gNB should instead perform required measurements and send those measurements to the LMF, which will handle the inference. This distinction aligns with the case definitions from the study​.

- Container: The new IE can be added as an optional IE in the NRPPa Positioning Request message (or equivalent control message used to initiate a positioning session). It will likely be defined in the ASN.1 as an enumerated type. Backwards compatibility is ensured by making it optional: legacy gNBs (without Rel-18/19 support) will ignore it, defaulting to legacy behavior (which implicitly corresponds to LMF-side computation for conventional positioning).

- Usage example: Upon receiving a request with the indicator set to “LMF-side-inference”, the gNB knows it should not attempt to calculate the UE’s position but rather gather detailed channel data as per the measurement configuration. Conversely, “gNB-side-inference” instructs the gNB that it is responsible for running the model (e.g., a trained neural network) and providing the resulting position estimate or an intermediate result.

2. Extend the NRPPa measurement reporting to support multi-path timing/angle information (top-k paths). The proposal is to enhance the Positioning Measurement Report (gNB→LMF) message structure as follows:

- Multi-path Measurement List IE: Introduce a new IE (or an extension of an existing measurement IE) that can carry a list of multiple path components. Each entry in the list would include key attributes per path:

- Relative time delay (or absolute Time of Arrival if a common time reference is used) of the path, likely with quantization as used in existing timing IEs.

- Received power (or power ratio) of that path.

- Angle of arrival or departure (if applicable and measured by the gNB’s antenna array) for that path, in azimuth/elevation or as a quantized index if needed.

- Optionally, a phase or other channel statistic if identified by RAN1 as useful for ML (noting RAN1 suggestions to report CIR samples including phase​).

- Number of paths (k): Up to a reasonable maximum (for example, 3 or 5 strongest paths) to limit message size. The IE could include an explicit count or be defined as a sequence list in ASN.1.

- Extensibility: Use an ASN.1 sequence of or an IE container that can be extended in later releases if more information per path is needed. Initially, delay, power, and possibly angle are prioritized as they were explicitly noted in study items (CIR power-time profile)​

- Interpretation: This multi-path list effectively conveys a partial channel impulse response. The first entry would normally correspond to the earliest arriving path (often the LoS path if it exists). Subsequent entries are likely reflections. An ML model at the LMF (Case 3b) could use this to infer position by recognizing patterns (e.g., a reflection cluster geometry). Even in traditional methods, this could be useful for advanced multilateration algorithms, but here it is primarily to feed ML. If the indicator (item 1 above) is “gNB-side-inference”, the gNB might not need to send the full list to LMF (since the gNB itself uses it internally). However, we propose that even in Case 3a, the gNB may send the multi-path info (or at least the primary path info and quality) to the LMF for transparency or auditing.

- Relation to existing IEs: If TS 38.455 already has something like MeasurementResult IE that encapsulates timing/angle, we propose to add an optional sub-structure there for multiple results. Alternatively, define a new IE MultipathMeasurementInfo as an extended IE. The aim is to implement the RAN1 recommendation to allow channel impulse response reporting via NRPPa​in a compact form (by sending only top components rather than raw sample arrays).

3. Include LoS/NLoS classification with confidence in the measurement report. To enhance context for positioning:

- LoS/NLoS indication per path or overall: The gNB should indicate whether it considers the direct path to the UE to be line-of-sight or not. If multi-path info is reported, this classification typically pertains to the earliest path. We propose reusing the existing LoS-NLoS Information IE for this purpose. If this IE is currently binary (LoS or NLoS), we keep that usage but augment it with a confidence level.

- Confidence metric: Introduce a new field for confidence, expressed either as a percentage (0–100%) or a numeric value (e.g., 0–1 in steps of 0.1) indicating the probability that the classification is correct. This could be added via an extension of the LoS-NLoS IE (e.g., LoS-NLoSInformation-ExtIEs container) to avoid impacting legacy usage. For instance, define LoS-NLoS-Confidence as an INTEGER (0..100) or ENUM (low/medium/high confidence).

- Semantics: A LoS classification with low confidence might signal to the LMF that conditions are ambiguous (perhaps partially blocked line-of-sight), whereas high confidence means the gNB’s ML or detection algorithm is fairly certain. This information can be used by the LMF’s positioning algorithm to weight measurements or decide whether to trust a direct path measurement or perhaps to request additional assistance (like switching to another positioning method if NLoS is certain).

- Reporting: The gNB can derive this classification via an ML model (trained to detect LoS from radio parameters such as RSRP, RMS delay spread, etc.). The LoS/NLoS Information IE in an NRPPa report may now be populated based on ML inference rather than only geometric line-of-sight determination. The confidence value is optional but recommended if the classification originates from an AI/ML method. For backward compatibility, older LMFs will ignore the confidence subfield if they are not programmed to read it (since it would be in an extension container).

- Per-path classification: In theory, each path in the multi-path list could be marked as LoS or NLoS (the first path LoS implies it’s the direct path, others by definition are NLoS reflections). However, since “LoS/NLoS” usually refers to the direct path condition, we will apply it as an overall condition of the UE-to-gNB link for the purpose of positioning. The spec text will clarify that it pertains to the direct path (or strongest path) in the measurement.

4. Reuse or clarify existing IEs for ML metadata where appropriate: Instead of creating entirely new IEs for every ML output, we propose to leverage and extend what exists:

- Measurement Quality IE for ML outputs: The Measurement Quality IE in NRPPa (used currently to indicate quality of measurements like RSTD or RTT) can be utilized to carry a quality indicator of the ML-based measurement or position result. For example, if the gNB in Case 3a returns a position, it could fill Measurement Quality with an estimate of position accuracy (e.g., ±x meters) or an ML confidence score for that position. If the gNB returns a timing measurement derived via ML processing, the quality IE could represent the confidence in that timing. We propose adding a note or an explicit definition that when AI/ML is involved, this IE may represent model confidence or predicted error rather than purely physical measurement variance. This is essentially a semantic extension; the ASN.1 might remain the same, but the description in the spec will clarify the usage. If needed, additional enumeration can be added to indicate the type of quality metric (e.g., one value could indicate “ML confidence” vs “RRM quality”). Since the Measurement Quality IE is already present in the spec (e.g., in Rel-17/18 docs), reusing it ensures minimal overhead.

- LoS/NLoS IE extension: As described, we reuse LoS-NLoS Information IE adding a confidence. If extension fields are not acceptable within that IE, we alternatively introduce a separate IE LoS/NLoS Confidence that is included alongside it. However, using the ExtIEs mechanism is preferred (the snippet suggests such an extension container exists for LoS-NLoS IE in similar protocols​, which we can mirror for NRPPa).

- No duplication of existing data: Ensure that if a new multi-path list is introduced, it does not conflict with existing single-path reporting IEs. Possibly, the spec can specify that if the multi-path list IE is present, the single Path Timing or Angle IE may be omitted or represents the first path only. This avoids confusion. The proposal is to clearly define their relationship.

- Overall position result handling: For Case 3a, where gNB provides a position estimate (AI/ML output), the existing NRPPa message for location result (if one exists, e.g. Positioning Result) can be used. We just need to ensure it can carry new types of positioning methods. Likely, the positioning result IE has a technology identifier (e.g., NR Multi-RTT, NR E-CID, etc.). We propose adding an identifier for “AI/ML-assisted positioning” so that the LMF knows the result was obtained via the ML model. This could be a simple codepoint in an existing IE (e.g., PositioningMethod enum extended to include “ai-ml-assistance”). This is a minor addition but important for clarity in logs and for any LCS service layer that might want to know how the position was obtained.

- All the above proposals are designed to be consistent with RAN3 agreements and RAN1 inputs. They ensure that the NRPPa protocol can support both modes of AI/ML positioning (gNB or LMF inference) and carry the richer information that ML algorithms rely on, such as multipath channel features and confidence metrics. The changes are additive and backward-compatible. Where possible, we extend existing IEs (MeasurementQuality, LoS/NLoS) rather than define entirely new messages, which keeps the impact on the spec minimal. Moreover, these proposals maintain alignment with the overall LCS framework – the LMF still orchestrates the procedure via NRPPa messages as before​, but now with enhanced content. To introduces one new IE (AI/ML mode indicator) and extends a few others (measurement report contents) to fulfill the objectives of AI/ML-enhanced positioning under the NR_AIML_air-Core work item. This provides the needed protocol support so that implementations can use advanced ML-based positioning methods to improve accuracy, especially in NLoS or rich-scattering environments​. The following section details the precise impacts on the specification documents.

Impact on Specifications

The proposed changes impact 3GPP TS 38.455 (NRPPa), specifically the message and IE definitions for LMF-to-gNB positioning procedures.

- The Positioning Request message (from LMF to NG-RAN node) will be updated to include the new optional IE AI/ML Mode Selection Indicator. In the ASN.1, this could appear as an addition in the ProtocolIE-Container for the request message. The presence is optional (denoted with "Presence: optional") and criticality likely set to ignore (so older nodes ignore it). The semantics description for this IE will be added, explaining the two modes (gNB-side or LMF-side inference). No existing IE is removed or changed; this is a purely additive change.

- Positioning Measurement Report/Result: The gNB’s response message(s) containing measurements or final result will be extended. If the spec currently has a NRPPa-PositioningMeasurementReport or similar, this is where the multi-path information IE and extended LoS/NLoS info will be included. Concretely:

- A new IE MultiPathInfoList (name TBC) will be added under this message, with its structure defined in the Information Elements section. This IE may itself be a sequence containing multiple PathInfo entries. Each PathInfo entry includes fields for timing advance or pseudo-range, received power, angle, etc., as described in the proposal. The ASN.1 for PathInfo and MultiPathInfoList will be introduced. The MultiPathInfoList IE is optional; its presence could be conditional on the Mode Indicator (e.g., primarily for Case 3b, or always included if available).

- The existing LoS-NLoSInformation IE definition will be updated via an extension. The ASN.1 LoS-NLoSInformation-ExtIEs will have a new field, e.g. confidenceDegree or similar, of type INTEGER or ENUM as defined. The description will clarify that this indicates confidence of the LoS/NLoS determination (applicable when an ML-based or measurement-based classification is done). The impact is that decoders need to handle the extended structure, but if they are built to ASN.1 automatically, backward compatibility is inherently handled by extension markers.

- The MeasurementQuality IE usage will be clarified in the context of ML. This might not require ASN.1 changes if it’s an existing field (just interpretation). However, if needed, an enumerated value can be added to indicate the quality refers to an ML confidence or error. For example, if MeasurementQuality currently encodes an uncertainty value, we might simply state that for AI/ML results it represents the model’s predicted error (e.g., 1-sigma error in meters). If it’s an index, define how an ML confidence maps to that index.

- If the gNB returns a final position (Case 3a scenario), the Positioning Result message (or equivalent) will have an indication of the method. The spec’s table for that message will have an entry updated or added: e.g., extend the LocationMethod or PositioningMethod IE to include an “AI-ML” option. Additionally, the MeasurementQuality IE in that result can carry the position accuracy as noted.

- ASN.1 updates: All new or modified IEs shoukd be captured in the ASN.1 module at the end of TS 38.455.

  - New enumeration for AIML-ModeIndicator ::= ENUMERATED { gNB-side, LMF-side } (exact naming to be decided, ensuring consistency with naming conventions).

  - New sequence PathMeasurementInfo with components (delay, power, angle, etc.) and PathMeasurementInfoList ::= SEQUENCE (SIZE(1..maxPath)) OF PathMeasurementInfo. A corresponding IE container definition PathMeasurementInfoListIE will be added in the NRPPa-PROTOCOL-IES definitions.

  - Extension of LoS-NLoSInformation (e.g., LoS-NLoSInformation::= SEQUENCE { ... , confidence INTEGER (0..100) OPTIONAL, ... } with an extension marker if needed).

  - Possibly extension of an existing enum for positioning method.

- No impact on TS 38.473 or other LCS specs: It assumes that any needed changes in the UE-facing protocol (NRPPa user-plane, LPP, etc.) are handled separately if at all. For example, TS 38.455 deals with LMF↔gNB; TS 38.473 (NRPPa for user plane) is not directly addressed here.

- Backwards compatibility and version handling: Networks and LMF/gNB implementations that support Rel-18 AI/ML positioning will likely negotiate capabilities (e.g., via OAM or configuration). The presence of the new IEs inherently indicates the support. There is minimal risk to legacy operation: if either the LMF or gNB doesn’t support these IEs, they will be ignored or not sent. For instance, a legacy gNB will ignore an unknown Mode Indicator in a request (treating it as unrecognized IE with criticality ignore), and thus default to standard positioning (which corresponds to LMF-side processing). Likewise, a legacy LMF won’t send the indicator at all. Therefore, the introduction of these parameters will not break existing positioning services.

# Conclusion

  Proposal 1 (AI/ML Mode Selection Indicator)

- Introduce a new optional indicator in LMF→gNB Positioning Request messages for specifying whether AI/ML inference is performed at the gNB (Case 3a) or at the LMF (Case 3b).

- Ensures both nodes have a clear, unified approach to measurement collection vs. ML-based location computation.

  Proposal 2 (Enhanced Multi-path Measurement Reporting)

- Extend NRPPa measurement reports to include top-k propagation paths (delay, power, angle) for improved accuracy, especially under NLoS conditions.

- Leverage ML-based classification to identify or weight each path (e.g., direct path vs. reflection).

  Proposal 3 (LoS/NLoS Classification & Confidence)

- Reuse the LoS/NLoS Information IE and add an optional confidence metric (e.g., probability of LoS) to enrich positioning data.

- Helps the LMF or gNB ML model handle uncertain propagation scenarios more effectively.

  Proposal 4 (Reuse of Existing NRPPa IEs for ML Metadata)

- Clarify usage of Measurement Quality IE to indicate ML-derived confidence or error estimates.

- Avoid redundant new messages by extending existing IEs (e.g., extension containers) for backward compatibility.