3GPP TSG-RAN WG3 Meeting #127-bis	R3-252176

# Wuhan, China,  7– 11 April, 2025

Title: 	Discussion on UE performance metric for both non-split and split architecture

Source: 	CMCC

Agenda item: 	11.4

Document Type:	Discussion

- Introduction

In RAN3 #124 meeting, agreements as following have been achieved for split architecture:

Transferring Measured UE performance from CU-UP to CU-CP.

And in RAN3 #125 meeting,

The details of signaling measured UE performance metrics from DU to CU-CP and/or from CU-UP To CU-CP needs further discussion during normative phase. The above overrides the agreement “Transferring Measured UE performance from CU-UP to CU-CP.” from RAN3 #124.

And in RP-242385, the WI description is agreed that:

The aim of this work item is to specify new AI/ML-based use cases and introduce further enhancements to finalize the Rel-18 leftovers based on the conclusions captured in TR 38.743.

The detailed objectives of the WI are listed as follows:

- Support of the Leftovers in Rel-18 AI/ML for NG-RAN [RAN3]: Split architecture support for Rel-18 use cases

Note: Coordination with RAN2, SA5 when needed if any.

In RAN3 #126 meeting, the UE performance metric including Average throughput DL/UL, Average Packet Loss DL, Average Packet Delay DL/UL are discussed and LS has been sent to SA5 on clarification of how to calculate the Average throughput DL/UL.

In RAN3 #127 meeting, the following agreement achieved:

Packet delay measured for UE Performance feedback is sent from CU-UP (the delay in DU side is aggregated in the CU-UP) to the CU-CP in the case of CP-UP split architecture.

Packet delay measured for UE Performance feedback is sent from DU to CU via DDDS in the case of CU-DU split architecture.

Stage 3 details of the solution are FFS.

.2	Discussion

This paper will focus on the discussion on transferring data between gNB-CU and gNB-DU for AI/ML Load Balance and Mobility Enhancement use cases. Since the topic is hardly discussed in previous R19 meetings, we propose to discuss the following with priority in this meeting.

In RAN3#125bis, RAN3 received LS R3-245021 from SA5, and SA5 provides the following information:

SA5 would like to response to the questions with the following:

RAN3 evaluated carefully the two specifications and made the following observations with respect to TS 28.552 and TS 28.558:

- TS 28.558 does not define M4 measurement.

- There seems to be a misalignment between the two specifications with respect to which entity performs the measurements. In particular, RAN3 identified the following cases:

- The “Average delay DL air-interface” is measured by the gNB-DU in TS 28.552 and by NRCellCU (for non-split and 2-split scenario) and GNBCUUPFunction (for 3-split scenario) in TS 28.558.

- The “Average delay DL in gNB-DU” is measured by the gNB-DU in TS 28.552 and by NRCellCU (for non-split and 2-split scenario) and GNBCUUPFunction (for 3-split scenario) in TS 28.558.

- The “UL PDCP SDU Loss Rate” is measured by the GNBCUUPFunction and NRCellCU in TS 28.552 and by GNBCUUPFunction in TS 28.558.

From RAN3 perspective, the above misalignment should be corrected in TS 28.558 based on TS 28.552.

From SA5 perspective, the misalignment between TS28.558 and TS 28.552 has been corrected in TS28.558v18.1.0. The missing M4 measurement has be added into TS28.558 as attached.

Observcation1: It is clear from this LS reply that Average delay DL needs to involve measurement collected from gNB-DU.

  - Average Throughput UL/DL

In RAN3 #126 meeting, it has been agreed that:

for Average UE Throughput DL/UL: these metrics could be calculated as specified in TS 28.558 clauses 6.3.1.4.1 and 6.3.1.4.2.

6.3.1.4.1	Average DL UE throughput in gNB

c)	This measurement is obtained according to the following formula based on the "ThpVolDl" and "ThpTimeDl" defined below. Separate counters are maintained for each mapped 5QI (or QCI for EN-DC architecture [16]) and for each supported S-NSSAI.

If TℎpTimeDl>0, TℎpVolDlTℎpTimeDl×1000 [kbit/s]

If TℎpTimeDl=0, 0 [kbit/s]

For small data bursts, where all buffered data is included in one initial HARQ transmission,, otherwise

6.3.1.4.2	Average UL UE throughput in gNB

c)	This measurement is obtained according to the following formula based on the "ThpVolUl" and "ThpTimeUl" defined below. Separate counters are maintained for each mapped 5QI (or QCI for EN-DC architecture [16]) and for each supported S-NSSAI.

If TℎpTimeUl>0, TℎpVolUlTℎpTimeUl×1000 [kbit/s]

If TℎpTimeUl=0, 0 [kbit/s]

For small data bursts, where all buffered data is included in one initial HARQ transmission otherwise

And LS has been sent to SA5 to clarify it:

For the Average UE Throughput DL and Average UE Throughput UL, RAN3 would like to ask SA5 whether the formulas in fields c) of clauses 6.3.1.4.1 and 6.3.1.4.2 in TS 28.558, respectively, refer to measurements that are specified with a per-UE granularity or with a per-DRB granularity as the measurement name in field e) of the same mentioned clauses starts with “DRB.”. RAN3 would appreciate an explanation of why the measurement name starts with the “DRB.” prefix.

Observation 2: The Average UE Throughput DL and UL are measured through the per-DRB level throughput for the UE, and averaged via the data volume/time among all these DRB for per slice or per UE.

According to TS 28.558 clauses 6.3.1.4.1 and 6.3.1.4.2., the Average UE Throughput DL and UL are calculated through the Data Volume in RLC level:

|  | The RLC level volume of a data burst, excluding the data transmitted in the slot when the buffer is emptied. A sample for ThpVolDl is the data volume, counted on RLC SDU level, in kbit successfully transmitted (acknowledged by UE) in DL for one DRB during a sample of ThpTimeDl. (It shall exclude the volume of the last piece of data emptying the buffer). |
| --- | --- |
|  | The RLC level volume of a data burst, excluding the data transmitted in the slot when the buffer is emptied. A sample for ThpVolUl is the data volume counted on RLC SDU level in kbit received in UL for one DRB during a sample of ThpTimeUl, (It shall exclude the volume of the last piece of data emptying the buffer). |

Observation 3: For non-split architecture, it is obvious that the Average UE Throughput DL and UL are measured in gNB-DU based on RLC level volume of a data burst.

In previous meetings, some companies proposed that “the gNB-CU-UP knows the PDCP SN and the packet size of the PDCP SDU. It can get the transmission status from gNB-DU via DDDS.” However, this method can be only used in split architecture. For non-split architecture, we shall follow the definition of SA5, to get the throughput through RLC level volume of a data burst, counting the RLC SDU. Then dis-alignment will be caused by DDDS method for split architecture which count PDCP SDU, and SA5 method for non-split architecture which count RLC SDU. The dis-alignment will cause the un-comparability between split and non-split architecture gNB.

Observation 4：Throughput measured through DDDS is based on PDCP SDU, it is different from SA5 defined throughput measurement through RLC SDU.

And to guarantee that for both split and non-split architecture, the Average UE Throughput DL and UL shall be measured in the same way and comparable, for split architecture, the UE throughput shall also base on the RLC level volume of data burst for per DRB. Which means the UE throughput shall be measured at gNB-DU, and transferred to gNB-CU-CP for aggregation.

Another concern about getting measurement through DDDS method is that for AIML model, it needs long term, continuous data collection. To get the measurement through DDDS means turn on the monitor function in user plane for long time,  it will be a big impact on the gNB performance. Let the user plane gather measurement may not be a good choice.

Proposal 1: For both non-split and split architecture, the average Throughput UL/DL are measured on RLC level in gNB-DU.

Proposal 2:It is proposed to transfer the Average Throughput UL/DL from gNB-DU to gNB-CU through F1 interface.  And gNB-CU aggregate the per UE or per slice level throughput UL/DL.

  - Average Packet Loss DL/UL

According to TS28.554 clause 6.3.1.3, for non-split architecture, the DL packet loss is measured as the DL packet loss rate on Uu interface (i.e., RLC SDU). For slicing scenario, it may include the loss that can’t meet the delay threshold packets. For split architecture, the DL packet loss also include the DL F1-U Packet Loss Rate, while the UL packet loss include the UL PDCP SDU Loss Rate and UL F1-U Packet Loss Rate.

Anyway, SA5 does not define the PDCP layer packet loss. So it is assumed that for non-split architecture, only the RLC packet loss is measured.

Some companies also proposed to use DDDS to measure the DL packet Loss in CU-UP. In DDDS, The gNB-CU-UP can get the number of bits based on the size of PDCP SDU and transmitted/delivered PDCP SN in DDDS. Which means the packet loss is measured in PDCP layer. According to the operation data, the PDCP layer loss is much more than the RLC layer packet loss, which make the RLC layer packet loss negligible. In this way, the split-architecture gNB packet loss will be significantly bigger than non-split architecture gNB packet loss measurement.

It is not reasonable to measure packet loss in different way for non-split and split gNB.

Proposal 3: For split architecture, the Average Packet Loss DL shall consider both DL packet loss rate on Uu interface(i.e., RLC SDU) and DL F1-U Packet Loss Rate.

Proposal 4: For both split and non-split architecture, the Average Packet Loss measurement shall be gotten on gNB-DU, and gNB-DU shall send them to gNB-CU for aggregation.

Currently, the Average Packet Loss UL is not taken as UE performance feedback for non-split architecture. However, for the split- architecture, the information on the UL PDCP SDU Loss Rate and F1-U Packet Loss Rate are still useful for the AI/ML as UE performance feedback.

Proposal 5: It is proposed to add Average Packet Loss UL as an optional UE Performance feedback. For split architecture, it shall consider both UL PDCP SDU Loss Rate and F1-U Packet Loss Rate.

In AI/ML for NGRAN, the UE level performance feedback and slice level performance feedback shall be collected. How to use the per-DRB level measurement to get the average UE level or slice level measurement is needed to be discussed in RAN3.  From TS 28.558 clauses 6.3.1.2, you can see for the DL Packet Loss Rate：

6.3.1.2.1	DL Packet Loss Rate on Uu

a)	This measurement provides the DL Packet (i.e., RLC SDU) Loss rate on Uu interface. The measurement is split into subcounters per QoS level (mapped 5QI or QCI in EN-DC architecture [16]) and per supported S-NSSAI. This measurement is also referred to as DL M7 in TS 37.320 [9].

b)	CC

c)	This measurement is obtained according to the definition in clause 4.2.1.5.1 of TS 38.314 [8], named "Packet Uu Loss Rate in the DL per DRB per UE". The measurement is performed per QoS level (mapped 5QI or QCI in EN-DC architecture [16]) and per supported S-NSSAI.

d)	Each measurement is an integer value. The number of measurements is equal to the number of QoS levels multiplied by the number of supported S-NSSAIs.

e)	DRB.PacketLossRateUuDlUe.Filter, 
Where Filter is a combination of QoS level and S-NSSAI, QoS level represents the mapped 5QI or QCI level, and SNSSAI represents S-NSSAI.

f)	NRCellDU

g)	5G-S-TMSI

h)	One usage of this measurement is to support ML training and performance evaluation.

And in in clause 4.2.1.5.1 of TS 38.314:

Protocol Layer: RLC

Table 4.2.1.5.1-1: Definition for Packet Uu Loss Rate in the DL per DRB per UE

| Definition | Uu Packet Loss Rate in the DL per DRB per UE. One packet corresponds to one RLC SDU. The measurement is done separately per DRB. Detailed Definition: M(T,drbid)=Dloss(T,drbid)∗1000000N(T,drbid)+Dloss(T,drbid), where explanations can be found in the table 4.2.1.5.1-2 below. |
| --- | --- |

Table 4.2.1.5.1-2: Parameter description for Packet Uu Loss Rate in the DL per DRB per UE

| M(T,drbid) | Packet Loss Rate in the DL per DRB per UE. Unit: number of lost packets per transmitted packets per DRB * 106, Integer. |
| --- | --- |
| Dloss(T,drbid) | Number of DL packets, of a data radio bearer with DRB Identity = drbid, for which at least a part has been transmitted over the air but not positively acknowledged, and it was decided during time period T that no more transmission attempts will be done. If transmission of a packet might continue in another cell, it shall not be included in this count. |
| N(T,drbid) | Number of DL packets, of a data radio bearer with DRB Identity = drbid, which has been transmitted over the air and positively acknowledged during time period T. |
| T | Time Period during which the measurement is performed, Unit: minutes. |
| drbid | The identity of the measured DRB. |

For different DRBs, the traffic is different, which means N(T, drbid) are different. To get an average DL Packet Loss Rate on Uu interface, it is reasonable to average all the Dloss through Packets for all related DRBs:

M(T)=∑Dloss(T,drbid)∗1000000∑(NT,drbid+DlossT,drbid)
If there is no traffic under a DRB within the measurement duration time, N and Dloss for this DRB will be 0 and it won’t impact the average UE or Slice-level Packet Loss Rate.

6.3.1.3.3	      DL F1-U Packet Loss Rate

a)	This measurement provides the fraction of PDCP SDU packets which are not successfully received at the gNB-DU). It is a measure of the DL packet loss on the F1-U interface. The measurement is optionally split into subcounters per QoS level (mapped 5QI or QCI in EN-DC architecture [16]), and subcounters per supported S-NSSAI. This measurement is also referred to as DL M7 in TS 37.320 [9].

b)	SI

c)	This measurement is obtained as: 1,000,000* Number of missing DL GTP sequence numbers (TS 29.281 [15]), representing packets that are not delivered to lower layers, of a data radio bearer, divided by Total number of DL GTP sequence numbers (also including missing sequence numbers) of a bearer, starting from the sequence number of the first packet delivered by gNB-CU-UP to gNB-DU until the GTP sequence number of the last packet. Separate counters are optionally maintained for mapped 5QI (or QCI for EN-DC architecture [16]) and per supported S-NSSAI.

d)	Each measurement is an integer value representing the loss rate multiplied by 1E6. The number of measurements is equal to one. If the optional QoS and S-NSSAI level measurement are performed, the measurements are equal to the number of mapped 5QIs and the number of supported S-NSSAIs.

e)	The measurement name has the form DRB.F1UpacketLossRateDlUe, and optionally DRB.F1UPacketLossRateDlUe.QoS where QoS identifies the target quality of service class, and DRB.F1UPacketLossRateDlUe.SNSSAI where SNSSAI identifies the S-NSSAI.

f)	NRCellDU

g)	5G-S-TMSI

h)	One usage of this measurement is to support ML training and performance evaluation.

In the similar way, the UE or Slice-level DL F1-U Packet Loss Rate shall also be averaged among all its DRBs in the same way, using all the Dloss to be divided by all the Packets: 1,000,000* Number of missing DL GTP sequence numbers (TS 29.281 [15]), representing packets that are not delivered to lower layers, of a data radio bearer, for all DRBs, divided by Total number of UL GTP sequence numbers (also including missing sequence numbers) of a bearer for all DRBs.

F1UPacketLossRateDlUe(T,drbid)=∑Dloss(T,drbid)∗1000000∑(NT,drbid+DlossT,drbid), here Dloss is the missing DL GTP sequence numbers, N is the Total number of DL GTP sequence numbers.

Observation 5: The UE or Slice-level Average DL Packet Loss Rate on Uu shall be the average among all related DRBs, using the total Dloss to be divided by total packets in these DRBs.

Observation 6: The UE or Slice-level Average DL F1-U Packet Loss Rate shall be the average among all related DRBs, using the total Dloss to be divided by total packets in these DRBs.

For UL Packet Loss Rate:

6.3.1.3.1	UL PDCP SDU Loss Rate

a)	This measurement provides the fraction of PDCP SDU packets which are not successfully received at gNB-CU-UP. It is a measure of the UL packet loss including any packet losses in the air interface, in the gNB-CU and on the F1-U interface. Only user-plane traffic (DTCH) and only PDCP SDUs that have entered PDCP (and given a PDCP sequence number) are considered. The measurement is optionally split into subcounters per QoS level (mapped 5QI or QCI in EN-DC architecture [16]), and subcounters per supported S-NSSAI. This measurement is also referred to as UL M7 in TS 37.320 [9].

b)	SI

c)	This measurement is obtained as: 1,000,000* Number of missing UL PDCP sequence numbers, representing packets that are not delivered to higher layers, of a data radio bearer, divided by Total number of UL PDCP sequence numbers (also including missing sequence numbers) of a bearer, starting from the sequence number of the first packet delivered by UE PDCP to gNB-CU-UP until the sequence number of the last packet. If transmission of a packet might continue in another cell, it shall not be included in this count. Separate counters are optionally maintained for mapped 5QI (or QCI for EN-DC architecture [16]) and per supported S-NSSAI.

d)	Each measurement is an integer value representing the loss rate multiplied by 1E6. The number of measurements is equal to one. If the optional QoS and S-NSSAI level measurements are performed, the measurements are equal to the number of mapped 5QIs and the number of supported S-NSSAIs.

e)	The measurement name has the form DRB.PacketLossRateUlUe, and optionally
DRB.PacketLossRateUlUe.QoS where QoS identifies the target quality of service class, and DRB.PacketLossRateUlUe.SNSSAI where SNSSAI identifies the S-NSSAI.

f)	GNBCUUPFunction; NRCellCU (applicable to all split scenarios);

NRCellCU (only applicable to 2-split scenario).g)	S-TMSI

h)	One usage of this measurement is to support ML training and performance evaluation.

6.3.1.3.2	UL F1-U Packet Loss Rate

a)	This measurement provides the fraction of PDCP SDU packets which are not successfully received at gNB-CU-UP. It is a measure of the UL packet loss on the F1-U interface. The measurement is optionally split into subcounters per QoS level (mapped 5QI or QCI in EN-DC architecture [16]) and subcounters per supported S-NSSAI. This measurement is also referred to as UL M7 in TS 37.320 [9].

b)	SI

c)	This measurement is obtained as: 1,000,000* Number of missing UL GTP sequence numbers (TS 29.281 [15]), representing packets that are not delivered to higher layers, of a data radio bearer, divided by Total number of UL GTP sequence numbers (also including missing sequence numbers) of a bearer, starting from the GTP sequence number of the first packet delivered by gNB-DU to gNB-CU-UP until the GTP sequence number of the last packet. Separate counters are optionally maintained for mapped 5QI (or QCI for EN-DC architecture [16]) and per supported S-NSSAI.

d)	Each measurement is an integer value representing the loss rate multiplied by 1E6. The number of measurements is equal to one. If the optional QoS and S-NSSAI level measurement are performed, the measurements are equal to the number of mapped 5QIs and the number of supported S-NSSAIs.

e)	The measurement name has the form DRB.F1UpacketLossRateUlUe, and optionally DRB.F1UPacketLossRateUlUe.QoS where QoS identifies the target quality of service class, and DRB.F1UPacketLossRateUlUe.SNSSAI where SNSSAI identifies the S-NSSAI.

f)	GNBCUUPFunction

g)	5G-S-TMSI

h)	One usage of this measurement is to support ML training and performance evaluation.

As the DL Packet Loss Rate, for different DRBs, the traffic is different, which means the number of PDCP SDUs are different. To get an average UL SDU Loss Rate, it is reasonable to average all the Dloss through Packets for all related DRBs under this UE or Slice:

PacketLossRateUlUe(T,drbid)=∑Dloss(T,drbid)∗1000000∑(NT,drbid+DlossT,drbid)

In the similar way, the UE or Slice-level UL F1-U Packet Loss Rate shall also be averaged among all its DRBs in the same way, using all the Dloss to be divided by all the Packets: 1,000,000* Number of missing UL GTP sequence numbers (TS 29.281 [15]), representing packets that are not delivered to higher layers, of a data radio bearer, divided by Total number of UL GTP sequence numbers (also including missing sequence numbers) of all DRBs under the UE or Slice.

Observation 7: The UE or Slice-level Average UL PDCP Packet Loss Rate, UL F1-U Packet Loss Rate shall be the average among all related DRBs within the UE or Slice, using the total Dloss to be divided by total packets in these DRBs in split architecture.

  - Average Packet Delay UL/DL

In RAN3 #127 meeting, the following agreement achieved:

Packet delay measured for UE Performance feedback is sent from CU-UP (the delay in DU side is aggregated in the CU-UP) to the CU-CP in the case of CU-UP split architecture.

Packet delay measured for UE Performance feedback is sent from DU to CU via DDDS in the case of CU-DU split architecture.

When CU-CP request the packet delay data collection to CU-UP for AIML use cases, CU-UP needs to initiate DDDS on DU immediately, and average the delay according to the data collection request time window.

Proposal 6: For the packet delay, new procedures need to be introduced in E1 interface, to send the packet delay gotten by DDDS from CU-UP to CU-CP.

Proposal 7: It is proposed to discuss how to initiate DDDS in user plane when CU-UP received the data collection request from CU-CP.

3	Summary

In this contribution, we provide additional input considerations on the remaining issues on AI/ML for NG-RAN split architecture use cases, following observations and proposals are made:

Observcation1: It is clear from this LS reply that Average delay DL needs to involve measurement collected from gNB-DU.

Observation 2: The Average UE Throughput DL and UL are measured through the per-DRB level throughput for the UE, and averaged via the data volume/time among all these DRB for per slice or per UE.

Observation 3: For non-split architecture, it is obvious that the Average UE Throughput DL and UL are measured in gNB-DU based on RLC level volume of a data burst.

Observation 4：Throughput measured through DDDS is based on PDCP SDU, it is different from SA5 defined throughput measurement through RLC SDU.

Proposal 1: For both non-split and split architecture, the average Throughput UL/DL are measured on RLC level in gNB-DU.

Proposal 2:It is proposed to transfer the Average Throughput UL/DL from gNB-DU to gNB-CU through F1 interface.  And gNB-CU aggregate the per UE or per slice level throughput UL/DL.

Proposal 3: For split architecture, the Average Packet Loss DL shall consider both DL packet loss rate on Uu interface(i.e., RLC SDU) and DL F1-U Packet Loss Rate.

Proposal 4: For both split and non-split architecture, the Average Packet Loss measurement shall be gotten on gNB-DU, and gNB-DU shall send them to gNB-CU for aggregation.

Proposal 5: It is proposed to add Average Packet Loss UL as an optional UE Performance feedback. For split architecture, it shall consider both UL PDCP SDU Loss Rate and F1-U Packet Loss Rate.

Observation 5: The UE or Slice-level Average DL Packet Loss Rate on Uu shall be the average among all related DRBs, using the total Dloss to be divided by total packets in these DRBs.

Observation 6: The UE or Slice-level Average DL F1-U Packet Loss Rate shall be the average among all related DRBs, using the total Dloss to be divided by total packets in these DRBs.

Observation 7: The UE or Slice-level Average UL PDCP Packet Loss Rate, UL F1-U Packet Loss Rate shall be the average among all related DRBs within the UE or Slice, using the total Dloss to be divided by total packets in these DRBs in split architecture.

Proposal 6: For the packet delay, new procedures need to be introduced in E1 interface, to send the packet delay gotten by DDDS from CU-UP to CU-CP.

Proposal 7: It is proposed to discuss how to initiate DDDS in user plane when CU-UP received the data collection request from CU-CP.

4. Reference

- 3GPP RAN3#125bis Chairman’s notes (RAN3_125bis_agenda_20241018_1430_EOM)

- 3GPP RAN3#127 Chairman’s notes (RAN3_127_agenda_20250221_1300_EOM)

- RP-242385 WID Enhancements for Artificial Intelligence (AI) Machine Learning (ML) for NG-RAN (ZTE, NEC), 3GPP RAN#105

- DRAFT R3-244848 TR38.743 of AIML for NG-RAN_v1 (Release 19)

- TS28.558 UE level measurements for 5G system

- TS38.425 NR user plane protocol