3GPP TSG-RAN WG3 Meeting #127bis	             R3-252184

Wuhan, CN, 7th – 11th Apr 2025

Agenda Item:	16.2

Source:	CMCC

Title:	Discussion on Inventory Procedure and Signaling

Document for:	Discussion

# 1	Introduction

During last meeting, we agreed the following agreements on Inventory Procedure and Signaling.

About Inventory Procedures: - WA: Introduce a NGAP Class 1 Inventory Request procedure. - Introduce INVENTORY REQUEST message, which includes Inventory Request Transfer IE. - Introduce INVENTORY RESPONSE message, which includes Inventory Response Transfer IE. - Introduce INVENTORY FAILURE messages, which includes Inventory Failure Transfer IE. - Introduce a NGAP Class 2 Inventory Report procedure. - Introduce INVENTORY REPORT message, which includes Inventory Report Transfer IE. - The above Transfer IEs are transparent to AMF in case of indirect communication. - FFS: For Inventory related messages, the inclusion of Correlation Identifier and AIOTF Identifier.

In this contribution, we will provide our views on signaling and procedure between A-IoT RAN and A-IoT CN to support inventory operation.

# 2	Discussion

As discussed in our A-IoT architecture contribution R3-252183, there is no problem to transfer A-IoT inventory information in NGAP, so we think the work assumption made in last meeting should be changed to an agreement, i.e., introducing a NGAP Class 1 Inventory Request procedure.

Proposal 1: Change the WA to an agreement, i.e., Introduce a NGAP Class 1 Inventory Request procedure.

As the Inventory procedure was introduced last time, this meeting will focus on the contents inside the Inventory procedure. In TR 38.769, we studied the fundamental information should be exchanged between A-IoT CN and gNB for inventory operation.

The Inventory Request from the A-IoT CN to the A-IoT RAN, may include the following: (1) A-IoT Device Identification (to find a single device, a group of devices, or all devices) NOTE 1: The definition of this identification is out of RAN3 scope. For topology 1, A-IoT RAN needs to store the A-IoT Device Identification received from the inventory request. NOTE 2: Whether to interpret and process AIoT Device Identification received from the inventory request needs further discussion. (2) Scope of inventory request (e.g., a certain area in which the inventory is to be triggered) Multiple individual A-IoT Device IDs (one ID per device) can be provided to the A-IoT CN via a single Inventory Report. NOTE 3: For topology 1, whether device ID received from the device is sent transparently to the A-IoT CN by the A-IoT RAN depends on RAN2 and SA2. NOTE 4: Whether the description in this clause apply to A-IoT enabled gNB in topology 2 needs further discussion and is dependent on SA2 and RAN2.

Considering the Inventory Request procedure, Inventory Request Transfer IE is included in INVENTORY REQUEST message to provide the A-IoT inventory request related information to the gNB. The contents of Inventory Request Transfer IE is discussed below.

Firstly, it can be seen that the Inventory Request Transfer IE shall contain the A-IoT Device Identification information, which is used to identify an A-IoT device or a group of A-IoT device(s). This IE can be designed as a CHOICE type IE with the choice to indicate whether the inventory applies to a single device, a group of devices, or all devices. The A-IoT Device can check the A-IoT Device Identification information to determine whether the inventory points to oneself. Assuming that AIOTF decides to inventory a single A-IoT device, the A-IoT Device Identification information would be the A-IoT Device Identifier. Assuming that AIOTF decides to inventory a group of devices, the A-IoT Device Identification information would be a Mask ID or a Group ID. Assuming that AIOTF decides to inventory all devices in an area, no A-IoT Device Identification information is included in Inventory Request Transfer IE only using NULL codepoint which signifies the inventory operation applies to all A-IoT devices.

Proposal 2: The Inventory Request Transfer IE should include the A-IoT Device Identification information.

Proposal 3: Assuming that AIOTF decides to inventory a single A-IoT device, the A-IoT Device Identification information would be the A-IoT Device Identifier.

Proposal 4: Assuming that AIOTF decides to inventory a group of devices, the A-IoT Device Identification information would be a Mask ID or a Group ID.

Proposal 5: Assuming that AIOTF decides to inventory all devices in an area, no A-IoT Device Identification information is included in Inventory Request Transfer IE only using NULL codepoint.

Besides, the request service area should be included in the Inventory Request Transfer IE. As we agreed in last meeting, the AIoT RAN node receives the requested service area information (encoded as area and/or reader ID list) from AIoT CN, so the requested service area information is encoded as A-IoT area and/or reader ID list.

Proposal 6: The Inventory Request Transfer IE should include the requested service area.

In addition to the information mentioned above, RAN3 can take some conclusions on the inventory procedure made by SA2 TS 23.xyz-020 into account.

## 5.4 Assistance information provided to AIoT RAN node The AIOTF provides the assistance information to the AIoT RAN together with the service operation requests. The AIOTF determines the assistance information provided to the AIoT RAN based on the information received from the AF. The Assistance information is used by the AIoT RAN for performing service operations, e.g. radio resource allocation. The following assistance information may be provided: - AIoT service type (e.g. Inventory, Command); - Optionally, approximate number of AIoT devices based on AF request; - Optionally, approximate D2R message size based on AF request.

From the SA2 TS, it can be observed that the AIOTF will provide some assistance information to gNB for performing service operations. As the service type already implicitly reflected in the procedures of both inventory and command procedure, the Inventory Request Transfer IE may include approximate number of A-IoT devices and/or approximate D2R message size if available. The approximate number of A-IoT devices from AF is beneficial for inventory operation. If the approximate number of devices is received from AIOTF, the gNB can estimate the resources that needs to be allocated in A-IoT paging. For example, if ten devices are expected to be paged, allocating the equal amount of access occasions can minimize the risk of collision. Likewise, the approximate D2R message size can make the gNB aware the length of device ID and assign the UL resource more efficiently. Thus, the approximate number of A-IoT devices can be optionally provided in the Inventory Request Transfer IE from the AIOTF to the A-IoT RAN node.

Proposal 7: The approximate number of A-IoT devices and approximate D2R message size can be optionally provided in Inventory Request Transfer IE.

In the last meeting, following the SA2 definitions, RAN3 also analyzed the feasibility of introducing a corelation identifier to enable the AIOTF to correlate the inventory responses with the corresponding requests. Assuming that AIOTF might initiate multiple inventory sessions toward the same gNB, the correlation identifier serves as a unique marker for the gNB to distinguish between different inventory sessions within the request messages from the AIOTF. Accordingly, the gNB must include the same correlation identifier in its response message or failure message to allow the AIOTF to verify whether the inventory operation has been successfully initiated. Upon successful initiation of the inventory operation, the gNB shall report the device ID alongside the correlation identifier in the inventory report message.

Proposal 8: The correlation identifier should be included in Inventory Request procedure and Inventory Report procedure.

In the indirect path deployment scenario where the AMF acts as an intermediate node between the AIOTF and the gNB, the AMF may have interfaces with multiple AIOTFs. Consequently, the gNB could receive inventory request messages from different AIOTFs through the same AMF. It is evident that solely incorporating a correlation identifier in inventory operation is insufficient, as identical corelation identifiers might be assigned by different AIOTFs. To ensure concurrent execution of multiple inventory operations from distinct AIOTFs, the AMF must additionally allocate an AIOTF identifier alongside the correlation identifier to associate each request with its originating AIOTF, thereby enabling the gNB to distinguish the inventory operations initiated by different AIOTFs. Furthermore, the AIOTF identifier allows the AMF to route the response message back to the requesting AIoTF that corresponds to the original request.

Proposal 9: The AIOTF identifier should be included in Inventory Request procedure and Inventory Report procedure.

# 3	Conclusion

Based on the discussions mentioned above, in this contribution we provide some observations and proposals on the inventory procedure and signaling:

Proposal 1: Change the WA to an agreement, i.e., Introduce a NGAP Class 1 Inventory Request procedure.

Proposal 2: The Inventory Request Transfer IE should include the A-IoT Device Identification information.

Proposal 3: Assuming that AIOTF decides to inventory a single A-IoT device, the A-IoT Device Identification information would be the A-IoT Device Identifier.

Proposal 4: Assuming that AIOTF decides to inventory a group of devices, the A-IoT Device Identification information would be a Mask ID or a Group ID.

Proposal 5: Assuming that AIOTF decides to inventory all devices in an area, no A-IoT Device Identification information is included in Inventory Request Transfer IE only using NULL codepoint.

Proposal 6: The Inventory Request Transfer IE should include the requested service area.

Proposal 7: The approximate number of A-IoT devices and approximate D2R message size can be optionally provided in Inventory Request Transfer IE.

Proposal 8: The correlation identifier should be included in Inventory Request procedure and Inventory Report procedure.

Proposal 9: The AIOTF identifier should be included in Inventory Request procedure and Inventory Report procedure.

# 4	Reference

[1] RP-240826, Revised SID: Study on solutions for Ambient IoT (Internet of Things) in NR, CMCC, Huawei, T-Mobile USA;

[2] 3GPP TR 23.700-13, Study on Architecture support of Ambient power-enabled Internet of Things;

[3] 3GPP TR 38.769 V19.0.0, Study on solutions for Ambient IoT (Internet of Things) in NR;