3GPP TSG-RAN WG3 Meeting #R3-127-bis       	                                                         R3-252208

Wuhan, China, Apr 07-11, 2025

Agenda Item:	17.3

Source: 	Rakuten

Title:  	Support for OD-SIB1 mechanism at Cell A and NES Cell

Document for:	Discussion and decision

# Introduction

From RAN3#125-bis, we have the following agreements:

The UL WUS configuration will be transferred over Xn via a new class1 defined procedure.

Cell A gNB can decide and signal to NES Cell gNB that it stops the UL WUS configuration broadcast in its SIB.

From RAN3#126, we have the following agreements:

The purpose of the new XnAP procedure is to enable an NG-RAN node1 to provide UL WUS configuration information to NG-RAN node2.

The procedure uses non UE-associated signaling.

Cell A stores the UL WUS configuration information after it has received it.

A Class 2 procedure is to be introduced for Cell A gNB to signal to NES Cell gNB that it stops the UL WUS configuration broadcast in its SIB.

The procedure uses non UE-associated signalling.

Reuse the existing IE “Served Cell Information” IE to include UL WUS configuration from gNB-DU to gNB -CU in F1 Setup Request and gNB-DU Configuration Update procedures.

From RAN#106, we have the following agreements:

RAN confirms that Case 2 also includes the case where a particular Cell A can switch to become a NES Cell. This means that Cell A can provide UL-WUS configuration for itself before becoming a NES cell

# NES Mode of a Cell

In the earlier RAN#3 meetings, discussions on NES topic for OD-SIB1 assumed that Cell A is a cell which does not go into a NES mode. And hence, it can support the NES functionality of an overlapping cell. However, in the latest RAN plenary, RAN#106, it was confirmed that a Cell A can switch to become a NES Cell.

Based on this, our understanding is that a cell can have the following modes:

- Default mode: The cell broadcasts periodic SIB1

- NES mode: This cell does not broadcast periodic SIB1.The cell listens for UL WUS (RACH) from UEs and broadcasts OD-SIB1 in response to it.

When the cell is transmitting periodic SIB1, it can be considered as Cell A. And when the cell has activated NES mode, it can be considered as a NES cell.

Observation 1: When the cell is in default mode transmitting periodic SIB1, it can be considered as Cell A. And when the cell has activated NES mode, it can be considered as a NES cell.

## NES Cell and Cell A mapping

The minimum requirement to activate the NES mode by a cell is the support of an overlapping coverage cell, Cell A, which agrees to transmit its UL WUS configuration to the UEs camped on it. Therefore, a cell needs to know which are the overlapping coverage cells. This information may be obtained from the inventory database, or it may be configured by the operator, or it may be algorithmically derived in the cell itself. We will assume this knowledge to be present in the gNB-CU of the NG RAN node.

To continue the rest of the discussion, we propose that gNB-CU maintains a list of all its cells, their current SIB1 transmission mode (default or NES), and their latest UL WUS Configuration. Additionally, for each of the cells, a list of overlapping cells with their corresponding SIB1 modes and UL WUS Configurations is also maintained.

We propose that gNB-CU maintains a list of all its cells, their current SIB1 transmission mode (default or NES), and their latest UL WUS Configuration. Additionally, for each of the cells, a list of overlapping cells with their corresponding SIB1 modes and UL WUS Configurations is also maintained.

Using the example from the figure below, gNB1-CU will maintain a table for Cells 101, 102 and 103 with respective NES modes and UL WUS configuration. And for each of the cells the details of overlapping cells are also maintained. For example, for Cell 103, the gNB1-CU will maintain the details of Cell 201 and 202 also (plus it already has the details of overlapping Cell 102).

To begin with, the gNB-CU may assume that all the neighboring gNB cells with overlapping coverage are in the default SIB1 mode transmitting. This changes when a cell decides to go into the NES mode and sends a request for UL WUS transmission to the gNBs of the overlapping cells.

## Transition to NES Mode

When a source NG RAN node decides to transition to a NES mode, it needs the support of the overlapping NG RAN nodes. The source NG RAN node sends its UL WUS Configuration to the target NG RAN node to broadcast to UEs in IDLE/INACTIVE state in the target NG RAN node. This makes the NES NG RAN node discoverable by the UEs in the target NG RAN node.

As agreed in the previous meeting, a request procedure (UL WUS CONFIGURATION TRANSMISSION REQUEST) is sent by source NG RAN node (potential NES Cell) to determine whether the target NG RAN node (potential Cell A) is willing to broadcast UL WUS Configuration to the UEs camped on the target NG RAN node. We propose that UL WUS Configuration transmission request contains the Cell ID of the source NG RAN node, and its UL WUS Configuration details. We also propose that an optional cause IE (NES-OD-SIB1) is mentioned in the request.

The UL WUS Configuration transmission request contains the Cell ID of the source NG RAN node, its UL WUS Configuration details, and an optional cause IE (NES-OD-SIB1).

When the target NG RAN node gNB-CU receives the request, it checks which of its cells are overlapping with the requesting cell. Since the overlapping cells are mutually listed in each other’s database maintained by their respective gNB-CUs, the target NG RAN node (gNB-CU) receiving the request can easily map which of its cells are intended to be potential Cell A for the requesting source cell.

If the overlapping cell in the target gNB is in NES SIB1 transmission mode, or has initiated a transition to the NES mode, then the gNB-CU at the target NG-RAN node sends a failure message over the Xn interface to the source NG RAN gNB-CU. If the overlapping cell in the target gNB is in default SIB1 transmission mode, then it can potentially become Cell A to the requesting cell.

There are two options about which entity may at the target NG RAN node decides this – the receiving gNB-CU or the gNB-DU. Using the information of the SIB1 mode of the cell, gNB-CU may decide to accept/reject the support for NES mode of the neighboring cell.

On the other hand, if the gNB-CU sends this request to the gNB-DU, the gNB-DU is armed with further information regarding the cell load and the traffic conditions. Moreover, the gNB-DU additionally is responsible for calculating the Energy Saving Telemetry data, therefore, the gNB-DU may use this information also to decide whether it is in a position to support/or continue supporting the NES mode of a neighboring cell. This option of gNB-DU making the decision results in extra signaling on the F1AP, but the decision will be a fully informed one. Hence, we propose that gNB-DU of the target NG RAN node decides on whether to accept or reject the request for NES mode support of the neighboring cell.

Additionally, we propose that the target cell (Cell A) gNB-DU decides when to stop the ongoing support of the NES mode of the NES cell. We believe that gNB-DU is best positioned to make this decision since it has the latest information regarding the cell load, the traffic conditions, and the energy consumption.

We propose that gNB-DU of the target NG RAN node decides whether to accept or reject the request for NES mode support of the neighbouring cell.

We propose that the target cell (Cell A) gNB-DU decides when to stop the ongoing support of the NES mode of the NES cell.

## Cell A Rejecting/Canceling the NES support

If the target NG RAN node (Cell A) is unable or unwilling to broadcast UL WUS Configuration to the UEs camped on the target NG RAN node, the target NG-RAN node shall send the UL WUS CONFIGURATION TRANSMISSION FAILURE message to the source NG-RAN node. If the target NG RAN node decides to stop support of the ongoing NES mode of the NES Cell, then it shall send the UL WUS CONFIGURATION TRANSMISSION CANCEL to the source NG RAN node (NES Cell). We propose that a cause IE be included in both these messages. This IE will assist the NES cell is deciding on what course of action to take further.

When Cell A rejects the request for supporting NES mode of a neighbouring cell or decides to stop the ongoing support of the NES mode of a neighbouring cell, in both cases, it sends messages including a cause IE.

## Signalling at NES Cell and Cell A for Failure or Cancel scenario

When NES Cell gNB-CU receives the Failure or Cancel message from the Cell A gNB-CU, we propose that it is forwarded to NES Cell gNB-DU. This is important since NES Cell gNB-DU is the entity which decides whether the cell should transition to NES mode or not, and what is the trigger for NES. The gNB-DU receiving a cancel/reject may devise a logic on when and whether to resend a request for support of its transition to NES Cell based on the cause IE.

We propose that the Failure/Cancel message from Cell A is forwarded to NES Cell gNB-DU from NES Cell gNB-CU over the F1 interface.

When a cell decides to transition to the NES mode and receives a failure notification from the potential Cell A gNB If the NES cell has not transitioned to the NES mode due to the failure response from Cell A, then it takes no further action. It does not ping this Cell A for further support. In future, if the NES Cell decides to again transition to the NES mode from the default mode, it may send a fresh request for support to Cell A.

When a cell in NES mode gets a cancel message from Cell A gNB

When the NES Cell gNB-DU receives a Cancel message from Cell A, it is already in NES mode. It may decide to continue in this mode if it has support from one or more other Cell As. In which case, there is no further action taken at the NES Cell. It does not ping Cell A for further support. In future, if NES Cell decides to again transition to the NES mode from the default mode, it may send a fresh request for support to Cell A.

If it has no support for NES from any other Cell A, then the NES cell gNB-DU is forced to transition out of the NES mode and switch to default SIB1 transmission mode, and it informs the gNB-CU of this change. The gNB-CU sends a notification of change of NES mode to all the overlapping cells gNB-CUs (which are currently supporting or not supporting the NES mode) based on the table maintained by the NES Cell gNB-CU.

When Cell A decides to resume support of NES mode of neighboring cells

We believe that the support of Cell A will not change frequently. However, if there is such a scenario that Cell A decides to resume its support for the neighbor’s cell NES mode (within a short time after rejecting or canceling a request for support), the Cell A gNB-DU informs the gNB-CU of the same. Cell A gNB-CU may check its database on whether the support for NES mode of neighboring cells was rejected or withdrawn, and what is the latest SIB1 transmission mode of the neighboring cells are. Cell A gNB-CU then informs the NES Cell gNB-CU of its intent to support the NES mode. Both the gNB-CU’s update their database with this information, and Cell A gNB-CU indicates to Cell A gNB-DU that it may begin broadcasting the NES’s cell UL WUS Configuration. Similarly, the NES Cell gNB-CU indicates to NES Cell gNB-DU that it may transition to NES mode, if it was in default SIB1 transmission mode. If the NES cell was already in NES mode, then there is no communication between NES cell gNB-CU and gNB-DU.

We propose that RAN3 discusses the likely scenarios that could occur at NES Cell and Cell A when Cell A sends a failure/cancel message. We have proposed the signalling to handle each of these cases.

# Conclusion

We have the following observations and proposals:

Observation 1: When the cell is in default mode transmitting periodic SIB1, it can be considered as Cell A. And when the cell has activated NES mode, it is a NES cell.

Proposal 1	We propose that gNB-CU maintains a list of all its cells, their current SIB1 transmission mode (default or NES), and their latest UL WUS Configuration. Additionally, for each of the cells, a list of overlapping cells with their corresponding SIB1 modes and UL WUS Configurations is also maintained.

Proposal 2	The UL WUS Configuration transmission request contains the Cell ID of the source NG RAN node, its UL WUS Configuration details, and an optional cause IE (NES-OD-SIB1).

Proposal 3	We propose that gNB-DU of the target NG RAN node decides whether to accept or reject the request for NES mode support of the neighbouring cell.

Proposal 4	We propose that the target cell (Cell A) gNB-DU decides when to stop the ongoing support of the NES mode of the NES cell.

Proposal 5	When Cell A rejects the request for supporting NES mode of a neighbouring cell or decides to stop the ongoing support of the NES mode of a neighbouring cell, in both cases, it sends messages including a cause IE.

Proposal 6	We propose that the Failure/Cancel message from Cell A is forwarded to NES Cell gNB-DU from NES Cell gNB-CU over the F1 interface.

Proposal 7	We propose that RAN3 discusses the likely scenarios that could occur at NES Cell and Cell A when Cell A sends a failure/cancel message. We have proposed the signalling to handle each of these cases.

# Reference

Chairman’s Notes RAN3#125-bis

Chairman’s Notes RAN3#126

Chairman’s Notes RAN#127