3GPP TSG-RAN WG3 Meeting #127-bis														R3-252213

Wuhan, China, 7-11 April, 2025

Agenda item:	19.2

Source: 	Huawei, China Telecom, China Unicom

Title: 	Further discussions on SBFD related information exchange

Document for:	Discussion

1. Introduction

In the RAN3#127 meeting, there were agreements about SBFD related information exchange, listed as follow:

RAN3 to confirm that SSB resource configuration has already been supported to be exchanged between gNBs in Served Cell Information NR IE. gNB-DU provides the SBFD time and frequency configuration to gNB-CU in Served Cell Information IE, and gNB-CU provides the SBFD time and frequency configuration info received over Xn interface from gNB-CU to gNB-DU, in Served Cell Information IE in Served Cell Information IE in gNB-CU Configuration Update message. The SBFD time and frequency configuration is encoded as container defined by RAN2. For exchange of SSB info over Xn, nothing to be done, already supported in RAN3 spec. For exchange of strongest DL beam information over Xn, SSB info is defined as SSB-Index INTEGER (0..63), NZP CSI-RS info is defined as NZP CSI-RS Resource Indicator INTEGER (1..64). To define CLI-mitigation request as an optional IE.

In this contribution, we will further discuss the RAN3 impacts, including SBFD configuration exchange, measurement procedure, and CLI mitigation procedure among gNBs.

2. Discussion

2.2	Exchange of measurement configuration

In the LS from RAN1, RAN3 is request to support the exchange of following information:

- Information exchange of measurement resource configuration, i.e., SSB and/or periodic NZP CSI-RS

In the last meeting, RAN3 has confirmed that SSB SSB resource configuration has already been supported to be exchanged between gNBs. While, the NZP CSI-RS resource configuration hasn’t been provided in inter-node RRC message Measurement Timing Configuration, the open issue was described as follows:

For exchange of NZP CSI-RS over Xn, to add a new IE in Served Cell Information NR IE with reference to RAN2 definition?

Currently, the NZP CSI-RS resource configuration is defined in NZP-CSI-RS-ResourceSet IE by RAN2, but RAN2 has not included it in the inter-node message. RAN3 could either refer to NZP-CSI-RS-ResourceSet IE directly, or ask RAN2 to include this IE in the inter-node message. As we could see that from the IE “Served Cell Information NR”, in which there is sub-IE “Measurement Timing Configuration” which further includes the MeasurementTimingConfiguration inter-node message for the served cell, which has included the SSB resource configuration. If RAN2 agree to introduce the NZP-CSI-RS-ResourceSet in Measurement Timing Configuration, the current XnAP and F1AP specification can support the exchange of SBFD time configuration naturally; otherwise, new filed need to be introduced to Served Cell Information NR IE. For simplicity, we think RAN3 can introduce new filed contained NZP-CSI-RS-ResourceSet to Served Cell Information NR IE in XnAP and F1AP specifications.

Proposal 1: RAN3 agree to introduce NZP-CSI-RS resource configuration in Served Cell Information NR IE of XnAP and F1AP specifications, which refers to the IE NZP-CSI-RS-ResourceSet defined in TS 38.331.

In addition, there is another open point, i.e. whether there is need to trigger the neighbour gNB to perform CLI measurement on which cell and on exactly which signal(s) (SSB or NZP-CSI-RS), in order to be simple, we think it could be left to gNB implementation.

2.3	Exchange of measurement result and CLI-mitigation request

In the LS from RAN1, RAN3 is request to support the exchange of following information:

- Information exchange of strongest DL beam information

- Information exchange of CLI-mitigation request

In the last meeting, RAN3 has agreed that “To define CLI-mitigation request as an optional IE.”. Then, RAN3 should discuss whether the strongest DL beam information and CLI-mitigation request can be sent together, the open issues were described as follows:

Exchange the strongest DL beam information and the CLI-mitigation request as two optional IEs in the same message?

How to exchange the strongest DL beam information and the CLI-mitigation request over Xn, e.g. to reuse existing procedures such as NG-RAN node Configuration Update procedure or Resource Status Reporting procedure, or, to introduce new procedures?

Technically, we think the above two fields can be provided together, since when the neighbour gNB gets the strongest DL beam information, it could also evaluate whether there is any CLI from other gNB or not, and decide to request CLI mitigation, thus the two parameters could be included together, but of course it is up to gNB to decide whether to include the two parameters together or not, if there is no interference, there is no need, i.e. the two parameters could be included in the same message as optional IEs.

Proposal 2: RAN3 agree to exchange the strongest DL beam information and the CLI-mitigation request as two optional IEs in the same message.

On the issue of chosen of exactly procedure to exchange the strongest DL beam information and the CLI-mitigation request between NG-RAN node Configuration Update procedure or Resource Status Reporting procedure, or, to introduce new procedures, we think NG-RAN node Configuration Update procedure and a new procedure are suitable to exchange the above information. We slightly support to introduce new procedure to provide the strongest DL beam information and the CLI-mitigation request.

Proposal 3: RAN3 agree to introduce new procedure to provide the strongest DL beam information and the CLI-mitigation request over XnAP, e.g. CLI MEASUREMENT UPDATE procedure.

For F1 interface, in the victim gNB side, the gNB-DU should inform its gNB-CU about the strongest beam info and possible CLI request, while in the aggressor gNB side, the gNB-CU should forward the received info to its gNB-DU. For the details design, similar to XnAP, we think the new procedure is suitable to provide the strongest DL beam information and the CLI-mitigation request, e.g. CLI MEASUREMENT UPDATE procedure.

Proposal 4: RAN agree to provide the strongest DL beam information and the CLI-mitigation request from gNB-DU to gNB-CU (requesting side) and from gNB-CU to gNB-DU (requested side).

Proposal 4bis: RAN agree to introduce new procedure to provide the strongest DL beam information and the CLI-mitigation request over F1AP, e.g. CLI MEASUREMENT UPDATE procedure.

Here stage 2 TPs and stage3 TPs to TS 38.423 could be referred in [2], and stage 2&3 TPs to TS 38.473 in [3].

3. Conclusion

In this contribution, we analyzed the specification impacts of gNB-gNB mitigation. The following observation and proposals were provided:

Exchange of measurement configuration

Proposal 1: RAN3 agree to introduce NZP-CSI-RS resource configuration in Served Cell Information NR IE of XnAP and F1AP specifications, which refers to the IE NZP-CSI-RS-ResourceSet defined in TS 38.331.

Exchange of memeasurement result and CLI-mitigation request

Proposal 2: RAN3 agree to exchange the strongest DL beam information and the CLI-mitigation request as two optional IEs in the same message.

Proposal 3: RAN3 agree to introduce new procedure to provide the strongest DL beam information and the CLI-mitigation request over XnAP, e.g. CLI MEASUREMENT UPDATE procedure.

Proposal 4: RAN agree to provide the strongest DL beam information and the CLI-mitigation request from gNB-DU to gNB-CU (requesting side) and from gNB-CU to gNB-DU (requested side).

Proposal 4bis: RAN agree to introduce new procedure to provide the strongest DL beam information and the CLI-mitigation request over F1AP, e.g. CLI MEASUREMENT UPDATE procedure.

4. Reference

[1]	RP-241614, WID revision: Evolution of NR duplex operation: Sub-band full duplex (SBFD)

[2]	R3-252214,	Stage 2&3 CR on the introduction of Evolution of NR duplex operation: Sub-band full duplex (SBFD), Huawei

[3]	R3-252215,	Stage 2&3 CR on the introduction of Evolution of NR duplex operation: Sub-band full duplex (SBFD), Huawei