3GPP TSG-RAN WG3 #127bis	             		                           R3-252223

Wuhan, China, 7th – 11th April 2025

Agenda Item:	19.2

Source:	Jio Platforms Ltd (JPL)

Title:		Enhancements for Cross-Link Interference Management in NR Duplex Evolution (F1AP interface)

Document for:	Discussion

# Introduction

## 1.1 Background

With the evolution of 5G New Radio (NR) toward Release 19, 3GPP has been studying how to further enhance cross-link interference (CLI) management in the context of Sub-band Full Duplex (SBFD) and more dynamic TDD operations (under the ongoing work item NR_duplex_evo-Core). Cross-link interference occurs when one cell’s downlink transmission overlaps with another cell’s uplink reception (and vice versa), potentially degrading performance and capacity. Release 17/18 introduced preliminary mechanisms to handle CLI (e.g. TDD pattern sharing, semi-static scheduling approaches), but more sophisticated solutions are needed for next-phase duplex enhancements.

## 1.2 Problem Statement

Current F1AP (3GPP TS 38.473) does not provide:

A standardized way for the gNB-DU to indicate detailed NZP CSI-RS (Non-Zero Power Channel State Information Reference Signal) measurement configuration to the gNB-CU for cross-link interference monitoring.

A mechanism for the gNB-DU to report strongest downlink beam information to the gNB-CU, enabling beam-specific interference coordination.

An explicit CLI-Mitigation Request IE in the gNB-DU Configuration Update procedure.

These gaps limit the ability for a victim gNB-DU to communicate important interference-related data or request active interference reduction from the controlling gNB-CU (and potentially from neighboring cells over Xn). As a result, operators cannot fully capitalize on SBFD or dynamic TDD, since real-time coordination signals are missing in the F1 interface.

# Background AND Discussion

## 2.1 Observed Requirements from Duplex Evolution

SBFD mandates sub-band level coordination to avoid saturating the UL with interfering DL transmissions. This calls for precise measurement signals and dynamic reconfigurations in time or frequency subbands.

Dynamic TDD adds complexity, since each cell can independently schedule DL/UL resources, increasing the chance of cross-link overlap.

Measurements and Beam Indications: The victim cell (or DU) needs to identify precisely which beam or subband is interfering, typically via reference signals. A single node’s vantage point (the gNB-DU receiving UL) can best measure real interference power. Hence, it must be able to share that measurement outcome and request help.

## 2.2 Existing Mechanisms and Their Shortcomings

F1AP in Rel-18 provides broad cell configuration parameters but has no dedicated IE for NZP CSI-RS reference signals used specifically for CLI measurement. Similarly, no standardized “Strongest DL Beam” field is defined in Served Cell Information to pinpoint interfering beams.

The gNB-DU Configuration Update procedure can carry optional IEs for changing cell configuration, but it lacks a CLI-Mitigation Request IE that signals the controlling CU: “I am experiencing cross-link interference; please mitigate.” As a result, any CLI mitigation is done via vendor-proprietary methods or partial Xn-based solutions.

While XnAP (TS 38.423) also is being enhanced for CLI exchange among gNBs, the internal coordination between DU and CU remains incomplete. A uniform approach is needed so the gNB-DU can push real-time interference data to the CU, which can then handle inter-cell negotiation or power/time adjustments.

## 2.3 Key Observations from Trials and Industry Feedback

Deploying TDD in dense or overlapping coverage areas face high CLI if a neighbor cell’s DL beam strongly overlaps with local UL scheduling.

NZP CSI-RS is increasingly used as an “interference measurement” signal because it allows more flexible placement (time/frequency) and improved measurement accuracy.

The ability to identify and request targeted mitigation (e.g., reduce power on “beam X” or “subband Y”) in near-real-time is crucial for advanced duplex modes.

A set of new optional IEs in F1AP can bridge the gap, ensuring minimal overhead and backward compatibility.

## 2.4 Proposals

Proposal 1: Introduce NZP CSI-RS Measurement Configuration in Served Cell Information

F1AP currently includes a ServedCellInformation IE, which enumerates various properties of each served cell (such as frequency info, TDD configuration).

To handle SBFD or dynamic TDD, the controlling gNB-CU (or neighboring nodes) often needs to know which NZP CSI-RS resources a gNB-DU is transmitting for interference monitoring.

Proposal Details:

Add a new optional IE named, for example, NZP-CSI-RS-MeasurementConfiguration (type: OCTET STRING or referencing the RRC-defined structure) to the extension container of ServedCellInformation.

In the F1 SETUP REQUEST and GNB-DU CONFIGURATION UPDATE procedures, the DU can supply the configuration details (resource ID, periodicity, time/frequency domain) for the CSI-RS resource used to measure or calibrate CLI.

The CU, upon receiving it, can utilize or forward this information to other nodes to facilitate interference measurement or coordination.

Benefits:

Provides a standardized method to share measurement resource setups over F1.

Backward compatible: If the IE is absent or unknown, a Release 18 node simply ignores it.

Enhances performance by ensuring that the controlling CU (and other nodes) know the exact resource used to measure interference, enabling finer real-time coordination.

Proposal 2: Add Strongest DL Beam Information IE to Served Cell Information

In 5G, a cell often has multiple DL beams (SSB-based or CSI-RS-based beam management). The gNB-DU sees which beam is strongest from local measurement or UE reports.

Identifying the strongest beam is crucial for CLI because that beam typically causes the highest interference to neighbor uplinks.

Proposal Details:

Introduce a new optional IE in ServedCellInformation, named, for example, StrongestDLBeamInformation.

This IE is a CHOICE type allowing either an SSB index (0..63) or a CSI-RS resource ID (1..64 or 1..128, as appropriate) to identify which beam is the “dominant” or “strongest” one.

The gNB-DU populates this field when it has internal knowledge of the beam that is actively used and has the highest power or coverage. The gNB-CU can use it in coordination with neighbor cells (over Xn) to mitigate CLI if that beam is interfering.

Benefits:

Simplifies beam coordination: the CU knows exactly which beam is most relevant for CLI.

Minimizes guesswork or vendor-proprietary solutions.

Aligns with advanced beam-based operations that are common in mid/high frequency bands.

Proposal 3: Define a CLI-Mitigation Request IE in gNB-DU Configuration Update

The gNB-DU Configuration Update procedure is used by the DU to inform the CU of changes, but currently lacks any direct mention of CLI mitigation.

In practice, DUs experiencing severe interference or “collisions” might want to request immediate action from the CU (which can talk to neighbors via Xn or control local scheduling).

This is especially critical for semi-static or dynamic SBFD scheduling, where the DU might detect that certain subbands or beams are heavily affected by neighbor transmissions.

Proposal Details:

Add a new optional IE named CLI-MitigationRequest in the GNB-DU Configuration Update message.

This IE can be a simple enumerated or boolean type indicating “CLI mitigation requested,” optionally with sub-fields (e.g., target neighbor cell ID, severity metric, recommended subband/time). For minimal complexity, a single enumerated value mitigationRequested suffices.

Upon receiving the request, the gNB-CU either triggers local scheduling changes or coordinates with neighbor gNBs over Xn to reduce interference (e.g., lower power, shift beams, or do TDD re-alignment).

Benefits:

Allows real-time or near real-time “bottom-up” feedback from the DU to the CU regarding interference conditions.

The CU can then orchestrate more effective, targeted solutions (like a short burst of power reduction or re-scheduling).

Provides a direct, standardized hook for advanced CLI solutions in the future, which might reference the same IE.

# Conclusion

- Proposal 1 – NZP CSI-RS Measurement Configuration

  - Adds an optional IE in ServedCellInformation for the DU to specify a downlink CSI-RS configuration used for interference measurement.

  - Enhances the ability of the CU and possibly other nodes to measure cross-link interference precisely.

- Proposal 2 – Strongest DL Beam Information

  - Introduces an optional IE in ServedCellInformation to inform the CU of the cell’s strongest downlink beam (via SSB index or CSI-RS resource ID).

  - Facilitates targeted beam-level CLI mitigation and better beam coordination across cells.

- Proposal 3 – CLI-Mitigation Request

  - Defines a new optional IE for the gNB-DU Configuration Update message, enabling the DU to request interference mitigation from the CU.

  - Empowers bottom-up reporting of critical interference scenarios, bridging the gap between local real-time measurement and system-level coordination.