3GPP TSG-RAN WG3 Meeting #127bis	R3-252253

Wuhan, China, April 7 - 11, 2025    									    		     Revison of R3-250568

Title:	(TP to TS 38.473) Unified solution to support L3 measurement based R18 LTM

Agenda Item:	9.2

Source:	Qualcomm Inc, Apple Inc, AT&T, Boost Mobile Networks (Dish),JIO Platforms, Fujitsu, Rakuten, Vivo, Tejas Networks, Xiaomi

Document for:	Discussion/Agreement

# 1	Introduction

In RAN3#125bis, RAN3 received an LS [1] from RAN2 about support for L3 measurement based LTM, and is given shown below:

1. Overall Description: In the late phase of Rel-18 LTM definition, it was decided, that LTM can be triggered based on Layer 3 measurements by the UE. Consequently, Layer 3 measurements are delivered via RRC to the CU and the CU needs to request the DU to send MAC CE to trigger LTM execution. During RAN2 discussion, it was recognised, that there is no message over F1 which would be sent from CU to DU requesting to trigger MAC CE for the case that the LTM mobility is based on L3 measurements. It was also recognised that such a message would be needed from Release 18 onwards. 2. Actions: RAN2 kindly asks RAN3 to introduce F1 support for the case where the CU needs to request the DU to send MAC CE to trigger LTM execution based on UE Layer 3 measurements.

During RAN3#125bis online discussion, some companies proposed CU to decide about triggering of L3 measurements based LTM and CU will inform DU to trigger MAC-CE based cell switch command. But some companies argued that it is enough for CU to provide L3 measurements to DU and then DU will make decisions about when to trigger Early TA Acquisition and MAC-CE based cell switch command etc.

Couple of companies argued that RAN2 LS does NOT talk about any Early TA Acquisition for L3 measurements based LTM and it is not applicable. Which is not aligned with many other companies understanding and as a result another LS R3-245710 was sent from RAN3 to RAN2 asking for clarification about whether Early TA Synchronization is applicable for L3 measurement based LTM or not.

RAN WG3 discussed the introduction of F1 support for the case where LTM is based on UE layer 3 measurements and acknowledge that this is not supported in Rel-18.

During the discussion, some companies raised the concern whether the early synchronization should be supported or not for L3 measurements based LTM.

RAN2 sent another reply LS R2-2409373 to RAN3 and given below.

RAN WG2 would like to thank RAN WG3 for their LS in R2-2409382. RAN WG2 understands that early synchronisation is also applicable to L3 measurements based LTM. It is also RAN WG2 understanding that for L3 measurements based LTM, the CU decides early synchronisation according to the results of L3 measurements and indicates that decision with the corresponding beam information to a DU and then the DU triggers early synchronisation.

Note that it is not RAN2 realm to make decision about whether CU or DU to make decisions about whether L3 measurements based LTM decisions like when to trigger early TA synchronization, MAC-CE based TCI state and MAC-CE based cell switch command. It is purely RAN3 to make such decision by taking LTM performance requirements into account and specify necessary F1 enhancements needed.

In RAN3#126, this issue was discussed and chair noted “Continue the discussion and decide the solution in next meeting.“

In Jan-2025, there was an offline email discussion among multiple companies to discuss issues of CU and DU based solution . Qualcomm has raised many issues with CU based solution.

In RAN3#127, RAN3 discussed without making any progress and chair decided not to support any solution in spite of 8 companies objected for not specifying any solution in not good practice.RAN3 sent reply LS to RAN2, cc: RAN4, RAN.

RAN WG3 has discussed the solutions, however does not reach a consensus to specify the support for L3 measurement based LTM for CU/DU distributed architecture. As a result, F1 support for L3 measurement based LTM will not be specified in Release 18

RAN#107 discussed the issue and provided following guidance to RAN3 in [16].

RAN kindly requests RAN3 to agree by RAN#108, a single standard solution as Rel-18 correction, to support the L3 measurement based LTM over the F1 interface (see WI Code above).

In this contribution, we discuss technical details of both CU and DU based solutions and provided conclusion to select one solution.

# 2	Discussion

In Rel-18, the main motivation for introducing LTM is to reduce the handover interruption latency and ability to switch cells/beams quickly by DU based on L1 filtered measurements received from UE.

Due to some initial development phase, some vendors claimed that they may not be able to support L1 measurements processing and some UEs may not be able to support L1 measurements reporting for all scenarios. Thus, to enable initial deployments, L3 measurements based LTM was also agreed to be supported in RAN4 and RAN2.

It is important to note that L3 measurements based LTM was agreed in RAN4 primarily for FR1 only and is not intended for FR2 case.

Observation 1 : L3 measurement based R18 LTM is supported only for FR1.

Overtime, it is possible that multiple gNB vendors supporting both L1 and L3 measurement based LTM configuration and different UEs based on their capability are expected to support L1 or L3 measurement based LTM for FR1. In such mixed deployment scenario, it is desirable to have one entity in gNB  should control LTM HO triggering for both L1 and L3 measurements based LTM.

Let us first try to understand what is difference between L1 measurement reports and L3 measurement reports from UE. Following Figure from TS 38.300 depicts UE processing of L1 and L3 measurements.

In case of L3 based measurements, UE does L3 based filtering which is long term average of reporting radio metrics (i.e., RSRP, RSRQ and SINR) and reports using RRC measurement reports. For L1 based measurements, UE does L1 based filtering (which is quick) and reports to gNB via either PUCCH or PUSCH in UCI. R18 Layer 1 LTM measurements are based on SSB only and UE reports L1-RSRP which is directly received by DU.

Between L1-RSRP and L3 RSRP, the only difference is what filter is being used by UE and these metric values meaning remains same. Example : RSRP of -90 dBm value is same whether it is L1 or L3 filtered and receiving node does not have to perform any additional decoding. Based on implementation-based filtering coefficient, DU can use different values based on whether it is L1-RSRP or L3-RSRP value. How DU makes decisions based on L3 measurement reports received from CU is upto DU implementation matter and is not standards issue.

Observation 2 : The difference between L1 vs L3 measurements is only about usage of L1 filter  vs L3 filter by the UE. At receiving RAN node, there is no additional de-filtering needed and meaning of absolute value of L1-RSRP Vs L3-RSRP remains same. DU will understand radio measurement very well and DU will handle processing of L3 measurements based on implementation.

DU also hosts control plane part which can understand RRC IEs and in many cases DU prepared RRC IEs including lower layer configuration and send to CU via “DU to CU RRC Information IE”. CU sends multiple RRC IEs  to DU in “CU to DU RRC Information IE” in many F1-AP messages. It is not correct to argue that DU does not understand L3 filtered measurement report and only CU can understand them.

In legacy L3 HO procedures, in existing F1-AP signalling messages CU to DU RRC Information IE -> Handover Preparation Information -> RRM-Config -> MeasResultList2NR is already sent from CU to DU and is not done for first time.

Observation 3 : control plane part of DU can understand RRC IEs and in many existing F1-AP signalling messages RRC IEs are already exchanged by CU and DU. In fact DU gets MeasResultList2NR IE within CU to DU RRC Information IE -> Handover Preparation Information -> RRM-Config -> MeasResultList2NR.

CU also forwards several SON reports such as RLF Report, RA Report, SHR, SPR as “RRC containers” in Access and Mobility Indication message over F1AP as shown below. This is based on the assumption that DU can also understand RRC containers, otherwise there would be no reason to forward these SON reports as “RRC containers” to DU.

Examples:

| >>RA Report Container | M |  | OCTET STRING | Includes the RA-ReportList-r16 IE as defined in subclause 6.2.2 in TS 38.331 [8]. | - |  |
| --- | --- | --- | --- | --- | --- | --- |

| >>NR UE RLF Report Container | M |  | OCTET STRING | Includes the nr-RLF-Report-r16 IE contained in the UEInformationResponse message defined in TS 38.331 [8]. | - |  |
| --- | --- | --- | --- | --- | --- | --- |

| >>Successful HO Report Container | M |  | OCTET STRING | Includes the SuccessHO-Report IE as defined in subclause 6.2.2 in TS 38.331 [8]. | - |  |
| --- | --- | --- | --- | --- | --- | --- |

| >>Successful PSCell Change Report Container | M |  | OCTET STRING | Includes the SuccessPSCell-Report IE as defined in TS 38.331 [8]. | - |  |
| --- | --- | --- | --- | --- | --- | --- |

Observation 4: CU forwards several SON reports such as RLF Report, RA Report, SHR, SPR as “RRC containers” in Access and Mobility Indication message over F1AP. This is based on the assumption that DU can understand RRC containers, otherwise there would be no reason to forward these SON reports as “RRC containers” to DU.

Key question is for FR1 L3 measurement based LTM, which network node has to make decision about LTM procedure triggers. i.e CU vs DU.

Before we discuss about L3 measurement based LTM solution, let us consider analysis of R18 FR1/2 Scell activation enhancements as well. This case is related to R18 L3 measurement based LTM discussion.

R18 FR1/FR2 Scell Activation Enhancement Analysis:

In addition to L3 measurement based LTM, RAN4/RAN2 introduced enhancements for “fast activation of Unknown FR2 Scells” using L3 Measurements. Based on this new enhancement, upon receiving MAC-CE Scell activation command from DU, UE MAC will indicate to RRC to trigger L3 measurement report. Whether to trigger L3 measurement reporting upon unknown Scell activation or not is controlled by new RRC IE ReportConfigNR -> ReportOnScellActivation-r18. Details available in RRC CR [14] and MAC CR [15].In split gNB Architecture, DU is responsible for MAC-CE based Scell activation/deactivation and is not known to CU. If UE sends L3 measurement reports, CU does not know what triggered UE to send L3 measurement reports for Scells. Additionally, if both L1 and L3 measurements are received, DU must have freedom to select which measurements to use to decide Scell TCI state activation for activated Scells. In later stage, RAN4 agreed to extend for FR1 Scells as well.

From RAN4 LS R4-2314338 [13],

Enhancement 1: network triggering report (of L3 measurement result) during FR2 unknown SCell activation

TCI activation command transmitted is based on L3 measurement results or L1 measurement results? Agreements: - If the L3 report is triggered after SCell activation, the TCI activation command is assumed based on: - the L3 report, if no L1-RSRP report is received before transmitting the TCI activation command. - the L3 report or L1-RSRP report (i.e., up to network implementation), if L3 report and L1-RSRP report is received before transmitting the TCI activation command.

In split gNB Architecture, DU is responsible for MAC-CE based Scell activation/deactivation and is not known to CU. If UE sends L3 measurement reports, CU does not know what triggered UE to send L3 measurement reports for Scells. Additionally, if both L1 and L3 measurements are received, DU must have freedom to select which measurements to use to decide Scell TCI state activation for activated Scells.

Observation 5: To support R18 fast Scell TCI state activation, DU should be able to process both L3 and L1 measurements since CU does not know when the DU sent Scell MAC-CE Activation command to UE.

R18 RACH-less HO :

Tdoc R3-250355 [12] from CATT, CMCC, CT provided following analysis for R18 RACH-less HO.

In Rel-18 RACH-less HO, the RRC handover command for the UE may contain a beam identifier for the beam to be used by the UE at the target cell. As specified by TS 38.300[4], the beam used by UE can be determined based on UE beam level measurement report.

Definition of RACH-less HO command in TS 38.331[5]:

RACH-LessHO-r18 ::=                 SEQUENCE {

targetNTA-r18                       ENUMERATED {zero, source}                                   OPTIONAL,   -- Need N

beamIndication-r18                  CHOICE {

tci-StateID-r18                     TCI-StateId,

ssb-Index-r18                       SSB-Index

}                                                                                               OPTIONAL,   -- Need N

...

}

In split architecture, the RRC handover command is generated by gNB-DU and gNB-DU can read the candidate cells’ L3 measurement report which is transferred via the CU to DU RRC information in Handover Preparation Information IE to determine the selected beam (TCI state or SSB) for UE.

It can be understood that gNB-DU determining the beam for UE based on L3 measurement report is already there from Rel-18. Also, L3 measurement report transfer from CU to DU is a legacy approach, though it is specifically used for RRC handover. Thus, Option 2 aligns with the DU and CU functionality split.

Observation 6: In R18 RACH-less HO, L3 measurements are already provided from CU to DU via F1-AP to assist DU to prepare RACH-LessHO IE as part of CellGroupConfig IE.

Based on above discussion,

Proposal 1 : In split gNB Architecture, one network node shall be allowed to process and trigger fast FR2 Scell TCI state activation and LTM HO decisions for both L1 and L3 measurements in both FR1 and FR2.

L3 measurement based LTM solution analysis:

For R18 L3 measurement based LTM, following are 2 possible solutions .

Option 1 : Based on L3 measurements received from UE, CU makes LTM decision and informs DU to trigger LTM procedures. (CU based solution)

Option 2 : CU forwards L3 measurements received from UE to DU via F1 signalling and DU makes all LTM decisions similar to L1 based measurements.(DU based solution)

For R18 LTM, following procedures are used at high level.

- LTM candidate cell pre-configuration via RRC

- Early DL/UL synchronization

  - MAC-CE based TCI state activation for DL synchronization.

  - PDCCH order tiggered early UL TA synchronization

- LTM cell switch execution upon receiving MAC-CE based cell switch command

- LTM cell switch completion.

R18 LTM HO latency reduction benefits will come primarily due to RACH-less LTM which requires early DL/UL synchronization before network sending MAC-CE based cell switch command and is depicted in below figure from TS 38.300, Annex G.

CU trigger based solution analysis:

DU is responsible for PHY/MAC scheduling operations and in case of R18 LTM, DU PHY/MAC scheduler is responsible when to trigger MAC-CE based TCI state activation, PDCCH order trigger and MAC-CE based cell switch command as well.

If CU makes LTM HO decision based on L3 measurements received from UE, there are multiple aspects need to be considered and decision should be based on careful system level end to end performance based technical analysis and  decision should not be based on legacy L3 HO decisions are made by CU and therefore follow the same principle for L3 measurements based LTM as well. This is not correct thing to do for LTM cell switch.

If CU has to make decision about when to trigger MAC-CE based TCI activation, PDCCH order for UL TA Acquisition and MAC-CE based cell switch command, basically CU is taking role of DU’s PHY/MAC scheduling decisions which is also violation of CU-DU functional split. In split gNB architecture, there are no instances where DU PHY/MAC scheduling and timing functions are controlled by CU.

Observation 6 : CU triggering  F1-AP signalling messages towards DU for MAC-CE based TCI state activation, PDCCH order and MAC-CE Cell switch command adds additional complexity without adding any real LTM performance benefit. DU shall be responsible for PHY/MAC scheduling decisions and CU taking control of DU scheduling functions is violation of CU-DU functional split for split gNB.

First let us analyse, what are issues associated with different F1-AP triggers sent from CU to DU.

“Pre DL-UL Sync and Cell Switch Command” as single trigger from CU to DU:

In this scenario, upon receiving single F1-AP trigger code point, DU is expected to tigger MAC-CE TCI state activation, early UL TA synchronization and followed by MAC-CE Cell Switch Command (CSC) including TA value.

In FR1 LTM, handover can be either Intra Freq or Inter Freq. early DL/UL synchronization caused delay and interruption  is dependent on whether it is intra or inter frequency LTM HO.

For MAC-CE based TCI activation for DL time/freq synchronization typically takes a minimum of ~25ms delay due to requirement of one SSB occasion for time/freq synchronization and it can cause some interruption as well. Only DU knows the timing of next immediate SMTC and SSB timing, where as CU does not know timing relationships. If MAC-CE can not be delivered to UE in 1st HARQ transmission then DU MAC will perform HARQ Re-transmission and this adds additional delay in addition to minimum of ~25ms. Total MAC-CE TCI state activation delay will be HARQ delay +~ 25ms.

If DL TCI state activation is skipped, it negatively impacts the early UL TA synchronization interruption and CSC interruption because UE has to process an additional SSB occasion to get DL T/F synchronization.

Using this single code point option, it is not possible to activate/deactivate MAC-CE TCI state for different candidate cells and it is important to allow flexibility to activate/deactivate TCI state due to the fact that UEs will support limited number of active TCI states based on UE capability and in dense deployments candidate cells/beams radio channel conditions change dynamically depending on UE mobility situation.

Observation 7 : CU triggering single F1-AP signalling message with single code point towards DU does not allow to activate/deactivate MAC-CE TCI state for multiple candidate cells and per cell level UL pre-sync cannot be performed in advance and thus defeats the purpose of early DL/UL sync.

For intra frequency UL TA pre-synchronization, UE side interruption may be very small. For inter frequency UL TA pre-synchronization (assuming UE UL contention free RACH preamble transmission to candidate cell is not within configured UE UL BWP), it can add approximately 5-10ms interruption at UE side and additional delay based on next available SSB, RACH occassion. Additionally, for both intra-frequency and inter-frequency cases, candidate cell DU has to send estimated UL TA value to CU and CU has to forward back to source cell DU. This latency may be variable depending on deployment dependent F1 delay (typical round trip F1 delay observed in some network is approximately 10-15ms and it can be longer in some deployments) and processing delay of candidate DU, CU forwarding delay and source DU processing delay as well.

If source DU/Cell does not get any TA information via F1 within implementation based timer, source cell may send another PDCCH order to UE to trigger PRACH preamble re-transmission with additional power and again source DU/cell has to wait for TA information from candidate DU. Note that CU does not know anything about TA value provided by candidate DU/Cell due to the fact that F1-AP TA Information transfer messages are non-UE specific messages and which preambles are allocated for given UE is only known to source DU/cell.

If TA value is received by source cell, then it sends MAC-CE CSC to UE including UL TA value. Based on CSC command provided TA value, UE triggers RACH-less LTM. If TA value is not provided to UE in CSC, UE will do RACH-based LTM and does not get benefit of LTM interruption reduction during LTM CSC execution. If pre-DL/UL sync is already performed then CSC execution causes interruption of ~13ms approximately. If pre-DL TCI state is not activated then it adds an additional 20ms interruption during CSC execution i.e. LTM CSC interruption of around 13ms + 20ms = 33ms.

Thus, performing these 3 steps in sequence causes DU to delay MAC-CE based cell switch command when compared to legacy L3 HO, increased LTM HO execution time and interruption defeats the purpose of introducing early DL/UL synchronization for LTM and it may even be longer than L3 based HO interruption due to involvement of network side UL TA information exchange delay. If DU skips early DL/UL synchronization to speed up MAC-CE based cell switch command, then UE has to perform RACH-based LTM, which causes additional LTM HO interruption and LTM latency reduction gains will be lost.

Due to delayed LTM MAC-CE CSC execution compared to legacy L3 HO, it is possible that UEs serving cell radio channel conditions may further deteriorate and may cause UE declaring RLF as well and it negatively impacts the UE performance.

Based on above analysis, it is not desirable/not beneficial for CU to send single trigger code point to DU to trigger all 3 events in sequence.

Observation 8: CU does not have any timing information related to SMTC/SSB availability, RACH Occasion , TA information and whether DL/UL Sync is successful or not is not known to CU and only DU knows all this information.

Observation 9: CU triggering single F1-AP signalling message with single code point towards DU for triggering MAC-CE based TCI state activation, PDCCH order and MAC-CE Cell switch command causes additional delay for DU to trigger MAC-CE based cell switch command , causes long delay to execute CSC and adds all interruptions in a sequence manner and deteriorating serving cell conditions may cause UE declaring RLF on serving cell and defeats the purpose of early DL/UL synchronization for LTM UE.

“DL TCI state activation, early UL TA sync and Cell Switch Command” as separate triggers from CU to DU:

With this option, it allows atleast some flexibility for CU based solution to trigger DL TCI state activation/deactivation, UL TA pre-synchronization and Cell Switch command independently. When more than one LTM candidate cell is configured for LTM UE, it allows flexibility to activate/deactivate DL TCI state, early UL TA synchronization in advance to sending cell switch command to a single cell. This means that there are additional F1-AP signalling messages but benefit is that there is no need for DU to perform DL TCI state activation, UL TA pre-sync and CSC in sequence and atleast addresses the drawback of having single code point trigger. But another drawback is that CU does not know whether DU successfully performed MAC-CE TCI state activation/deactivation, early TA synchronization due to lack of feedback from DU to CU.

Based on current RAN4 requirements, For PRACH transmission during early UL TA sync, if the UE is not configured with L1-RSRP measurements for the candidate cell at all, it will always add one SSB reception timeline delay irrespective of whether the DL TCI state associated with the PRACH has been activated or not, as shown in the second column in the table below.

| TSSB | L1-RSRP measurement is NOT configured | L1-RSRP measurement is configured |  |
| --- | --- | --- | --- |
|  |  | TCI state is NOT activated and L1-RSRP meas cycle > 160ms | Otherwise |
| FR1 LTM candidate cell | Tfirst-SSB_RACH | Tfirst-SSB_RACH | 0 |
| FR2 LTM candidate cell | Tfirst-SSB_RACH | Tfirst-SSB_RACH |  |
| Tfirst-SSB_RACH - If SSB is within active BWP, time to the first SSB occasion, after [1slot from] the end of the slot of the PDCCH - Otherwise, time to the first SSB occasion, overlapped with MGL after [2]ms and [1slot from] the end of the slot of the PDCCH |  |  |  |

However, even for this approach, since CU does not know the timing relation between MAC-CE TCI state activation command and availability of next SMTC/SSB occasion and DU has to make all these timing decisions. For early UL TA sync, CU does not know timing relation of next available SMTC/SSB occasion of candidate cells, RACH Occasion (RO) and TA is not visible to CU. DU has to handle all these tasks because CU has requested DU to perform UL TA pre-sync. In fact it is serving cell/DU responsibility to keep track of when and for which candidate cells DL/UL TA Sync is performed and TA value as well. There is no real performance benefit of this solution either when compared to DU autonomously making decisions for DL/UL sync and Cell Switch Command based on UEs L3 measurement reports received from CU.

Observation 10: CU sending separate F1-AP singling messages to DU for DL TCI state activation, UL TA pre-sync and cell switch command offers some flexibility but it DOES NOT provide any additional LTM performance benefits compared to DU autonomously making decisions based on L3 measurements.

For LTM Mobility, CU determines candidate cells to be configured for LTM UE based on L3 measurement reports. During LTM preparation phase, CU proposes candidate cells to DU and DU is expected to perform admission control based on available radio resources and provides configuration of all requested candidate cells or subset of CU proposed candidate cells. Then CU provides all candidate cells configuration to UE via RRC signalling procedure. For all configured cells, at the time of sending MAC-CE based CSC to UE, there is no further admission control needed. This is same philosophy used for L1 based LTM, legacy CHO as well. The same principle applies to both R18 L3 measurement based LTM and R19 C-LTM as well. Due to radio resource unavailability, if any configured candidate DU/Cell will not be able to admit anymore new UEs into its cell, candidate DU/cell is expected to send LTM cancelation request to CU and CU releases that particular candidate cell configuration for LTM UE. Thus, it is not correct to say that CU will perform admission control before sending cell switch command, so it CU has to control L3 measurement based LTM.

Observation 11: For both L3 and L1 measurement based LTM, admission control will be performed by candidate cells at the time of candidate cell preparation and MAC-CE Cell Switch Command will be sent to UE for any of the candidate cells without any additional admission control.

DU based solution analysis:

If CU simply forwards L3 measurements received from UE to DU, it is simple for DU to process L3 filtered measurement reports and DU schedular autonomously can make decisions whether and when to trigger MAC-CE based TCI state activation/deactivation, PDCCH order based early UL TA acquisition and MAC-CE based cell switch command as DU knows all associated timing and scheduling information.  This simplifies both CU and DU implementation and DU will keep control of when to trigger these 3 events for LTM UE. As we already explained above, DU should be able to process RRC based L3 measurements and DU is not required to perform any additional de-filtering for L3 filtered radio measurements. i.e RSRP, RSRQ and SINR. Based on RRC configuration, it is already possible for UE to report both beam level and cell level measurements, which DU will better understand than CU. Note that for L1 measurements, UE reports measurements at beam level for a number of cells and is controlled by UE capability. In another alternative, it is possible that CU can parse RRC L3 measurement results and provide both serving cell and candidate cell measurements to DU as F1-AP IEs. This enables CU to process L3 measurement reports and still DU will make its own decisions based on processed L3 measurement information received from CU.

In DU implementation, same LTM algorithm can be better optimized for both L1 and L3 measurement report based LTM and meet common LTM RAN4 timeline and interruption requirements for UE to maximize LTM performance.

Observation 12 : CU forwarding L3 filtered measurements to DU (either parsed or octet string) and DU scheduler can autonomously make decision about when to trigger DL/UL pre-synchronization and MAC-CE based cell switch command depending on either L3 or L1 measurements.

Analysis for UEs supporting Intra-Frequency L1 measurements and Inter-Frequency L3 measurements:

Whether to support intra-frequency and inter-frequency L1 measurements by UE is based on its capability. This means some UEs may support only L1 measurement based intra-frequency LTM and L3 measurement based inter-frequency LTM. In such cases, if intra-frequency L1 measurements are configured then DU makes L1 measurement based intra-freq LTM decisions and whether CU or DU makes decision for inter-freq LTM needs to be analysed carefully.

Assume that CU makes decision for L3 measurement based LTM for inter-freq case and DU makes decision for L1 measurement based intra-freq LTM, then upon DU receiving F1-AP triggers from CU (for L3 based inter-freq LTM) and at the same time DU will be receiving intra-frequency L1 measurement reports from UE, DU should be allowed freedom to decide whether to perform intra-frequency LTM or Inter-frequency LTM. This means that in such mixed scenarios, DU should have visibility of both L3 measurements and L1 measurements as well to make proper decisions. Additionally, even if CU sends F1-AP trigger to DU, if DU decides to perform L1 measurement based intra-frequency LTM then DU should be allowed not to follow CU decision. F1-AP signalling should enable DU to inform CU about unable to perform L3 measurement based LTM.

Based on above discussion, we really do not see any benefits for CU based solution for either single code point trigger and atleast multiple code point solution offers some flexibility even though it is not adding any additional LTM performance benefits when compared to DU based solution.

Additionally, for the case of L1 based intra-freq LTM and L3 based inter-freq LTM, it is not effective for CU making decisions for inter-freq HO. This adds unwanted complexity and DU will have to make proper decision when both intra and inter freq LTM HO conditions are met at the same time. Instead, if L3 measurements are provided to DU, it can make either L1 or L3 based measurements. CU based solution causes fragmentation of L1/L3 measurement based LTM, FR2 fast Scell activation and R18 RACH-less decisions into different nodes and violates the principles of unified solution approach.

Observation 13: Adopting CU based solution for L3 measurement based LTM will create multiple node solution of LTM, Scell TCI state activation and which must be avoided.

Thus we propose DU based solution as single solution,which is also aligned with R18 fast FR2 Scell activation procedures.

Proposal 2: For L3 measurement based LTM, DU makes decision about when to trigger DL/UL pre-synchronization and MAC-CE based Cell Switch Command. (i.e., CU will not instruct DU when to trigger DL/UL pre-synchronization and MAC-CE based cell switch command).

Proposal 3: RAN3 to discuss following 2 options for sending L3 filtered measurements from CU to DU to enable DU autonomously make LTM pre-sync and Cell Switch Command decisions.

Option A: F1-AP message carries RRC L3 Measurement Results as Octet String

Option B: CU parses L3 RRC measurement results and provides both serving and candidate cell measurement results as F1-AP IEs.

Proposal 4: For L3 measurements based R18 LTM, specify F1-AP signalling enhancements to support DU based solution as baseline solution.  Corresponding option A CRs provided in R3-252149, R3-252150, R3-252151. Option B F1-AP CRs can be discussed further based on proposal 2 outcome.

# 3	Conclusion

In this contribution, we discussed how to support L3 measurement based LTM and following are key observations and proposals for discussion.

Observation 1 : L3 measurement based R18 LTM is supported only for FR1.

Observation 2 : The difference between L1 vs L3 measurements is only about usage of L1 filter  vs L3 filter by the UE. At receiving RAN node, there is no additional de-filtering needed and meaning of absolute value of L1-RSRP Vs L3-RSRP remains same. DU will understand radio measurement very well and DU will handle processing of L3 measurements based on implementation.

Observation 3 : control plane part of DU can understand RRC IEs and in many existing F1-AP signalling messages RRC IEs are already exchanged by CU and DU. In fact DU gets MeasResultList2NR IE within CU to DU RRC Information IE -> Handover Preparation Information -> RRM-Config -> MeasResultList2NR.

Observation 4: CU forwards several L3 SON reports such as RLF Report, RA Report, SHR, SPR as “RRC containers” in Access and Mobility Indication message over F1AP. This is based on the assumption that DU can understand RRC containers, otherwise there would be no reason to forward these SON reports as “RRC containers” to DU.

Observation 5: To support R18 fast Scell TCI state activation, DU should be able to process both L3 and L1 measurements and CU does not know when the DU sent Scell MAC-CE Activation command to UE.

Observation 6: In R18 RACH-less HO, L3 measurements are already provided from CU to DU via F1-AP to assist DU to prepare RACH-LessHO IE as part of CellGroupConfig IE.

Observation 7 : CU triggering  F1-AP signalling messages towards DU for MAC-CE based TCI state activation, PDCCH order and MAC-CE Cell switch command adds additional complexity without adding any real LTM performance benefit. DU shall be responsible for PHY/MAC scheduling decisions and CU taking control of DU scheduling functions is violation of CU-DU functional split for split gNB.

Observation 8 : CU triggering single F1-AP signalling message with single code point towards DU does not allow to activate/deactivate MAC-CE TCI state for multiple candidate cells and per cell level UL pre-sync cannot be performed in advance and thus defeats the purpose of early DL/UL sync.

Observation 9 : CU does not have any timing information related to SMTC/SSB availability, RACH Occasion , TA information and whether DL/UL Sync is successful or not is not known to CU and only DU knows all this information.

Observation 10 : CU triggering single F1-AP signalling message with single code point towards DU for triggering MAC-CE based TCI state activation, PDCCH order and MAC-CE Cell switch command causes additional delay for DU to trigger MAC-CE based cell switch command , causes long delay to execute CSC and adds all interruptions in a sequence manner and deteriorating serving cell conditions may cause UE declaring RLF on serving cell and defeats the purpose of early DL/UL synchronization for LTM UE.

Observation 11: CU sending separate F1-AP singling messages to DU for DL TCI state activation, UL TA pre-sync and cell switch command offers some flexibility even though it DOES NOT provide any additional LTM performance benefits compared to DU autonomously making decisions based on L3 measurements.

Observation 12 : CU forwarding L3 filtered measurements to DU (either parsed or octet string) and DU scheduler can autonomously make decision about when to trigger DL/UL pre-synchronization and MAC-CE based cell switch command depending on either L3 or L1 measurements.

Observation 13 : CU forwarding L3 filtered measurements to DU simplifies F1 signalling design and DU scheduler can autonomously make decision about when to trigger DL/UL pre-synchronization and MAC-CE based cell switch command depending on either L3 or L1 measurements.

Observation 13: Adopting CU based solution for L3 measurement based LTM will create multiple node solution of LTM, Scell TCI state activation and RACH-less HO and which must be avoided.

Thus, based on above observations, we propose

Proposal 1 : In split gNB Architecture, one network node shall be allowed to process and trigger fast FR2 Scell TCI state activation and LTM HO decisions for both L1 and L3 measurements in both FR1 and FR2.

Proposal 2: For L3 measurement based LTM ,DU makes decision about when to trigger DL/UL pre-synchronization and MAC-CE based Cell Switch Command. (i.e., CU will not instruct DU when to trigger DL/UL pre-synchronization and MAC-CE based cell switch command).

Proposal 3: RAN3 to discuss following 2 options for sending L3 filtered measurements from CU to DU to enable DU autonomously make LTM pre-sync and Cell Switch Command decisions.

Option A: F1-AP message carries RRC L3 Measurement Results as Octet String

Option B: CU parses L3 RRC measurement results received from UE and provides both serving and candidate cell measurement results as F1-AP IEs.

Proposal 4: For L3 measurements based R18 LTM, specify F1-AP signalling enhancements to support DU based solution as baseline solution.  Corresponding option A CRs provided in R3-252149, R3-252150, R3-252151.  Option B F1-AP CRs can be discussed further based on proposal 2 outcome and is provided in Annex for reference.

# 4	Reference

[1] R2-2407606:	L3 measurement based LTM support over F1

[2] R2-2409373:	L3 measurement based LTM support over F1

[3] R3-245710 :   Reply LS on L3 measurement based LTM support over F1

[4] R3-245626 : Discussion on L3 measurement based LTM support over F1 (Huawei)

[5] R3-245627 : Support of L3 measurement based LTM over F1 (Huawei)

[6] R3-245629 : Support of L3 measurement based LTM over F1 (Huawei)

[7] R3-245631: Support of L3 measurement based LTM over F1 (Huawei)

[8] R3-244090: LTM interwork with L3 measurements (Nokia)

[9] R3-250563: Support of L3 measurement based LTM over F1_stage2 CR for DU based solution

[10] R3-250565: Support of L3 measurement based LTM over F1 for DU based solution

[11] R3-250566 : Stage-2 Support of L3 measurement based LTM for DU based solution

[12] R3-250355: (draft CR to F1AP) Discussion on L3 measurement report for SCell activation (CATT, CMCC,CT)

[13] R4-2314338: LS on FR2 SCell activation enhancements

[14] R2-2313936: Introduction of FR2 SCell enhancements (RRC CR)

[15] R2-2313937: Introduction of FR2 SCell enhancements (MAC CR)

[16] RP-250790 : Reply LS on L3 measurement based LTM support over F1

[17] R3-250879 : Reply LS on L3 measurement based LTM support over F1

# 4	Annex: F1-AP Stage-3 TP for TS 38.473.

### 8.3.X	LTM Assistance Information

#### 8.3.X.1	General

The purpose of the LTM Assistance Information procedure is to enable the gNB-CU to inform the gNB-DU about the L3 measurement results of the candidate cells. The procedure uses UE-associated signalling.

#### 8.3.X.2	Successful Operation

Figure 8.3.11.2-1: LTM Assistance Information procedure for L3 Measurement based LTM. Successful operation.

The gNB-CU initiates the procedure by sending a LTM ASSISTANCE INFORMATION message.

Upon reception of the LTM ASSISTANCE INFORMATION message, the gNB-DU shall, if supported, use the assistance information for triggering the L3 measurement based LTM procedure.

#### 8.3.X.3	Unsuccessful Operation

Not applicable.

/************Next change*************************/

Method 1:

#### 9.2.2.X1 	LTM ASSISTANCE INFORMATION

This message is sent by the gNB-CU to inform the gNB-DU about the LTM assistance information.

Direction: gNB-CU  gNB-DU

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.3.1.1 |  | YES | reject |
| gNB-CU UE F1AP ID | M |  | 9.3.1.4 |  | YES | reject |
| gNB-DU UE F1AP ID | M |  | 9.3.1.5 |  | YES | reject |
| Serving Cell Measurement List |  | (1 … maxNoofServingCells) |  |  |  |  |
| >Serving Cell ID | M |  | NR CGI 9.3.1.12 |  | - | - |
| >> L3 Measurement Result | M |  | OCTET STRING | Includes the MeasResultNR IE as defined in subclause 6.3.2 in TS 38.331 [8]. | - | - |
| LTM Candidate Cell Measurement List |  | (1 … maxNoofLTMCells) |  |  |  |  |
| >Candidate Cell ID | M |  | NR CGI 9.3.1.12 |  | - | - |
| >>L3 Measurement Result | M |  | OCTET STRING | Includes the MeasResultNR IE as defined in subclause 6.3.2 in TS 38.331 [8]. | - | - |

| Range bound | Explanation |
| --- | --- |
| maxNoofServingCells | Maximum no. of NR Serving Cells, the maximum value is 32. |
| maxNoofLTMCells | Maximum no. of LTM Candidate Cells, the maximum value is 8. |

Method 2:

#### 9.2.2.X2 	LTM ASSISTANCE INFORMATION

This message is sent by the gNB-CU to inform the gNB-DU about the LTM assistance information.

Direction: gNB-CU  gNB-DU

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.3.1.1 |  | YES | reject |
| gNB-CU UE F1AP ID | M |  | 9.3.1.4 |  | YES | reject |
| gNB-DU UE F1AP ID | M |  | 9.3.1.5 |  | YES | reject |
| Serving Cell Measurement List |  | (1 … maxNoofServingCells) |  |  |  |  |
| >Serving Cell ID | M |  | NR CGI 9.3.1.12 |  | - | - |
| >> L3 NR Measurement Result | M |  | 9.2.2.Y |  | - | - |
| LTM Candidate Cell Measurement List |  | (1 … maxNoofLTMCells) |  |  |  |  |
| >Candidate Cell ID | M |  | NR CGI 9.3.1.12 |  | - | - |
| >>L3 NR Measurement Result | M |  | 9.2.2.Y |  | - | - |

| Range bound | Explanation |
| --- | --- |
| maxNoofServingCells | Maximum no. of NR Serving Cells, the maximum value is 32. |
| maxNoofLTMCells | Maximum no. of LTM Candidate Cells, the maximum value is 8. |

#### 9.2.2.Y 	L3 NR Measurement Result

The L3 NR Measurement Result IE contains information about Cell and Beam level measurements information.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Physical Cell ID |  |  | OCTET STRING | Includes the PhysCellID IE as defined in subclause 6.3.2 in TS 38.331 [8]. | - | - |
| Cell Level Measurements |  |  |  |  |  |  |
| >SSB based Measurements | O |  | OCTET STRING | Includes the MeasQuantityResults as defined in subclause 6.3.2 in TS 38.331 [8]. | - | - |
| >CSI-RS based Measurements | O |  | OCTET STRING | Includes the MeasQuantityResults as defined in subclause 6.3.2 in TS 38.331 [8]. | - | - |
| Beam Level Measurements |  |  |  |  |  |  |
| SSB Based Measurement List |  | (0 … maxNoofSSBBeams) |  |  |  |  |
| >SSB ID | O |  | INTERGER (1..64) |  | - | - |
| >> SSB Measurement Result | O |  | OCTET STRING | Includes the MeasQuantityResults as defined in subclause 6.3.2 in TS 38.331 [8]. | - | - |
| CSI-RS Based Measurements List |  | (0 … maxNoofNZPCSIRSBeams) |  |  |  |  |
| >CSI-RS Resource ID | O |  | INTERGER (1..192) |  | - | - |
| >> CSI-RS Measurement Results | O |  | OCTET STRING | Includes the MeasQuantityResults as defined in subclause 6.3.2 in TS 38.331 [8]. | - | - |

| Range bound | Explanation |
| --- | --- |
| maxNoofSSBBeams | Maximum no. of SSB Beams, the maximum value is 64. |
| maxNoofNZPCSIRSBeams | Maximum no. of CSI-RS Resources , the maximum value is 192. |

Note that once  we agree stage-2 and high level message structure then ASN.1 will be written later.