3GPP TSG RAN WG3#127bis	R3-252320

7th April – 11th April - 2025

Wuhan, China

Agenda Item:	13.2

Source:	China Telecom

Title:	Summary of offline discussion on MobilityEnh_LTM

Document for:	Approval

Introduction

CB: # MobilityEnh_LTM

- discuss the open issues and DC parts.

- capture the agreements and update the TPs

- capture the left open issues for next meeting.

(Moderator - CT)

Offline summary in R3-252320

For the Chairman’s Notes

Propose the following:

New BLCR for TS38.423 to support inter-CU LTM in DC – BLCR rapporteur Lenovo

Reply LS to SA3 on security handling in R3-252440 - Agreed

TP for TS38.300, R3-251533 rev [in R3-252424] – Agreed

TP for TS38.423, R3-251964 rev [in R3-252431] – Agreed

TP for TS38.473, R3-252118 rev [in R3-252433] – Agreed

TP for TS38.420, R3-251881 rev [in R3-252444] – Agreed

TP for TS38.470, R3-252434 – Agreed

TP for TS38.401, R3-251863 rev [in R3-252438] – Agreed

TP for TS37.340, R3-251837 rev [in R3-252439] – Agreed

TP for TS38.423 (for DC), R3-251947 rev [in R3-252445] – Agreed

Propose to capture the following:

Reword the online agreement to: For the initial inter-CU LTM preparation, the source gNB computes the KgNB*(s) (per candidate cell) for each candidate gNB, and forwards the {KgNB*, NCC} pair(s) to the candidate gNB via Handover Request message as legacy.

Reword the online agreement to: The new source gNB needs to provide the new KgNB*(s) to candidate gNBs via LTM configuration update procedure. FFS on the procedure design either per cell or per gNB.

Whether to define additional information from CU to DU to deliver NCC value.

Turn the WA into agreement: Introduce a new non-UE associated class 2 procedure on Xn, namely TA information Transfer message, to transfer the TA information from the candidate gNB-CU to the source gNB-CU.

Include the following information in the TA information transfer message:

- Candidate Cell ID

- TA value

- Preamble index

- RA-RNTI

FFS on the XnAP IDs.

Turn the WA into agreement: Use a single UE association (e.g. identified by a pair of {Source NG-RAN node UE XnAP ID IE and Target NG-RAN node UE XnAP ID IE}) for multiple LTM handover request to the same candidate gNB.

For subsequent LTM, the candidate gNB uses the old target UE XnAP ID (the target UE AP ID it previously allocated between it and the old serving gNB) to identify the UE context when receiving the LTM Configuration Update message from the new serving gNB. FFS on how to deliver the old target UE XnAP ID(s) to the new serving gNB.

Data Forwarding Information IE is a list including multiple data forwarding addresses from each candidate gNB(s).

Include the following information in SN Change Required (Source SN to MN) message:

- LTM request indication (explicit way)

- Suggested PScell list (implicit way in INM, FFS on explicit way)

- Measurement results (implicit way in INM)

- Upper limit number of PScell(s) for each candidate SN (FFS)

- CSI Resource configuration request (explicit way)

- Early sync information request (explicit way)

Include the following information in SN Addition Request (MN to Candidate SN) message:

- LTM request indication (explicit way)

- Suggested PScell list (implicit way in INM, FFS on explicit way)

- Measurement results (implicit way in INM)

- Upper limit number of PScell(s) for the candidate SN (explicit way)

- CSI Resource configuration request (explicit way)

- Early sync information request (explicit way)

It is the source SN to generate the LTM configuration ID mapping list for the accepted candidate PSCell(s), and send it to the MN for subsequent LTM via SN modification request acknowledge message. Then the MN forwards the LTM configuration ID mapping list to the candidate SN via SN Modification Request message.

For SN-initiated inter-CU SCG LTM, the source SN can provide the SCG reference configuration via SN Change Required message. If the source SN does not provide the SCG reference configuration, MN can request SCG reference configuration from one of the candidate SN(s), FFS on whether MN can request SCG reference configuration from the source SN.

The TA information transfer message can be reused and enhanced to transfer the TA information from the candidate SN to the source SN via the MN.

Include the target PSCell ID and corresponding TCI State ID(s) in the Cell Switch Notification message.

Discussion

During the online session, we reached following agreement and open issues:

For the initial inter-CU LTM preparation, the source gNB computes the KgNB* for each candidate gNB, and forwards the {KgNB*, NCC} pair to the candidate gNB via Handover Request message as legacy. The new source gNB needs to provide the new KgNB* to candidate gNBs via LTM configuration procedure. The new NCC value needs to be sent from gNB-CU to the gNB-DU via UE context modification procedure. Reuse the LTM configuration update procedure to transfer UE’s 5G security capabilities and/or UE’s UP security policy from the new serving gNB to the other candidate gNBs in subsequent LTM. Introduce two new Class-1 UE-associated procedures over F1AP to support the source gNB-DU to trigger the activation/deactivation of the SP CSI-RS transmission in the candidate cell(s). Introduce a new class-1 UE-associated procedures over XnAP for the SP CSI-RS activation/deactivation. Open issues on the granularity of the CSI-RS resource/resource set, and need to be checked with RAN2 progress.

In this offline, we plan to continue the discussion towards inter-CU LTM including the DC procedures and update TPs based on the agreements we reached.

Support of inter-CU LTM in Standalone scenario

Security key handling

During the online session, LGE raise a comment on the additional information from CU to DU to deliver NCC value, while there are other company concerns other alternative for DU to identify the intra-CU/inter-CU case:

Reuse the LTM configuration update procedure to transfer UE’s 5G security capabilities and/or UE’s UP security policy from the new serving gNB to the other candidate gNBs in subsequent LTM. LGE: need additional information from CU to DU to deliver NCC value. Lenovo: candidate solution can be discussed offline. CATT: whether DU can identify the intra-gNB/inter-gNB switched by the cell id. QC, E///: there is no need to define additional information. Offline discuss on whether to define additional information from CU to DU to deliver NCC value.

In addition, according to RAN2 agreement in this meeting, the R19 set ID is used (i.e. for serving gNB-DU) to distinguish whether to send the NCC to the UEs, and RAN2 agreed to send an LS to RAN3:

- NCC is included in the LTM cell switch command MAC CE if the R19 set ID is different between the target cell and source cell. Conversely, if the R19 set ID is same for both cells, the NCC will not be included. [Lenovo]: Wonder if we need to send an LS to inform RAN2 progress? [Apple]: Support sending LS to RAN3 to inform the related RAN2 progress. [AT129b][109][MOB] (ZTE) Scope: Prepare LS to RAN3 to inform RAN2 progress that is related to security aspect. Intended outcome: LS to RAN3 in R2-2503012. Deadline: Comeback in CB session.

Since this is the first time for RAN3 to address this issue, moderator propose to left this as an open issue for next meeting. Companies are appreciated to keep this issue in mind, and encouraged to provide preference solution in the next meeting.

Question: Can we leave the additional information from CU to DU to deliver NCC value as the open issue for the next meeting?

Open issues for the next meeting: Whether to define additional information from CU to DU to deliver NCC value.

Update the online agreement into following:

For the initial inter-CU LTM preparation, the source gNB computes the KgNB*(s) (per candidate cell) for each candidate gNB, and forwards the {KgNB*, NCC} pair(s) to the candidate gNB via Handover Request message as legacy.

The new source gNB needs to provide the new KgNB*(s) (per candidate cell of the candidate gNB) to candidate gNBs via LTM configuration procedure.

Moderator summary:

For the initial inter-CU LTM preparation, the source gNB computes the KgNB*(s) (per candidate cell) for each candidate gNB, and forwards the {KgNB*, NCC} pair(s) to the candidate gNB via Handover Request message as legacy.

The new source gNB needs to provide the new KgNB*(s) to candidate gNBs via LTM configuration update procedure. FFS on the procedure design either per cell or per gNB.

SP CSI-RS configuration

During the online session, we confirmed to have new Class-1 UE associated F1AP and XnAP procedures to support the source gNB-DU to trigger the activation/deactivation of the SP CSI-RS transmission in the candidate cell(s). And there is an open issue on the granularity of the CSI-RS resource/resource set, and need to be checked with RAN2. Moderator propose the leave this open issue for the next meeting.

Open issues for the next meeting: Open issues on the granularity of the CSI-RS resource/resource set, and need to be checked with RAN2 progress.

Moderator summary:

Reference configuration

In the last RAN3 meeting, we reached following agreements and left an open issue for reference configuration for inter-CU LTM case:

The source gNB can generate reference configuration and provide a reference configuration for LTM in a Handover Request and LTM configuration update message. The candidate gNB indicates whether a LTM candidate configuration is a complete candidate configuration in the Handover Request Acknowledge and LTM configuration update acknowledge message. FFS on whether the source gNB can request a candidate gNB to provide a reference configuration.

During the online session, we discussion the following alternatives, and it is the common understanding that Approach 2 is the baseline approach and companies want to further check the feasibility of Approach1 and Approach3, therefore, moderator propose to leave this issue open for the next meeting, and companies are encouraged to provide preference approach in the next meeting.

Potential approaches for reference configuration are listed as below:

- Approach 1: Similar as F1AP, the source gNB can request a candidate-gNB to provide a reference configuration. [Nokia, Google, CATT, China Telecom, ZTE, Lenovo, Huawei, NTT Docomo, Samsung, CMCC]

- Approach 2: For inter-CU LTM, only the source gNB can provide the reference configuration. [Qualcomm, Ericsson]

- Approach 3: If the source CU does not provide the reference configuration, it is up to the candidate DU to provide the reference configuration to the source CU and the source DU can decide which one to adopt as a reference configuration. [LG Electronics]

Open issues for the next meeting: FFS on whether the source gNB can request a candidate gNB to provide a reference configuration.

Moderator summary:

TA Information Transfer

In the last RAN3 meeting, we reached a working assumption to introduce a new non-UE associated class 2 procedure on Xn, which is also aligned with the F1AP.

WA: Introduce a new non-UE associated class 2 procedure on Xn, namely TA information Transfer message, to transfer the TA information from the candidate gNB-CU to the source gNB-CU.

According to companies’ contributions, many companies propose to turn this working assumption into agreements, while in [R3-251963, Ericsson], it proposed to use UE-associated class-2 procedure to transfer the list of TA values. Since we have been stuck in this issue for several meetings, and majority companies prefer to follow the principle of R18 F1AP design, moderator wants to check we can reach agreement on this issue and agree the following proposal:

Proposal: Turn the WA into agreement: Introduce a new non-UE associated class 2 procedure on Xn, namely TA information Transfer message, to transfer the TA information from the candidate gNB-CU to the source gNB-CU.

Moderator summary:

Turn the WA into agreement: Introduce a new non-UE associated class 2 procedure on Xn, namely TA information Transfer message, to transfer the TA information from the candidate gNB-CU to the source gNB-CU.

In addition, we need to discuss what information needed to be carried in this TA information Transfer message, it is obvious that the parameters (e.g. candidate Cell ID, TA value, Preamble index, and RA-RNTI) in F1AP TA Information Transfer messages are reused by the Xn TA Information Transfer message for inter-CU LTM. Besides, according to R3-251963 [Ericsson], it proposes to include the UE XnAP IDs of the source and target gNB in the TA information transfer message. Here we can check what information to be included in the TA information transfer message, for example:

- Candidate Cell ID

- TA value

- Preamble index

- RA-RNTI

- UE XnAP IDs

And further check if any other information is needed to be included.

Moderator summary:

CATT: How can the candidate cell know the XnAP IDs.

Include the following information in the TA information transfer message:

- Candidate Cell ID

- TA value

- Preamble index

- RA-RNTI

FFS on the XnAP IDs.

Handling of multiple UE associations

Issue1: UE association setup during initial handover preparation

In the last RAN3 meeting, we reached a working assumption that use a single UE association for multiple LTM handover request to the same candidate gNB.

WA: Use a single UE association for multiple LTM handover request to the same candidate gNB.

According to companies’ proposals, there are following approaches to achieve single UE association in parallel transaction:

Approach 1: Add an LTM Indicator IE and a Target NG-RAN node UE XnAP ID IE to the HANDOVER REQUEST. [R3-251963, Ericsson]

- an IE similar to the LTM Indicator IE included in F1AP, with two codepoints, e.g., “first-candidate”, “other-candidate”,

- a Target NG-RAN node UE XnAP ID IE, present if the value of the LTM Indicator is “other-candidate”.

| >LTM Indicator | M |  | ENUMERATED (first-candidate, other-candidate, …) |  | - |  |
| --- | --- | --- | --- | --- | --- | --- |
| >Target NG-RAN node UE XnAP ID | C-ifOtherCand |  | NG-RAN node UE XnAP ID 9.2.3.16 | Allocated at the target NG-RAN node. | - |  |

| Condition | Explanation |
| --- | --- |
| ifOtherCand | This IE shall be present if the LTM Indicator IE is present and set to "other-candidate". |

Approach 2: Use the same {Source NG-RAN node UE XnAP ID IE and Target NG-RAN node UE XnAP ID IE} pair as well as different Target cell ID to indicate multiple candidate cells within the same candidate gNB. [R3-251881, ZTE]

If the Target NG-RAN node UE XnAP ID IE is contained in the LTM Handover Information Request IE included in the HANDOVER REQUEST message, the target NG-RAN node shall consider that the source NG-RAN node has requested the LTM information for the same UE-associated signaling connection associated with the Target NG-RAN node UE XnAP ID allocated in the previous Handover Preparation procedure.

Approach 3: Allow parallel transactions using the same source UE AP ID only, with each identified by a unique target cell ID. [R3-251742, LG Electronics]

### 8.2.1 Handover Preparation #### 8.2.1.1 General This procedure is used to establish necessary resources in an NG-RAN node for an incoming handover. If the procedure concerns a conditional handover, parallel transactions are allowed. Possible parallel requests are identified by the target cell ID when the source UE AP IDs are the same. If the procedure concerns an LTM, parallel transactions are allowed using the same source UE AP ID only. In this case, possible parallel requests are identified by the target cell ID, and only one UE-associated signaling connection is established and maintained between the source NG-RAN node and the target NG-RAN node. The procedure uses UE-associated signalling.

Approach 4: To use the existing mandatory NG-C UE associated signalling reference IE in the XnAP HANDOVER REQUEST as the single UE association for multiple LTM handover request to the same candidate gNB. [R3-251670, NEC]

Here we can discuss the feasibility to use only one UE association for one UE when multiple LTM candidate cells are prepared in the same gNB based on the above approaches, and further check whether to chance the WA in to agreement:

Proposal: Turn the WA into agreement: Use a single UE association for multiple LTM handover request to the same candidate gNB.

Moderator summary:

Turn the WA into agreement: Use a single UE association (e.g. identified by a pair of {Source NG-RAN node UE XnAP ID IE and Target NG-RAN node UE XnAP ID IE}) for multiple LTM handover request to the same candidate gNB.

Issue2: UE association setup after each LTM cell switch (i.e. UE association between the new serving gNB and candidate gNBs)

For subsequent LTM, after each cell switch, the UE association needs to be established between the new serving gNB and candidate gNB(s). In RAN3#126 meeting, it was agreed to reuse the LTM Configuration Update procedure to establish UE association between the new source gNB and the other candidate gNB(s) after each inter-CU LTM Cell Switch to support subsequent LTM.

RAN3 #126 To support subsequent LTM, the LTM Configuration Update procedure is reused to establish UE association between the new source gNB and the other candidate gNB(s) after each inter-CU LTM Cell Switch.

For subsequent LTM, the candidate gNB should identify that the UE is the same as the previously associated between the old serving gNB and the current candidate gNB. Since there is only one UE association among source gNB and candidate gNB, the question needs to be discussed is how the candidate gNB identify that the UE is the same as the previously associated between the old serving gNB and the current candidate gNB. Based on companies’ proposals, there are following options:

- Option 1: Use the old target UE XnAP ID for each candidate cell to the corresponding candidate gNB.

- Option 2: Use the old source UE XnAP ID for each candidate cell to the corresponding candidate gNB.

- Option 3: Use the pair of <old source UE XnAP ID, old target UE XnAP ID> for each candidate cell to the corresponding candidate gNB via the LTM Configuration Update message after the LTM cell switch.

- Option 4: All the above three options are feasible.

In this session, we can discuss which option to adopt.

Moderator summary:

HW, CATT, Lenovo, ZTE, Nokia, Samsung, Ericsson: support option1.

NEC: support option3.

Use the old target UE XnAP ID for each candidate cell(s) to the corresponding candidate gNB.

FFS on how to deliver the candidate UE XnAP ID(s) to the new serving gNB.

Data forwarding

To support data forwarding in case of inter-CU LTM, the Source gNB needs to transfer information regarding PDU Session Resource Information List of all other Candidate gNBs to the Target gNB to execute data forwarding for subsequent LTM. Similarly, RAN3 has agreed this information to be included within LTM CONFIGURATION UPDATE message with the exact timing left to implementation. However, some Stage-3 details were left open.

Reuse the LTM Configuration Update procedure to sync up configurations among candidate gNBs for subsequent LTM, including early sync configuration, configuration ID, and data forwarding addresses, etc. It is up to the network implementation when the sync up is performed: during the preparation step, or during the cell switch execution step, or after successful cell switch. Normal data forwarding may be initiated after the source gNB decides to trigger the LTM cell switch for the UE, and when exactly it is initiated is left to implementation.

In latest BL CR for XnAP 38.423, the existing Data Forwarding Info IE defined in section 9.2.1.16 was introduced to the LTM CONFIGURATION UPDATE message with remark of “FFS if list is needed”. Many companies propose to update this IE to contain a list of Data Forwarding Info IEs, each corresponding to a different target gNB to where data forwarding should be performed. Here, we can check whether the following proposal is agreeable:

Proposal: Data Forwarding Information IE is a list including multiple data forwarding addresses from each candidate gNB.

Moderator summary:

Nokia: Also delver the data forwarding information in the cell switch notification message.

Ericsson: this is related to the granularly of LTM configuration update procedure.

Data Forwarding Information IE is a list including multiple data forwarding addresses from each candidate gNB(s).

Support of inter-CU LTM in NR-DC scenario.

SN initiated inter-CU SCG LTM without MCG changes

The figure of SN initiated inter-CU SCG LTM procedure from the endorsed running CR R2-2410928 in RAN2 #128 is cited as below:

Figure 10.5.2-5 SN initiated inter-SN SCG LTM

## Issue1: Enhancement on SN Change Required and SN Addition Request message

In the last RAN3 meeting, we reach following agreement related to the inter-CU SCG preparation:

For SN initiated inter-CU SCG LTM, the source SN initiates the inter-CU SCG LTM preparation procedure by sending an SN Change Required message to the MN. Within the list of cells suggested by the source SN, the candidate SN provides the SCG part configuration of each candidate PSCell and may also provide the L1 RS (e.g. a list of SSB or a list of CSI-RS) configuration for L1 measurement, early UL sync configuration or TCI-state configuration, to the MN, via SN addition request ACK message.

In the last meeting, we agreed that it is the source SN to initiate the inter-CU SCG LTM preparation procedure by sending an SN Change Required message to the MN. However, what information should be provided from source SN to the candidate SN(s) via MN is not discussed, based on companies’ proposal, there are following alternatives:

  - Suggested candidate PSCells and the associated measurement results, in the CG-Config and CG-ConfigInfo: Google,

  - Upper limit for the number of PSCells for each candidate SN: CATT, CT,

  - Suggested candidate PSCell ID list: Huawei

  - Request for CSI resource configuration: Lenovo

  - Early sync configuration request: Lenovo, Huawei

  - Others (i.e. CSI resource configuration)

In addition, RAN2 agreement on inter-node message is cited as below for reference:

RAN2#128(2024.11):

11.	RAN2 assumes that how to indicate the list of candidate PSCells from source SN to MN is up to RAN3. From RAN2 perspective, in INM, source SN may send measurement results of candidate PSCells to the MN. The MN then forwards the measurement results to the candidate SN(s), and then the candidate SN(s) determines the LTM candidate cells based on the measurement results and the upper limit for the number of PSCells that can be prepared by each candidate SN. The existing IEs defined in INM can be reused as a baseline.

Based on the above information, we can discuss what information is needed to transmitted from source SN to MN via Handover Required message, and then from MN to candidate SN(s) via SN Addition Request message, implicit way or explicit way, the candidate information is listed as below:

- LTM request indication

- Suggested PScell list

- Measurement results

- Upper limit number of PScell(s)

- CSI Resource configuration request

- Early sync information request

Moderator summary:

Include the following information in SN Change Required (source SN to MN)message:

- LTM request indication (explicit way)

- Suggested PScell list (implicit way in INM)

- Measurement results (implicit way in INM)

- Upper limit number of PScell(s) for each candidate SN (FFS)

- CSI Resource configuration request (explicit way)

- Early sync information request (explicit way)

Include the following information in SN Addition Request (MN to candidate SN) message:

- LTM request indication (explicit way)

- Suggested PScell list (implicit way in INM)

- Measurement results (implicit way in INM)

- Upper limit number of PScell(s) for the candidate SN (explicit way)

- CSI Resource configuration request (explicit way)

- Early sync information request (explicit way)

In addition, according to [R3-251837, CATT], it is necessary to have coordination between the MN and SN for transferring the LTM configuration ID since intra-CU SCG LTM and inter-CU SCG LTM may coexist. A reasonable way is to let the source SN generate the LTM configuration ID mapping list for the accepted candidate PSCell(s), and send it to the MN for subsequent LTM (i.e. in step6).

5. The MN sends the received L1 RS configuration, early UL/DL sync configuration of candidate cells to the source SN via SN Modification Request message. 6. The source SN generates the common CSI resource configuration for L1 measurement on candidate PSCells. The source SN sends the generated common CSI resource configuration and the updated source SCG configuration to the MN via SN modification request acknowledge message. The source SN sends the configuration ID mapping list for the accepted candidate PSCell(s) to the MN for subsequent LTM.

Here we can check if following Proposal1 is agreeable:

Proposal 1: It is the source SN to generate the LTM configuration ID mapping list for the accepted candidate PSCell(s), and send it to the MN for subsequent LTM.

Moderator summary:

It is the source SN to generate the LTM configuration ID mapping list for the accepted candidate PSCell(s), and send it to the MN for subsequent LTM via SN modification request acknowledge message. Then the MN forwards the LTM configuration ID mapping list to the candidate SN via SN Modification Request message.

## Issue2: SCG Reference configuration

According to RAN2 agreement, it is the SN (source or candidate) generates the SCG part of the reference configuration:

RAN2 #127bis agreement： The MN generates the MCG part of the reference configuration (if any), while the SN (source or candidate) generates the SCG part of the reference configuration. - The MN is responsible for the reference configuration generation for SN initiated inter-CU SCG LTM. It can be up to the NW implementation whether to include the MCG part. - The MN can request an SCG reference configuration from any of the involved SNs.

In [R3-251759, Qualcomm], it proposed that similar as in Rel-18 Subsequent CPAC, the MN can request an SCG reference configuration from either the source SN or a candidate SN. Similar proposal also provided in [R3-251701, Google] and [R3-252240, ZTE].

Here we can check if Proposal2 is agreeable:

Proposal 2. For inter-CU LTM in DC, the MN can request an SCG reference configuration from either the source SN or a candidate SN.

Moderator summary:

For SN-initiated inter-CU SCG LTM, the source SN can provide the SCG reference configuration via SN Change Required message. If the source SN does not provide the SCG reference configuration, MN can request SCG reference configuration from either source SN or one of the candidate SN(s).

## Issue3: TA Information Transfer

In step 13/14a, the UE performs DL/UL sync with the LTM candidate cell(s) before receiving the cell switch command. For the PDCCH order triggered early RACH. In step 14b, the candidate SN(s) needs to transfer the calculated TA value of candidate cell(s) to the source SN via the MN. The existing TA information transfer procedure via F1 interface can be extended to Xn interface to transfer the TA information between the MN and the source/candidate SN.

Here we can check if Proposal3 is agreeable:

Proposal 3: The TA information transfer message can be reused to transfer the TA information from the candidate SN to the source SN via the MN.

Moderator summary:

The TA information transfer message can be reused and enhanced to transfer the TA information from the candidate SN to the source SN via the MN.

## Issue4: Security key updates for SCG LTM

According to RAN2 agreement, the SCPAC-similar security update configuration is introduced for inter-CU SCG LTM:

RAN2#127bis agreement: - The SCPAC-similar security update configuration is introduced for inter-CU SCG LTM, i.e. similar to IEs sk-CounterConfiguration, servingSecurityCellSetId and securityCellSetId. The names of the new IEs are to be discussed in stage3.

In case of subsequent inter-CU SCG LTM, the configuration of the candidate PSCell remains in effect even after the UE has connected to the target cell, allowing the UE to connect to the same SN later without requiring any additional network reconfiguration. To support it, the MN needs to provide a list of unique security keys and sk-counter values for each SN to the UE, where each of these values is used when the UE connects to the target SCG. The candidate SN also needs to sync the security key and sk-counter value with the MN over the Xn interface.

To support subsequent SCG LTM, the security keys and sk-counter values for candidate SN needs to be transfer from MN to the candidate SN/target SN, based on companies’ proposals there are following two options:

- Option 1: MN sends a list of {S-key and SK-counter} to each of candidate SN via SN addition/modification request message. [Lenovo, ZTE]

- Option 2: MN sends security key and a list of sk-counters to the target SN the SN via the CELL SWITCH NOTIFICATION message. [Ericsson]

Here we can discuss which option to adopt.

Moderator summary:

CATT: why send a list, only one key is enough.

Skip this issue, leave it open.

## Issue5: Cell Switch Notification

In the last RAN3 meeting, we reach following agreement related to Cell Switch Notification procedure:

The Cell Switch Notification message can be reused from the source SN to the target SN via the MN. The detailed IE can be further discussed.

According to [R3-251862, China Telecom] and [R3-252119, Huawei], similar as inter-CU LTM in standalone scenario, the target PSCell ID and corresponding TCI State ID(s) need to be included in the Cell Switch Notification message.

Here we can check if Proposal4 is agreeable:

Proposal 4: Include the target PSCell ID and corresponding TCI State ID(s) in the Cell Switch Notification message.

Moderator summary:

Include the target PSCell ID and corresponding TCI State ID(s) in the Cell Switch Notification message.

## Issue6: Access Success

According to [R3-251817, CATT], in current BCLR for TS37.340, the step 23 ACCESS SUCCESS message is used for target gNB to inform source gNB that UE has accessed. However, RAN3 never defined the Access success in XnAP, and usually there is no directly interaction between source S-SN and T-SN. The key issue is whether need and how to inform S-SN that the UE access to T-SN successfully. In S-CPAC case, the MN will send a SN MODIFICATION REQUEST message to S-SN after receive the RRCReconfigurationcomplete from UE the main purpose of this step is to initial late data forwarding and update the data forwarding address.

According to [R3-251963, Ericsson], it proposed that at completion of SN initiated inter-CU SCG LTM, the target SN sends a HANDOVER SUCCESS to the MN, which forwards it to the source SN, to inform the source SN the UE successfully accessed the target cell.

Based on the above status, there are possible following options to be further discussed:

- Option 1: Reuse the SN MODIFICATION REQUEST message to inform the source SN that UE successfully accessed the target PSCell.

- Option 2: Reuse the HANDOVER SUCCESS message to inform the source SN the UE successfully accessed the target PSCell.

- Option 3: No need to inform the access success information to the source SN.

Here we can down select from the above options.

Moderator summary:

LG, Ericsson: Option2.

CATT: Prefer Option1.

Leave it open.

Avoid simultaneous MCG and SCG LTM

In RAN3 #123 meeting, RAN3 agreed that inter-CU LTM is not configured in both MCG and SCG at the same time, which is also confirmed by RAN2 agreement and the updated WID.

For coexistence of MCG and SCG LTM, following agreements were reached in RAN2#127bis meeting:

- RAN2 confirms to support coexistence of following cases, it is up to network implementation to ensure simultaneous execution for both MCG and SCG will not happen:

- Inter-MN LTM and intra-SN LTM

- Inter-SN LTM and intra-MN LTM

Besides, in RAN3#123bis meeting, we reached following agreement related to inter-CU LTM in NR-DC:

- Confirm the case that inter-CU LTM is not configured in both MCG and SCG at the same time.

Regarding this issue, we need to design solutions to avoid simultaneous configuration for inter-CU MCG and SCG LTM.

During the online section, we mentioned following two options:

- Option 1: The MN/source SN needs to inform the other node once it decides to initiate an inter-CU LTM.

- Option 2: The MN rejects the SN-initiated inter-CU SCG LTM request with a proper cause value.

- Option 3: Up to RAN2(i.e. implicit indication via Inter-Node RRC signaling).

In this section, we can down select from the above options.

Moderator summary:

Skip this issue.

TP/BLCRs

Prepare following TPs based the agreements in this meeting:

TP for 38.300 Nokia

TP for 38.423 Ericsson

TP for 38.473 Huawei

TP for 38.420 ZTE

TP for 38.470 Huawei

TP for 38.401 China Telecom (if needed)

Inter-CU SCG LTM:

New BLCR for TS38.423 to support inter-CU LTM in DC – BLCR rapporteur Lenovo

TP for 37.340 CATT

TP for 38.423 Lenovo

Conclusion, Recommendations [if needed]

If needed

References

| R3-251507 | LS on activation/deactivation of semi-persistent CSI-RS resource for LTM candidate cells (RAN1(Fujitsu)) | LS in |
| --- | --- | --- |
| R3-251525 | Reply LS on security handling for inter-CU LTM in non-DC cases (SA3(CATT)) | LS in |
| R3-251533 | TP (BL CR TS 38.300) Inter-CU LTM Procedure (Nokia) | other |
| R3-251582 | Discussion on Inter-CU LTM security (Nokia) | discussion |
| R3-251629 | Data forwarding for Inter-CU LTM DC scenarios (Ofinno, LLC) | discussion |
| R3-251630 | Issues on SN initiated inter-CU SCG LTM (Ofinno, LLC) | discussion |
| R3-251633 | Discussion on Inter-CU LTM (Rakuten Mobile, Inc) | discussion |
| R3-251670 | (TP to BL CR for TS 38.423 on Inter-CU LTM) Rel-19 inter-CU LTM issues (NEC) | other |
| R3-251671 | discussion Rel-19 inter-CU LTM in DC scenario (NEC) | discussion |
| R3-251707 | (TP to BL CR 38.423) Support subsequent mobility in inter-CU LTM (Google) | other |
| R3-251741 | (TP for NR_Mob_Ph4 TS 38.473) Discussions on key distribution and NCC delivery for Inter-CU LTM (LG Electronics Inc.) | other |
| R3-251742 | Discussions on key remaining aspects of Inter-CU LTM (LG Electronics Inc.) | discussion |
| R3-251759 | Signalling enhancements for Inter-CU LTM handover (Qualcomm Incorporated) | discussion |
| R3-251835 | (TP for 38.473, 38.423) Discussion for security handling on non-DC case (CATT) | discussion |
| R3-251836 | [Draft LS] Reply LS to SA3 on the security handling for inter-CU LTM in non-DC case (CATT) | LS out To: SA3 CC: RAN2 |
| R3-251837 | (TP for 37.340) Discussion for Inter-CU LTM (CATT) | discussion |
| R3-251862 | Discussion on signaling enhancement for inter-CU LTM (China Telecom) | discussion |
| R3-251863 | (TP to TS38.401) Support of activation or deactivation of SP CSI-RS for inter-DU LTM (China Telecom) | other |
| R3-251881 | (TP for 38.423, 38.300, 38.420, 38.470) Inter-CU LTM (ZTE Corporation) | other |
| R3-251946 | [TP to BLCR for TS 38.423] Inter-CU LTM (Lenovo) | other |
| R3-251947 | [TP to BLCR for TS 38.423] Inter-CU LTM in DC (Lenovo) | other |
| R3-251963 | Further discussion on inter-CU LTM (Ericsson) | discussion |
| R3-251964 | (TP for LTM BL CR for TS 38.423, TS 38.473, TS 38.300, TS 37.340) – Support for inter-CU LTM (Ericsson) | other |
| R3-251972 | gNB-DU initiated LTM candidate cell reconfiguration (Rakuten Mobile, Qualcomm Inc) | discussion |
| R3-251986 | Issues on source gNB initiated LTM cancel and UE association setup (Ofinno, LLC) | discussion |
| R3-252088 | Support for Inter-CU L1/L2 Triggered Mobility (LTM) over Xn (Jio Platforms Ltd (JPL)) | discussion |
| R3-252089 | Xn support for inter-CU L1/L2 Triggered Mobility (Jio Platforms Ltd (JPL)) | CR1476r, TS 38.423 v18.5.0, Rel-18, Cat. C |
| R3-252118 | (TP for LTM BLCR for TS38.473, TS38.300, TS38.470): Further discussion on inter-CU LTM procedure (Huawei) | other |
| R3-252119 | (TP for LTM BLCR for TS37.340, TS38.423): Discussion on inter-CU SCG LTM (Huawei) | other |
| R3-252148 | Discussion on inter-CU LTM (NTT DOCOMO INC..) | discussion |
| R3-252152 | Discussion on inter-gNB-CU LTM (Samsung) | discussion |
| R3-252153 | (TP to BLCR for TS38.423 and TS38.473) Inter-gNB-CU LTM (Samsung) | other |
| R3-252228 | CSI-RS network coordination (Ericsson, Nokia, LG Electronics) | discussion |
| R3-252229 | (TP for TS 38.423) – Xn support for Semi-persistent CSI-RS transmission (Ericsson, Nokia, LG Electronics) | other |
| R3-252230 | (TP for TS 38.473) – F1 support for Semi-persistent CSI-RS transmission (Ericsson, Nokia, LG Electronics) | other |
| R3-252240 | (TP for 38.423, 37.340) SN initiate SCG LTM (ZTE Corporation) | other |

Appendix

Previous meeting agreements:

RAN3#123bis:

Prioritize to support inter-CU LTM over Xn interface, and RAN3 specify the inter-CU LTM solutions for standalone scenario first.

Reuse existing Xn Handover Request and Handover Request ACK for Inter-CU LTM initial preparation.

Confirm the case that inter-CU LTM is not configured in both MCG and SCG at the same time.

Early data forwarding can be supported for inter-CU LTM.

Cell Switch Notification from source DU to target DU (in different gNB from source) for LTM execution.

RAN3#124:

Source gNB-CU initiates the handover preparation procedure for inter-gNB-CU LTM.

Introduce a new procedure on Xn to transfer the TA information.

A new XnAP class 2 procedure, namely LTM Cell Switch Notification is introduced on Xn to forward the target Cell ID and target TCI state ID(s) from the source gNB-CU to the target gNB-CU.

Reuse Handover Success procedure over Xn for Rel-19 Inter-CU LTM, to tell the source CU that the UE has accessed to the target Cell.

The Handover Cancel message is reused by the source gNB to release the reserved resource for LTM candidate cells in the candidate gNBs.

Early data forwarding can be triggered before the Source gNB triggers a MAC CE Command to the UE to change cells, timing is left to implementation.

Early sync configuration (TCI state and RACH configuration) can be obtained during the LTM preparation phase through the handover request and handover request acknowledge messages.

A candidate gNB can initiate cancellation of configured LTM candidate cell(s) of its own. Details are FFS.

FFS on the LTM modification procedures.

FFS on whether to reuse the existing XnAP UE CONTEXT RELEASE message at the source gNB if no LTM candidate cell(s) exist in the source gNB.

RAN3#125:

Introduce a new UE associated Class-1 XnAP procedure to update the LTM configurations for subsequent LTM.

Change the name of LTM Cell Switch Notification message to Cell Switch Notification message.

Reuse the Early Status Transfer and SN Status Transfer message for inter-CU LTM.

Adopt Class-2 procedure for candidate gNB-initiated LTM cancellation. Down select from Option1 and Option3 in the next meeting:

Option 1: Reuse CHO Cancel

Option 2: Rename CHO Cancel

Option 3: Introduce new procedure

RAN3#125bis:

Current SSB information in Xn Setup and Configuration Update procedures can be reused for LTM preparation phase.

WA: For inter-CU LTM mobility, a separate LTM request message (i.e. HANDOVER REQUEST message) is used for each candidate cell.

The error handling of multiple UE associations need to be considered.

The LTM Configuration IDs are allocated by the source CU.

WA: Reuse the existing XnAP UE CONTEXT RELEASE message at the source gNB if no LTM candidate cell(s) exist in the source gNB.

Follow F1AP, the source gNB-CU sends the CSI resource configuration of candidate cells to candidate gNB-CUs via Handover Request message for subsequent LTM, and the candidate gNB-CU sends the CSI report configuration to the source gNB-CU via Handover Request ACK message.

How the source gNB-CU sends the reference configuration to all candidate gNBs is pending on RAN2 progress.

Confirm the name of the new procedure as “LTM Configuration Update”.

Introduce a new procedure for candidate gNB-initiated LTM cancellation.

- Allow UE association in-between candidate CUs (in case the Xn connectivity existed) for subsequent LTM. When and how to establish the UE association is FFS.

RAN3#126:

RAN3 move forward on Legacy framework with PDCP change/switch for inter-CU LTM in this release, not considering the PDCP anchor based solution.

The source CU can request candidate CU to provide CSI-RS configuration in HANDOVER REQUEST message, and candidate CU signals the CSI-RS configuration in HANDOVER REQUEST ACKNOWLEDGEMENT message.

The source CU generates common CSI-RS Resource Configuration and sends it to candidate CU in LTM CONFIGURATION UPDATE message, the candidate CU signals the CSI-RS Report Configuration in LTM CONFIGURATION UPDATE ACKNOWLEDGEMENT message.

Turn the WA into agreement: For inter-CU LTM mobility, a separate LTM request message (i.e. HANDOVER REQUEST message) is used for each candidate cell.

To support subsequent LTM, the LTM Configuration Update procedure is reused to establish UE association between the new source gNB and the other candidate gNB(s) after each inter-CU LTM Cell Switch.

Confirm the message name as LTM Cancel for candidate gNB-initiated LTM cancellation.

Turn the WA into agreement: Reuse the existing XnAP UE CONTEXT RELEASE message at the source gNB if no LTM candidate cell(s) exists in the source gNB.

Late data forwarding may be initiated after the source gNB decides to trigger the LTM Cell Switch Command to the UE, when exactly it is initiated is left to implementation?

RAN3#127:

For both inter-CU and intra-CU cases:

For the network Semi-Persistent CSI-RS coordination, source gNB-DU/source gNB triggers the activation/deactivation of the CSI-RS transmission in the candidate cell.

For the activation/deactivation procedure, a class 1 procedure is needed from the source gNB-DU/source gNB. FFS for reusing existing one or a new one.

The candidate gNB-CU responds the full SSB Time/Frequency Configuration (in SSB Information IE) of candidate cells to the source gNB-CU in the Handover Request ACK message.

The source gNB-CU sends the pair of (gNB ID, new ID of early RACH configuration resource (to be further discussed)) to the candidate gNB-CU to request early RACH configuration.

WA: Introduce a new non-UE associated class 2 procedure on Xn, namely TA information Transfer message, to transfer the TA information from the candidate gNB-CU to the source gNB-CU.

RAN3 agree the following scenarios to support LTM with NR-DC:

1.SN initiated inter-CU SCG LTM without MCG changes (high priority)

2.Inter-CU MCG LTM without SN release

3.Inter-CU MCG LTM with SN release

4.Inter-CU MCG LTM with SN addition

The format of the new introduced IE is same with the gNB-DU ID.

The source gNB can generate reference configuration and provide a reference configuration for LTM in a Handover Request and LTM configuration update message.

The candidate gNB indicates whether a LTM candidate configuration is a complete candidate configuration in the Handover Request Acknowledge and LTM configuration update acknowledge message.

FFS on whether the source gNB can request a candidate gNB to provide a reference configuration.

WA: Use a single UE association for multiple LTM handover request to the same candidate gNB.

Reuse the LTM Configuration Update procedure to sync up configurations among candidate gNBs for subsequent LTM, including early sync configuration, configuration ID, and data forwarding addresses, etc.

It is up to the network implementation when the sync up is performed: during the preparation step, or during the cell switch execution step, or after successful cell switch.

Normal data forwarding may be initiated after the source gNB decides to trigger the LTM cell switch for the UE, and when exactly it is initiated is left to implementation.

For SN initiated inter-CU SCG LTM, the source SN initiates the inter-CU SCG LTM preparation procedure by sending an SN Change Required message to the MN.

The MN requests each candidate SN to allocate resources for the UE via SN Addition request message.

Within the list of cells suggested by the source SN, the candidate SN provides the SCG part configuration of each candidate PSCell and may also provide the L1 RS (e.g. a list of SSB or a list of CSI-RS) configuration for L1 measurement, early UL sync configuration or TCI-state configuration, to the MN, via SN addition request ACK message.

In order to support subsequent inter-CU SCG LTM, the MN needs to transfer the common CSI resource configuration and the collected information of candidate cells to the candidate SN(s), via SN modification request message. Accordingly, the candidate SN(s) responds with the updated candidate SCG configuration to the MN via SN modification request ACK message.

The Cell Switch Notification message can be reused from the source SN to the target SN via the MN. The detailed IE can be further discussed.