3GPP TSG-RAN WG3 Meeting #127 Bis	             R3-252366

Wuhan, China, 7 – 11 April 2025

Agenda Item:	21.3

Source:	CMCC, Nokia, Nokia Shanghai Bell, Huawei, CATT, Xiaomi, Ofinno, China Telecom, ZTE, Ericsson, Qualcomm, Samsung

Title:	(TP to BL CR for TS 37.483) Support of PDU set based QoS handling enhancement

Document for:	Approval

# Introduction

This contribution provides TP for E1AP to reflect the change for supporting of  alternative PDU Set QoS as listed in the updated WID in RP-250107. The corresponding objective is copied below

- Support of PDU set based QoS handling enhancement [RAN3]: - Support of DL PDU Set marking without PDU Set QoS - Support of Alternative PDU Set QoS, which may contain UL and/or DL PDU Set QoS Parameters (i.e. UL PSDB, DL PSDB, UL PSER and/or DL PSER). ……

Alternative QoS Profiles shall include PDU Set QoS parameters (PSDB, PSER) on the same direction(s), if QoS Profile includes PDU Set QoS parameters (PSDB, PSER) for either uplink or downlink or both.

# Annex: TP to BL CR For TS 37.483 For PDU set based QoS handling enhancement

<<<<<<<<<<<<<<<<<<<<<<<<<<<<<<<<<<start of change>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>

#### 9.3.1.93	Alternative QoS Parameters Set List

This IE contains alternative sets of QoS parameters which the NG-RAN node can indicate to be fulfilled when notification control is enabled and it cannot fulfil the requested list of QoS parameters.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Alternative QoS Parameters Set Item |  | 1..<maxnoofQoSparaSets> |  |  |  |  |
| >Alternative QoS Parameters Set Index | M |  | INTEGER (1..8,..) |  | – |  |
| >Guaranteed Flow Bit Rate Downlink | O |  | Bit Rate 9.3.1.20 |  | – |  |
| >Guaranteed Flow Bit Rate Uplink | O |  | Bit Rate 9.3.1.20 |  | – |  |
| >Packet Delay Budget | O |  | 9.3.1.47 |  | – |  |
| >Packet Error Rate | O |  | 9.3.1.48 |  | – |  |
| >Maximum Data Burst Volume | O |  | 9.3.1.50 | For details see TS 23.501 [20]. This IE shall be included if the Delay Critical IE is set to “delay critical” and is ignored otherwise. | YES | ignore |
| >PDU Set Delay Budget Downlink | O |  | 9.3.1.79 |  | - |  |
| >PDU Set Delay Budget Uplink | O |  | 9.3.1.79 |  | - |  |
| >PDU Set Error Rate Downlink | O |  | 9.3.1.48 |  | - |  |
| >PDU Set Error Rate Uplink | O |  | 9.3.1.48 |  | - |  |

| Range bound | Explanation |
| --- | --- |
| maxnoofQoSParaSets | Maximum no. of alternative sets of QoS Parameters allowed for the QoS under Notification Control. Value is 8. |

<<<<<<<<<<<<<<<<<<<<<<<<<<<<<<<<<<<<<<<<<<next change >>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>

### 9.4.5	Information Element Definitions

-- ASN1START

-- **************************************************************

--

#### -- Information Element Definitions

--

-- **************************************************************

E1AP-IEs {

itu-t (0) identified-organization (4) etsi (0) mobileDomain (0)

ngran-access (22) modules (3) e1ap (5) version1 (1) e1ap-IEs (2) }

DEFINITIONS AUTOMATIC TAGS ::=

BEGIN

//omitted text unchanged//

id-PSIbasedDiscardTimer,

id-UserPlaneErrorIndicator,

id-MaximumDataBurstVolume,

id-PDCPSNGapReport,

id-UserPlaneFailureIndication,

id-PduSetDelayBudgetDownlink,

id-PduSetDelayBudgetUplink,

id-PduSetErrorRateDownlink,

id-PduSetErrorRateUplink,

maxnoofMBSAreaSessionIDs,

maxnoofSharedNG-UTerminations,

maxnoofMRBs

//omitted text unchanged//

#### -- A

//omitted text unchanged//

AlternativeQoSParaSetList ::= SEQUENCE (SIZE(1..maxnoofQoSParaSets)) OF AlternativeQoSParaSetItem

AlternativeQoSParaSetItem ::= SEQUENCE {

alternativeQoSParameterIndex		INTEGER(1..8,...),

guaranteedFlowBitRateDL				BitRate					OPTIONAL,

guaranteedFlowBitRateUL				BitRate					OPTIONAL,

packetDelayBudget					PacketDelayBudget		OPTIONAL,

packetErrorRate						PacketErrorRate			OPTIONAL,

iE-Extensions		ProtocolExtensionContainer { {AlternativeQoSParaSetItem-ExtIEs} }	OPTIONAL,

...

}

AlternativeQoSParaSetItem-ExtIEs E1AP-PROTOCOL-EXTENSION ::= {

{ ID id-MaximumDataBurstVolume	CRITICALITY ignore	EXTENSION MaxDataBurstVolume		PRESENCE optional		}|

{ ID id-PduSetDelayBudgetDownlink 	CRITICALITY ignore	EXTENSION ExtendedPacketDelayBudget		PRESENCE optional		}|

{ ID id-PduSetDelayBudgetUplink		CRITICALITY ignore	EXTENSION ExtendedPacketDelayBudget		PRESENCE optional		}|

{ ID id-PduSetErrorRateDownlink		CRITICALITY ignore	EXTENSION PacketErrorRate				PRESENCE optional		}|

{ ID id-PduSetErrorRateUplink		CRITICALITY ignore	EXTENSION PacketErrorRate				PRESENCE optional		},

...

}

<<<<<<<<<<<<<<<<<<<<<<<<<<<<<<<<<<<<<<<<<<next change >>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>

### 9.4.7	Constant Definitions

-- ASN1START

-- **************************************************************

--

#### -- Constant definitions

--

-- **************************************************************

E1AP-Constants {

itu-t (0) identified-organization (4) etsi (0) mobileDomain (0)

ngran-access (22) modules (3) e1ap (5) version1 (1) e1ap-Constants (4) }

DEFINITIONS AUTOMATIC TAGS ::=

BEGIN

//omitted text unchanged//

id-F1U-TNL-InfoToRelease-List									ProtocolIE-ID ::= 212

id-BroadcastF1U-ContextReferenceE1								ProtocolIE-ID ::= 213

id-PSIbasedDiscardTimer											ProtocolIE-ID ::= 214

id-UserPlaneErrorIndicator										ProtocolIE-ID ::= 215

id-MaximumDataBurstVolume 										ProtocolIE-ID ::= 216

id-BCBearerContextNGU-TNLInfoatNGRAN-Request					ProtocolIE-ID ::= 217

id-PDCPSNGapReport												ProtocolIE-ID ::= 218

id-UserPlaneFailureIndication									ProtocolIE-ID ::= 219

id-PduSetDelayBudgetDownlink									ProtocolIE-ID ::= a1

id-PduSetDelayBudgetUplink										ProtocolIE-ID ::= a2

id-PduSetErrorRateDownlink										ProtocolIE-ID ::= a3

id-PduSetErrorRateUplink										ProtocolIE-ID ::= a4

END

-- ASN1STOP

<<<<<<<<<<<<<<<<<<<<<<<<<<<<<<<<<<<<<<<<<<end of change >>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>