3GPP TSG-RAN WG3 Meeting #127bis	R3-252443

Wuhan, China, 7 – 11 April 2025

# Title:	[DRAFT] LS on AI/ML Positioning Case 3b

# Response to:

# Release:	Release 19

# Work Item:	NR_AIML_air-Core

Source:	Ericsson [to be RAN3]

To:	SA2

Cc:

Contact Person:

#### Name:	Yazid Lyazidi

#### Tel. Number:

#### E-mail Address:	Yazid.lyazidi@ericsson.com

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

# Attachments:

1. Overall Description:

As part of the work on use case 3a for AI/ML assisted positioning, included in the WI NR_AIML_air-Core, RAN3 is working on solutions to retrieve positioning data at the NG-RAN. Such data can be used to manage NG-RAN sided models, e.g. to train or to monitor such models.

RAN3 has identified the following solutions and RAN3 would like to ask SA2 whether they are feasible.

Solution 1: Positioning Information collection for UEs under positioning

Solution 1: Positioning Information collection for UEs undergoing positioning that involves the gNB

Step 1. A positioning process is ongoing for a UE. The gNB involved in the positioning process determines that positioning information, e.g. UE location information, is needed for the UE under positioning.

2. gNB sends the Positioning Information Required message to the LMF over NRPPa, including the NRPPa Step Measurement ID associated to the UE for which positioning data is requested.

Step 3. LMF sends the Positioning Information Update message to the gNB, including the requested positioning data.

Solution 2: Positioning Information collection for UEs served by the NG-RAN which are not being positioned

- The gNB is currently serving one or more UE(s) for which positioning measurements (i.e., UL SRS measurements) is available or for which they can be collected via a positioning session. The gNB wishes to receive positioning information, consisting of labels (e.g. UE location information) to form a data set for training. Based on gNB implementation, the gNB decides when/if to trigger a (new) NG: POSITIONING INFORMATION REQUEST message to the serving AMF of the served UE(s). - The POSITIONING INFORMATION REQUEST message is an NGAP non-UE associated message containing e.g. the NGAP UE IDs of the UEs for which positioning information is required, type of requested information, e.g. UE position, time stamp. - The AMF receives the request message from the NG-RAN node. If the requested information is already available at AMF as part of UE positioning context, the AMF responds directly with the message in step 6. Otherwise, it selects an LMF to request positioning of each UE for Positioning Information collection purposes. The AMF triggers a Positioning Information Request towards the LMF (it could consist of the (legacy) Nlmf_Location_DetermineLocation service operation) to request the current location of the UE. - The LMF performs one or more of the positioning procedures described in clause 6.11.1, 6.11.2 and 6.11.3 of TS 23.273. - The LMF returns the requested information towards the AMF. The message Nlmf_Location_DetermineLocation Response might be reused. - The AMF returns the requested label(s) to the NG-RAN node in a POSITIONING INFORMATION RESPONSE message. The AMF may return instead a failure message with appropriate cause value if the procedure does not succeed. - The gNB combines the received label with the TRP measurements to form the data needed.

2. Actions:

To SA2

ACTION: 	RAN3 asks SA2 to provide feedback on feasibility of the solutions described above.

3. Date of Next TSG RAN WG3 Meetings:

TSG-RAN3 Meeting #128		May 19-23, 2025			Malta, MT

TSG-RAN3 Meeting #129		August 25-28, 2025			Bangalore, India