3GPP TSG-RAN WG3 Meeting #127bis	R3-252490

Wuhan, China, 07 - 11 April, 2025

Agenda Item:	21.3

Source:	Ericsson, Nokia, Nokia Shanghai Bell, CMCC, China Telecom, Huawei, ZTE, Qualcomm Inc., Samsung

Title:	(TP to BL CR for TS 38.423) Support of uplink rate control

Document for:	Other

# 1	Introduction

Regarding UL bit rate control, it is agreed to introduce an indication in NGAP, to allow the SMF to inform the gNB which QoS flow(s) are subject to uplink rate control.

Introduce the same indication in XnAP to inform target gNB.

# 2	References

[1] RP-250107, Revised WID on XR (eXtended Reality) for NR Phase 3, Nokia (Rapporteur), 3GPP TSG RAN Meeting #107.

Start of first change

#### 9.2.3.5	QoS Flow Level QoS Parameters

This IE defines the QoS Parameters to be applied to a QoS flow.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| CHOICE QoS Characteristics | M |  |  |  | – |  |
| >Non Dynamic 5QI |  |  |  |  |  |  |
| >>Non dynamic 5QI Descriptor | M |  | 9.2.3.8 |  | – |  |
| >Dynamic 5QI |  |  |  |  |  |  |
| >>Dynamic 5QI Descriptor | M |  | 9.2.3.9 |  | – |  |
| Allocation and Retention Priority | M |  | 9.2.3.7 |  | – |  |
| GBR QoS Flow Information | O |  | 9.2.3.6 | This IE shall be present for GBR QoS flows and is ignored otherwise. | – |  |
| Reflective QoS Attribute | O |  | ENUMERATED (subject to, ...) | Reflective QoS is specified in TS 23.501 [7]. This IE applies to Non-GBR bearers only and is ignored otherwise. | – |  |
| Additional QoS flow Information | O |  | ENUMERATED (more likely, …) | If this IE is set to "more likely", this indicates that traffic for this QoS flow is likely to appear more often than traffic for other flows established for the PDU session. This IE may be present in case of Non-GBR flows only and is ignored otherwise. | – |  |
| QoS Monitoring Request | O |  | ENUMERATED (UL, DL, Both, …) | Indicates to measure UL, or DL, or both UL/DL delays for the associated QoS flow. | YES | ignore |
| QoS Monitoring Reporting Frequency | O |  | INTEGER (1.. 1800, …) | Indicates the Reporting Frequency for RAN part delay for Qos monitoring. Unit: second | YES | ignore |
| QoS Monitoring Disabled | O |  | ENUMERATED(true, ...) | Indicates to stop the QoS monitoring. | YES | ignore |
| PDU Set QoS Parameters |  | 0..1 |  | Indicates the PDU Set QoS Parameters. | YES | ignore |
| >UL PDU Set QoS Information | O |  | PDU Set QoS Information 9.2.3.203 |  | – |  |
| >DL PDU Set QoS Information | O |  | PDU Set QoS Information 9.2.3.203 |  | – |  |
| Indication of Bitrate Adaptation | O |  | ENUMERATED (uplink, …) | Indicates that the QoS Flow allows rate adaptation in the indicated direction. | YES | ignore |

Next change

### 9.3.5	Information Element definitions

-- ASN1START

-- **************************************************************

--

-- Information Element Definitions

--

-- **************************************************************

//omitted text unchanged//

id-PDUSetQoSParameters,

id-N6JitterInformation,

id-ECNMarkingorCongestionInformationReportingRequest,

id-TAISliceUnavailableCellList,

id-MobileIABCell,

id-XR-Bcast-Information,

id-MaximumDataBurstVolume,

id-CPAC-Preparation-Type,

id-MN-only-MDT-collection,

id-BarringExemptionforEmerCallInfo,

id-Transmission-Bandwidth-asymmetric,

id-NRPPaPositioningInformation,

id-Indication-of-bitrate-adaptation,

Next change

-- I

//omitted text unchanged//

ImplicitFormat	::= SEQUENCE	{

dUFSlotformatIndex 			DUFSlotformatIndex,

iE-Extensions		ProtocolExtensionContainer { { ImplicitFormat-ExtIEs } } OPTIONAL,

...

}

ImplicitFormat-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {

...

}

Indication-of-bitrate-adaptation ::= ENUMERATED {uplink, ...}

//omitted text unchanged//

QoSFlowLevelQoSParameters ::= SEQUENCE {

qos-characteristics			QoSCharacteristics,

allocationAndRetentionPrio	AllocationandRetentionPriority,

gBRQoSFlowInfo				GBRQoSFlowInfo														OPTIONAL,

reflectiveQoS				ReflectiveQoSAttribute												OPTIONAL,

additionalQoSflowInfo		ENUMERATED {more-likely, ...}										OPTIONAL,

iE-Extensions				ProtocolExtensionContainer { {QoSFlowLevelQoSParameters-ExtIEs} } OPTIONAL,

...

}

QoSFlowLevelQoSParameters-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {

{ID id-QoSMonitoringRequest					CRITICALITY ignore	EXTENSION QosMonitoringRequest					PRESENCE optional}|

{ID id-QosMonitoringReportingFrequency		CRITICALITY ignore	EXTENSION QosMonitoringReportingFrequency		PRESENCE optional}|

{ID id-QoSMonitoringDisabled				CRITICALITY ignore	EXTENSION QoSMonitoringDisabled					PRESENCE optional}|

{ID id-PDUSetQoSParameters					CRITICALITY ignore	EXTENSION PDUSetQoSParameters						PRESENCE optional}|

{ID id-Indication-of-bitrate-adaptation		CRITICALITY	ignore	EXTENSION		Indication-of-bitrate-adaptation	PRESENCE optional},

...

}

Next change

### 9.3.7	Constant definitions

-- ASN1START

-- **************************************************************

--

-- Constant definitions

--

-- **************************************************************

//omitted text unchanged//

id-RegistrationRequestForDataCollection																ProtocolIE-ID ::= 460

id-ReportCharacteristicsForDataCollection															ProtocolIE-ID ::= 461

id-ReportingPeriodicityForDataCollection															ProtocolIE-ID ::= 462

id-NodeAssociatedInfoResult																			ProtocolIE-ID ::= 463

id-SLPositioning-Ranging-Services-Info																ProtocolIE-ID ::= 464

id-XR-Bcast-Information					 															ProtocolIE-ID ::= 465

id-PDUSessionsListToBeReleased-UPError																ProtocolIE-ID ::= 466

id-MaximumDataBurstVolume																			ProtocolIE-ID ::= 467

id-CPAC-Preparation-Type																			ProtocolIE-ID ::= 468

id-UserPlaneFailureIndication																		ProtocolIE-ID ::= 469

id-MN-only-MDT-collection																			ProtocolIE-ID ::= 470

id-BarringExemptionforEmerCallInfo																	ProtocolIE-ID ::= 471

id-Transmission-Bandwidth-asymmetric																ProtocolIE-ID ::= 472

id-SRSPositioningConfigOrActivationRequest															ProtocolIE-ID ::= 473

id-NRPPaPositioningInformation																		ProtocolIE-ID ::= 474

id-Indication-of-bitrate-adaptation												ProtocolIE-ID ::= XXX

End of change