3GPP TSG-RAN WG3 Meeting #127bis	R3-252535

# Wuhan, China, 7th – 11th April 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.300 | CR | Draft | rev | - | Current version: | 18.5.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | X | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Support of XR enhancements |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Nokia, Nokia Shanghai Bell, Lenovo, Qualcomm, Xiaomi, CATT, Ericsson, CMCC, Samsung, Huawei, China Telecom |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R3 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_XR_Ph3-Core |  |  |  |  |  | Date: |  |  | 2025-04-24 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | Add support of enhancements for XR. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Add the clarification that the source gNB should forward the End of Data Burst Indication to target gNB during the data forwarding for handover. This enables the target gNB to push the UE back to sleep as early as possible. (R3-251578) - Add the support for MMSID (R3-252351) - Add the support for BSSize and TTNB (R3-252370) |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | XR enhancements are not supported. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 16.15.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  | X |  | Other core specifications |  |  |  | TS 37.340 CR TS 37.483 CR 0171 TS 38.413 CR 1282 TS 38.423 CR 1269 TS 38.473 CR 1485 TS 38.415 CR 0056 TS 38.425 CR 0156 |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

-----------------Start of the Changes-------------------

### 16.15.2	Awareness

XR-Awareness relies on QoS flows, PDU Sets, Data Bursts and traffic assistance information (see TS 23.501 [3]).

The following PDU Set QoS Parameters may be provided by the SMF to the gNB as part of the QoS profile of the QoS flow, and to enable PDU Set based QoS handling at least one of them shall be provided:

-	PDU Set Delay Budget (PSDB): as defined in TS 23.501 [3], upper bound for the duration between the reception time of the first PDU (at the UPF for DL, at the UE for UL) and the time when all PDUs of a PDU Set have been successfully received (at the UE in DL, at the UPF in UL). When available, supersedes the PDB of the QoS flow.

-	PDU Set Error Rate (PSER): as defined in TS 23.501 [3], upper bound for a rate of non-congestion related PDU Set losses between RAN and the UE. When available, it supersedes the PER of the QoS flow.

NOTE 1:	In this release, a PDU set is considered as successfully delivered only when all PDUs of a PDU Set are delivered successfully.

-	PDU Set Integrated Handling Information (PSIHI): indicates whether all PDUs of the PDU Set are needed for the usage of PDU Set by application layer, as defined in TS 23.501 [3].

NOTE 2:	For a given QoS flow, the PDU Set QoS parameters are common for all PDU Sets but can be different for UL and DL.

The Multi-modal Service ID may also be provided by the SMF to the gNB as part of the QoS parameters of the QoS flow. It is up to the gNB’s implementation to use this information for multi-modality service.

During the Xn-handover preparation procedure, the source gNB sends the stored PDU Set QoS Parameters as part of the QoS profile to the target NG-RAN node. For NG handover, the AMF provides the PDU Set QoS parameters to the target gNB by means of the NGAP HANDOVER REQUEST message.

In addition, the UPF can identify PDUs that belong to PDU Sets, and may indicate to the gNB the following PDU Set Information in the GTP-U header:

-	PDU Set Sequence Number;

-	Indication of End PDU of the PDU Set;

-	PDU Sequence Number within a PDU Set;

-	PDU Set Size in bytes;

-	PDU Set Importance (PSI), which identifies the relative importance of a PDU Set compared to other PDU Sets within the same QoS Flow.

5GC may provide XR traffic assistance information to gNB through NG AP TSC Assistance Information (TSCAI) as specified in clause 5.37.8 of TS 23.501[3] (for both GBR and non-GBR QoS flows):

-	UL and/or DL Periodicity;

-	N6 Jitter Information (i.e. between UPF and Data Network) associated with the DL Periodicity.

This assistance information can be used by the gNB to configure DRX to enable better UE power saving.

In addition, 5GC may provide the following information through NG-U as specified in clause 5.37.5.2 of TS 23.501[3]:

-	Indication of End of Data Burst in the GTP-U header of the last PDU in downlink.

-	Indication of Data Burst Size in the GTP-U header of the first PDU of the data burst in downlink.

-	Time To Next Burst in the GTP-U header of the PDU in downlink.

The information can be used by the gNB to push the UE back to sleep when possible. During the data forwarding for handover, the source gNB provides the Indication of End of Data Burst, Indication of Data Burst Size, and Time to Next Burst received from the UPF to target gNB in case one or more PDUs of the data burst is forwarded to target gNB.

In the uplink, the UE needs to be able to identify PDU Sets and Data Bursts dynamically, including PSI. How this is done is left up to UE implementation but when possible for a QoS flow, this is indicated to the gNB via UE Assistance Information.

-----------------End of the Change-------------------