3GPP TSG RAN WG3 Meeting #127bis	R3-252397

Wuhan, China, 07-11 April, 2025

Title:	LS on Average Window for Alternative QoS

Response to:	-

Release:	REL-18

Work Item:	NR_newRAT-Core, TEI18

Source:	RAN3

To:	SA2

Cc:	-

Contact person:	liuziqiao@catt.cn

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

Attachments:	-

1	Overall description

RAN3 notices the misalignment on Average Window between RAN3 and SA2 specifications.

Current SA2 specification (e.g. TS 23.501) has following statement but RAN3 specifications do not include Averaging Window in the Alternative QoS Parameters.

An Alternative QoS Profile represents a combination of QoS parameters PDB, PER, Averaging Window and GFBR to which the application traffic is able to adapt.

Companies in RAN3 discussed whether to include averaging window information in the Alternative QoS Parameter set and there are two different understandings:

- Understanding 1: NG-RAN node will re-use the same value as the averaging window configured in the QoS profile to determine whether the QoS profile can be fulfilled and which alternative QoS profile can be used as reference if the QoS profile is not fulfilled.

- Understanding 2: The Alternative QoS Parameter sets may contain different averaging window configurations. It means that the NG-RAN will use different durations to determine whether the QoS profile can be fulfilled and which alternative QoS profile can be used as reference if the QoS profile is not fulfilled. This represents a new requirement to NG-RAN.

RAN3 could not reach consensus on introducing averaging window in the alternative QoS Profile, and respectfully requests SA2 to provide clarification on introducing averaging window in the alternative QoS Profile.

2	Actions

To SA2

ACTION: 	RAN3 kindly asks SA2 to provide feedback on the above RAN3 concern.

3	Dates of next TSG RAN WG3 meetings

RAN3#128		19 - 23 May 2025			Malta, MT

RAN3#129		25 - 29 August 2025		Bangalore, IN