3GPP TSG-RAN WG3 Meeting #127-bis	R3-2522474

Wuhan, China, 7-11 April, 2025

Agenda item:	19.2

Source: 	Ericsson, Huawei, Samsung, Qualcomm, ZTE, Huawei, CATT, Nokia, CMCC, LGE,

Title: 	(TP to BL CR for 38.300) introduction of Evolution of NR duplex operation: Sub-band full duplex (SBFD)

Document for:	Discussion & Approval

# Introduction

This TP tries to capture the agreements achieved for SBFD.

# Annex:	TP for TS 38.300

/*********************Start of changes***********************/

## 17.2	Cross-Link Interference Management

When different TDD DL/UL patterns are used between neighbouring cells, UL transmission in one cell may interfere with DL reception in another cell: this is referred to as Cross Link Interference (CLI).

To mitigate CLI, gNBs can exchange and coordinate their intended TDD DL-UL configurations over Xn and F1 interfaces; and the victim UEs can be configured to perform CLI measurements. There are two types of CLI measurements:

-	SRS-RSRP measurement in which the UE measures SRS-RSRP over SRS resources of aggressor UE(s);

-	CLI-RSSI measurement in which the UE measures the total received power observed over RSSI resources.

Layer 3 filtering applies to CLI measurement results and both event triggered and periodic reporting are supported.

In case of Sub-band full duplex (SBFD) operation CLI may also be present. A gNB reports CLI related information of its serving cells to neighbour gNBs, the neighbour gNB should evaluate and take potential interference mitigation measures when necessary.

/*********************End of changes***********************/