3GPP TSG RAN WG3 Meeting #127-bis	R3-251506

Wuhan, China, 07-11 April, 2025

3GPP TSG-CT WG1 Meeting #153	C1-250820

# Athens, Greece, 17 – 21 February 2025

# Title:	LS on PWS enhancements for MWAB and MBSR

# Release:	Rel-19, Rel-18

# Work Item:	VMR_Ph2, VMR

Source:	CT1

To:	SA2

CC:	RAN3

Contact Person:

Name:	Ivo Sedlacek

E-mail Address:	ivo dot sedlacek at ericsson dot com

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

1. Overall Description:

CT1 identified that TS 23.502 states:

------------

In addition to the above, if the UE is served by an authorized MSBR and the AMF serving this UE receives the Location Report including the TAI/NR CGI for the MBSR that UE is accessing, the AMF shall, if supported, update the corresponding Warning Area List NG-RAN specified in TS 23.041 [86]

------------

and

------------

In addition to the above, if the UE is served by an MWAB-gNB and the AMF serving this UE receives the Location Report including the TAI/NR CGI for the MWAB that UE is accessing, the AMF shall, if supported, update the corresponding Warning Area List NG-RAN specified in TS 23.041 [86].

------------

Those TS 23.502 statements are not aligned with TS 23.041 statements:

------------

2.	Using the "impacted area" information, the CBCF identifies which AMFs need to be contacted and determines the information to be placed into the Warning Area List NG-RAN Information Element. The CBCF sends a Write-Replace Warning Request NG-RAN message containing the warning message to be broadcast and the delivery attributes (Message Identifier, Serial Number, list of NG-RAN TAIs, Warning Area List NG-RAN, OMC ID, CWM Indicator, Send Write-Replace-Warning-Indication, Global RAN Node ID, Warning Area Coordinates) to AMFs.

...

The Warning Area List NG-RAN shall be a list of Cell IDs or a list of NG-RAN TAIs or one or more Emergency Area IDs. The Warning Area List NG-RAN is only used by the NG-RAN node. The NG-RAN node is configured with the NG-RAN TAI(s) and Cell ID(s) it serves and the Emergency Area ID(s) that it belongs to. The NG-RAN node checks for any match of the contents of the Warning Area List NG-RAN with these IDs to identify the cells where to distribute the warning message. The Warning Area List NG-RAN is an optional information element. If the Warning Area List NG-RAN is absent, it shall be interpreted as "all cells on the NG-RAN node". The number of cell IDs will be limited by the message size on N50 and N2. An Emergency Area ID is unique within the PLMN or SNPN.

------------

CT1 would like to remind SA2 that CT1 is responsible for stage-2 for PWS. Thus, any stage-2 requirements for PWS need to be agreed by CT1.

2. Actions:

To SA2 group.

ACTION: 	Given that CT1 is responsible for stage-2 for PWS, CT1 would like to ask SA2 to remove the TS 23.502 statements above (1st statement from TS 23.502 Rel-18 and Rel-19, 2nd statement from TS 23.502 Rel-19).

3. Date of Next CT1 Meetings:

CT1#154                           7th April- 11th April 2025...............................China

CT1#155                           19th May- 23rd  May 2025.............................Bratislava, Slovakia

CT1#156                           25th August- 29th August 2025.....................Goteborg, Sweden

CT1#157                           13th October- 17th October 2025..................Sophia Antipolis, France

CT1#158                           17th November- 21st November 2025............Dallas, US

CT1#159                           9th February- 13th February 2025..................India