3GPP TSG-RAN WG3 Meeting #127	R3-250078

Athens, Greece, EU, 17-21 Feb 2025

Agenda item:	10.3.2

Source:	Nokia

Title:	(MDT TP to BL CR to TS 38.300) MDT solution for slice support and slice-related mobility enhancements

Document for:	Discussion

# 1	Introduction

At RAN3#125, the discussion focused on signalling-based MDT and adding the slice to the area scope information. It is expected that the logged MDT will be discussed next. Therefore, in this paper, we remind the proposals related to the logged MDT and handling of the slice discontinuity.

Some potential mobility problems in network slicing deployments were earlier presented in [1]. At RAN3 #126, RAN3 decided to postpone involving RAN2 and to discuss in RAN3 the NSAG-based cell reselection. Because of that, in this paper, we provide more details on the solution, possible problem and the information that may help identify them.

# 2	Discussion

## 2.1	Logged MDT Enhancements for Slice-based Cell Reselection

Rel-17 introduced network slice-based cell reselection, where the UE is provided with one or more network slice groups and associated frequency priorities [1], [2] and [3]. Based on network slice(s) the UE is interested in and on the configured prioritisation, it performs frequency selection and tries to camp on the frequency that the network prioritizes for the prioritized slice group. The goal is to avoid long delay and extensive signalling during the connection setup that may occur when redirecting the UE to the right frequency, if the one it camps on is not appropriate for the requested slice.

However, if these cell re-selection configurations are not correctly defined and optimised, this goal may not be achieved: it could happen that the UE is not able to connect to its required slice in the cell where it is camping based on the slice-specific frequency priorities provided by the network. The problems that the UE may encounter are, for example:

- Despite following the NSAG priorities, the UE selects a cell where the needed slice is not supported or not allowed.

- Despite following the NSAG priorities, the UE still must be redirected at the connection setup.

- After following the priorities of the highest priority NSAG of the UE, the UE is forced to revert to lower-priority NSAG config and repeat cell reselection process.

Observation 1: NSAG configuration is meant to decrease connection setup delay and related signalling. It may, however, be non-optimal that hinders to achieve the intended goals.

Therefore, it is important that the network gets some feedback from the UE about how well the slice-based cell reselection configurations are working and if the UE is experiencing some slice connectivity problems. In RAN3 #124, it was agreed to enhance logged MDT for slice-based cell reselection. We propose to enable logging at least following information to trace cases of suboptimal NSAG config:

- The UE did not find a suitable cell on the prioritized frequency and the reselection procedure had to be repeated according to other NSAG configuration.

- Selected cell does not support the required network slice

- Service request for the required slice was not accepted in the selected cell

Proposal 1: A UE shall be able to log cases where the NSAG configuration did not enable fast connection setup and what kind of problem the UE faced.

Furthermore, it shall be possible to collect information that may help trace where the problem occurred. For this, following information may help:

- Indication that the UE followed NSAG prioritization and which NSAGs were used by the UE to identify the frequency priority configuration should be logged to help the network identify the corresponding slice groups that could be having problematic configurations

- The UE should also log the used frequency priorities along with the slice group(s), because the mapping of slice(s) to slice group as well as frequency priority associated with slice group can be changed dynamically by the network therefore, it is important to log this information to exactly capture what configurations the UE used.

- Possibly, number of cell measurements or number of frequencies measured.

Proposal 2: A UE shall be able to log additional information helpful to trace the cause of the recorded problem.

The enhancement of the logged MDT cannot be implemented in RAN3. It is therefore required to ask RAN2 to provide the needed changes.

Proposal 3: RAN3 kindly requests RAN2 to specify the above measurement enhancements for logged MDT for slice-based cell reselection.

## 2.2	Successful Handovers with Slice Discontinuity

Rel-18 network slicing enhancements introduced support of network slice deployments with service areas that are smaller than the Tracking Area (TA) [2]. This Rel-18 feature is important to deploy highly customized and localized network slices without the need for TA re-design. In such deployments, one potential mobility issue at network slice border is, for example, how to select the neighbour cell for handover (HO). For example, based on UE measurements, the source Cell 1 may have two HO candidates, Cell 2 and Cell 3, but only Cell 3 supports all the slices the UE needs. However, as the source cell does not know in advance the UE trajectory, it has to also prepare Cell 2. This is especially relevant for Conditional HO (CHO) type of mobility as there is a delay between HO preparation and execution. Therefore, Cell 2 cannot be excluded from prepared cells to avoid Radio Link Failure (RLF) and total loss of service even if a HO to Cell 2 would mean loss of service for some of the required slices. Also, currently, in the CHO execution phase, the UE cannot make any prioritization of potential target cells based on the required slices of the UE. In Rel-18 networks, this becomes critical as the slices may only be supported in a small number of cells and above situation of mobility across the slice border could appear more often.

Observation 2: In case of CHO, the source may need to prepare also cells that do not have all the necessary slices available, and it does not know if the selected non-supporting target was indeed the only option for the UE.

Once the UE successfully completes the HO, the source cell would know which target was selected by the UE and therefore would also know which slices of the UE will continue in the selected target cell and which slices will not continue, as the source cell knows the slice support of all its neighbor cells. However, based on current information, the source cell cannot detect how far the UE was from the candidates that do have all the slices available. For proper root cause analysis, it is crucial to differentiate the cases of UE1, UE2 and UE3 in Figure 1.

- For UEs with execution location similar to UE1, it might help to adjust the mobility parameters like CIO to prioritize HO from Cell 1 to Cell 3 instead of Cell 2 to ensure service continuity for all the slices of the UE.

- However, for UEs with mobility similar to UE2 or UE3, it might not be sufficient to change the mobility parameters only, as these UEs are far away from Cell 3 at the point of HO. However, if majority of the UEs that are connected to Slice B follow this pattern, this could also indicate a coverage problem in the Slice B planning.

Figure 1: UE Mobility at Network Slice Border

Observation 3: Service degradation due to handovers to cells that do not support necessary slices can be avoided if the source cell can analyze the condition at the execution of CHO of the majority of the UEs associated with a certain network slice.

One potential way to record above mentioned measurements is to enhance the Successful HO report (SHR) as the HO was successful from radio perspective. The enhancements could include:

- Adding a new trigger condition to record an SHR i.e. if the UE observes after decoding the HO command at the point of HO execution that one or more of its connected slices will not be supported by the selected target cell, the UE triggers logging of SHR.

- The SHR cause value would also need to be enhanced to let the network know that it was due to non-support of the slice at target cell and service discontinuity.

Once the selected target cell detects that the SHR was generated for network slice service continuity problem, the target cell also forwards the SHR to the original source cell for proper root cause analysis and corrective action, as defined for the SHR handling.

Proposal 4: The SHR mechanism is enhanced to enable recording the SHR in case when some of the used slices are not supported in the target cell and that a slice-related cause value is added.

# 3	Conclusions

In this paper, we address two slice-related problems discussed within the Work Item. First, we propose what enhancements are needed to enhance MDT reporting of slice support. We observe that:

Observation 1: NSAG configuration is meant to decrease connection setup delay and related signalling. It may, however, be non-optimal that hinders to achieve the intended goals.

And we make following proposals:

Proposal 1: A UE shall be able to log cases where the NSAG configuration did not enable fast connection setup and what kind of problem the UE faced.

Proposal 2: A UE shall be able to log additional information helpful to trace the cause of the recorded problem.

Proposal 3: RAN3 kindly requests RAN2 to specify the above measurement enhancements for logged MDT for slice-based cell reselection.

Then, we analyse options to avoid unnecessary handovers to cells not supporting all needed slices:

Observation 2: In case of CHO, the source may need to prepare also cells that do not have all the necessary slices available, and it does not know if the selected non-supporting target was indeed the only option for the UE.

Observation 3: Service degradation due to handovers to cells that do not support necessary slices can be avoided if the source cell can analyze the condition at the execution of CHO of the majority of the UEs associated with a certain network slice.

Based on the above, we propose:

Proposal 4: The SHR mechanism is enhanced to enable recording the SHR in case when some of the used slices are not supported in the target cell and that a slice-related cause value is added.

The above proposals are implemented in the two annexes below: Annex 1 proposes a TP related to mobility enhancements, while Annex 2 proposes an LS to RAN2 to request needed support.

# References

- R3-243146, RAN3 #124

- 3GPP TS 23.501

- 3GPP TS 38.300

- 3GPP TS 38.331

# Annex 1: Text proposal to TS 38.300 for SHR enhancement

The below text is proposed to be added to the TS 38.300.

First change, ommited text not changed

#### 15.5.2.7	Successful HO

One of the functions of Mobility Robustness Optimization is to detect a sub-optimal successful handover event. The aim is to identify underlying conditions during successful ordinary handovers, successful DAPS handovers, or successful Conditional handovers.

For analysis of successful handover, the UE may collect Successful Handover Report (SHR) based on configuration by network, if stored, and makes the SHR available to the network as specified in TS 38.331 [12]. The UE stores the SHR until it is fetched by the network or for 48 hours after the SHR is recorded.

For SHR collected during intra-NR handover, if the target NR node fetches the SHR from the UE and the trigger of SHR is T310/T312 or slice unavailability, it may forward the information to the source NR node, i.e. the node handling the cell reported as source cell in this SHR, by using the ACCESS AND MOBILITY INDICATION message over Xn or by means of the Uplink RAN configuration transfer procedure and Downlink RAN configuration transfer procedure over NG.

If the NG-RAN node that fetches the SHR from the UE is neither the source node nor the target node of the handover, it forwards the information to the node(s) which configured the SHR trigger causing the SHR to be generated, by using the ACCESS AND MOBILITY INDICATION message over Xn or by means of the Uplink RAN configuration transfer procedure and Downlink RAN configuration transfer procedure over NG.

In case of failure shortly after successful Handover, the same mobility event may generate both a SHR and a RLF report. In this case, the node(s), which configured the SHR trigger causing the SHR, may take the duplication into account e.g. ignore the SHR.

Upon retrieval of an SHR, the receiving node may analyse whether its mobility configuration needs adjustment.

The SHR report can be used to detect one case of Intra-system Too Late Handover, namely when DAPS HO is configured but an RLF is detected in the source cell during a successful DAPS HO.

The SHR report can be collected for intra-NR handover and for handover from NR to E-UTRA.

Remaining text not changed

# Annex 2: Draft LS to RAN2

3GPP TSG-RAN WG3 #127		R3-25xxxx

Athens, Greece, EU, 17-21 Feb 2025

Title:	[DRAFT] LS on SON for Network Slicing

Response to:

Release:	Release 19

Work Item:	NR_ENDC_SON_MDT_Ph4-Core

Source:	Nokia [to be RAN3]

To:	RAN2

Cc:

Contact Person:	Krzysztof Kordybach (krzysztof.kordybach@nokia.com)

Send any reply LS to:	3GPP Liaisons Coordinator, 3GPPLiaison@etsi.org

Attachments:	R3-25xxxx

1. Overall Description:

RAN3 would like to inform RAN2 about decisions made in the context of SON for Network Slicing.

RAN3 agreed to work on enhancements for slice-based cell reselection with logged MDT measurements

RAN3 identified following enhancements in MDT mechanism:

- New logging trigger for slice-based cell reselection measurements;

- Logging slice groups used for identifying reselection priorities;

- Used frequency priorities;

- Logging problems to connect to needed slice;

- Logging measurements in relation to frequency priorities;

RAN3 agreed to work on network slice mobility optimization utilizing Successful HO report

RAN3 identified following enhancements to SHR mechanism:

- Adding a trigger to record SHR in case of successful execution of a HO to a cell that does not support all the needed slices;

- Adding a new SHR cause to enable identification of slice-related reports;

RAN3 has agreed a TP to the BL CR to TS 38.300 related to the SHR enhancement, as attached.

2. Actions:

To RAN2

RAN3 kindly asks RAN2 to enable the needed signalling.

3. Date of Next TSG-RAN3 Meetings:

TSG-RAN3 Meeting #127-bis			7-11 April 2025						China

TSG-RAN3 Meeting #128			19-23 May 2025					Malta, EU