3GPP TSG-RAN WG3 Meeting #127	R3-250088

Athens, Greece, 17-21 February 2025

Agenda Item:		11.2

Source:		Ofinno

Title:	AI/ML enabled slicing

Document for:		Discussion and decision

# Introduction

After RAN3 #126 meeting, the following progress has been achieved on the agenda item 11.2 AI/ML enabled slicing:

Enhance the Data Collection Reporting Initiation procedure and Data Collection Reporting procedure to enable requesting and reporting of the predicted radio resource status per slice and predicted slice available capacity over the Xn interface.

WA: UE performance per each PDU session associated to a requested S-NSSAI is reported from target node to source node.

Introduce Slice To Report List for Data Collection IE including PLMN ID and S-NSSAI in DATA COLLECTION REQUEST message.

RAN3#126:

First Bit (Predicted Radio Resource Status) can be reused for requesting predicted slice radio resource status with list of Slice in the DATA COLLECTION REQUEST message.

Introduce the new bit for Predicted slice available capacity in the Report Characteristics for Data Collection IE in the DATA COLLECTION REQUEST message.

Introduce a Predicted Slice Available Capacity IE in the DATA COLLECTION UPDATE message to report the requested predicted slice available capacity to the requesting node.

Update the Semantics Description of Predicted Radio Resource Status IE in the DATA COLLECTION UPDATE message to indicate that this IE also includes the Slice Radio Resource Status List IE.

Source node can request per UE per S-NSSAI granularity for UE performance feedback.

There is no consensus if UE performance can be measured in per UE per S-NSSAI and reported from target node to source node.

This contribution discusses several aspects related to UE performance feedback.

# Discussion

## 2.1	UE performance feedback per slice

RAN3 has agreed to introduce slice-specific prediction reporting to the DATA COLLECTION procedure.

In particular, Slice To Report List for Data Collection is included to the DATA COLLECTION REQUEST:

| Cell To Report List for Data Collection |  | 0..1 |  | Cell ID list to which the request applies. | YES | ignore |
| --- | --- | --- | --- | --- | --- | --- |
| >Cell To Report Item for Data Collection |  | 1 .. <maxnoofCellsinNG-RANnode> |  |  | – |  |
| >>Cell ID | M |  | Global NG-RAN Cell Identity 9.2.2.27 | Indicates an NR Cell Identity. | – |  |
| >>Slice To Report List for Data Collection |  | 0..1 |  | S-NSSAI list to which the request applies. | YES | ignore |
| >>>Slice To Report Item for Data Collection |  | 1 .. < maxnoofBPLMNs > |  |  | - |  |
| >>>>PLMN Identity | M |  | 9.2.2.4 | Broadcast PLMN | - |  |
| >>>>S-NSSAI List |  |  | 1 |  | - |  |
| >>>>>S-NSSAI Item |  | 1 .. < maxnoofSliceItems> |  |  | - |  |
| >>>>>>S-NSSAI | M |  | 9.2.3.21 |  | - |  |

Also, Tenth Bit for requesting the Predicted Slice Available Capacity is added to the Report Characteristics for Data Collection:

| Report Characteristics for Data Collection | C-ifRegistrationRequestForDataCollectionStart |  | BITSTRING (SIZE(32)) | Each position in the bitmap indicates the object the NG-RAN node2 is requested to report. First Bit = Predicted Radio Resource Status, Second Bit = Predicted Number of Active UEs, Third Bit = Predicted RRC Connections Fourth Bit = Average UE Throughput DL, Fifth Bit = Average UE Throughput UL, Sixth Bit = Average Packet Delay, Seventh Bit = Average Packet Loss DL Eighth Bit = Energy Cost Ninth Bit = Measured UE Trajectory Tenth Bit = Predicted Slice Available Capacity Other bits are ignored by the NG-RAN node2. | YES | reject |
| --- | --- | --- | --- | --- | --- | --- |

This allows NG-RAN node1 to request NG-RAN node2 to report predicted Slice Radio Resource Status and predicted Slice Available Capacity:

-	the SSB Area Radio Resource Status List IE, excluding the DL scheduling PDCCH CCE usage IE and UL scheduling PDCCH CCE usage IE, included in the Predicted Radio Resource Status IE, if the first bit, "Predicted Radio Resource Status" of the Report Characteristics for Data Collection IE included in the DATA COLLECTION REQUEST message is set to "1" and if the measurement object is admitted by NG-RAN node2. If the Slice To Report List for Data Collection IE is included for a cell, the Predicted Radio Resource Status IE for such cell shall, if supported, include the Slice Radio Resource Status Item IE.

-	the Predicted Slice Available Capacity IE, if the tenth bit, "Predicted Slice Available Capacity" of the Report Characteristics for Data Collection IE included in the DATA COLLECTION REQUEST message is set to "1"and if

- the Slice To Report List for Data Collection IE is included for the cell, and

- the measurement object is admitted by NG-RAN node2.

This allows, for example, AI/ML in the NG-RAN node1 to make, e.g., mobility, energy saving, load balancing, CCO decisions considering slice-level granularity. After such decisions, it is beneficial for the NG-RAN node1 to get slice-level feedback from the NG-RAN node2.

One form of such feedback would be slice-level measurements from the NG-RAN node2 (e.g., Slice Available Capacity measurements and Slice Radio Resource Status List in RESOURCE STATUS UPDATE message). But it is also beneficial for the AI/ML in the NG-RAN node1 to understand how its decisions have impacted UEs handed over from the NG-RAN node1 to the NG-RAN node2. For this purpose, it is beneficial to have a possibility for the NG-RAN node1 to request the NG-RAN node2 to report the UE performance feedback per slice.

Proposal 1: Source node can request target node per slice reporting of UE performance feedback. If requested and supported, the target node can report to the source node per slice UE performance feedback.

Specific methods to calculate per slice UE performance feedback should be harmonized/clarified, in cooperation with SA5 when needed.

## 2.2	Slice To Report List for Data Collection

Slice To Report List for Data Collection has been included to the DATA COLLECTION REQUEST:

| Cell To Report List for Data Collection |  | 0..1 |  | Cell ID list to which the request applies. | YES | ignore |
| --- | --- | --- | --- | --- | --- | --- |
| >Cell To Report Item for Data Collection |  | 1 .. <maxnoofCellsinNG-RANnode> |  |  | – |  |
| >>Cell ID | M |  | Global NG-RAN Cell Identity 9.2.2.27 | Indicates an NR Cell Identity. | – |  |
| >>Slice To Report List for Data Collection |  | 0..1 |  | S-NSSAI list to which the request applies. | YES | ignore |
| >>>Slice To Report Item for Data Collection |  | 1 .. < maxnoofBPLMNs > |  |  | - |  |
| >>>>PLMN Identity | M |  | 9.2.2.4 | Broadcast PLMN | - |  |
| >>>>S-NSSAI List |  |  | 1 |  | - |  |
| >>>>>S-NSSAI Item |  | 1 .. < maxnoofSliceItems> |  |  | - |  |
| >>>>>>S-NSSAI | M |  | 9.2.3.21 |  | - |  |

Currently, it is intended for reporting predicted Slice Radio Resource Status and predicted Slice Available Capacity.

The Slice To Report List for Data Collection may not be directly applicable for reporting UE performance feedback per slice. For example, the UE may not use all slices listed in the Slice To Report List for Data Collection.

Two solutions are possible in this case.

Option 1:

The NG-RAN node2 (target node) may report the NG-RAN node1 (source node) UE performance feedback per slice only for those slices from the Slice To Report List for Data Collection, that are applicable for the UE handed over from the NG-RAN node1 to the NG-RAN node2. The NG-RAN node2 signals the list of applicable slices inside Slice UE Performance IE when reporting the UE performance feedback per slice.

Option 2:

The NG-RAN node1 (source node) may explicitly signal to the NG-RAN node2 (target node), a UE-associated list of slices for reporting the UE performance feedback. For example, this may be included in the same HANDOVER REQUEST message and S-NODE ADDITION REQUEST message that initiates UE performance feedback reporting by the Data Collection ID IE.

Proposal 2: Consider two options to select a list of slices for requesting/reporting the UE performance feedback per slice:

Option 1: Target node reports the UE performance feedback per slice only for the applicable slices from the Slice To Report List for Data Collection. The target node signals the list of applicable slices inside Slice UE Performance IE when reporting the UE performance feedback per slice.

Option 2: Source node explicitly signals list of slices for a specific UE when initiating the reporting of the UE performance feedback in, e.g., HANDOVER REQUEST message or S-NODE ADDITION REQUEST message.

# Proposals

Proposal 1: Source node can request target node per slice reporting of UE performance feedback. If requested and supported, the target node can report to the source node per slice UE performance feedback.

Proposal 2: Consider two options to select a list of slices for requesting/reporting the UE performance feedback per slice:

Option 1: Target node reports the UE performance feedback per slice only for the applicable slices from the Slice To Report List for Data Collection. The target node signals the list of applicable slices inside Slice UE Performance IE when reporting the UE performance feedback per slice.

Option 2: Source node explicitly signals list of slices for a specific UE when initiating the reporting of the UE performance feedback in, e.g., HANDOVER REQUEST message or S-NODE ADDITION REQUEST message.

# Reference

[1] R3-250001, RAN3#127 Meeting Agenda

[2] R3-250002, RAN3#126 Meeting Report

[3] R3-247928, (BL CR to TS 38.423) Support of enhancements on AI/ML for NG-RAN

[4] R3-247785, SoD of CB AI based Slicing