3GPP TSG-RAN3 Meeting #127	R3-250101

Athens, Greece, Feb 17th – 21nd, 2025

Agenda item:	22.2

Source:	Qualcomm Incorporated

Title:	Support for Multi-hop L2 U2N Relays

Document for:	Discussion and Decision

# Introduction

The following is the Rel-19 WI scope:

Specify mechanisms to support up to two additional hops relays on top of Rel-17 U2N relay.

The work starts with one additional hop relay (i.e., remote UE -> first relay UE -> last relay UE -> gNB) until RAN#107 and further check will be made in RAN#107 if it can be easily extended to two additional hops relays (i.e., remote UE -> first relay UE -> second relay UE -> last relay UE -> gNB).

A necessary criterion for the specified mechanisms is easy extensibility to support two additional hop relays for the work done until RAN#107 and to be forward compatible for future extensions for additional relays.

- Relay discovery and (re)selection [RAN2]

- Signaling support for relay UEs and remote UE authorization if SA2 concludes it is needed [RAN3]

- Impact on SRAP and QoS handling for multi-hop [RAN2]

- Control plane procedures [RAN2, RAN3]

# Discussion

  - Terminologies

RAN2 agreed the following:

n-hop relaying involves n relay UEs by definition.

Distinguish the “last” relay UE as the U2N relay UE directly connected to the gNB, for RAN2 discussion

Distinguish the “intermediate” relay UEs as the other relay UEs, for RAN2 discussion

Distinguish the “first” relay UE as the intermediate relay UE directly connected to the remote UE, for RAN2 discussion

The following figures show the above definitions pictorially:

2-hop relay

3-hop relay

It is proposed for RAN3 to follow the definitions agreed by RAN2 and therefore the following is proposed:

RAN3 to use the terminologies agreed by RAN2 on “last relay UE”, “intermediate relay UE” and “first relay UE” for RAN3 discussions.

  - Authorization

SA2 sent the LS R3-250070 to RAN3

SA2 would like to inform RAN2 and RAN3 that SA2 agreed to provide the following service authorization information from AMF to NG-RAN for Layer-2 multi-hop UE-to-Network relay operations as attached CR:

- whether the UE is authorized to act as a 5G ProSe Layer-2 UE-to-Network Relay supporting 5G ProSe Layer-2 multi-hop UE-to-Network Relay;

- whether the UE is authorized to act as a 5G ProSe Layer-2 Intermediate UE-to-Network Relay;

- whether the UE is authorized to act as a 5G ProSe Layer-2 Remote UE supporting 5G ProSe Layer-2 multi-hop UE-to-Network Relay.

SA2 agreed to provide the following service authorization information from AMF to NG-RAN for Layer-2 multi-hop UE-to-Network relay operations:

- whether the UE is authorized to act as a 5G ProSe Layer-2 UE-to-Network Relay supporting 5G ProSe Layer-2 multi-hop UE-to-Network Relay;

- whether the UE is authorized to act as a 5G ProSe Layer-2 Intermediate UE-to-Network Relay;

- whether the UE is authorized to act as a 5G ProSe Layer-2 Remote UE supporting 5G ProSe Layer-2 multi-hop UE-to-Network Relay.

Over NGAP, XnAP and F1AP, there is a “5G ProSe Authorized” IE that provides information on the authorization status of the UE to use different 5G ProSe services.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| 5G ProSe Direct Discovery | O |  | ENUMERATED (authorized, not authorized, ...) | Indicates whether the UE is authorized for 5G ProSe Direct Discovery | - |  |
| 5G ProSe Direct Communication | O |  | ENUMERATED (authorized, not authorized, ...) | Indicates whether the UE is authorized for 5G ProSe Direct Communication | - |  |
| 5G ProSe Layer-2 UE-to-Network Relay | O |  | ENUMERATED (authorized, not authorized, ...) | Indicates whether the UE is authorized for 5G ProSe Layer-2 UE-to-Network Relay | - |  |
| 5G ProSe Layer-3 UE-to-Network Relay | O |  | ENUMERATED (authorized, not authorized, ...) | Indicates whether the UE is authorized for 5G ProSe Layer-3 UE-to-Network Relay | - |  |
| 5G ProSe Layer-2 Remote UE | O |  | ENUMERATED (authorized, not authorized, ...) | Indicates whether the UE is authorized for 5G ProSe Layer-2 Remote UE | - |  |
| 5G ProSe Layer-2 Multipath | O |  | ENUMERATED (authorized, not authorized, ...) | Indicates whether the 5G ProSe Layer-2 Remote UE is authorized for 5G ProSe multipath transmission | YES | ignore |
| 5G ProSe Layer-2 UE-to-UE Relay | O |  | ENUMERATED (authorized, not authorized, ...) | Indicates whether the UE is authorized for 5G ProSe Layer-2 UE-to-UE Relay UE | YES | ignore |
| 5G ProSe Layer-2 UE-to-UE Remote | O |  | ENUMERATED (authorized, not authorized, ...) | Indicates whether the UE is authorized for 5G ProSe Layer-2 UE-to-UE Remote UE. | YES | ignore |

We propose to extend the existing "5G ProSe Authorized" IE in NGAP, XnAP and F1AP to support the 3 new authorization information as requested by SA2

Extend "5G ProSe Authorized" IE in NGAP, XnAP and F1AP to support the below  authorization information for L2 multi-hop U2N relays:

  - whether the UE is authorized to act as a 5G ProSe Layer-2 UE-to-Network Relay supporting 5G ProSe Layer-2 multi-hop UE-to-Network Relay;

  - whether the UE is authorized to act as a 5G ProSe Layer-2 Intermediate UE-to-Network Relay;

  - whether the UE is authorized to act as a 5G ProSe Layer-2 Remote UE supporting 5G ProSe Layer-2 multi-hop UE-to-Network Relay.

  - Multi-hop path addition and release

In Rel-17/18, RAN3 looked at F1AP enhancements to support indirect path addition (for single-hop) and for indirect path release (from single hop). Also, RAN3 looked at how L2 configurations (PC5/Uu RLC configurations, SRAP configurations, PC5/Uu QoS split) are configured and impacts to F1AP procedures to support these.

In Rel-19, for multi-hop relaying support, RAN3 should look at F1AP enhancements needed to support intra-gNB multi-hop path establishment and intra-gNB multi-hop release. Further, RAN3 should look at a hop-by-hop addition/release or a direct multi-hop addition/release. Therefore, the following is proposed:

RAN3 to consider F1AP enhancements needed to support intra-gNB multi-hop path establishment

  - Direct  single-hop  multi-hop

  - Direct  multi-hop

RAN3 to consider F1AP enhancements to support intra-gNB multi-hop path release

  - Multi-hop  single-hop  Direct

  - Multi-hop  Direct

  - Multi-hop service continuity scenarios

RAN2 agreed the following on service continuity scenarios:

The intra-gNB switching from indirect to direct path procedure defined in TS 38.300 for single hop case can be used as the baseline for multi-hop indirect to direct path switching.​ Event X1 /X2 can be reused in multi-hop indirect to direct path switching with the understanding that the “first relay UE” in multi-hop relay link is “serving L2 U2N Relay UE” to be reported.​

The intra-gNB switching from indirect path to indirect path procedure defined in TS 38.300 in single hop case can be used as baseline for multi-hop indirect to single hop indirect path switching procedure.​ The following measurement events can be reused in multi-hop indirect to single-hop indirect path switching:​ Event Y2​, Event Z1 with the understanding that the “first relay UE” in multi-hop relay link is “serving L2 U2N Relay UE” to be reported.

Multi-hop direct path-switch

RAN3 to consider F1AP enhancements to support multi-hop  direct path-switch scenarios:

  - Intra-gNB multi-hop indirect to direct path switching

  - Intra-gNB direct to multi-hop indirect path switching

Multi-hop  Single-hop path-switch

RAN3 to consider F1AP enhancements to support multi-hop  single-hop following path-switch scenarios:

  - Intra-gNB multi-hop indirect to single hop indirect path switching

  - Intra-gNB single-hop indirect to multi-hop indirect path switching

RAN2 supports the case that the target U2N relay UE is a new relay UE which is not on the source relay path, and existing Rel-18 indirect path to indirect path switching can be reused.​

RAN2 will support the case that the target single-hop relay UE is the last relay UE on the source path using the existing Rel-18 i2i path switching.  Spec impact will be minimised.​

RAN2 does not specify anything to support the case that that the target U2N relay UE is an intermediate relay UE which is on the source relay path using a single procedure.  ​This case can be handled in the baseline control plane model, if necessary and subject to network implementation, by sequential path switches for the remote UE and the target intermediate relay UE.

As per the above RAN2 agreements, RAN2 considered the following cases:

Sub-case 1: Target U2N relay UE is a new relay UE which is not on the source relay path

Sub-case 2: Target single-hop relay UE is the last relay UE on the source path

We propose RAN3 to consider the above sub-cases during multi-hop  single-hop path switching scenarios

RAN3 to consider if there are any F1AP enhancements to support the following the two sub-cases during multi-hop  single-hop path switching scenarios:

  - Sub-case 1: Target relay UE is a new relay UE which is not on the source relay path

  - Sub-case 2: Target relay UE is the last relay UE on the source path

# Conclusion

Proposal 1:	RAN3 to use the terminologies agreed by RAN2 on “last relay UE”, “intermediate relay UE” and “first relay UE” for RAN3 discussions.

Observation 1.	SA2 agreed to provide the following service authorization information from AMF to NG-RAN for Layer-2 multi-hop UE-to-Network relay operations:

o whether the UE is authorized to act as a 5G ProSe Layer-2 UE-to-Network Relay supporting 5G ProSe Layer-2 multi-hop UE-to-Network Relay;

owhether the UE is authorized to act as a 5G ProSe Layer-2 Intermediate UE-to-Network Relay;

owhether the UE is authorized to act as a 5G ProSe Layer-2 Remote UE supporting 5G ProSe Layer-2 multi-hop UE-to-Network Relay.

Proposal 2:	Extend "5G ProSe Authorized" IE in NGAP, XnAP and F1AP to support the below  authorization information for L2 multi-hop U2N relays:

a. whether the UE is authorized to act as a 5G ProSe Layer-2 UE-to-Network Relay supporting 5G ProSe Layer-2 multi-hop UE-to-Network Relay;

b. whether the UE is authorized to act as a 5G ProSe Layer-2 Intermediate UE-to-Network Relay;

c. whether the UE is authorized to act as a 5G ProSe Layer-2 Remote UE supporting 5G ProSe Layer-2 multi-hop UE-to-Network Relay.

Proposal 3:	RAN3 to consider F1AP enhancements needed to support intra-gNB multi-hop path establishment

a. Direct  single-hop  multi-hop

b. Direct  multi-hop

Proposal 4:	RAN3 to consider F1AP enhancements to support intra-gNB multi-hop path release

a. Multi-hop  single-hop  Direct

b. Multi-hop  Direct

Proposal 5:	RAN3 to consider F1AP enhancements to support multi-hop  direct path-switch scenarios:

a. Intra-gNB multi-hop indirect to direct path switching

b. Intra-gNB direct to multi-hop indirect path switching

Proposal 6:	RAN3 to consider F1AP enhancements to support multi-hop  single-hop following path-switch scenarios:

a. Intra-gNB multi-hop indirect to single hop indirect path switching

b. Intra-gNB single-hop indirect to multi-hop indirect path switching

Proposal 7: RAN3 to consider if there are any F1AP enhancements to support the following two sub-cases during multi-hop  single-hop path switching scenarios:

a. Sub-case 1: Target relay UE is a new relay UE which is not on the source relay path

b. Sub-case 2: Target relay UE is the last relay UE on the source path

# 4. References