3GPP TSG-RAN WG3 #127                                          R3-250110

Athens, GR, 17th – 21st Feb, 2024

Source:	CATT

Title:	Discussion on A-IoT reader selection

Agenda Item:	16.2

Document for:	Decision

Introduction

On support of A-IoT, a study item has been completed in the last RAN WG meetings. Corresponding solutions on the architecture, signaling, locating, etc. are captured in the TR38.769 [1]. In RAN#106, a new work item for Ambient IoT has been approved (WID in [2]), aimed to standardize RAN aspects of Ambient IoT. In the WID, we will only focus on Topology 1, the major RAN3 scope are highlighted below:

- RAN3 scope:

  - Specify necessary architectural aspects, and signaling and procedures between A-IoT RAN and A-IoT CN to support the A-IoT functions, assuming an architecture of aggregated gNB, including:

    - Inventory and command operations

    - Device location reporting at reader ID granularity

Note: The above A-IoT functions are supported over the existing NG interface, based on architecture(s) defined by RAN3/SA2.

A key issue from the Study Item (SI) phase is determining which node performs A-IoT reader selection, as this impacts information exchange between A-IoT RAN and A-IoT CN.

SA2 also discussed the issue on A-IoT reader selection and sent an LS[3] to RAN3, as below:

[Question to RAN3]

SA2 has concluded for topology 1, the AIOTF selects the AIOT RAN node(s) and optionally provides a list of the BS readers to AIOT RAN. SA2 assumes that OAM can configure the AIOTF with needed information for this selection. SA2 have further discussed whether the AIOT RAN may provide the AIOT RAN information e.g., supported AIOT RAN serving area, supported BS reader ID list and corresponding BS reader serving area to the core network.

SA2 kindly asks RAN3 to clarify whether RAN3 plan to support the above AIOT RAN information exchange with AIOTF. If so, SA2 kindly ask RAN3 to provide the definition of the appropriate parameters.

In this contribution, we would discuss how to perform Reader selection in Topology 1, and provide our observations and proposals, and draft LS reply accordingly.

Discussion

  - Awareness of A-IoT RAN information in AIOTF

AS discussed in LS [3], there are two solutions on providing A-IoT RAN information in AIOTF

- Signaling based solution.

- OAM based solution.

For signalling based solution, A-IoT RAN information is provided from A-IoT RAN to A-IoT CN via NG interface message. For OAM based solution, it is configured to AIOTF by OAM which may need manual work. But considering the reader(s) in Topology 1 are stationary, OAM based solution is slightly preferred for simplicity.

Proposal 1: A-IoT CN could aware of A-IoT RAN information by signalling based solution or OAM based solution. OAM based solution is slightly preferred for simplicity.

  - Reader selection procedure

First of all, when inventory or command service is triggered to certain area, A-IoT CN should select proper A-IoT RAN to send inventory or command request message according to service area.

During inventory and command procedure, reader information could be sent to A-IoT CN and A-IoT CN could store the relationship between reader and device for subsequent operation.

Proposal 2: AIOTF could acquire the relationship between reader and device via inventory or command procedure.

Then for the subsequent operation, A-IoT CN could select the proper reader(s) within this area. In another word, A-IoT CN could take the responsibility for reader selection, in case the reader information is available.

Proposal 3: If reader information is available in A-IoT CN, the A-IoT CN could take the responsibility for reader selection, and provide the selected Reader ID(s) to A-IoT RAN when initiating an A-IoT service.

In case A-IoT CN lacks reader information, it may need to provide intended service area information to A-IoT RAN to perform the A-IoT services in proper area. A-IoT RAN may use this information to select proper Readers it holds for corresponding A-IoT service.

Proposal 4: If reader information is not available in A-IoT CN, the A-IoT CN may need to provide the intended service area information to A-IoT RAN to perform the A-IoT service in proper area.

As for the intended service area, it may be defined as cell list, TA list or other new defined serving area. In LS [3], SA2 also suggests defining AIOT RAN serving area, supported BS reader ID list and corresponding BS reader serving area.

In our understanding, not necessary to define new serving area identity in specification, for example, GNSS area, floor or warehouse, etc. For GNSS area, it cannot to indicate reader deployed vertically. For floor or warehouse, it could only be used in certain scenarios. It is hard to define an area suitable for all available scenarios.

Proposal 5: Not necessary to define new serving area (e.g. GNSS area) for A-IoT services.

Proposal 6: If Reader Information is not available in the AIoT CN, Cell list, TAI list may be used to indicate the indicated service area from AIoT CN to AIoT RAN.

Base on the discussion above, it’s proposed to reply the LS to SA2:

- RAN3 assumed that AIOTF should have necessary information to select proper AIoT RAN node, and probably the intended BS Readers to perform A-IoT services. For Topology 1, Corresponding information (e.g. supported AIOT RAN serving area, supported BS reader ID list and corresponding BS reader serving area) are assumed to be static, which could be configured to AIoTF via OAM configuration. Then, AIoTF is able to select proper AIoT RAN node(s) and proper BS Reader(s) to perform A-IoT services.

- In case A-IoT CN lacks AIOT RAN information, AIoTF may also use the cell list, TAI list as the intended service area which would be provide to A-IoT RAN to perform the A-IoT services in proper area. Then A-IoT RAN can use this information to select proper BS readers it holds for corresponding A-IoT service(s).

Corresponding draft LS reply is provided in the Annex, RAN3 is requested to discuss and agree the draft LS.

Proposal 7: Discuss and agree the draft LS reply to SA2 on BS reader selection, draft LS is provided in the Annex.

Conclusions

In this contribution, we mainly discussed the issue on UE reader selection. Based on the discussion above, the following proposals are provided:

Proposal 1: A-IoT CN could aware of A-IoT RAN information by signalling based solution or OAM based solution. OAM based solution is slightly preferred for simplicity.

Proposal 2: AIOTF could acquire the relationship between reader and device via inventory or command procedure.

Proposal 3: If reader information is available in A-IoT CN, the A-IoT CN could take the responsibility for reader selection, and provide the selected Reader ID(s) to A-IoT RAN when initiating an A-IoT service.

Proposal 4: If reader information is not available in A-IoT CN, the A-IoT CN may need to provide the intended service area information to A-IoT RAN to perform the A-IoT service in proper area.

Proposal 5: Not necessary to define new serving area (e.g. GNSS area) for A-IoT services.

Proposal 6: If Reader Information is not available in the AIoT CN, Cell list, TAI list may be used to indicate the indicated service area from AIoT CN to AIoT RAN.

Proposal 7: Discuss and agree the draft LS reply to SA2 on BS reader selection, draft LS is provided in the Annex.

Reference

- TR 38.769 TR 38.769 Study on solutions for ambient IoT (Internet of Things)

- RP-243326 New Work Item: Solutions for Ambient IoT (Internet of Things) in NR

- S2-2501343 “LS on SA2 Ambient IoT TR conclusion update” 3GPP SA WG2 Meeting #166

Draft LS Reply to SA2

Title:	[Draft] Reply LS on SA2 Ambient IoT TR conclusion update

Response to:	S2-2501343

Release:	Rel-19

Work Item:	Ambient_IoT_solutions-Core

Source:	CATT (to be RAN3)

To:	SA2

Cc:

Contact Person:

Name:	Jiancheng Sun

E-mail Address:	sunjiancheng@catt.cn

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

Attachments:	None

1. Overall Description:

RAN3 thanks SA2 for the LS on SA2 Ambient IoT TR conclusion update. RAN3 assumed that AIOTF should have necessary information to select proper AIoT RAN node, and probably the intended BS Readers to perform A-IoT services. For Topology 1, Corresponding information (e.g. supported AIOT RAN serving area, supported BS reader ID list and corresponding BS reader serving area) are assumed to be static, which could be configured to AIoTF via OAM configuration. Then, AIoTF is able to select proper AIoT RAN node(s) and proper BS Reader(s) to perform A-IoT services.

In case A-IoT CN lacks AIOT RAN information, AIoTF may also use the cell list, TAI list as the intended service area which would be provide to A-IoT RAN to perform the A-IoT services in proper area. Then A-IoT RAN can use this information to select proper BS readers it holds for corresponding A-IoT service(s).

2. Actions:

To SA2:

ACTION: 	RAN3 kindly asks SA2 to take the above feedback into consideration.

3. Date of Next TSG-RAN3 Meetings:

3GPP TSG RAN WG3#127        17 - 21 Feb 2025				Athens, GR

3GPP TSG RAN WG3#127-bis		 07 - 11 Apr 2025				Wuhan, CN