3GPP TSG-RAN WG3 Meeting #127                             	  R3-250111

Athens, GR, 17th – 21st Feb, 2025

Source:	CATT

Title:	Discussion on locating of A-IoT device

Agenda Item:	16.3

Document for:	Discussion and Decision

- Introduction

On support of A-IoT, a study item has been completed in the last RAN WG meetings. Corresponding solutions on the architecture, signallings, locating, etc. are captured in the TR38.769 [1]. In RAN#106, a new work item for Ambient IoT has been approved(WID in [2]), aimed to standardize RAN aspects of Ambient IoT. In the WID, we will only focus on Topology 1, the major RAN3 scope are highlighted below:

- RAN3 scope:

  - Specify necessary architectural aspects, and signaling and procedures between A-IoT RAN and A-IoT CN to support the A-IoT functions, assuming an architecture of aggregated gNB, including:

    - Inventory and command operations

    - Device location reporting at reader ID granularity

Note: The above A-IoT functions are supported over the existing NG interface, based on architecture(s) defined by RAN3/SA2.

In this contribution, we will discuss device location reporting at reader ID granularity, and provide the observations and proposals accordingly.

- Discussion

In the study phase, locating of A-IoT device has been studied and captured in the TR 38.769 [1], as below:

6.9	Locating ambient IoT devices

6.9.1	General

A-IoT device location information may be used for the following purposes:

(a) improving the A-IoT operation itself, e.g., by A-IoT CN sending a Command to one or more readers (e.g., the last reader(s)) associated to the device rather than sending it blindly.

(b) providing location information to the consumer of the A-IoT service.

Locating an Ambient IoT device at "reader ID granularity" is useful for both purposes and is to be supported.

NOTE: "reader ID granularity" is likely not sufficient for purpose (b) and may be refined in the future.

6.9.2	Topology 1

For support of A-IoT device location, the A-IoT RAN node may report the A-IoT RAN node ID to the A-IoT CN in the Inventory Report or Command Response messages.

The A-IoT RAN node may additionally report a Reader ID to the A-IoT CN in the Inventory Report and/or the Command Response messages, which corresponds to the reader to which the A-IoT device responded.

NOTE 1: Whether multiple Reader ID(s) can be included by the A-IoT RAN node in the Inventory Report/Command Response message needs further discussion.

NOTE 2: Definition and format of the A-IoT RAN node ID and Reader ID needs further discussion.

NOTE 3: Whether and how to have awareness of Reader location in A-IoT CN needs further discussion.

During the A-IoT study phase, it’s discussed and agreed to support RAN node ID + reader ID granularity on locating of an A-IoT device. We also discussed whether and how to support finer granularity, and not concluded then.

In the WID [2], the objective is quite clear, to support A-IoT device location reporting at reader ID granularity. So, we should only focus on the A-IoT device location reporting at reader ID granularity in the work item phase, not to discuss the potential solutions for finer granularity.

Proposal 1: Only support A-IoT device location reporting at reader ID granularity, finer granularity is not considered in Rel-19.

For support of A-IoT device location, the A-IoT RAN node may report a Reader ID to the A-IoT CN in the Inventory Report and/or the Command Response messages, which corresponds to the reader to which the A-IoT device responded in the AIoT Radio interface.

Proposal 2: the reader ID where the A-IoT device responded in the AIoT Radio interface could be included in the Inventory Report and/or the Command Response messages towards AIoT CN.

For Topology 1, one AIoT RAN node may hold one or multiple readers. The information of the reader(s), e.g. the ID, the location, the coverage, etc. are assumed to be static, which could be obtained  by the AIoT CN via OAM configuration or XXAP signalling.

Proposal 3: Reader Information, e.g. Reader ID, location, etc. could be provided to the AIoT CN via OAM or XXAP signalling.

On definition of Reader ID, we understand the reader ID is only used in the XXAP between AIoT RAN and AIoT CN to uniquely identify a Reader of one AIoT RAN node.  Here provide two possible options to define the Reader ID:

- Option 1: RAN node ID + Reader identifier (e.g. 1..255);

- Option 2: Reader identifier (e.g. 1..255);

Both of the options are feasible, considering the XXAP signalling is between the AIoT RAN and AIoT CN, to be simple, we could select option 2, as the reader ID transferred over the XXAP is just used to identify a Reader of a specific AIoT RAN node.

Proposal 4: Reader ID is used to uniquely identify a Reader associated to a certain AIoT RAN node, which could be defined as an INTEGER, e.g. INTEGER (0..255).

- Conclusion

Based on the discussion in section 2, the following observations and proposals are provided:

Proposal 1: Only support A-IoT device location reporting at reader ID granularity, finer granularity is not considered in Rel-19.

Proposal 2: the reader ID where the A-IoT device responded in the AIoT Radio interface could be included in the Inventory Report and/or the Command Response messages towards AIoT CN.

Proposal 3: Reader Information, e.g. Reader ID, location, etc. could be provided to the AIoT CN via OAM or XXAP signalling.

Proposal 4: Reader ID is used to uniquely identify a Reader associated to a certain AIoT RAN node, which could be defined as an INTEGER, e.g. INTEGER (0..255).

- Reference

- TR 38.769 Study on solutions for ambient IoT (Internet of Things)

- RP-243326 New Work Item: Solutions for Ambient IoT (Internet of Things) in NR