3GPP TSG RAN WG3 Meeting #127	R3-250117

Athens, Greece, 17 – 21 Feb, 2025

Title: 	(TP to TS37.340 and TS38.423) SN initiated inter-CU SCG LTM

Source: 	ZTE Corporation

Agenda item:	13.2

Document for:	Discussion and Decision

- Introduction

In this meeting, RAN3 has also received an LS (R3-250008) from RAN2, which includes the following progress from RAN2 on the inter-CU LTM.

- RAN2#128:

LS (R3-250008)

Agreements on inter-CU LTM:

- Rel-19 Set ID is configured for a candidate configuration. For DRB:

- Inter-CU MCG LTM: When Rel-19 Set ID of candidate cell is different from serving cell,

> UE performs PDCP re-establishment to all radio bearers.

- Inter-CU SCG LTM: When Rel-19 Set ID of candidate cell is different from serving cell,

> For MN terminated bearer without change of termination point, UE does not perform PDCP re-establishment. PDCP recovery is needed if it is split bearer.

> For SN terminated bearer without change of termination point, UE performs PDCP re-establishment.

> If there is change of termination point for a radio bearer (the keyToUse in the RadioBearerConfig is different from the keyToUse in the current UE configuration), UE performs PDCP re-establishment.

- For SRBs in inter-CU SCG LTM, Rel-19 ID is used to determine whether PDCP re-establishment or PDCP SDU discard is performed for LTM execution for SRB3.

- For SRB1/2 in inter-CU SCG LTM, PDCP re-established is not performed based on NW configuration (PDCP re-establishment flag and SDU discard).

- RAN2 confirms that inter-CU MCG LTM with SCG addition is supported assuming no much specification effort is required. If there are much specification efforts, we will not have it.

- RAN2 confirms that the inter-CU MCG LTM with intra-SN PSCell change is supported in Rel19.

- From RAN2 perspective, the following coexistence cases in NR-DC can be supported:

- Case 1: Intra-CU MCG LTM + Inter-CU MCG LTM

- Case 2: Intra-CU SCG LTM + Inter-CU SCG LTM

7.	In coexistence cases of inter-CU MCG/SCG LTM and intra-CU MCG/SCG LTM, when inter-CU MCG or SCG LTM is executed, it’s up to the NW to ensure that maintained LTM candidate configurations are valid, e.g. reconfigure or release invalid intra-CU MCG/SCG LTM candidate configurations. UE does not autonomously release invalid intra-CU candidate configurations.

8.	RAN2 to support intra-CU SCG LTM in MN RRC message (i.e. MN RRCReconfiguration message), in addition to SN RRC message.

9.	RAN2 to support intra-CU MCG LTM with SCG configuration.

10.	It’s up to NW to ensure that the complete configuration includes the MCG part and SCG part configuration when UE combines the reference and candidate configuration for inter-CU SCG LTM.

11.	RAN2 assumes that how to indicate the list of candidate PSCells from source SN to MN is up to RAN3. From RAN2 perspective, in INM, source SN may send measurement results of candidate PSCells to the MN. The MN then forwards the measurement results to the candidate SN(s), and then the candidate SN(s) determines the LTM candidate cells based on the measurement results and the upper limit for the number of PSCells that can be prepared by each candidate SN. The existing IEs defined in INM can be reused as a baseline.

- RAN2#127bis:

Agreements on inter-CU LTM

- The Rel18 handling on failure is reused in R19 if the UE selects an intra-CU LTM candidate cell after intra-CU LTM failure; for other cases, e.g. inter-CU LTM failure, the failure handling is FFS (related to SA3’s inputs).

- For non-DC case, if the new Rel-19 IDs of the serving cell and the target cell have same values, the UE compares the ltm-ServingCellNoResetID and ltm-NoResetID and performs the corresponding L2 reset operation as defined in Rel-18.

- The SCPAC-similar security update configuration is introduced for inter-CU SCG LTM, i.e. similar to IEs sk-CounterConfiguration, servingSecurityCellSetId and securityCellSetId. The names of the new IEs are to be discussed in stage3.

- Regarding the candidate and reference configuration generation and signaling design, the following SCPAC-similar principles can be applied for inter-CU SCG LTM as baseline:

- The reference configuration for inter-CU SCG LTM at least include SCG part, FFS on MCG part.

- FFS: Network ensures that when UE combines the reference and candidate configuration for inter-CU SCG LTM, the configuration generated by UE must contain both MCG and SCG part configurations.

- The candidate configuration and reference configuration are modeled as an MN RRCReconfiguration message.

- Upon inter-CU SCG LTM, the UE performs reconfiguration with sync towards SCG, but the reconfiguration with sync in MCG is not allowed.

- The MN generates the MCG part of the reference configuration (if any), while the SN (source or candidate) generates the SCG part of the reference configuration.

- The MN is responsible for the reference configuration generation for SN initiated inter-CU SCG LTM. It can be up to the NW implementation whether to include the MCG part.

- The MN can request an SCG reference configuration from any of the involved SNs.

5.	For SN initiated inter-CU SCG LTM, the candidate SN provides the SCG part configuration of each candidate PSCell, and may also provide the L1 RS (e.g. a list of SSB or a list of CSI-RS) configuration for L1 measurement, early UL sync configuration or TCI-state configuration, to the MN.

6.	The source SN is responsible to generate the common CSI resource configuration for L1 measurement on candidate SCG cells.

7.	The MN sends the received L1 RS configuration, early UL sync configuration, or TCI-state configuration of candidate cells to the source SN. And the source SN responds with the common CSI resource configuration to the MN.

8.	In order to support subsequent inter-CU SCG LTM, the MN needs to transfer, during the LTM preparation phase, the common CSI resource configuration and the collected information of candidate cells to the candidate SN(s). Accordingly, the candidate SN(s) responds with the updated candidate SCG configuration to the MN.

9.	Upon execution of inter-SN SCG LTM, the UE sends an MN RRCReconfigurationComplete message to the MN, which includes an SN RRCReconfigurationComplete message.

10.	Re-use legacy LTM Cell Switch Command MAC CE for inter-SN LTM.

11.	RAN2 confirms to support coexistence of following cases, it is up to network implementation to ensure simultaneous execution for both MCG and SCG will not happen:

- Inter-MN LTM and intra-SN LTM

- Inter-SN LTM and intra-MN LTM

- RAN2#127:

Agreements on inter-CU LTM

Reference configuration:

- Inter-CU LTM re-uses the reference configuration from Rel-18 LTM. No additional reference configurations (no multiple reference configurations) are supported.

CSI resource and report configuration:

- The Rel-18 signaling structure for LTM CSI resource and report configuration is reused for inter-CU LTM, i.e. a common CSI resource configuration and cell-specific CSI report configuration.

- The source CU is responsible to generate the common CSI resource configuration.

Early DL sync:

- For inter-CU LTM, the R18 candidate TCI State activation/deactivation design (including MAC CE and related UE handling) is reused.

Inter-CU LTM switch:

- For inter-CU LTM cell switch, it’s the source DU that triggers the MAC CE and informs the source CU about the target LTM cell.

RLC and PDCP re-establishment in inter-CU LTM:

- If the security key update is required, the UE shall perform MAC reset, RLC re-establishment and PDCP re-establishment. As baseline introduce a new Rel-19 ID in RRC: if the Rel-19 ID is different for the source cell and the target cell, the UE performs PDCP re-establishment, including security key update, however dependent on SA3 response, we can revisit it.

Handling of candidate configuration after inter-CU LTM cell switch:

- As in Rel-18 LTM, the UE keeps its LTM candidate cell configurations after at least a inter-CU LTM cell switch procedure where the UE is not configured with DC, unless these are explicitly released by the network.

DRB/PDU session mismatch in subsequent inter-CU LTM:

- RAN2 understand NW implementation can handle the concern raised in P6, R2-2407421.

LTM and L3 HO:

- L3 mobility (including both the network triggered L3 HO and CHO) can be configured to UE, while the inter-CU LTM is configured (w/o DC), and the following items can be considered (follow Rel-18 intra-CU LTM):

- When performing the L3 mobility (HO or CHO), the UE does not autonomously release inter-CU LTM configurations, unless these are explicitly released by the network.

- The RRCReconfiguration message to execute an L3 mobility (HO or CHO) procedure may reconfigure inter-CU LTM configurations.

- For the execution order between CHO and LTM, Rel-18 principle is applied.

Inter-CU SCG LTM:

- Inter-CU SCG LTM preparation can be initiated by source SN.

- The inter-CU SCG LTM configuration, SN generates SCG part configuration, MN includes it into its MN RRC configuration message.

- For inter-CU SCG LTM, the LTM cell switch command MAC CE is sent by source SN.

- RAN2 understands for the security key update of inter-CU SCG LTM, SCPAC security key update mechanism is taken as baseline. We will send LS to SA3 to ask them to take it into account for their works.

- Only SN-initiated inter-SN LTM (including LTM configuration, early DL/UL synch and LTM execution) is supported in Rel-19.

Inter-CU MCG LTM:

- SCG configuration can be changed in inter-CU MN and leave how to handle SCG part up to NW implementation (e.g. release or reconfiguration).

- Upon execution of inter-CU MN LTM with DC, the UE is required to perform refresh of security key, re-establishment of RLC and PDCP, and MAC reset at both MN and SN side (i.e. Rel-15 principle is applied).

- For the SN key update in inter-CU MN LTM with DC, the UE applies legacy R15 RRC reconfiguration with sync procedure.

This paper discusses SN initiated inter-CU SCG LTM, based on the RAN2 progress.

- Discussion

The following figure is abstracted from the endorsed running CR R2-2410928 in RAN2 #128

SN initiated inter-SN SCG LTM The SN initiated inter-SN SCG LTM procedure is used to configure inter-SN SCG LTM. Figure 10.5.2-5 SN initiated inter-SN SCG LTM

LTM Preparation

In step 1~3, the source SN initiates the inter-CU SCG LTM preparation procedure by sending a SN Change Required message to the MN. Then the MN requests each candidate SN to allocate resources for the UE by means of the SN Addition procedure. Within the list of cells suggested by the source SN, the candidate SN decides the list of PSCell(s) to prepare for SCG LTM, considering of the maximum number indicated by the MN. For each prepared PSCell, the candidate SN provides the corresponding SCG radio resource configuration, and may also provide the L1 RS (e.g. SSB or CSI-RS) configuration for L1 measurement, early UL sync configuration, or TCI-state configuration. The candidate SN sends those configurations to the MN via SN Addition Request Acknowledge message. And then the MN may send the Xn-U Address Indication message to the candidate SN(s) to provide the information for data forwarding and/or trigger the early data forwarding.

5.	For SN initiated inter-CU SCG LTM, the candidate SN provides the SCG part configuration of each candidate PSCell, and may also provide the L1 RS (e.g. a list of SSB or a list of CSI-RS) configuration for L1 measurement, early UL sync configuration or TCI-state configuration, to the MN.

Proposal 1: For SN initiated inter-CU SCG LTM, the source SN initiates the inter-CU SCG LTM preparation procedure by sending a SN Change Required message to the MN.

Proposal 2: Then the MN requests each candidate SN to allocate resources for the UE via SN Addition request message.

Proposal 3: Within the list of cells suggested by the source SN, the candidate SN provides the SCG part configuration of each candidate PSCell, and may also provide the L1 RS (e.g. a list of SSB or a list of CSI-RS) configuration for L1 measurement, early UL sync configuration or TCI-state configuration, to the MN, via SN addition request ACK message.

In step 5-6, the MN initiate the SN modification procedure towards the source SN and candidate SN(s), to transfer the LTM related configuration of candidate cells.

For inter-CU LTM in NR SA, we have agreed that the source CU to generate a common CSI resource configuration for L1 measurement on candidate cells, based on the L1 RS configuration from each candidate cell. Besides, in order to support subsequent LTM, the source CU shall also send the common CSI resource configuration and the information for candidate cells from multiple candidate CUs to each candidate CU, to allow the candidate CU updating the configurations of candidate cells, e.g. the L1 CSI report configuration. The similar principle can be reused for SN initiated inter-CU SCG LTM. Considering that only the SN initiated inter-SN LTM (including LTM configuration, early DL/UL sync and LTM execution) is supported in Rel-19, the source SN needs to know the L1 measurement configuration and early sync configuration of candidate cells.

Observation 1: For SN initiated inter-SN LTM, the LTM configuration, early DL/UL sync and LTM execution are triggered by the source SN, so the source SN needs to know the L1 measurement configuration and early sync configuration of candidate cells.

Thus, it’s natural to let the source SN generate the common CSI resource configuration for candidate cells. The MN can transfer the received L1 RS (e.g. SSB or CSI-RS) configuration, early UL sync configuration, or TCI-state configuration to the source SN via MN initiated SN modification procedure, as the step 4 in Figure 1. Based on the L1 RS configuration for each candidate cell, the source SN can generate the common CSI resource configuration, as well as the L1 CSI report configuration provided within the source SCG configuration. Then the source SN transfers the generated common CSI resource configuration and the updated source SCG configuration (e.g. including the L1 CSI report configuration) to the MN via SN modification request acknowledge message, i.e. as the step 5 above.

6.	The source SN is responsible to generate the common CSI resource configuration for L1 measurement on candidate SCG cells.

7.	The MN sends the received L1 RS configuration, early UL sync configuration, or TCI-state configuration of candidate cells to the source SN. And the source SN responds with the common CSI resource configuration to the MN.

Proposal 4: The MN sends the received L1 RS configuration, early UL sync configuration, or TCI-state configuration of candidate cells to the source SN, e.g. via SN modification request message, and the source SN responds with the common CSI resource configuration to the MN via SN modification request ACK message.

In step 7-8, in order to support subsequent inter-CU SCG LTM, the MN needs to transfer the common CSI resource configuration and the collected information of candidate cells to the candidate SN(s), e.g. via MN initiated SN modification procedure. And then the candidate SN can update the candidate SCG configuration (e.g. generating the L1 CSI report configuration), accordingly.

8.	In order to support subsequent inter-CU SCG LTM, the MN needs to transfer ,during the LTM preparation phase, the common CSI resource configuration and the collected information of candidate cells to the candidate SN(s). Accordingly, the candidate SN(s) responds with the updated candidate SCG configuration to the MN.

Proposal 5: In order to support subsequent inter-CU SCG LTM, the MN needs to transfer the common CSI resource configuration and the collected information of candidate cells to the candidate SN(s), via SN modification request message. Accordingly, the candidate SN(s) responds with the updated candidate SCG configuration to the MN via SN modification request ACK message.

In step 9/10, the MN sends the RRCReconfiguration message to the UE including the SCG LTM configuration. And the UE responses an RRCReconfigurationComplete message to the MN. The MN RRCReconfigurationComplete message includes an SN RRCReconfigurationComplete message to the source SN.

In step 9, the MN informs the source SN with the SN RRCReconfigurationComplete message via SN Change Confirm message, to indicate that SCG LTM is prepared.

Early sync

In step 13/14a, the UE performs DL/UL sync with the LTM candidate cell(s) before receiving the cell switch command. For the PDCCH order triggered early RACH. In step 14b, the candidate SN(s) needs to transfer the calculated TA value of candidate cell(s) to the source SN via the MN. The existing TA information transfer procedure via F1 interface can be extended to Xn interface to transfer the TA information between the MN and the source/candidate SN.

RAN3 has agreed that: Introduce a new procedure on Xn to transfer the TA information.

Observation 2: RAN3 has agreed to extend the existing TA information transfer procedure via F1 interface to Xn interface to transfer the TA information from the candidate SN to the source SN via the MN.

LTM cell switch execution

In step 15, the UE sends the L1 measurement report(s) to the source SN (i.e. source SN DU). In step 16/17, the source SN decides to execute LTM cell switch to the target cell by sending an LTM cell switch command MAC CE to the UE.

In step 18/19, the source SN sends a notification message to the target SN via the MN, to indicate the triggering of LTM to the UE. The message may include the target cell ID, the selected TCI-state ID(s) of the target cell. We can simply extend the existing cell switch notification procedure via F1 interface to Xn interface.

RAN3 has agreed that Cell Switch Notification from source DU to target DU (in different gNB from source) for LTM execution

Observation 3: RAN3 has agreed to extend the existing cell switch notification procedure via F1 interface to Xn interface to inform the selected target cell from the source SN to the target SN via the MN.

LTM cell switch completion

In step 20, the UE performs the random access procedure towards the target cell, if UE does not have valid TA of the target cell. If the UE has valid TA of the target cell, the UE skips random access procedure towards the target cell, i.e. RACH-less LTM. The order the UE sends the MN RRCReconfigurationComplete message and performs the Random Access procedure towards the target SCG is not defined. If the UE has performed a RA procedure in step 20, the UE considers that LTM cell switch execution is successfully completed when the random access procedure towards the target SCG is successfully completed. For RACH-less LTM, the UE considers that LTM cell switch execution is successfully completed when the UE determines that the target SN has successfully received its first UL data.

In step 21, the UE sends an MN RRCReconfigurationComplete message to the MN, which includes an SN RRCReconfigurationComplete message. In step 22, and then the MN informs the target SN that the UE has completed the reconfiguration procedure successfully via SN Reconfiguration Complete message, including the SN RRCReconfigurationComplete message.

In step 27, the data forwarding procedure is performed.

In step 28, if applicable, the update of the UP path towards the core network is performed via a PDU session path update procedure.

RAN3 has agreed that Turn the WA into agreement: Reuse the existing XnAP UE CONTEXT RELEASE message at the source gNB if no LTM candidate cell(s) exists in the source gNB. So, in step 29, the MN initiates XnAP UE CONTEXT RELEASE message at the source gNB if no LTM candidate cell(s) exists in the source gNB, to inform the execution of SCG LTM. The detailed procedure is similar to SN initiated inter-SN SCPAC.

Based on the description of each step above, we proposed to take flow charts and procedures for SN initiated inter-CU SCG LTM in Figure 1 as baseline.

Observation 4: 	RAN3 has agreed to extend the existing TA information transfer procedure via F1 interface to Xn interface to transfer the TA information from the candidate SN to the source SN via the MN.

Observation 5: 	RAN3 has agreed to extend the existing cell switch notification procedure via F1 interface to Xn interface to inform the selected target cell from the source SN to the target SN via the MN.

Proposal 6: The flow charts and procedures in Figure 1 are taken as baseline for SN initiated inter-CU SCG LTM.

- Conclusion

After the above analysis, we provide the following observations and proposals.

Observation 1: For SN initiated inter-SN LTM, the LTM configuration, early DL/UL sync and LTM execution are triggered by the source SN, so the source SN needs to know the L1 measurement configuration and early sync configuration of candidate cells.

Observation 2: RAN3 has agreed to extend the existing TA information transfer procedure via F1 interface to Xn interface to transfer the TA information from the candidate SN to the source SN via the MN.

Observation 3: RAN3 has agreed to extend the existing cell switch notification procedure via F1 interface to Xn interface to inform the selected target cell from the source SN to the target SN via the MN.

Observation 4: 	RAN3 has agreed to extend the existing TA information transfer procedure via F1 interface to Xn interface to transfer the TA information from the candidate SN to the source SN via the MN.

Observation 5: 	RAN3 has agreed to extend the existing cell switch notification procedure via F1 interface to Xn interface to inform the selected target cell from the source SN to the target SN via the MN.

Proposal 1: For SN initiated inter-CU SCG LTM, the source SN initiates the inter-CU SCG LTM preparation procedure by sending a SN Change Required message to the MN.

Proposal 2: Then the MN requests each candidate SN to allocate resources for the UE via SN Addition request message.

Proposal 3: Within the list of cells suggested by the source SN, the candidate SN provides the SCG part configuration of each candidate PSCell, and may also provide the L1 RS (e.g. a list of SSB or a list of CSI-RS) configuration for L1 measurement, early UL sync configuration or TCI-state configuration, to the MN, via SN addition request ACK message.

Proposal 4: The MN sends the received L1 RS configuration, early UL sync configuration, or TCI-state configuration of candidate cells to the source SN, e.g. via SN modification request message, and the source SN responds with the common CSI resource configuration to the MN via SN modification request ACK message.

Proposal 5: In order to support subsequent inter-CU SCG LTM, the MN needs to transfer the common CSI resource configuration and the collected information of candidate cells to the candidate SN(s), via SN modification request message. Accordingly, the candidate SN(s) responds with the updated candidate SCG configuration to the MN via SN modification request ACK message.

Proposal 6: The flow charts and procedures in Figure 1 are taken as baseline for SN initiated inter-CU SCG LTM.

- Text Proposal to TS37.340

SN initiated inter-SN SCG LTM

The SN initiated inter-SN SCG LTM procedure is used to configure inter-SN SCG LTM.

Figure 10.5.2-5 SN initiated inter-SN SCG LTM

Figure 10.5.2-5 shows an example signalling flow for the inter-SN SCG LTM initiated by the SN:

Editor’s Note: RAN2 assumes that the figure should be anchored together with RAN3. The final figure is up to the progresses made in RAN3.

1.	The source SN initiates the inter-SN SCG LTM procedure by sending the SN Change Required to the MN providing a list of candidate PSCell(s) for inter-SN SCG LTM preparation. The message may include an SCG reference configuration. Source SN may additionally send measurement results of candidate PSCells to the MN.

2.	The MN requests each candidate SN to allocate resources for the UE by means of the SN Addition procedure. The MN also provides a list of KSN and associated sk-Counter values for each candidate SN, and forwards the received measurement results to each candidate SN(s). The MN may select one of the candidate SN(s) and request providing the reference SCG configuration as part of the SN Addition procedure. Once obtained, the MN provides the reference configuration to other candidate SN(s).

3.	The candidate SN(s) determines the LTM candidate cells based on the measurement results (if provided) and the upper limit for the number of PSCells that can be prepared by each candidate SN. The candidate SN provides the SCG part configuration of each candidate PSCell, and may also provide the L1 RS (e.g. a list of SSB or a list of CSI-RS) configuration for L1 measurement, early UL/DL sync configuration, the reference SCG configuration if request, by sending SN Addition Request Acknowledge message to the MN.

4.	For SN terminated bearers using MCG resources, the MN may provide Xn-U DL TNL address information in the Xn-U Address Indication message to the candidate SN(s).

5.	The MN sends the received L1 RS configuration, early UL/DL sync configuration of candidate cells to the source SN via SN Modification Request message.

6.	The source SN generates the common CSI resource configuration for L1 measurement on candidate PSCells. The source SN sends the generated common CSI resource configuration and the updated source SCG configuration to the MN via SN modification request acknowledge message.

7.	The MN transfers, during the LTM preparation phase, the common CSI resource configuration and the collected information of candidate cells to each candidate SN via SN Modification Request message.

8.	The candidate SN(s) responds with the updated candidate SCG configuration to the MN via SN modification request acknowledge message.

9.	The MN generates the reference configuration for SN initiated inter-CU SCG LTM, which may include the MCG part of the reference configuration generated by the MN (if any), together with the SCG part of the reference configuration generated by the source SN or candidate SN. The MN sends an RRCReconfiguration message to the UE including the inter-SN SCG LTM configuration, i.e. a list of RRCReconfiguration* messages, in which each RRCReconfiguration* message contains the SCG configuration in the RRCReconfiguration** message received from the candidate SN in step 3 and possibly an MCG configuration. Besides, the RRCReconfiguration message can also include an updated MCG configuration, as well as the NR RRCReconfiguration*** message generated by the source SN, e.g., to configure the required measurements.

10.	The UE applies the RRCReconfiguration message received in step 9, stores the inter-SN SCG LTM configuration and replies to the MN with an RRCReconfigurationComplete message, including an SN RRCReconfigurationComplete*** message to the source SN.

11.	The MN informs the source SN with the SN RRCReconfigurationComplete*** message via SN Change Confirm message, to indicate that SCG LTM is prepared.

12.	RAN3 will discuss whether Early Data Fowarding is supported.

13.	The UE may perform DL synchronization with LTM candidate cell(s) before receiving the cell switch command, as specified in clause 9.2.3.5.2 in TS 38.300 [3].

14a.The UE may perform UL synchronization with LTM candidate cell(s) before receiving the cell switch command, as specified in clause 9.2.3.5.2 in TS 38.300 [3].

14b. The candidate SN(s) transfers the calculated TA value of candidate cell(s) to the source SN via the MN, by Xn: TA Information Transfer message.

15.	The UE performs L1 measurements on the configured LTM candidate cell(s) and transmits L1 measurement reports to the source SN, if the L1 measurement configuration in RRCReconfiguration is received in step 9.

16.	The source SN decides to execute cell switch to a target PSCell.

17.	The source SN transmits an LTM cell switch command MAC CE triggering cell switch by including a target configuration ID and other related information for the target PSCell, as specified in clause 9.2.3.5.2 in TS 38.300 [3]. The UE applies the candidate configuration indicated by the target configuration ID including PSCell change.

18/19.	The source SN sends a Cell Switch Notification message to the target SN via the MN, to indicate the triggering of LTM to the UE.

20.	The UE performs the random access procedure towards the target PSCell, if the UE does not have valid TA of the target PSCell.

21. The UE sends an MN RRCReconfigurationComplete* message to the MN, which includes an SN RRCReconfigurationComplete** message to the target SN.

22.	The MN informs the target SN that the UE has completed the reconfiguration procedure successfully via SN Reconfiguration Complete message, including the SN RRCReconfigurationComplete** message.

23~29. FFS. Editor’s Note: Details for related procedures and descriptions are up to RAN3 discussion.

NOTE X:	The steps 12-29 can be performed multiple times for subsequent SCG LTM cell switch execution using the SCG LTM candidate configuration(s) provided in step 9.

Editor’s Note: In the above descriptions and signalling flow for the overall procedure of SN-initiated inter-SN SCG LTM, the detail procedures and messages between MN, source or candidate SN(s) are up to RAN3 discussion.

- Text Proposal to TS38.423

----------------------------------------< Start of the change>-------------------------------------

9.1.2.1	S-NODE ADDITION REQUEST

This message is sent by the M-NG-RAN node to the S-NG-RAN node to request the preparation of resources for dual connectivity operation for a specific UE.

Direction: M-NG-RAN node  S-NG-RAN node.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.2.3.1 |  | YES | reject |
| M-NG-RAN node UE XnAP ID | M |  | NG-RAN node UE XnAP ID 9.2.3.16 | Allocated at the M-NG-RAN node | YES | reject |
| -------------<Skip unchanged part>----------------------- |  |  |  |  |  |  |
| CHO Information SN Addition | O |  |  |  | YES | reject |
| >Source M-NG-RAN node ID | M |  | Global NG-RAN Node ID 9.2.2.3 |  | – |  |
| >Source M-NG-RAN node UE XnAP ID | M |  | NG-RAN node UE XnAP ID 9.2.3.16 | Allocated at the source M-NG-RAN node | – |  |
| >Estimated Arrival Probability | O |  | INTEGER (1..100) |  | – |  |
| SCG Activation Request | O |  | 9.2.3.154 |  | YES | ignore |
| Conditional PSCell Addition Information Request | O |  |  |  | YES | reject |
| >Maximum Number of PSCells To Prepare | M |  | INTEGER (1..8, ...) | Indicates the maximum number of PSCells that the target SN may prepare. | – |  |
| >Estimated Arrival Probability | O |  | INTEGER (1..100) | Indicates the arrival probability for the UE towards the candidate target SN. | – |  |
| >S-CPAC Request Information | O |  | 9.2.3.192 | Indicates that SN addition is for S-CPAC preparation. | YES | reject |
| >S-CPAC Reference Configuration Request | O |  | ENUMERATED (request, …) | Indicates that the reference configuration for S-CPAC is requested. | YES | ignore |
| S-NG-RAN node UE Slice Maximum Bit Rate | O |  | UE Slice Maximum Bit Rate List 9.2.3.167 | This IE indicates the S-NG-RAN node portion of the UE Slice Aggregate Maximum Bit Rate as specified in TS 23.501 [7] | YES | reject |
| F1-terminating IAB-donor Indicator | O |  | ENUMERATED (true, ...) | This IE applies only if the UE is an IAB-MT. | YES | reject |
| Selected NID | O |  | NID 9.2.2.65 | This IE, together with the Selected PLMN IE, indicates the SNPN proposed for the SCG to the S-NG-RAN node. | YES | ignore |
| QMC Coordination Request | O |  | 9.2.3.197 | This IE contains information for managing configuration and reporting of one or more QoE and/or RAN visible QoE measurements at the S-NG-RAN node subject to addition. | YES | ignore |
| Source SN to Target SN QMC Information | O |  | QMC Configuration Information 9.2.3.156 | This IE contains S-NG-RAN node-related QMC Configuration Information to be forwarded to the target S-NG-RAN node. | YES | ignore |
| Source M-NG-RAN node ID | O |  | Global NG-RAN Node ID 9.2.2.3 | The NG-RAN Node ID of the source M-NG-RAN node in e.g. NR-DC to NR-DC handover. | YES | ignore |
| IAB Authorization Status | O |  | ENUMERATED (authorized, not authorized, …) | Indicates the IAB node´s authorization status. | YES | ignore |
| SCG LTM Information Request | O |  | 9.2.1.yy1 |  | YES | reject |
| SCG Early Sync Information Request |  |  | 9.2.1.xx3 | Note: Seen in BLCR to 38.423 | YES | ignore |

| Range bound | Explanation |
| --- | --- |
| maxnoofPDUSessions | Maximum no. of PDU sessions. Value is 256 |

| Condition | Explanation |
| --- | --- |
| ifSNterminated | This IE shall be present if there is at least one PDU Session Resource Setup Info – SN terminated in the PDU Session Resources To Be Added List IE. |

9.1.2.2	S-NODE ADDITION REQUEST ACKNOWLEDGE

This message is sent by the S-NG-RAN node to confirm the M-NG-RAN node about the S-NG-RAN node addition preparation.

Direction: S-NG-RAN node  M-NG-RAN node.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.2.3.1 |  | YES | reject |
| M-NG-RAN node UE XnAP ID | M |  | NG-RAN node UE XnAP ID 9.2.3.16 | Allocated at the M-NG-RAN node | YES | reject |
| S-NG-RAN node UE XnAP ID | M |  | NG-RAN node UE XnAP ID 9.2.3.16 | Allocated at the S-NG-RAN node | YES | reject |
| -------------<Skip unchanged part>----------------------- |  |  |  |  |  |  |
| Conditional PSCell Addition Information Acknowledge | O |  |  |  | YES | ignore |
| >Candidate PSCell List |  | 1 |  | Ignored, if the Candidate PSCell with Other Information List IE is included. | – |  |
| >>Candidate PSCell Item |  | 1 .. <maxnoofPSCellCandidate> |  |  | – |  |
| >>>PSCell ID | M |  | NR CGI 9.2.2.7 |  | – |  |
| >Candidate PSCell with Other Information List |  | 0..1 |  |  | YES | reject |
| >>Candidate PSCell with Other Information Item |  | 1 .. <maxnoofPSCellCandidate> |  |  | – |  |
| >>>PSCell ID | M |  | NR CGI 9.2.2.7 |  | – |  |
| >>>S-CPAC Complete Candidate Configuration Indicator | O |  | Complete Candidate Configuration Indicator 9.2.3.194 |  | – |  |
| SN Mobility Information | O |  | BIT STRING (SIZE (32)) | Information related to PSCell change; T-SN provides it in order to enable later analysis of the conditions that led to wrong PSCell change. | YES | ignore |
| QMC Coordination Response | O |  | 9.2.3.198 | This IE contains the response of the S-NG-RAN node to the QMC coordination request. | YES | ignore |
| CHO Information SN Addition Acknowledge | O |  |  |  | YES | reject |
| >PCell ID | O |  | Global NG-RAN Cell Identity 9.2.2.27 | PCell indicated in the corresponding S-NODE ADDITION REQUEST message. | – |  |
| Direct Forwarding Path Availability with source M-NG-RAN node | O |  | ENUMERATED (direct path available, …) | Indicates direct forwarding path is available between the target S-NG-RAN node and source M-NG-RAN node. | YES | ignore |
| SCG LTM Information Response | O |  | 9.2.1.yy2 |  | YES | reject |
| SCG Early Sync Information Response |  |  | 9.2.1.xx4 | Note: Seen in BLCR to 38.423 | YES | ignore |

| Range bound | Explanation |
| --- | --- |
| maxnoofPDUSessions | Maximum no. of PDU sessions. Value is 256 |
| maxnoofPSCellCandidate | Maximum no, of PSCell candidate. Value is 8 |

----------------------------------------< Next change>-------------------------------------

9.1.2.5	S-NODE MODIFICATION REQUEST

This message is sent by the M-NG-RAN node to the S-NG-RAN node to either request the preparation to modify S-NG-RAN node resources for a specific UE, or to query for the current SCG configuration, or to provide the S-RLF-related information to the S-NG-RAN node.

Direction: M-NG-RAN node  S-NG-RAN node.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.2.3.1 |  | YES | reject |
| M-NG-RAN node UE XnAP ID | M |  | NG-RAN node UE XnAP ID 9.2.3.16 | Allocated at the M-NG-RAN node | YES | reject |
| S-NG-RAN node UE XnAP ID | M |  | NG-RAN node UE XnAP ID 9.2.3.16 | Allocated at the S-NG-RAN node | YES | reject |
| -------------<Skip unchanged part>----------------------- |  |  |  |  |  |  |
| CHO Information SN Modification | O |  |  |  | YES | ignore |
| >Conditional Reconfiguration | M |  | ENUMERATED (intra-MN-CHO, ...) |  | – |  |
| >Estimated Arrival Probability | O |  | INTEGER (1..100) |  | – |  |
| SCG Activation Request | O |  | 9.2.3.154 |  | YES | ignore |
| Conditional PSCell Addition Information Modification Request | O |  |  | This IE may be sent to the candidate SN. | YES | ignore |
| >Maximum Number of PSCells To Prepare | O |  | INTEGER (1..8, ...) | Indicates the maximum number of PSCells that the candidate SN may prepare. | – |  |
| >Estimated Arrival Probability | O |  | INTEGER (1..100) | Indicates the arrival probability for the UE towards the candidate SN. |  |  |
| >S-CPAC Request Information | O |  | 9.2.3.192 | Indicates that SN modification is for S-CPAC preparation or modification. | YES | reject |
| >S-CPAC Reference Configuration Request | O |  | ENUMERATED (request, …) | Indicates that the reference configuration for S-CPAC is requested. | YES | ignore |
| >S-CPAC Inter-SN Execution Notification | O |  | ENUMERATED (executed, …) | Indicates that inter-SN S-CPAC was executed. | YES | reject |
| Conditional PSCell Change Information Update | O |  |  | This IE may be sent to the source SN in SN-initiated inter-SN CPC, or sent to a candidate SN in S-CPAC. | YES | ignore |
| >Multiple Target S-NG-RAN Node List |  | 1 |  |  | – |  |
| >>Multiple Target S-NG-RAN Node Item |  | 1 .. <maxnoofTargetSNs> |  |  | – |  |
| >>>Target S-NG-RAN node ID | M |  | Global NG-RAN Node ID 9.2.2.3 |  | – |  |
| >>>Candidate PSCell List |  | 1 |  |  | – |  |
| >>>>Candidate PSCell Item |  | 1 .. <maxnoofPSCellCandidate> |  |  | – |  |
| >>>>>PSCell ID | M |  | NR CGI 9.2.2.7 |  | – |  |
| S-NG-RAN node UE Slice Maximum Bit Rate | O |  | UE Slice Maximum Bit Rate List 9.2.3.167 | This IE indicates the S-NG-RAN node portion of the UE Slice Aggregate Maximum Bit Rate as specified in TS 23.501 [7] | YES | ignore |
| Management Based MDT PLMN Modification List | O |  | MDT PLMN Modification List 9.2.3.169 |  | YES | ignore |
| Selected NID | O |  | NID 9.2.2.65 | This IE, together with the Selected PLMN IE, indicates the SNPN proposed for the SCG to the S-NG-RAN node. | YES | ignore |
| QMC Coordination Request | O |  | 9.2.3.197 | This IE contains information for managing configuration and reporting of one or more QoE and/or RAN visible QoE measurements at the S-NG-RAN node subject to modification. | YES | ignore |
| Source SN to Target SN QMC Information Inquiry | O |  | ENUMERATED (true, ...) | This IE contains a request for S-NG-RAN node-related QMC Configuration Information. The information is to be forwarded to the target S-NG-RAN node. | YES | ignore |
| IAB Authorization Status | O |  | ENUMERATED (authorized, not authorized, …) | Indicates the IAB node´s authorization status. | YES | ignore |
| SCG LTM configuration update Request |  |  | 9.2.1.yy3 |  |  |  |

| Range bound | Explanation |
| --- | --- |
| maxnoofPDUSessions | Maximum no. of PDU sessions. Value is 256 |
| maxnoofPSCellCandidate | Maximum no. of PSCell candidates. Value is 8 |
| maxnoofTargetSNs | Maximum no. of the target S-NG-RAN nodes. Value is 8 |

9.1.2.6	S-NODE MODIFICATION REQUEST ACKNOWLEDGE

This message is sent by the S-NG-RAN node to confirm the M-NG-RAN node’s request to modify the S-NG-RAN node resources for a specific UE.

Direction: S-NG-RAN node  M-NG-RAN node.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.2.3.1 |  | YES | reject |
| M-NG-RAN node UE XnAP ID | M |  | NG-RAN node UE XnAP ID 9.2.3.16 | Allocated at the M-NG-RAN node | YES | ignore |
| S-NG-RAN node UE XnAP ID | M |  | NG-RAN node UE XnAP ID 9.2.3.16 | Allocated at the S-NG-RAN node | YES | ignore |
| -------------<Skip unchanged part>----------------------- |  |  |  |  |  |  |
| Conditional PSCell Addition Information Modification Acknowledge | O |  |  | This IE may be sent from the target SN. | YES | ignore |
| >Candidate PSCell List |  | 1 |  | Ignored, if the Candidate PSCell with Other Information List IE is included. | – |  |
| >>Candidate PSCell Item |  | 1 .. <maxnoofPSCellCandidate> |  |  | – |  |
| >>>PSCell ID | M |  | NR CGI 9.2.2.7 |  | – |  |
| >Candidate PSCell with Other Information List |  | 0..1 |  |  | YES | reject |
| >>Candidate PSCell with Other Information Item |  | 1 .. <maxnoofPSCellCandidate> |  |  | – |  |
| >>>PSCell ID | M |  | NR CGI 9.2.2.7 |  | – |  |
| >>>S-CPAC Complete Candidate Configuration Indicator | O |  | Complete Candidate Configuration Indicator 9.2.3.194 |  | – |  |
| QMC Coordination Response | O |  | 9.2.3.198 | This IE contains the response of the S-NG-RAN node to the QMC coordination request. | YES | ignore |
| Source SN to Target SN QMC Information | O |  | QMC Configuration Information 9.2.3.156 | This IE contains S-NG-RAN node-related QMC Configuration Information to be forwarded to the target S-NG-RAN node. | YES | ignore |
| SCG LTM configuration update Response |  |  | 9.2.1.yy4 |  |  |  |

| Range bound | Explanation |
| --- | --- |
| maxnoofPDUSessions | Maximum no. of PDU sessions. Value is 256 |
| maxnoofPSCellCandidate | Maximum no. of PSCell candidates. Value is 8 |

----------------------------------------< Next change>-------------------------------------

9.1.2.11	S-NODE CHANGE REQUIRED

This message is sent by the S-NG-RAN node to the M-NG-RAN node to trigger the change of the S-NG-RAN node.

Direction: S-NG-RAN node  M-NG-RAN node.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.2.3.1 |  | YES | reject |
| M-NG-RAN node UE XnAP ID | M |  | NG-RAN node UE XnAP ID 9.2.3.16 | Allocated at the M-NG-RAN node | YES | reject |
| S-NG-RAN node UE XnAP ID | M |  | NG-RAN node UE XnAP ID 9.2.3.16 | Allocated at the S-NG-RAN node | YES | reject |
| Target S-NG-RAN node ID | M |  | Global NG-RAN Node ID 9.2.2.3 | This IE shall be ignored if the Conditional PSCell Change Information Required IE is present. | YES | reject |
| Cause | M |  | 9.2.3.2 |  | YES | ignore |
| PDU Session SN Change Required List |  | 0..1 |  |  | YES | ignore |
| >PDU Session SN Change Required Item |  | 1 .. <maxnoofPDUsessions> |  | NOTE: If the PDU Session Resource Change Required Info – SN terminated IE is not present in a PDU Session SN Change Required Item IE, abnormal conditions as specified in clause 8.3.5.4 apply. | – |  |
| >>PDU Session ID | M |  | 9.2.3.18 |  | – |  |
| >>PDU Session Resource Change Required Info – SN terminated | O |  | 9.2.1.18 |  | – |  |
| S-NG-RAN node to M-NG-RAN node Container | M |  | OCTET STRING | Includes the CG-Config message as defined in subclause 11.2.2 of TS 38.331 [10]. This IE shall be ignored if the Conditional PSCell Change Information Required IE is present. | YES | reject |
| SCG UE History Information | O |  | 9.2.3.151 |  | YES | ignore |
| SN Mobility Information | O |  | BIT STRING (SIZE (32)) | Information related to PSCell change; S-NG-RAN node provides it in order to enable later analysis of the conditions that led to wrong PSCell change. | YES | ignore |
| Source PSCell ID | O |  | Global NG-RAN Cell Identity 9.2.2.27 |  | YES | ignore |
| Conditional PSCell Change Information Required | O |  |  |  | YES | ignore |
| >Multiple Target S-NG-RAN Node List |  | 1 |  |  | – |  |
| >>Multiple Target S-NG-RAN Node Item |  | 1 .. <maxnoofTargetSNs> |  |  | – |  |
| >>>Target S-NG-RAN node ID | M |  | Global NG-RAN Node ID 9.2.2.3 |  | – |  |
| >>>CPC Indicator | M |  | ENUMERATED (CPC-initiation, CPC-modification, CPC-cancellation, ...) |  | – |  |
| >>>Maximum Number of PSCells To Prepare | M |  | INTEGER (1..8, …) | Indicates the maximum number of PSCells that the target SN may prepare. | – |  |
| >>>Estimated Arrival Probability | O |  | INTEGER (1..100) | Indicates the arrival probability for the UE towards the candidate target SN. | – |  |
| >>>S-NG-RAN node to M-NG-RAN node Container | M |  | OCTET STRING | Includes the CG-Config message as defined in subclause 11.2.2 of TS 38.331 [10]. | – |  |
| >S-CPAC Request | O |  | ENUMERATED (initiation, …) | Indicates that SN change is for S-CPAC preparation. | YES | reject |
| Source SN to Target SN QMC Information | O |  | QMC Configuration Information 9.2.3.156 | This IE contains S-NG-RAN node-related QMC Configuration Information to be forwarded to the target S-NG-RAN node. | YES | ignore |
| SCG LTM Information Required | O |  | 9.2.1.yy5 |  | YES | reject |
| SCG Early Sync Information Required |  |  | 9.2.1.xx3 | Note: Seen in BLCR to 38.423 | YES | ignore |

| Range bound | Explanation |
| --- | --- |
| maxnoofPDUsessions | Maximum no. of PDU sessions. Value is 256 |
| maxnoofTargetSNs | Maximum no. of the target S-NG-RAN nodes. Value is 8 |

----------------------------------------< Next change>-------------------------------------

9.2.1.yy1	SCG LTM Information Request

| IE/Group Name | Presence | Range | IE Type and Reference | Semantics Description |
| --- | --- | --- | --- | --- |
| Candidate PSCell List To Prepare |  | 1 |  |  |
| >Candidate PSCell Item |  | 1 .. <maxnoofPSCellCandidate> |  |  |
| >>PSCell ID | M |  | NR CGI 9.2.2.7 |  |
| SCG Reference Configuration | O |  | OCTET STRING |  |
| SCG Reference Configuration Request | O |  | ENUMERATED (request, …) | Indicates that the reference configuration for SCG LTM is requested. |
| RRC Container[FFS] | O |  | OCTET STRING | Contains measurement results of candidate PSCells, pending to RAN2 |
| SCG LTM Multiple Target S-NG-RAN Node List |  | 0 .. <maxnoofTargetSNsMinusOne> |  |  |
| >Target S-NG-RAN node ID | M |  | Global NG-RAN Node ID 9.2.2.3 | Other candidate SN under preparation for SCG LTM. |
| >Recommended Candidate PSCells | M |  | OCTET STRING | Includes the candidateCellList in case of SN-initiated inter-SN SCG LTM, contained in the CG-ConfigInfo message as defined in TS 38.331 [10],[FFS] |
| SCG LTM Security Configurations List |  |  | 9.2.3.ZZ1 | Indicates the security configurations for SCG LTM, which will replace the previous configuration if any. |

| Range bound | Explanation |
| --- | --- |
| maxnoofPSCellCandidate | Maximum no, of PSCell candidate. Value is 8 |
| maxnoofTargetSNsMinusOne | Maximum no. of the target S-NG-RAN nodes minus 1. Value is 7 |

----------------------------------------< Next change>-------------------------------------

9.2.1.yy2	SCG LTM Information Response

| IE/Group Name | Presence | Range | IE Type and Reference | Semantics Description |
| --- | --- | --- | --- | --- |
| SCG Reference Configuration | O |  | OCTET STRING |  |
| FFS |  |  |  |  |

9.2.1.yy3	SCG LTM configuration update Request

| IE/Group Name | Presence | Range | IE Type and Reference | Semantics Description |
| --- | --- | --- | --- | --- |
| CSI-RS Resource Configuration | O |  | OCTET STRING |  |
| FFS |  |  |  |  |

9.2.1.yy4	SCG LTM configuration update Response

| IE/Group Name | Presence | Range | IE Type and Reference | Semantics Description |
| --- | --- | --- | --- | --- |
| CSI-RS Report Configuration | O |  | OCTET STRING (FFS) |  |
| FFS |  |  |  |  |

----------------------------------------< Next change>-------------------------------------

9.2.1.yy5 SCG LTM Information Required

Editor’s note: Details on configurations need to be continued.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| Multiple Target S-NG-RAN Node List |  | 1 |  |  |
| >Multiple Target S-NG-RAN Node Item |  | 1 .. <maxnoofTargetSNs> |  |  |
| >>Target S-NG-RAN node ID | M |  | Global NG-RAN Node ID 9.2.2.3 |  |
| >>Candidate PSCell List |  | 1 |  |  |
| >>>Candidate PSCell Item |  | 1 .. <maxnoofPSCellCandidate> |  |  |
| >>>>PSCell ID | M |  | NR CGI 9.2.2.7 |  |
| >>SCG Reference Configuration | O |  | OCTET STRING |  |
| >>RRC Container[FFS] | O |  | OCTET STRING | Contains measurement results of candidate PSCells, pending to RAN2 |

| Range bound | Explanation |
| --- | --- |
| maxnoofTargetSNs | Maximum no. of the target S-NG-RAN nodes. Value is 8 |
| maxnoofPSCellCandidate | Maximum no, of PSCell candidate. Value is 8 |

9.2.3. ZZ1	SCG LTM Security Configurations List

| IE/Group Name | Presence | Range | IE Type and Reference | Semantics Description |
| --- | --- | --- | --- | --- |
| SCG LTM Security Configurations List |  | 1 |  |  |
| >S-SCG LTM Security Configurations Item |  | 1 .. <maxnoofSecurityConfigurations> |  |  |
| >>S-NG-RAN node Security Key | M |  | BIT STRING (SIZE(256)) | The S-KSN which is provided by the M-NG-RAN node, see TS 33.501 [28]. |
| >>SK-counter | M |  | INTEGER (0..65535) |  |

| Range bound | Explanation |
| --- | --- |
| maxnoofSecurityConfigurations | Maximum no. of SCG LTM security configurations. Value is 8. |

----------------------------------------< End of the change>-------------------------------------