3GPP TSG RAN WG3 Meeting #127	R3-250121

Athens, Greece, 17 – 21 Feb, 2025

Title: 	Discussion on Location Report

Source: 	ZTE Corporation

Agenda item:	16.3

Document for:	Discussion and Decision

- Introduction

In the RAN #106 plenary meeting, the WID (New Work Item: Solutions for Ambient IoT (Internet of Things) in NR) has approved in RP-243326.

The RAN3’s scope is as below.

- RAN3 scope:

  - Specify necessary architectural aspects, and signaling and procedures between A-IoT RAN and A-IoT CN to support the A-IoT functions, assuming an architecture of aggregated gNB, including:

    - Inventory and command operations

    - Device location reporting at reader ID granularity

Note: The above A-IoT functions are supported over the existing NG interface, based on architecture(s) defined by RAN3/SA2.

This paper addresses to discuss the location report.

- Discussion

  - General

TR38.769 6.9.2 Topology 1 For support of A-IoT device location, the A-IoT RAN node may report the A-IoT RAN node ID to the A-IoT CN in the Inventory Report or Command Response messages. The A-IoT RAN node may additionally report a Reader ID to the A-IoT CN in the Inventory Report and/or the Command Response messages, which corresponds to the reader to which the A-IoT device responded. NOTE 1: Whether multiple Reader ID(s) can be included by the A-IoT RAN node in the Inventory Report/Command Response message needs further discussion. NOTE 2: Definition and format of the A-IoT RAN node ID and Reader ID needs further discussion. NOTE 3: Whether and how to have awareness of Reader location in A-IoT CN needs further discussion.

In this R19 WI, only T1 is supported, the reader type is one kind of network node, e.g., AIoT RAN, BS. TRP.

For the network node as reader, it is deployed by operator, its position is unchanged. Otherwise, the network deployment and network optimisation cannot be operated, e.g., SON.

Observation 1: The position of T1 reader is unchanged, it can be exchanged between RAN and CN by NGAP signalling or OAM.

  - NGAP signalling vs OAM

The network node as T1 reader is deployed and configured by operator, the position of T1 reader is never changed.

As legacy, the configuration of network node is static and normally handled by OAM. On the other hand, NGAP signalling is normally used for dynamic or semi-static configuration.

Proposal 1: The position of T1 reader is exchanged by OAM.

  - Device location reporting method

After an Inventory procedure is successfully handled, the AIoT RAN node will send the Inventory report message to the AIoTF, which include the latest association between reader and device(s). Then the AIoTF can establish, update and store the latest association between reader and device(s).

Proposal 2: During Inventory procedure, the AIoT RAN can send the latest association between reader and device(s) to the AIoTF, via Inventory report message.

Proposal 3: The AIoTF can acquire the device location based on its associating reader and the reader’s position.

- Conclusion

Based on the above analysis, we provide the following observation and proposals.

Observation 1: The position of T1 reader is unchanged, it can be exchanged between RAN and CN by NGAP signalling or OAM.

Proposal 1: The position of T1 reader is exchanged by OAM.

Proposal 2: During Inventory procedure, the AIoT RAN can send the latest association between reader and device(s) to the AIoTF, via Inventory report message.

Proposal 3: The AIoTF can acquire the device location based on its associating reader and the reader’s position.