3GPP TSG-RAN WG3 Meeting #127	R3-250132

Athens, GR, 17-21 Feb, 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 29.413 | CR | 0035 | rev |  | Current version: | 18.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | x | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction of N3IWF user location information |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, Nokia, Nokia ShanghaiBell, CATT |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R3 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_newRAT-Core, TEI17 |  |  |  |  |  | Date: |  |  | 2025-02-07 |
| Category: | A |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | As agreed in NGAP CR1196 (see R3-247082), the N3IWF user location information IE was renamed as N3IWF user location information with port number IE to include the port number, and a new N3IWF user location information without port number IE was agreed to not include the port number. This is due to the fact that the port number is optional for the N3IWF user location information, as described in TS 23.501. However, the above agreements were not reflected over the procedure texts yet, where the N3IWF user location information IE was described. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Correct the N3IWF user location information IE name to align with TS 38.413 - Add procedural texts on the N3IWF user location information without port number IE. Impact Analysis: Impact assessment towards the previous version of the specification (same release): This CR has isolated impact with the previous version of the specification (same release) because it impacts the non-3GPP user location information. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The procedure texts are not consistent for the N3IWF user location information. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.3 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

Change Begins

## 5.3	Exceptions for NGAP message contents and information element coding when used for non-3GPP access

<<<<<<<<<<<<<<<<<<<< Unmodified Text Omitted >>>>>>>>>>>>>>>>>>>>

The Global RAN Node ID IE in the applicable NGAP messages includes the following IEs as specified in TS 38.413 [2]:

-	Global N3IWF ID IE for the untrusted non-3GPP access.

-	Global TNGF ID IE for the trusted non-3GPP access.

-	Global TWIF ID IE for the trusted WLAN access.

-	Global W-AGF ID IE for the wireline 5G access.

The User Location Information IE in the applicable NGAP messages includes the following IEs as specified in TS 38.413 [2]:

-	N3IWF User Location Information with port number IE or the N3IWF user location information without port number IE for the untrusted non-3GPP access.

-	TNGF User Location Information IE for the trusted non-3GPP access.

-	TWIF User Location Information IE for the trusted WLAN access.

-	W-AGF User Location Information IE for the wireline 5G access.

The Security Key IE in the applicable NGAP messages includes the KN3IWF, or the KTNGF, or the KTWIF, or the KWAGF as specified in TS 33.501 [5].

The RAN UE NGAP ID IE in the applicable NGAP messages identifies the UE association over the NG interface within the N3IWF node, or the TNGF node, or the TWIF node, or the W-AGF node, as specified in TS 38.413 [2].

<<<<<<<<<<<<<<<<<<<< Unmodified Text Omitted >>>>>>>>>>>>>>>>>>>>

Change Ends