3GPP TSG-RAN WG3 Meeting #127	R3-250146

Athens, Greece, 17-21 Feb 2025

Agenda Item:	20.2

Source:	ZTE Corporation

Title:	[TP to 38.455 & 38.473 & 38.401] Support of AI/ML assisted Positioning (case 3a & 3b)

Document for:	Discussions & Approval

- Introduction

The objectives of AI/ML for air interface WI [1] are listed as follows:

- Positioning accuracy enhancements, encompassing [RAN1/RAN2/RAN3]:

  - Direct AI/ML positioning:

    - (1st priority) Case 1: UE-based positioning with UE-side model, direct AI/ML positioning

    - (2nd priority) Case 2b: UE-assisted/LMF-based positioning with LMF-side model, direct AI/ML positioning

    - (1st priority) Case 3b: NG-RAN node assisted positioning with LMF-side model, direct AI/ML positioning

  - AI/ML assisted positioning

    - (2nd priority) Case 2a: UE-assisted/LMF-based positioning with UE-side model, AI/ML assisted positioning

    - (1st priority) Case 3a: NG-RAN node assisted positioning with gNB-side model, AI/ML assisted positioning

  - Specify necessary measurements, signalling/mechanism(s) to facilitate LCM operations specific to the Positioning accuracy enhancements use cases, if any

  - Investigate and specify the necessary signalling of necessary measurement enhancements (if any)

  - Enabling method(s) to ensure consistency between training and inference regarding NW-side additional conditions (if identified) for inference at UE for relevant positioning sub use cases

  - Note: RAN1 will not discuss any proposals targeting 2nd priority issues in Q4 of 2024.

Based on the current discussions in RAN1 and RAN2, the topic that requires normative work in RAN3 is AI/ML-based positioning. Further analysis indicates that Case 3a and Case 3b may have impacts on RAN3.

In previous meetings, the initial RAN3 progress was as follows:

The existing LCS framework is re-used for AI/ML positioning.

Case 3a agreements:

- LMF provides ground truth label and related data (i.e., part B information as called by RAN1) to NG-RAN.

FFS on the content of the ground truth label and related data pending on RAN1 progress. FFS on the signalling design.

- How the LMF obtains the ground truth label and related data information is outside of RAN3 scope.

- NG-RAN obtains measurements from TRP (i.e., part A information as called by RAN1).

The details of the measurements are still pending and subject to further discussion in RAN1.

- RAN3 will proceed with Option A: NG-RAN node performs monitoring metric calculation for its own model.

- Signalling of measurements as the inference output for case 3a can re-use the existing procedures for positioning measurement specified over NRPPa.

- WA: The LMF starts a NRPPa transaction. The gNB determines that data collection is needed.

For case 3b:

- NG-RAN gets measurements (i.e., part A information as called by RAN1) from TRP and provides them to LMF.

The details of the measurements are still pending and subject to further discussion in RAN1.

In this contribution, we provide our understanding on the case 3a & 3b about assisted AI/ML positioning.

- Discussion

  - Case 3b (Direct AI/ML positioning)

Below are RAN1's agreements on Case 3b, which have implications for RAN3.

Agreement in RAN1 For Rel-19 AI/ML based positioning Case 3b, regarding sample-based measurement (if supported), from RAN1 perspective, - LMF can signal parameter values of Nt, Nt', k to gNB via NRPPa. For Rel-19 AI/ML based positioning, for Case 3b, in addition to path-based measurement that is referring to the measurement in the existing specifications (up to Rel-18), additionally support the following enhancement to the measurement, - The measurement is composed of Nt' values of the estimated channel response in time domain. The Nt’ values are selected from a list of Nt consecutive channel response values, which have timing granularity T. - The timing information for the Nt' values are reported with a timing granularity T, where T=2kxTc. k represents the timing reporting granularity factor. Tc is the basic time unit for NR. - The associated measurement (e.g., power if reported) corresponds to the measurement for the reported Nt' values. - The timing information is defined relative to a reference time, same as the path-based measurement. - The Nt’ selected time domain channel measurement values are expected to be those with the highest power. - The starting time of the list of Nt consecutive values is determined as: starting time = first detected path rounded down with timing granularity T. - LMF can signal parameter values of Nt, Nt', k to gNB via NRPPa. Candidate set values: - Nt'<=24. FFS: Nt' values. - Nt = {32, 64, 128} - FFS: k - The gNB/TRP may use different Nt', Nt and/or k values other than the signalled parameter for measurement reporting. In this case, it’s up to LMF implementation to process the reported measurement - FFS: whether transmit offset from gNB to LMF Note: measurement by UE is a separate discussion. Note: the purpose of the time domain channel measurements, such as for Rel-19 AI/ML based positioning, is not specified From RAN1 perspective, for model inference of AI/ML positioning Case 3b, at least the following are mandatorily or optionally supported in a measurement report from gNB to LMF: - (Mandatory) Channel measurement; - (Optional) Quality of the channel measurement; - FFS: details of the quality - (Mandatory) Time stamp of the channel measurement.

It is explicitly observed that the LMF can signal the parameter values Nt, Nt', and k to the gNB via NRPPa for sample-based measurements. In the legacy procedure, the LMF signals the measurement configuration through the MEASUREMENT REQUEST message. Hence, it is straightforward that the LMF can also signal the parameter values Nt, Nt', and k to the gNB via the MEASUREMENT REQUEST message. The same enhancement is applied to F1AP from CU to DU.

- Enhance the MEASUREMENT REQUEST message from LMF to gNB to involve the parameter values Nt, Nt', and k.

- Enhance the POSITIONING MEASUREMENT REQUEST message from CU to DU to involve the parameter values Nt, Nt', and k.

Additionally, from RAN1 perspective, the measurement report from the gNB to the LMF includes the following:

- (Mandatory) Channel measurement

- (Mandatory) Timestamp of the channel measurement

- (Optional) Quality of the channel measurement

It is widely acknowledged that the existing Time Stamp IE, Measurement Quality IE, and LoS/NLoS Indicator IE in the TRP measurement result can be reused for Case 3a. Therefore, the same principle applies to Case 3b.

- For case 3b, the existing Time Stamp IE, Measurement Quality IE can be reused in the TRP measurement result.

- Introduce the Channel Measurement IE as one of TRP Measurement Result over NRPPa and F1AP.

  - Case 3a (AI/ML assisted positioning)

2.2.1 Model training

In the last meeting, there was a heated discussion regarding Case 3a (AI/ML-assisted positioning). The main point of contention was how the gNB should request training data from the LMF and whether the gNB can trigger the request message to the LMF without adhering to the principle that the LMF must always initiate the transaction ID.

As specified in the TS 38.305,

Positioning and data acquisition transactions between a LMF and NG-RAN node are modelled by using procedures of the NRPPa protocol. There are two types of NRPPa procedures:

-	UE associated procedure, i.e. transfer of information for a particular UE, including the procedures supporting the Positioning Information Transfer, E-CID Location Information Transfer and Measurement Preconfiguration Information Transfer functions;

-	Non UE associated procedure, i.e. transfer of information applicable to the NG-RAN node and associated TRP, including the procedures supporting the OTDOA Information Transfer, Assistance Information Transfer, TRP Information Transfer, Measurement Information Transfer and PRS Information Transfer functions.

Parallel transactions between the same LMF and NG-RAN node are supported; i.e. a pair of LMF and NG-RAN node may have more than one instance of an NRPPa procedure in execution at the same time.

For possible extensibility, the protocol is considered to operate between a generic "access node" (e.g. gNB, ng-eNB) and a "server" (e.g. LMF). An NRPPa transaction is only initiated by the server.

The NRPPa transaction can only be initiated by the LMF. Therefore, for AI/ML-based positioning, this NRPPa transaction must also be initiated by the LMF. Otherwise, it would violate the traditional positioning mechanism. Whether in the context of AI/ML for Air or AI/ML for NG-RAN, AI/ML is merely an enhancement tool. We should not disrupt the legacy mechanism to enhance a specific AI/ML case. Instead, AI/ML should be introduced within the framework of legacy operations.

- For AI/ML-based positioning, the NRPPa transaction must also be initiated by the LMF, following the principle of legacy positioning mechanism.

Therefore, two candidate solutions can be considered to support request and report training information between NG-RAN node and LMF:

- 1st Sol: The request from NG-RAN node is embedded in the legacy procedure, and the new Class 2 message from LMF is used for reporting.

- 2nd Sol: Introduce new data collection initiation and reporting procedure over NRPPa, enabling NG-RAN request part B information from LMF, as similar mechanisms as data collection procedure over XnAP.

1st Solution:

Based on traditional positioning principles, the gNB needs to request positioning training data from the LMF. Since model training is not necessarily real-time, it will only begin once sufficient data has been collected. Additionally, model accuracy validation is also required. From our perspective, if an NG-RAN node needs to collect data from the LMF for model training, it can feedback to the LMF during each positioning procedure, indicating that it requires additional positioning information (Part B) for that instance.

- If an NG-RAN node needs to collect data (part B) from the LMF for model training, it can feedback the request requirements to the LMF during each positioning procedure.

The request indication from gNB to LMF can be following procedures:

Figure 1. The request training data requirement from gNB to LMF

An NG-RAN node with model training needs can inform the LMF about its data collection requirements through a response message during the positioning procedure. The requirements can be conveyed in the TRP INFORMATION RESPONSE, POSITIONING INFORMATION RESPONSE, and MEASUREMENT RESPONSE. From our perspective, it is preferable to use the MEASUREMENT RESPONSE message to carry the request indication for training data collection, as the NG-RAN node has the full picture of the SRS configuration at that time.

- The NG-RAN node can inform the LMF about its data collection requirements through one of the NRPPa positioning procedures, preferably using the MEASUREMENT RESPONSE message.

After calculating the UE location, the LMF determines whether user consent allows the provision of training data information to the NG-RAN node. If consent is granted, the LMF reports the training data information associated with the UE via the DATA COLLECTION REPORT message. If consent is not granted, the LMF sends a DATA COLLECTION FAILURE message to the NG-RAN node, including a failure cause.

- If consent is granted, the LMF reports the UE's training data information via the new DATA COLLECTION REPORT message; otherwise, it sends a new DATA COLLECTION FAILURE message to the NG-RAN node with a failure cause.

Besides, whether LMF should notify the NG-RAN node that data collection is available can be further discussed. Since user consent is controlled by the LMF, but the request initiation is controlled by the NG-RAN node, the NG-RAN node will ultimately be informed whether the LMF is allowed to provide training data information. Therefore, having the LMF proactively inform the NG-RAN node about data collection availability before the positioning procedure is merely redundant reference information. However, if it is indeed necessary, the LMF can inform the NG-RAN node through an existing request message or a new NRPPa message.

- Having the LMF proactively inform the NG-RAN node about data collection availability before the positioning procedure is merely redundant reference information.

2nd Solution:

The general procedure of data generation and collection (data collection/generation node can be different entities in different use cases) can be listed as follows (refer to Figure 1):

| - Step1: Data collection node indicates the requirement for data collection - Step2 (UE/PRU/gNB’s behavior): Data generation node generates data based on the configured requirement - Step3: Data generation node reports the collected data | Figure 1 Data generation and collection procedure |
| --- | --- |

It is feasible to let the data collection node configure the data collection requirement for AI/ML positioning model considering the positioning accuracy requirement. More specifically, AI/ML models with higher QoS requirements can collect data with higher quality requirements during the data collection procedure, where the requirements may include quality requirement of the channel measurement, report periodicity, etc. And the data collection/generation entity can be different among use cases.

For case 3a model training, LMF need to provide ground truth label and related data (i.e., part B information as called by RAN1) to NG-RAN. Therefore it is proposed to introduce new data collection initiation and reporting procedure over NRPPa, as similar mechanisms as data collection procedure over XnAP.

The key to this solution is whether the NG-RAN node is allowed to store the NRPPa transaction ID and proactively trigger the DATA COLLECTION REQUEST message to the LMF.

- If it is allowed that NG-RAN node to trigger the request message with the stored NRPPa transaction ID LMF initiated before, introduce new data collection initiation and reporting procedure over NRPPa, enabling NG-RAN request part B information from LMF, as similar mechanisms as data collection procedure over XnAP.

2.2.2 Model performance evaluation

In the last meeting, RAN3 has achieved a working assumption that RAN3 will proceed with Option A: NG-RAN node performs monitoring metric calculation for its own model.

And following is the RAN1 agreement on this issue:

Agreement

For AI/ML positioning Case 3a, for performance monitoring metric calculation in label-based monitoring, from RAN1 perspective, Option A and Option B are feasible,

- Option A.	NG-RAN node performs monitoring metric calculation for its own model.

- Option B.	LMF performs monitoring metric calculation for the model located at the NG-RAN node.

Note: Final selection of Option A and Option B is out of RAN1 scope. Potential support of Option A and/or Option B is pending RAN3 confirmation.

RAN3 now agreed that NG-RAN node performs monitoring metric calculation for its own model. Now, there are 2 different ways for model performance evaluation：

- Option 1: gNB performs model monitoring metric calculation and makes monitoring decision

- Option 2: gNB performs model monitoring metric calculation and LMF makes monitoring decision.

- RAN3 is kindly asked to discuss which option is the appropriate way for model performance evaluation for case 3a:

Option 1: gNB performs model monitoring metric calculation and makes monitoring decision

Option 2: gNB performs model monitoring metric calculation and LMF makes monitoring decision.

For model performance evaluation, RAN3 discussed this topic during the R17 and R18 stages and concluded that model performance is out of RAN3's scope. This wording to reflect this into stage2 specification is still under discussion regarding whether it needs further revision, but the principle is to avoid transmitting model performance metrics over NG-RAN interfaces (e.g., loss, accuracy, etc.).

- Option 1 (gNB performs model monitoring metric calculation and makes the monitoring decision), also referred to as Option A, is the appropriate approach for model performance evaluation in Case 3a.

RAN1 requests confirmation from RAN3 regarding the performance monitoring metric calculation.

- Send the LS to RAN1 to inform RAN3 preference on the option of model performance monitoring metrics calculation for case 3a.

  - Stage 2 impact

For the AI/ML topic, it is important to specify which network entity is responsible for model training and model inference. In the case of AI/ML for NG-RAN, the deployment is already documented in the specification. Therefore, the deployment of AI/ML-assisted positioning should also be explicitly mentioned in the specification.

For AI/ML-assisted positioning (Case 3a):

- Model inference is performed at the gNB-CU.

- Model training is performed at either the gNB-CU or OAM.

- Capture the deployment for AI/ML assisted positioning in the TS 38.401.

- Conclusion

Following is our proposals and observations.

Case 3b (Direct Positioning):

- Enhance the MEASUREMENT REQUEST message from LMF to gNB to involve the parameter values Nt, Nt', and k.

- Enhance the POSITIONING MEASUREMENT REQUEST message from CU to DU to involve the parameter values Nt, Nt', and k.

- For case 3b, the existing Time Stamp IE, Measurement Quality IE can be reused in the TRP measurement result.

- Introduce the Channel Measurement IE as one of TRP Measurement Result over NRPPa and F1AP.

Case 3a (Assisted Positioning):

- For AI/ML-based positioning, the NRPPa transaction must also be initiated by the LMF, following the principle of legacy positioning mechanism.

- If an NG-RAN node needs to collect data (part B) from the LMF for model training, it can feedback the request indication to the LMF during each positioning procedure.

- The NG-RAN node can inform the LMF about its data collection requirements through one of the NRPPa positioning procedures, preferably using the MEASUREMENT RESPONSE message.

- If user consent is granted, the LMF reports the UE's training data information via the DATA COLLECTION REPORT message; otherwise, it sends a DATA COLLECTION FAILURE message to the NG-RAN node with a failure cause.

- Having the LMF proactively inform the NG-RAN node about data collection availability before the positioning procedure is merely redundant reference information.

- If it is allowed that NG-RAN node to trigger the request message with the stored NRPPa transaction ID LMF initiated before, introduce new data collection initiation and reporting procedure over NRPPa, enabling NG-RAN request part B information from LMF, as similar mechanisms as data collection procedure over XnAP.

- RAN3 is kindly asked to discuss which option is the appropriate way for model performance evaluation for case 3a:

Option 1: gNB performs model monitoring metric calculation and makes monitoring decision

Option 2: gNB performs model monitoring metric calculation and LMF makes monitoring decision.

- Option 1 (gNB performs model monitoring metric calculation and makes the monitoring decision), also referred to as Option A, is the appropriate approach for model performance evaluation in Case 3a.

- Send the LS to RAN1 to inform RAN3 preference on the option of model performance monitoring metrics calculation for case 3a.

- Capture the deployment for AI/ML assisted positioning in the TS 38.401.

- Text Proposal – NRPPa TS 38.455

9.1.4.1	MEASUREMENT REQUEST

This message is sent by the LMF to request the NG-RAN node to configure a positioning measurement.

Direction: LMF  NG-RAN node.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.2.3 |  | YES | reject |
| NRPPa Transaction ID | M |  | 9.2.4 |  | - |  |
| LMF Measurement ID | M |  | INTEGER (1..65536, …) |  | YES | reject |
| TRP Measurement Request List |  | 1 |  |  | YES | reject |
| >TRP Measurement Request Item |  | 1..<maxnoofMeasTRPs> |  |  | - |  |
| >>TRP ID | M |  | 9.2.24 |  | - |  |
| >>Search Window Information | O |  | 9.2.26 |  | - |  |
| >>Cell ID | O |  | NR CGI 9.2.9 | The Cell ID of the TRP identified by the TRP ID IE. | YES | ignore |
| >>AoA Search Window Information | O |  | UL-AoA Assistance Information 9.2.66 |  | YES | ignore |
| >>Number of TRP Rx TEGs | O |  | ENUMERATED (2, 3, 4, 6, 8, …) |  | YES | ignore |
| >>Number of TRP RxTx TEGs | O |  | ENUMERATED (2, 3, 4, 6, 8, …) |  | YES | ignore |
| Report Characteristics | M |  | ENUMERATED (OnDemand, Periodic, ...) |  | YES | reject |
| Measurement Periodicity | C-ifReportCharacteristicsPeriodic |  | ENUMERATED (120ms, 240ms, 480ms, 640ms, 1024ms, 2048ms, 5120ms, 10240ms, 1min, 6min, 12min, 30min, 60min,…, 20480ms, 40960ms, extended) | The codepoint 120ms, 240ms, 480ms, 1024ms, 2048ms, 1min, 6min, 12min, 30min, and 60min are not applicable | YES | reject |
| TRP Measurement Quantities |  | 1 |  |  | YES | reject |
| >TRP Measurement Quantities Item |  | 1 .. <maxnoPosMeas> |  |  | - |  |
| >>TRP Measurement Type | M |  | ENUMERATED (gNB-RxTxTimeDiff, UL-SRS-RSRP, UL-AoA, UL-RTOA,…, Multiple UL-AoA, UL SRS-RSRPP, UL-RSCP, Channel Measurement) | The UL-RSCP measurement is applicable only when the UL-RTOA and/or gNB-RxTxTimeDiff measurement(s) is also requested. | - |  |
| >>Timing Reporting Granularity Factor | O |  | INTEGER (0..5) | Value (0..5) corresponds to (k0..k5) TS 38.133 [16]. This IE is ignored when the Timing Reporting Granularity Factor Extended IE is included. | - |  |
| >>Timing Reporting Granularity Factor Extended | O |  | INTEGER (-6..-1, …) | Value -6 corresponds to kminus6, value -5 corresponds to kminus5 and so on, see TS 38.133 [16] | - |  |
| >>Channel Measurement Configuration | O |  | 9.2.xx |  | YES | ignore |
| SFN initialisation Time | O |  | Relative Time 1900 9.2.36 | If this IE is not present, the TRP may assume that the value is same as its own SFN initialisation time. | YES | ignore |
| SRS Configuration | O |  | 9.2.28 |  | YES | ignore |
| Measurement Beam Information Request | O |  | ENUMERATED (true,...) | This IE is ignored when the Measurement Characteristics Request Indicator IE is included. | YES | ignore |
| System Frame Number | O |  | INTEGER(0..1023) |  | YES | ignore |
| Slot Number | O |  | INTEGER(0..79) |  | YES | ignore |
| Measurement Periodicity Extended | C-ifMeasPerExt |  | ENUMERATED (160ms, 320ms, 1280ms, 2560ms, 61440ms, 81920ms, 368640ms, 737280ms, 1843200ms, …) |  | YES | reject |
| Response Time | O |  | 9.2.68 | This IE is ignored when the Report Characteristics IE is set to “periodic”. | YES | ignore |
| Measurement Characteristics Request Indicator | O |  | 9.2.81 |  | YES | ignore |
| Measurement Time Occasion | O |  | ENUMERATED (o1, o4, …) |  | YES | ignore |
| Measurement Amount | O |  | ENUMERATED (0, 1, 2, 4, 8, 16, 32, 64) | This IE is ignored if the Report Characteristics IE is set to ‘OnDemand’. Value 0 represents an infinite number of periodic reporting. | YES | ignore |
| Time Window Information Measurement List | O |  | 9.2.91 |  | YES | ignore |

| Condition | Explanation |
| --- | --- |
| ifReportCharacteristicsPeriodic | This IE shall be present if the Report Characteristics IE is set to the value "Periodic". |
| ifMeasPerExt | This IE shall be present if the Measurement Periodicity IE is set to the value "extended". |

| Range bound | Explanation |
| --- | --- |
| maxnoPosMeas | Maximum no. of measured quantities that can be configured and reported with one positioning measurement message. Value is 16384. |
| maxnoofMeasTRPs | Maximum no. of TRPs that can be included within one message. Value is 64. |

9.2.xx	Sample based Measurement Configuration

The IE contains the configuration for sample-based measurements for AI/ML assisted positioning.

| IE/Group Name | Presence | Range | IE Type and Reference | Semantics Description |
| --- | --- | --- | --- | --- |
| Selected Channel Response Count | M |  | INTEGER(FFS) |  |
| Total Channel Response Count | M |  | ENUMERATED (32, 64,128) |  |
| Timing Reporting Granularity Factor | M |  | FFS |  |

9.1.4.2	MEASUREMENT RESPONSE

This message is sent by the NG-RAN node to report positioning measurements.

Direction: NG-RAN node  LMF.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.2.3 |  | YES | reject |
| NRPPa Transaction ID | M |  | 9.2.4 |  | - |  |
| LMF Measurement ID | M |  | INTEGER (1..65536, …) |  | YES | reject |
| RAN Measurement ID | M |  | INTEGER (1..65536, …) |  | YES | reject |
| TRP Measurement Response List |  | 0..1 |  |  | YES | reject |
| >TRP Measurement Response Item |  | 1..<maxnoofMeasTRPs> |  |  | - |  |
| >>TRP ID | M |  | 9.2.24 |  | - |  |
| >>TRP Measurement Result | M |  | 9.2.37 |  | - |  |
| >>Cell ID | O |  | NR CGI 9.2.9 | The Cell ID of the TRP identified by the TRP ID IE. | YES | ignore |
| Criticality Diagnostics | O |  | 9.2.2 |  | YES | ignore |

| Range bound | Explanation |
| --- | --- |
| maxnoofMeasTRPs | Maximum no. of TRPs that can be included within one message. Value is 64. |

9.1.4.3	MEASUREMENT FAILURE

This message is sent by the NG-RAN node to report measurement failure.

Direction: NG-RAN node  LMF.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.2.3 |  | YES | reject |
| NRPPa Transaction ID | M |  | 9.2.4 |  | - |  |
| LMF Measurement ID | M |  | INTEGER (1..65536, …) |  | YES | reject |
| Cause | M |  | 9.2.1 |  | YES | ignore |
| Criticality Diagnostics | O |  | 9.2.2 |  | YES | ignore |

9.1.4.4	MEASUREMENT REPORT

This message is sent by the NG-RAN node to report positioning measurements for the target UE.

Direction: NG-RAN node  LMF.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.2.3 |  | YES | ignore |
| NRPPa Transaction ID | M |  | 9.2.4 |  | - |  |
| LMF Measurement ID | M |  | INTEGER (1..65536, …) |  | YES | reject |
| RAN Measurement ID | M |  | INTEGER (1..65536, …) |  | YES | reject |
| TRP Measurement Response List |  | 1 |  |  | YES | reject |
| >TRP Measurement Response Item |  | 1..<maxnoofMeasTRPs> |  |  | - |  |
| >>TRP ID | M |  | 9.2.24 |  | - |  |
| >>TRP Measurement Result | M |  | 9.2.37 |  | - |  |
| >>Cell ID | O |  | NR CGI 9.2.9 | The Cell ID of the TRP identified by the TRP ID IE. | YES | ignore |

9.2.37	TRP Measurement Result

This information element contains the measurement result.

| IE/Group Name | Presence | Range | IE Type and Reference | Semantics Description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Measured Result Item |  | 1 .. <maxnoPosMeas> |  |  | - |  |
| >CHOICE Measured Results Value | M |  |  |  | - |  |
| >>UL Angle of Arrival |  |  | 9.2.38 |  |  |  |
| >>UL SRS-RSRP |  |  | INTEGER (0..126) |  |  |  |
| >>UL RTOA Measurement |  |  | 9.2.39 |  |  |  |
| >>gNB Rx-Tx Time Difference |  |  | 9.2.40 |  |  |  |
| >>Z-AoA |  |  | 9.2.67 |  | YES | reject |
| >>Multiple UL-AoA |  |  | 9.2.71 |  | YES | reject |
| >>UL SRS-RSRPP |  |  | 9.2.72 |  | YES | reject |
| >>UL RSCP |  |  | 9.2.92 |  | YES | reject |
| >>Channel Measurement |  |  | FFS |  | Yes | reject |
| >Time Stamp | M |  | 9.2.42 |  | - |  |
| >Measurement Quality | O |  | 9.2.43 |  | - |  |
| >Measurement Beam Information | O |  | 9.2.57 |  | - |  |
| >SRS Resource type | O |  | 9.2.73 |  | YES | ignore |
| >ARP ID | O |  | 9.2.75 |  | YES | ignore |
| >LoS/NLoS Information | O |  | 9.2.77 |  | YES | ignore |
| >Mobile TRP Location Information | O |  | 9.2.88 |  | YES | ignore |
| >Measured Frequency Hops | O |  | ENUMERATED (singleHop, multiHop, …) |  | YES | ignore |
| >Aggregated Positioning SRS Resource ID List |  | 0..1 |  | Indicates the used Positioning SRS resources across aggregated carriers. | YES | ignore |
| >>Aggregated Positioning SRS Resource ID Item |  | 2..< maxnoaggregatedPosSRS-Resources > |  |  | - |  |
| >>>Positioning SRS Resource ID | M |  | INTEGER (0..63) |  |  |  |
| >>>Point A | M |  | INTEGER (0..3279165) |  | YES | ignore |
| >>>SCS Specific Carrier |  | 1 |  |  | YES | ignore |
| >>>>Offset To Carrier | M |  | INTEGER (0..2199, …) |  |  |  |
| >>>>Subcarrier Spacing | M |  | ENUMERATED(kHz15, kHz30, kHz60, kHz120, …, kHz480, kHz960) |  |  |  |
| >>>>Carrier Bandwidth | M |  | INTERGER (1..275, …) |  |  |  |
| >>>PCI | O |  | INTEGER (0..1007) |  | YES | ignore |
| >Measurement Based On Aggregated Resources | O |  | ENUMERATED(true, …) |  | YES | ignore |

- Text Proposal – F1AP TS 38.473

// mirror TP to TS38.455

- Text Proposal – TS 38.401

7.6	Positioning

The NG-RAN supports the positioning functionality as specified in TS 38.305 [25].

To support of AI/ML assisted positioning, in case of CU-DU split architecture, the following scenarios may be supported:

-	AI/ML Model Training is located in the OAM and AI/ML Model Inference is located in the gNB-CU.

-	AI/ML Model Training and Model Inference are both located in the gNB-CU.

- 7	Draft LS to RAN1

3GPP TSG-RAN WG3 Meeting #127	R3-25xxxx

Athens, Greece, 17-21 Feb 2025

Title:	[Draft] LS on AI/ML assisted Positioning about model performance monitoring

Response to:	-

Release:	Rel-19

Work Item:	NR_AIML_air-Core

Source:	ZTE Corporation (will be RAN3)

To:	RAN1

Cc:	RAN2

Contact person:	Chen Jiajun

Chen.jiajun1@zte.com.cn

Send any reply LS to:  3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

Attachments: N/A

- Overall description

RAN3 discussed the following options for model performance monitoring in the case of case 3a,

- Option 1: gNB performs model monitoring metric calculation and makes monitoring decision

- Option 2: LMF performs model monitoring metric calculation and makes monitoring decision

- Option 3: LMF performs model monitoring metric calculation and gNB makes monitoring decision

- Option 4: gNB performs model monitoring metric calculation and LMF makes monitoring decision

RAN3 prefers the option 1 that gNB performs model monitoring metric calculation and makes monitoring decision to avoid transmitting model performance metrics over NG-RAN interfaces.

- Actions

To RAN1:

ACTION: 	RAN3 kindly asks RAN1 to take the above information into consideration.

3	Dates of next TSG SA WG 5 meetings

RAN3#128	17 February 	- 21 February   2025		China