3GPP TSG-RAN WG3 Meeting #127	R3-250147

Athens, Greece, 17 – 21 February 2025

Agenda item:	16.2

Source:	Xiaomi

Title:	Discussion on support of AIoT topology 1

Document for:	Discussion

# 1	Introduction

In this paper, we would like to discuss the issue and potential solutions to support AIoT topology 1 based on RAN WID RP-243326.

Specify necessary architectural aspects, and signaling and procedures between A-IoT RAN and A-IoT CN to support the A-IoT functions, assuming an architecture of aggregated gNB, including: Inventory and command operations Device location reporting at reader ID granularity Note: The above A-IoT functions are supported over the existing NG interface, based on architecture(s) defined by RAN3/SA2.

The following sub-topics are discussed:

- Architecture and protocol stacks

- Overall NG Functions for AIoT

- NG Setup between AIoTF and gNB for direct path

- Inventory and command operation

- Reader selection

# 2	Discussion

### Architecture and Protocol stacks

According to the SA2 conclusion in TR 23.700-13[1], both direct path and indirect path between AIoT RAN and AIoTF are supported.

Figure 1 RAN architecture for direct path

Figure 2 RAN architecture for indirect path

The protocol stacks for the different architecture can be as follows:

Figure 3 Protocol stacks for direct path

Figure 4 Protocol stacks for indirect path

AIoT information can be part of information in NGAP. For indirect path, the necessary enhancements between AMF and AIoTF are needed, e.g., AMF needs to forward and route the AIoT related information between gNB and AIoTF, similar to SM information, RAN3 needs to send LS to SA2 and CT4 and ask them to specify the interactions between AMF and AIoTF.

Proposal 1, Specify AIoT information as NGAP containers for both direct path and indirect path.

Proposal 2, Liaise with SA2/CT4 to define AMF-AIoTF interactions, treating AIoT information (e.g., inventory/command) as N2 message containers.

### Overall NG Functions for AIoT

The content of the AIoT information depends on the functions to support the above architecture and protocols, the following functions needs to be supported for AIoT operation.

| Functions | Direct path | Indirect path |
| --- | --- | --- |
| NG management | New/enhancement on NG setup | Existing NG setup |
| AIoT Inventory | common AIoT Inventory procedure |  |
| AIoT Inventory Report | common AIoT Report procedure |  |
| AIoT Command | common AIoT Command procedure |  |
| Reader Information Exchange | common Reader Information procedure |  |

For NG interface management, there’re differences between direct path and indirect path.

For AIoT related functions, i.e., AIoT inventory, inventory report, command, reader information exchange can be common for both direct path and indirect path.

Proposal 3, except NG management, AIoT functions, i.e., AIoT inventory, inventory report, command, and reader information exchange are common for both direct path and indirect path.

### NG Setup between AIoTF and gNB for direct path

The main difference between direct path and indirect path is that new NG or enhanced NG setup procedure needs to be introduced for direct path. And there’re two options for NG setup

- option 1 RAN-initiated (reuses existing NG setup but includes incompatible IEs).

- option 2 AIoTF-initiated (preferred, with SCTP association preconfigured by OAM).

If RAN initiates NG setup, the existing NG setup procedure can be re-used, however, there’re some mandatory IEs marked in yellow below not suitable for AIoTF,

Information in NG setup request

| IE/Group Name | Presence |
| --- | --- |
| Message Type | M |
| Global RAN Node ID | M |
| RAN Node Name | O |
| Supported TA List |  |
| >Supported TA Item |  |
| >>TAC | M |
| >>Broadcast PLMN List |  |
| >>>Broadcast PLMN Item |  |
| >>>>PLMN Identity | M |
| >>>>TAI Slice Support List | M |
| Default Paging DRX | M |

Information in NG setup response

| IE/Group Name | Presence |
| --- | --- |
| Message Type | M |
| AMF Name | M |
| Served GUAMI List |  |
| >Served GUAMI Item |  |
| >>GUAMI | M |
| Relative AMF Capacity | M |
| PLMN Support List |  |
| >PLMN Support Item |  |
| >>PLMN Identity | M |
| >>Slice Support List | M |

AIoTF initiates NG setup is more suitable for AIoT operation, as the AIoT session is always initiated by AIoTF. The SCTP association between AIoTF and gNB can be preconfigured by OAM, then the AIoTF initiates the AIoT NG setup when needed.

For SCTP association, in NR system, there’re multiple associations between gNB and AMF including UE associations and non-UE associations. For topology 1 in AIoT system, it’s enough to only have non-UE association, so that multiple SCTP association is not needed between gNB and AMF.

Observation 1, an AIoTF session is always initiated by core network.

Observation 2, there is no UE-associated SCTP associations between AIoTF and gNB for topology 1.

Proposal 4, RAN3 agree to introduce AIoTF-initiated NG setup procedures “e.g., AIoT NG setup” procedure between gNB and AIoTF.

Proposal 5, RAN3 discuss whether to support multiple SCTP association between gNB and AIoTF.

### Inventory and command operation

According to the conclusion in study phase in TR 38.769[2], to support inventory operation, the following procedures are needed:

- inventory request/response/failure

- inventory report

For the detail information, there’re some LS between RAN2 and SA2, according to the SA2’s response in [3].

SA2 thanks RAN2 for the LS (R2-2411232) on Assistance information from CN to the reader. SA2 would like to provide the answer to the questions in the LS as following: Q1: Is it feasible for CN to provide the following information to the reader? - the service type of A-IoT (e.g. inventory, command) - whether the service is targeted for a single or multiple devices; - the approximate number of target devices (if available). SA2 answer: CN can provide the information listed above to the reader if available.

To support command operation, the procedure for command request/response/failure is needed, in addition, the command is at least aimed for one specific device, the RAN level device ID is needed, it may be generated during inventory procedure and can be used to identify a device in RAN level.

For the information inventory request,

| Service type of A-IoT (e.g. inventory, command) | can be realized by using different AIoT messages over NGAP, no need to introduce additional IE for this purpose. |
| --- | --- |
| whether the service is targeted for a single or multiple devices | Introduce a targeted type IE in inventory request message. |
| approximate number of target devices | Introduce an approximate number of target devices IE in inventory request message. |

Proposal 6, RAN3 agree to introduce a class 1 procedure for inventory request/response/failure and a class 2 procedure for inventory report.

Proposal 6bis, RAN3 agree to introduce targeted type IE and approximate number of target devices IE in inventory request message.

Proposal 7, RAN3 agree to introduce a class 1 procedure for command request/response/failure.

Proposal 8, RAN3 agree to introduce RAN level device ID to identify a device in RAN side.

Observation 3, other information in the inventory and command related message pending to the further discussion in RAN2 and SA2.

### Reader selection

There’s an LS from SA2 [4] about reader selection, from SA2 perspective, it’s the AIoTF that performs reader selection with the preconfigured reader information by OAM and/or some reader information from gNB. RAN3 needs to discuss whether gNB needs to provide some reader information to AIoTF

[Question to RAN3] SA2 has concluded for topology 1, the AIOTF selects the AIOT RAN node(s) and optionally provides a list of the BS readers to AIOT RAN. SA2 assumes that OAM can configure the AIOTF with needed information for this selection. SA2 have further discussed whether the AIOT RAN may provide the AIOT RAN information e.g., supported AIOT RAN serving area, supported BS reader ID list and corresponding BS reader serving area to the core network. SA2 kindly asks RAN3 to clarify whether RAN3 plan to support the above AIOT RAN information exchange with AIOTF. If so, SA2 kindly ask RAN3 to provide the definition of the appropriate parameters.

We think it’s beneficial for AIoTF to be aware of reader information from the gNB, not just from the OAM, RAN3 agreed that a gNB may have multiple readers, for topology 1, the readers are antennas in different locations, the locations of the readers are fixed, but the serving area (depending on the transmitting power) and some other radio capabilities (e.g., bandwidth) for a reader may be determined by gNB based on the radio conditions.

The reader information from gNB can be used for not only the reader selection but also locating a device at reader ID granularity, so it’s suggested to introduce a Reader information exchange procedure between gNB and AIoTF.

The procedures for direct path and indirect path can be as follows

Figure 5 reader information exchange for direct path

Figure 6 reader information exchange for indirect path

We assume that the reader ID can be represented by TRP ID, the detail discussion can be found in R3-250147. For reader selection, at least the following information are needed:

- TRP ID, used to identify a reader.

- TRP location, used to identify the geographical location of a reader

- AIoT radio capabilities (frequency, bandwidth, power), used to indicate the radio capabilities of a reader

The serving area of a reader can be determined by the TRP location and power of AIoT radio capability.

For locating a device, if the TRP ID and TRP location of a reader is known by the AIoTF via reader information exchange procedure, if a device is reached by a specific reader, the AIoTF can know the device location by knowing the reader location.

Proposal 9, RAN3 agree to introduce a class 1 procedure for reader information exchange.

Observation 4, The TRP of a reader can be regarded a set of geographically co-located antennas supporting AIoT Transmission Point and/or AIoT Reception Point functionalities.

Proposal 10, the following information can be included in the reader information provided by gNB for reader selection and locating AIoT device:

- TRP ID, used to identify a reader.

- TRP location, used to identify the geographical location of a reader

- AIoT radio capabilities (frequency, bandwidth, power), used to indicate the radio capabilities of a reader.

# 3	Conclusion

In this contribution, we discussed the issue and potential solutions to support AIoT in topology 1 and have the following observations and proposals.

Architecture and Protocol stacks

Proposal 1, Specify AIoT information as NGAP containers for both direct path and indirect path.

Proposal 2, Liaise with SA2/CT4 to define AMF-AIoTF interactions, treating AIoT information (e.g., inventory/command) as N2 message containers.

Overall NG Functions for AIoT

Proposal 3, except NG management, AIoT functions, i.e., AIoT inventory, inventory report, command, and reader information exchange are common for both direct path and indirect path.

NG Setup between AIoTF and gNB for direct path

Observation 1, an AIoTF session is always initiated by core network.

Observation 2, there is no UE-associated SCTP associations between AIoTF and gNB for topology 1.

Proposal 4, RAN3 agree to introduce AIoTF-initiated NG setup procedures “e.g., AIoT NG setup” procedure between gNB and AIoTF.

Proposal 5, RAN3 discuss whether to support multiple SCTP association between gNB and AIoTF.

Inventory and command operation

Proposal 6, RAN3 agree to introduce a class 1 procedure for inventory request/response/failure and a class 2 procedure for inventory report.

Proposal 6bis, RAN3 agree to introduce targeted type IE and approximate number of target devices IE in inventory request message.

Proposal 7, RAN3 agree to introduce a class 1 procedure for command request/response/failure.

Proposal 8, RAN3 agree to introduce RAN level device ID to identify a device in RAN side.

Observation 3, other information in the inventory and command related message pending to the further discussion in RAN2 and SA2.

Reader selection

Proposal 9, RAN3 agree to introduce a class 1 procedure for reader information exchange.

Observation 4, The TRP of a reader can be regarded a set of geographically co-located antennas supporting AIoT Transmission Point and/or AIoT Reception Point functionalities.

Proposal 10, the following information can be included in the reader information provided by gNB for reader selection and locating AIoT device:

- TRP ID, used to identify a reader.

- TRP location, used to identify the geographical location of a reader

- AIoT radio capabilities (frequency, bandwidth, power), used to indicate the radio capabilities of a reader.

# 4	Reference

[1]	 TR 23.700-13 V1.1.0, Study on Architecture support of Ambient power-enabled Internet of Things

[2]  TR 38.769, Study on solutions for ambient IoT (Internet of Things)

[3] R3-250069/S2-2501241 Reply LS on Assistance Information from CN to the Reader

[4] R3-250075/S2-2501343LS on SA2 Ambient IoT TR conclusion update, SA2.

# 5	Annex Draft Reply LS

Title: Reply LS on SA2 Ambient IoT TR Conclusion Update
Response to: LS from SA2 (Ref: [Original LS Reference])
Release: Release 19
Work Item: FS_AmbientIoT, FS_Ambient_IoT_solutions

Source: RAN3
To: SA2, CT4
Cc: [Relevant Groups, if any]

Contact Person:
Name: [RAN3 Contact Name]
E-mail Address: [Contact Email]

Send any reply LS to: 3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

1. Response to SA2 Question:

RAN3 thanks SA2 for the liaison and provides the following feedback:

On SA2’s question regarding support for AIoT RAN information exchange with AIoTF:
RAN3 agrees that enabling the AIoTF to receive reader information (e.g., TRP ID, location, radio capabilities) from the gNB (AIoT RAN) is beneficial for reader selection and device localization. RAN3 confirms its intent to support the exchange of this information and proposes the following:

Parameters for Reader Information Exchange:

- TRP ID: Uniquely identifies a reader (aligned with existing TRP definitions in RAN specifications).

- TRP Location: Geographical coordinates of the reader (e.g., latitude, longitude, altitude).

- AIoT Radio Capabilities:

  - Frequency bands supported.

  - Bandwidth and transmit power configurations.

Procedures for Information Exchange:

- Direct Path: A new Class 1 procedure (e.g., via NGAP or dedicated signaling) between the gNB and AIoTF.

- Indirect Path: reader information transfer between AMF and AIoTF should be supported by SA2 and CT4.

2. Actions:

To SA2 and CT4:

- RAN3 asks SA2 and CT4 to update their specifications accordingly.

Date of Next RAN3 Meetings:

- RAN3 #127-bis: [Dates/Location]

- RAN3 #128: [Dates/Location]