3GPP TSG-RAN WG3 Meeting #127	R3-250148

Athens, Greece, 17 – 21 February 2025

Agenda item:	16.2

Source:	Xiaomi

Title:	(TP for TS 38.413) Support of A-IoT

Document for:	Other

# 1	Introduction

As discussed in R3-250147, the following new procedures are proposed:

- introduce AIoTF initiates AIoT NG setup procedure between gNB and AIoTF.

- introduce a class 1 procedure for inventory request/response/failure and a class 2 procedure for inventory report.

- introduce a class 1 procedure for command request/response/failure.

-  introduce RAN level device to identify a device.

-  introduce a class 1 procedure for reader information exchange.

# 2	TP to TS 38.413

<<<<<<<<<<<<<<<<<<<<<<<<<<<<<<<<<<<<<<Change starts>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>

## 3.2	Abbreviations

For the purposes of the present document, the abbreviations given in 3GPP TR 21.905 [1] and the following apply. An abbreviation defined in the present document takes precedence over the definition of the same abbreviation, if any, in 3GPP TR 21.905 [1].

5GC	5G Core Network

5QI	5G QoS Identifier

ACL	Access Control List

AMF	Access and Mobility Management Function

CAG	Closed Access Group

CGI	Cell Global Identifier

CP	Control Plane

DAPS	Dual Active Protocol Stacks

DC	Dual Connectivity

DL	Downlink

EPC	Evolved Packet Core

FN-RG	Fixed Network Residential Gateway

GUAMI	Globally Unique AMF Identifier

HFC	Hybrid Fiber-Coax

IAB	Integrated Access and Backhaul

IMEISV	International Mobile station Equipment Identity and Software Version number

LMF	Location Management Function

MBS	Multicast/Broadcast Service

MT	Mobile Terminated

N3IWF	Non 3GPP InterWorking Function

NB-IoT	Narrow Band Internet of Things

NGAP	NG Application Protocol

NID	Network Identifier

NPN	Non-Public Network

NRPPa	NR Positioning Protocol Annex

NSAG	Network Slice AS Group

NSCI	New Security Context Indicator

NSSAI	Network Slice Selection Assistance Information

OTDOA	Observed Time Difference of Arrival

PEIPS	Paging Early Indication with Paging Subgrouping

PNI-NPN	Public Network Integrated Non-Public Network

ProSe	Proximity Services

PSCell	Primary SCG Cell

PTM	Point to Multipoint

PTP	Point to Point

QMC	QoE Measurement Collection

QoE	Quality of Experience

RedCap	Reduced Capability

RIM	Remote Interference Management

RIM-RS	RIM Reference Signal

RSN	Redundancy Sequence Number

RSPP	Ranging/SL Positioning Protocol

SCG	Secondary Cell Group

SCTP	Stream Control Transmission Protocol

SgNB	Secondary gNB

SMF	Session Management Function

S-NG-RAN node	Secondary NG-RAN node

SNPN	Stand-alone Non-Public Network

S-NSSAI	Single Network Slice Selection Assistance Information

TAC	Tracking Area Code

TAI	Tracking Area Identity

TNAP	Trusted Non-3GPP Access Point

TNGF	Trusted Non-3GPP Gateway Function

TNLA	Transport Network Layer Association

TSS	Timing Synchronisation Status

TWAP	Trusted WLAN Access Point

TWIF	Trusted WLAN Interworking Function

UL	Uplink

UP	User Plane

UPF	User Plane Function

V2X	Vehicle-to-Everything

W-AGF	Wireline Access Gateway Function

WUS	Wake Up Signal

AIoTF	Ambient Internet of Things Function

<<<<<<<<<<<<<<<<<<<<<<<<<<<<<<<<<<<<<<<Next change>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>

## 8.1	List of NGAP Elementary Procedures

In the following tables, all EPs are divided into Class 1 and Class 2 EPs (see subclause 3.1 for explanation of the different classes):

Table 8.1-1: Class 1 procedures

| Elementary Procedure | Initiating Message | Successful Outcome | Unsuccessful Outcome |
| --- | --- | --- | --- |
|  |  | Response message | Response message |
| AMF Configuration Update | AMF CONFIGURATION UPDATE | AMF CONFIGURATION UPDATE ACKNOWLEDGE | AMF CONFIGURATION UPDATE FAILURE |
| RAN Configuration Update | RAN CONFIGURATION UPDATE | RAN CONFIGURATION UPDATE ACKNOWLEDGE | RAN CONFIGURATION UPDATE FAILURE |
| Handover Cancellation | HANDOVER CANCEL | HANDOVER CANCEL ACKNOWLEDGE |  |
| Handover Preparation | HANDOVER REQUIRED | HANDOVER COMMAND | HANDOVER PREPARATION FAILURE |
| Handover Resource Allocation | HANDOVER REQUEST | HANDOVER REQUEST ACKNOWLEDGE | HANDOVER FAILURE |
| Initial Context Setup | INITIAL CONTEXT SETUP REQUEST | INITIAL CONTEXT SETUP RESPONSE | INITIAL CONTEXT SETUP FAILURE |
| NG Reset | NG RESET | NG RESET ACKNOWLEDGE |  |
| NG Setup | NG SETUP REQUEST | NG SETUP RESPONSE | NG SETUP FAILURE |
| Path Switch Request | PATH SWITCH REQUEST | PATH SWITCH REQUEST ACKNOWLEDGE | PATH SWITCH REQUEST FAILURE |
| PDU Session Resource Modify | PDU SESSION RESOURCE MODIFY REQUEST | PDU SESSION RESOURCE MODIFY RESPONSE |  |
| PDU Session Resource Modify Indication | PDU SESSION RESOURCE MODIFY INDICATION | PDU SESSION RESOURCE MODIFY CONFIRM |  |
| PDU Session Resource Release | PDU SESSION RESOURCE RELEASE COMMAND | PDU SESSION RESOURCE RELEASE RESPONSE |  |
| PDU Session Resource Setup | PDU SESSION RESOURCE SETUP REQUEST | PDU SESSION RESOURCE SETUP RESPONSE |  |
| UE Context Modification | UE CONTEXT MODIFICATION REQUEST | UE CONTEXT MODIFICATION RESPONSE | UE CONTEXT MODIFICATION FAILURE |
| UE Context Release | UE CONTEXT RELEASE COMMAND | UE CONTEXT RELEASE COMPLETE |  |
| Write-Replace Warning | WRITE-REPLACE WARNING REQUEST | WRITE-REPLACE WARNING RESPONSE |  |
| PWS Cancel | PWS CANCEL REQUEST | PWS CANCEL RESPONSE |  |
| UE Radio Capability Check | UE RADIO CAPABILITY CHECK REQUEST | UE RADIO CAPABILITY CHECK RESPONSE |  |
| UE Context Suspend | UE CONTEXT SUSPEND REQUEST | UE CONTEXT SUSPEND RESPONSE | UE CONTEXT SUSPEND FAILURE |
| UE Context Resume | UE CONTEXT RESUME REQUEST | UE CONTEXT RESUME RESPONSE | UE CONTEXT RESUME FAILURE |
| UE Radio Capability ID Mapping | UE RADIO CAPABILITY ID MAPPING REQUEST | UE RADIO CAPABILITY ID MAPPING RESPONSE |  |
| Broadcast Session Setup | BROADCAST SESSION SETUP REQUEST | BROADCAST SESSION SETUP RESPONSE | BROADCAST SESSION SETUP FAILURE |
| Broadcast Session Modification | BROADCAST SESSION MODIFICATION REQUEST | BROADCAST SESSION MODIFICATION RESPONSE | BROADCAST SESSION MODIFICATION FAILURE |
| Broadcast Session Release | BROADCAST SESSION RELEASE REQUEST | BROADCAST SESSION RELEASE RESPONSE |  |
| Distribution Setup | DISTRIBUTION SETUP REQUEST | DISTRIBUTION SETUP RESPONSE | DISTRIBUTION SETUP FAILURE |
| Distribution Release | DISTRIBUTION RELEASE REQUEST | DISTRIBUTION RELEASE RESPONSE |  |
| Multicast Session Activation | MULTICAST SESSION ACTIVATION REQUEST | MULTICAST SESSION ACTIVATION RESPONSE | MULTICAST SESSION ACTIVATION FAILURE |
| Multicast Session Deactivation | MULTICAST SESSION DEACTIVATION REQUEST | MULTICAST SESSION DEACTIVATION RESPONSE |  |
| Multicast Session Update | MULTICAST SESSION UPDATE REQUEST | MULTICAST SESSION UPDATE RESPONSE | MULTICAST SESSION UPDATE FAILURE |
| Timing Synchronisation Status | TIMING SYNCHRONISATION STATUS REQUEST | TIMING SYNCHRONISATION STATUS RESPONSE | TIMING SYNCHRONISATION STATUS FAILURE |
| MT Communication Handling | MT COMMUNICATION HANDLING REQUEST | MT COMMUNICATION HANDLING RESPONSE | MT COMMUNICATION HANDLING FAILURE |
| Broadcast Session Transport | BROADCAST SESSION TRANSPORT REQUEST | BROADCAST SESSION TRANSPORT RESPONSE | BROADCAST SESSION TRANSPORT FAILURE |
| AIoT NG Setup | AIOT NG SETUP REQUEST | AIOT NG SETUP RESPONSE | AIOT NG SETUP FAILURE |
| AIoT Inventory | AIOTF INVENTORY REQUEST | AIOTF INVENTORY RESPONSE | AIOTF INVENTORY FAILURE |
| AIoT Command | AIOT COMMAND REQUEST | AIOTF COMMAND RESPONSE | AIOT COMMAND FAILURE |
| Reader Information Exchange | READER INFORMATION REQUEST | READER INFORMATION RESPONS | READER INFORMATION FAILURE |

Table 8.1-2: Class 2 procedures

| Elementary Procedure | Message |
| --- | --- |
| Downlink RAN Configuration Transfer | DOWNLINK RAN CONFIGURATION TRANSFER |
| Downlink RAN Status Transfer | DOWNLINK RAN STATUS TRANSFER |
| Downlink NAS Transport | DOWNLINK NAS TRANSPORT |
| Error Indication | ERROR INDICATION |
| Uplink RAN Configuration Transfer | UPLINK RAN CONFIGURATION TRANSFER |
| Uplink RAN Status Transfer | UPLINK RAN STATUS TRANSFER |
| Handover Notification | HANDOVER NOTIFY |
| Initial UE Message | INITIAL UE MESSAGE |
| NAS Non Delivery Indication | NAS NON DELIVERY INDICATION |
| Paging | PAGING |
| PDU Session Resource Notify | PDU SESSION RESOURCE NOTIFY |
| Reroute NAS Request | REROUTE NAS REQUEST |
| UE Context Release Request | UE CONTEXT RELEASE REQUEST |
| Uplink NAS Transport | UPLINK NAS TRANSPORT |
| AMF Status Indication | AMF STATUS INDICATION |
| PWS Restart Indication | PWS RESTART INDICATION |
| PWS Failure Indication | PWS FAILURE INDICATION |
| Downlink UE Associated NRPPa Transport | DOWNLINK UE ASSOCIATED NRPPA TRANSPORT |
| Uplink UE Associated NRPPa Transport | UPLINK UE ASSOCIATED NRPPA TRANSPORT |
| Downlink Non UE Associated NRPPa Transport | DOWNLINK NON UE ASSOCIATED NRPPA TRANSPORT |
| Uplink Non UE Associated NRPPa Transport | UPLINK NON UE ASSOCIATED NRPPA TRANSPORT |
| Trace Start | TRACE START |
| Trace Failure Indication | TRACE FAILURE INDICATION |
| Deactivate Trace | DEACTIVATE TRACE |
| Cell Traffic Trace | CELL TRAFFIC TRACE |
| Location Reporting Control | LOCATION REPORTING CONTROL |
| Location Reporting Failure Indication | LOCATION REPORTING FAILURE INDICATION |
| Location Report | LOCATION REPORT |
| UE TNLA Binding Release | UE TNLA BINDING RELEASE REQUEST |
| UE Radio Capability Info Indication | UE RADIO CAPABILITY INFO INDICATION |
| RRC Inactive Transition Report | RRC INACTIVE TRANSITION REPORT |
| Overload Start | OVERLOAD START |
| Overload Stop | OVERLOAD STOP |
| Secondary RAT Data Usage Report | SECONDARY RAT DATA USAGE REPORT |
| Uplink RIM Information Transfer | UPLINK RIM INFORMATION TRANSFER |
| Downlink RIM Information Transfer | DOWNLINK RIM INFORMATION TRANSFER |
| Retrieve UE Information | RETRIEVE UE INFORMATION |
| UE Information Transfer | UE INFORMATION TRANSFER |
| RAN CP Relocation Indication | RAN CP RELOCATION INDICATION |
| Connection Establishment Indication | CONNECTION ESTABLISHMENT INDICATION |
| AMF CP Relocation Indication | AMF CP RELOCATION INDICATION |
| Handover Success | HANDOVER SUCCESS |
| Uplink RAN Early Status Transfer | UPLINK RAN EARLY STATUS TRANSFER |
| Downlink RAN Early Status Transfer | DOWNLINK RAN EARLY STATUS TRANSFER |
| Multicast Group Paging | MULTICAST GROUP PAGING |
| Broadcast Session Release Required | BROADCAST SESSION RELEASE REQUIRED |
| Timing Synchronisation Status Report | TIMING SYNCHRONISATION STATUS REPORT |
| RAN Paging Request | RAN PAGING REQUEST |
| AIoT Inventory Report | AIOT INVENTORY REPORT |

<<<<<<<<<<<<<<<<<<<<<<<<<<<<<<<<<<<<<<<Next change>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>

### 8.7.x	AIoT NG Setup

#### 8.7.x.1	General

The purpose of the AIoT NG Setup procedure is to exchange application level data needed for the NG-RAN node and the AIoTF to correctly interoperate on the NG-C interface. This procedure shall be the first NGAP procedure triggered after the TNL association has become operational. The procedure uses non-UE associated signalling. This procedure applies only if the NG-RAN node is a gNB.

#### 8.7.x.2	Successful Operation

Figure 8.7.1.2-1: AIoT NG setup: successful operation

The AIoTF initiates the procedure by sending an AIOT NG SETUP REQUEST message including the appropriate data to the NG-RAN node. The NG-RAN node responds with an AIOT NG SETUP RESPONSE message including the appropriate data.

#### 8.7.x.3	Unsuccessful Operation

Figure 8.7.1.3-1: AIoTF NG setup: unsuccessful operation

If the NG-RAN node cannot accept the setup, it should respond with an AIOT NG SETUP FAILURE message and appropriate cause value.

#### 8.7.x.4	Abnormal Conditions

Void.

<<<<<<<<<<<<<<<<<<<<<<<<<<<<<<<<<<<<<<<Next change>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>

## 8.x	AIoT Procedures

### 8.x.1	AIoT Inventory

#### 8.x.1.1	General

The purpose of the AIoT inventory procedure is to perform inventory over NG-RAN node. The purpose of the AIoT Inventory procedure is to request the NG-RAN node to allocate AIoT radio resources for a AIoT inventory operation. The procedure uses non-UE associated signalling. This procedure applies only if the NG-RAN node is a gNB.

#### 8.x.1.2	Successful Operation

Figure 8.x.1.2-1: AIoT inventory: successful operation

Either the AMF or AIoTF initiates the procedure by sending an AIOTF INVENTORY REQUEST message to the NG-RAN node, the NG-RAN node allocates and coordinates the usage of A-IoT radio resources and sends an AIOTF INVENTORY RESPONSE message to the AMF or AIoTF.

Either notes: the detail procedure text depends on the IEs which needs to be further discussed.

#### 8.x.1.3	Unsuccessful Operation

Figure 8.x.1.3-1: AIoT inventory: unsuccessful operation

If the NG-RAN node is not able to perform AIoT inventory operation, it shall consider the procedure as failed and reply with the AIOT INVENTORY FAILURE message.

<<<<<<<<<<<<<<<<<<<<<<<<<<<<<<<<<<<<<<<Next change>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>

### 8.x.2	AIoT Inventory Report

#### 8.x.2.1	General

The purpose of the AIoT Inventory Report procedure is to provide the inventory results to the AMF or AIoTF. The procedure uses non-UE associated signalling. This procedure applies only if the NG-RAN node is a gNB.

#### 8.x.2.2	Successful Operation

Figure 8.x.2.2-1: AIoT Inventory Report

The NG-RAN node initiates the procedure by sending an AIOT INVENTORY REPORT message to the AMF or AIoTF.

#### 8.x.2.3	Abnormal Conditions

Void.

<<<<<<<<<<<<<<<<<<<<<<<<<<<<<<<<<<<<<<<Next change>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>

### 8.x.3	AIoT Command

#### 8.x.3.1	General

The purpose of the AIoT Command procedure is to request the NG-RAN node to allocate AIoTF radio resources for a AIoT command operation. The procedure uses non-UE associated signalling. This procedure applies only if the NG-RAN node is a gNB.

#### 8.x.3.2	Successful Operation

Figure 8.x.3.2-1: AIoT Command: successful operation

Either AMF or AIoTF initiates the procedure by sending a AIOT COMMAND REQUEST message to the NG-RAN node, the NG-RAN node allocates and coordinates the usage of A-IoT radio resources and sends an AIOTF COMMAND RESPONSE message to the AMF or A-IoTF.

Either notes: the detail procedure text depends on the IEs which needs to be further discussed.

#### 8.x.1.3	Unsuccessful Operation

Figure 8.x.1.3-1: AIoT Command: unsuccessful operation

If the NG-RAN node is not able to perform AIoT command operation, it shall consider the procedure as failed and reply with the AIOT COMMAND FAILURE message.

<<<<<<<<<<<<<<<<<<<<<<<<<<<<<<<<<<<<<<<Next change>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>

### 8.x.4	Reader Information Exchange

#### 8.x.4.1	General

The purpose of the Reader Information Exchange procedure is to allow the AMF or AIoTF to request the NG-RAN node to provide information for readers hosted by the NG-RAN node. This procedure applies only if the NG-RAN node is a gNB.

#### 8.x.4.2	Successful Operation

Figure 8.x.3.2-1: Reader Information Exchange: successful operation

The AMF or AIoTF initiates the procedure by sending a READER INFORMATION REQUEST message. The NG-RAN node responds with a READER INFORMATION RESPONSE message.

Either notes: the detail procedure text depends on the IEs which needs to be further discussed.

#### 8.x.4.3	Unsuccessful Operation

Figure 8.x.1.3-1: Reader Information Exchange: unsuccessful operation

If the NG-RAN node cannot provide any of the requested information for any Reader, it shall consider the procedure as failed and reply with the READER INFORMATION FAILURE message.

<<<<<<<<<<<<<<<<<<<<<<<<<<<<<<<<<<<<<< Change ends>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>