3GPP TSG-RAN WG3 #127	R3-250161

Athens, Greece, 17th – 21st February 2025

Agenda item:		13.2 Support of inter-CU LTM

Title:		discussion Rel-19 inter-CU LTM in DC scenario

Source:		NEC

Document for:		Discussion and Decision

# 1. Introduction

The Rel-19 Inter-CU LTM WID (RP-242356) its objective has the description:

- Specify support for inter-CU Layer1/Layer 2 Triggered Mobility (LTM) [RAN2, RAN3] - Prioritize the case when CU is acting as MN when DC is not configured - When DC is configured, inter-CU LTM can be configured either in MN or in SN but not both at the same time. For such cases: - As secondary priority, support the case where CU is acting as SN and MN is unchanged - As secondary priority, support the case where CU is acting as MN and SN is unchanged or SN is released - Specify support for subsequent LTM mobility procedures aiming to avoid RRC configuration between cell switches as per Rel-18 LTM - Coordination with SA3 needed with respect to security key handling - Note: Rel. 18 intra-CU LTM procedure is considered as baseline for adding inter-CU support

In last RAN3#125bis/126 meeting, the moderator summary the contributions, but was not discussed due to lack to time. In this contribution we discuss some issues related with the inter-CU LTM when DC is configured.

2. Discussion

According to the WID [1],

o	When DC is configured, inter-CU LTM can be configured either in MN or in SN but not both at the same time. For such cases:

 As secondary priority, support the case where CU is acting as SN and MN is unchanged

 As secondary priority, support the case where CU is acting as MN and SN is unchanged or SN is released

We call it here:

 Scenario 1: support the case where CU is acting as SN and MN is unchanged (can be understood as Inter-CU SCG LTM with MCG is unchanged or in short, SN initiated inter-CU SCG LTM)

 Scenario 2: support the case where CU is acting as MN and SN is unchanged or SN is released (can be understood as inter-CU MCG LTM with SCG unchanged or SCG release)

2.0	Prioritization of the work

The Rel-19 Inter-CU LTM prioritizes the case when CU is acting as MN when DC is not configured. The DC cases in the Rel-19 Inter-CU LTM have been put into the second priority as described in the WID RP-242356 [1].

Still in the first priority case i.e. the inter-CU LTM when DC is not configured, RAN3 is still having plenty of detail issues to discuss and work, additionally intra-CU condition LTM work is also started. Considering to have better quality of the specifications, it is still in the situation that it would be good to prioritize the work for Rel-19 inter-CU LTM when with DC is not configured.

Observation 1: it is still in the same situation to prioritize the work on the Rel-19 Inter-CU LTM when DC is not configured.

Nevertheless, for the inter-CU LTM when DC is configured, is discussed here.

The “Scenario 1: support the case where CU is acting as SN and MN is unchanged” may be similar with the S-CPAC and mostly the procedures for S-CPAC can be reused, with only need to add the LTM specific Early Sync procedure during the whole inter-CU SCG LTM.

The “Scenario 2: support the case where CU is acting as MN and SN is unchanged or SN is released” is like a CHO with SN, with the difference that no SN addition/change is considered for this Rel-19 inter-CU MCG LTM with SCG unchanged or SCG release. Most signalling procedures for the CHO with SN can be reused, but RAN3 need to first finalize the Inter-CU LTM as this will be the base.

Proposal 1: if to work on Inter-CU LTM when DC is configured, consider lower priority for “Scenario 2: support the case where CU is acting as MN and SN is unchanged or SN is released (can be understood as inter-CU MCG LTM with SCG unchanged or SCG release)”.

2.1	inter-CU LTM is not configured in both MCG and SCG at the same time

RAN3 has confirmed that ““Confirm the case that inter-CU LTM is not configured in both MCG and SCG at the same time”.”

To avoid both MCG and SCG are configured at the same time for the Inter-CU LTM, the MN and SN need to coordinate before initiating the inter-CU MCG LTM and inter-CU-SCG LTM respectively.

One principle to draw is that, which node will be the canter control of such situation. It has been more or less understood that the MN is the node who will control for the dual connectivity configure to the UE, e.g. whether to add SCG for the UE or not. Therefore to have MN has the centre control, it would natural to do like:

- In case of NR-DC, about inter-CU MCG LTM, MN indicate/Notify to the SN,

- In case of NR-DC, about inter-CU SCG LTM, SN send a request to MN which may be accepted or rejected by the MN.

Proposal 2: to avoid both MCG and SCG are configured at the same time for the inter-CU MCG LTM or Inter-CU SCG LTM, the MN is the centre control / decision so the SN need to request to the MN if can initiating inter-CU SCG LTM.

2.2	Discussion on Scenario 1: support the case where CU is acting as SN and MN is unchanged (can be understood as Inter-CU SCG LTM with MCG is unchanged or SN initiated inter-CU SCG LTM)

This “Scenario 1: support the case where CU is acting as SN and MN is unchanged (can be understood as Inter-CU SCG LTM with MCG is unchanged of in short SN initiated inter-CU LTM)” can reuse the existing SN initiated conditional SN Change whole procedures as the base, with additional LTM specific procedure.

Preparation phase: RAN2 [R3-247111 LS from RAN2] agreed that “Only SN-initiated inter-SN LTM (including LTM configuration, early DL/UL synch and LTM execution) is supported in Rel-19.”, can reuse the existing SN initiated conditional SN Change whole procedures as the base.

Proposal 3: for “Scenario 1: support the case where CU is acting as SN and MN is unchanged (can be understood as Inter-CU SCG LTM with MCG is unchanged or in short, SN initiated inter-CU SCG LTM)”, can reuse the existing SN initiated conditional SN Change whole procedures as the base, with additional LTM specific procedure.

Early Sync Phase: this will need to have a new Xn procedure, but can use the LTM Configuration Update procedure and TA Information Transfer procedure that will be newly introduced for Rel-19 Inter-CU LTM.

Proposal 4-1: For the early sync, in the Xn procedure, use the LTM Configuration Update procedure and TA Information Transfer procedure that will be newly introduced for Rel-19 Inter-CU LTM

Execution Phase: RAN2 [R3-247111 LS from RAN2] agreed that “For inter-CU SCG LTM, the LTM cell switch command MAC CE is sent by source SN”. Then source SN (source DU) need to send Cell Switch Notification to the MN (CU), then the MN send Cell Switch Notification to the target SN (target CU then to target DU).

Proposal 4-2: The source SN (source DU) need to send Cell Switch Notification to the MN (CU), then the MN send Cell Switch Notification to the target SN (target CU then to target DU).

Completion phase: RAN2 [R3-247111 LS from RAN2] agreed: “Upon execution of inter-SN SCG LTM, the UE sends an MN RRCReconfigurationComplete message to the MN, which includes an SN RRCReconfigurationComplete message.”, then MN need to consider to keep the S-SN for subsequent SCG LTM which means the UE context release procedure towards S-SN is optional.

Proposal 4-3: After the completion of the inter-SN SCG LTM, the MN need to consider to keep the S-SN for subsequent SCG LTM which means the UE context release procedure towards S-SN is optional.

3. Conclusion and proposals

Observation 1: it is still in the same situation to prioritize the work on the Rel-19 Inter-CU LTM when DC is not configured.

Proposal 1: if to work on Inter-CU LTM when DC is configured, consider lower priority for “Scenario 2: support the case where CU is acting as MN and SN is unchanged or SN is released (can be understood as inter-CU MCG LTM with SCG unchanged or SCG release)”.

Proposal 2: to avoid both MCG and SCG are configured at the same time for the inter-CU MCG LTM or Inter-CU SCG LTM, the MN is the centre control / decision so the SN need to request to the MN if can initiating inter-CU SCG LTM.

Proposal 3: for “Scenario 1: support the case where CU is acting as SN and MN is unchanged (can be understood as Inter-CU SCG LTM with MCG is unchanged)”, can reuse the existing SN initiated conditional SN Change whole procedures as the base, with additional LTM specific procedure.

Proposal 4-1: For the early sync, in the Xn procedure, use the LTM Configuration Update procedure and TA Information Transfer procedure that will be newly introduced for Rel-19 Inter-CU LTM

Proposal 4-2: The source SN (source DU) need to send Cell Switch Notification to the MN (CU), then the MN send Cell Switch Notification to the target SN (target CU then to target DU).

Proposal 4-3: After the completion of the inter-SN SCG LTM, the MN need to consider to keep the S-SN for subsequent SCG LTM which means the UE context release procedure towards S-SN is optional.

# References

[1] RP-242356 Revised WID: NR mobility enhancements Phase 4 (WID [NR_Mob_Ph4-Core])

[2] R3-247824, Summary of offline discussion on MobilityEnh

[3] R3-250008== R2-2411134, RAN2 agreements on Inter-CU LTM and Conditional LTM

[4] R3-250050 == R3-247932 (BL CR to 38.300) Inter-CU LTM

[5] R3-250051 == R3-247933 (BL CR to 38.423) Xn Support for Inter-CU LTM

[6] R3-250048 == R3-247590 (BL CR to 37.483) Introducing Rel-19 Mobility Enhancement

[7] R3-247111 LS on RAN2 agreements for inter-CU LTM (From RAN2 to RAN3)

[8] R3-250049 == R3-247514 (BL CR to 38.470) Introducing Rel-19 Mobility Enhancement

Annex: only for reference:

TS37.340v18.3.0

10.5.2	MR-DC with 5GC

SN initiated conditional SN Change

Figure 10.5.2-4: Conditional SN change procedure - SN initiated

10.19.2	MR-DC with 5GC

Conditional Handover with Secondary Node procedure

Figure 10.19.2-1: Conditional Handover with Secondary Node procedure

10.20	Subsequent Conditional PSCell Addition or Change

SN initiated subsequent CPAC

The subsequent CPAC procedure is initiated by the SN for subsequent CPAC configuration and subsequent CPAC execution.

Figure 10.20-2: Subsequent CPAC - SN initiated