3GPP TSG-RAN WG3 #127	R3-250162

Greece, Athens, 17th – 21st February 2025

Agenda item:		13.3 Support for Conditional LTM

Title:		Discussion of Rel-19 intra-CU Conditional LTM

Source:		NEC

Document for:		Discussion and Decision

# 1. Introduction

For the Rel-19 Inter-CU LTM, we continue to discuss and proposal to progress the project, this contribution especially discuss for the intra-CU conditional LTM.

13. NR Mobility Enhancements WI WID [NR_Mob_Ph4-Core]: RP-242356 (target: RAN #109) [TU: 1 ( 1, 1, 1, 0.5)] QUOTA: 3

13.3. Support for Conditional LTM Specify support of conditional Intra-CU LTM [RAN2, RAN3, RAN1] - Specify UE evaluated conditions for triggering LTM - Aim to support conditional LTM including subsequent LTM - Limit specifying the conditional LTM to the scenario where the UE is in non-DC - Checkpoint at RAN#107 to review the objective on whether Intra-CU conditional LTM can be specified to DC scenarios and if so, to which cases. RAN WG work to not start before this checkpoint

2. Discussion

2.1	Support for Conditional LTM

This meeting RAN3 received a RAN2 LS [R3-250008] shows the progress and agreements on C-LTM in RAN2.

Agreements on C-LTM: 1. The triggering condition of conditional LTM can be based on L3 measurement. 2. CondEventA3 and CondEventA5 conditions can be baseline for the conditional LTM execution. 3. The L1 execution condition of a candidate cell is associated to only one triggering event. 4. For L3 execution condition, it may consist of one or two triggering condition(s). If there are two triggering conditions associated with the same candidate cell, the UE shall consider the execution condition is fulfilled only when both triggering conditions are met. Only single RS type is supported and at most two different trigger quantities can be configured simultaneously for the evaluation of execution condition of a single candidate cell. 5. To support initial and subsequent conditional LTM, the following items can be considered for the configuration of execution condition: - The CLTM configuration of each candidate cell shall include the execution condition for initial conditional LTM, which is generated by the initial source cell to trigger the CLTM for the candidate cell. - The CLTM configuration of each candidate cell may include execution conditions for subsequent conditional LTM, which is generated by the candidate cell to trigger the CLTM for other candidate cells when the candidate cell becomes a serving cell. 6. The network can configure measurement reports e.g., L1 periodic, semi-persistent, aperiodic and event triggered report, or L3 measurement reports for conditional LTM, e.g., to trigger PDCCH ordered early RACH. 7. For CLTM, the Candidate Cell TCI States Activation/Deactivation MAC CE is re-used for the early activation/deactivation of TCI state(s) of a CLTM candidate configuration. 8. The Early TA is signalled to the UE from the source cell (i.e., not from the candidate cell directly to the UE). This agreement will be included in the LS to RAN1/3/4. 9. The network can inform the candidate cell’s TA information to UE via new MAC CE, which is the TA value when UE switches to that candidate cell during CLTM. 10. Candidate cell TA is maintained by a new timer. 11. For L1-based conditional LTM the condition evaluation is at MAC level and for L3-based conditional LTM the condition evaluation is at RRC level.

- L1 execution condition:

For L1 based condition LTM, L1 execution condition is to be generated by the DU, the CU will need to ask the DU to generated L1 execution condition.

RAN2 agreed that:

- The CLTM configuration of each candidate cell shall include the execution condition for initial conditional LTM, which is generated by the initial source cell to trigger the CLTM for the candidate cell. - The CLTM configuration of each candidate cell may include execution conditions for subsequent conditional LTM, which is generated by the candidate cell to trigger the CLTM for other candidate cells when the candidate cell becomes a serving cell.

So the L1 execution condition for the initial LTM execution is to be provided by the source DU, the CU need to ask the source DU to general L1 execution condition by the UE CONTEXT MODIFICATION REQUEST message, then the source DU generate the L1 execution condition and send it to the CU by UE CONTEXT MODIFICATION RESPONSE message.

Proposal 1: In F1 interface add an L1 Execution Condition Request IE is added in the UE CONTEXT MODIFICATION REQUEST message for the CU to ask source DU to provide L1 execution condition information.

Proposal 2: In F1 interface an L1 Execution Condition Information IE is added in the UE CONTEXT MODIFICATION RESPONSE message for the source DU to provide L1 execution condition information to the CU.

For the subsequent conditional LTM, the L1 execution condition is generated by the candidate cell for other candidate cells when the candidate cell will become a serving cell. Therefore, similarly that the CU need to ask the candidate DU to general L1 execution condition by the UE CONTEXT SETUP REQUEST message, then the candidate DU generate the L1 execution condition and send it to the CU by UE CONTEXT SEUTP RESPONSE message.

Proposal 3: In F1 interface an L1 Execution Condition Request IE is added in the UE CONTEXT SETUP REQUEST message for the CU to request candidate DU to provide L1 execution condition information.

Proposal 4: In F1 interface an L1 Execution Condition Information IE is added in the UE CONTEXT SETUP RESPONSE message for the candidate DUs to provide L1 execution condition information to the CU.

The content of the L1 Execution Condition Information provided by the DU, following what RAN2 has defined, is to be the LTM-CSI-ReportConfigId-r18 corresponds to the LTM-CSI-ReportConfig-r18, which will then need CU to set in part of the newly defined CLTM-ExecutionConditions-r19 in the RRCReconfiguration message to be sent to the UE.

From the RRC running CR after RAN2#128 (Nov 2024):

CLTM-ExecutionConditions-r19 ::= SEQUENCE (SIZE (1..maxNrofLTM-Configs-r18)) OF ExecutionConditions-r19 ExecutionConditions-r19 ::= SEQUENCE { ltm-CandidateId-r19 LTM-CandidateId-r18, executionCondition-r19 CHOICE { l1-Conditions-r19 LTM-CSI-ReportConfigId-r18, l3-Conditions-r19 SEQUENCE (SIZE (1..2)) OF MeasId } OPTIONAL, -- Need R

So the content of the L1 Execution Condition Information IE in F1AP, follow the RAN2 definition, has a list of cell and will be each ID (LTM-CSI-ReportConfigId-r18) correspond to each cell. This is proposed in the following chapter 2.3 Possible Stage 3 change for support for Conditional LTM.

Proposal 5: In F1 interface the L1 Execution Condition Information provided by the DU to CU, following the RAN2 agreement, has a list of cell and is to be each ID (LTM-CSI-ReportConfigId-r18) correspond to each cell.

- TA Information related

RAN2 LS said “the Early TA is signalled to the UE from the source cell, the network can inform the candidate cell’s TA information to UE via new MAC CE, which is the TA value when UE switches to that candidate cell during CLTM.”   So the TA information is sent from candidate DU to CU then from CU to source DU, this can be done by the existing DU-CU/CU-DU TA Information Transfer procedure.

Proposal 6: In F1 interface for the conditional LTM, the TA Information transferring can be done by the existing DU-CU/CU-DU TA Information Transfer procedure.

2.2	Possible Stage 2 change for support for Conditional LTM

Possible the stage 2 change:

Inter DU Conditional LTM, based on 38.401 figure 8.2.1.5-1: Inter-gNB-DU LTM

1.	The UE sends a MeasurementReport message (L3 measurement result) to the source gNB-DU containing measurements of neighbouring cells. The source gNB-DU sends an UL RRC MESSAGE TRANSFER message conveying the received MeasurementReport message to the gNB-CU.

2.	The gNB-CU determines to initiate LTM configuration.

3.	The gNB-CU sends a UE CONTEXT SETUP REQUEST message to the candidate gNB-DU(s) for each candidate cell, containing one candidate cell ID and the CSI resource configuration for subsequent LTM. The gNB-CU may provide the LTM configuration ID mapping list to the candidate gNB-DU(s). The gNB-CU may request PRACH resources from the candidate gNB-DU(s). The gNB-CU may request the candidate gNB-DU(s) to provide the lower layer configuration for the purpose of generating the reference configuration or provide the lower layer part of the reference configuration to the candidate gNB-DU(s). The gNB-CU may request the candidate gNB-DU(s) to provide the L1 execution condition for the purpose of Condition LTM.

4.	If the candidate gNB-DU accepts the request of LTM configuration, it responds with a UE CONTEXT SETUP RESPONSE message including the generated lower layer RRC configurations for the accepted target candidate cell. The candidate gNB-DU may include the L1 execution condition information.

NOTE 1:	The CU-initiated UE Context Modification procedure may be initiated for preparing candidate cells in the source gNB-DU as specified in step 3 and 4 in 8.2.1.4 Intra-gNB-DU LTM.

5.	The gNB-CU sends a UE CONTEXT MODIFICATION REQUEST message to the source gNB-DU including the information related to early sync and the LTM configuration ID mapping list for the accepted target candidate cell(s). The gNB-CU may send the updated CSI resource configuration to the source gNB-DU. The gNB-CU may inform the source gNB-DU about intra-DU L2 reset configuration. The gNB-CU may request the source gNB-DU(s) to provide the L1 execution condition for the purpose of Condition LTM.

6.	The source gNB-DU responds with a UE CONTEXT MODIFICATION RESPONSE message which includes an updated lower layer configuration, e.g., containing the updated CSI report configuration of the source cell. The source gNB-DU may include the L1 execution condition information.

7.	The gNB-CU may send a UE CONTEXT MODIFICATION REQUEST message for each candidate cell accepted in the candidate gNB-DU(s), containing the information for subsequent LTM or for updating the configurations of candidate cells. The gNB-CU may also provide the lower layer part of the reference configuration to the candidate gNB-DU(s). The gNB-CU may inform the candidate gNB-DU(s) about intra-DU L2 reset configuration.

8.	The candidate gNB-DU responds with a UE CONTEXT MODIFICATION RESPONSE message including the updated lower layer configuration, e.g., containing the updated CSI report configuration of the requested candidate cell.

NOTE 2: Step 7 may also be triggered after step 19, or after step 22 by implementation for subsequent LTM.

9.	The gNB-CU sends a DL RRC MESSAGE TRANSFER message to the source gNB-DU, which includes the generated RRCReconfiguration message with the LTM configuration. (the RRCReconfiguration message may include the L1 execution condition information)

10.	The source gNB-DU forwards the received RRCReconfiguration message to the UE.

11.	The UE responds to the source gNB-DU with an RRCReconfigurationComplete message.

12.	The source gNB-DU forwards the RRCReconfigurationComplete message to the gNB-CU via an UL RRC MESSAGE TRANSFER message.

13.	Early TA acquisition to the candidate cell(s) may be performed as specified in TS 38.300 [2].

14.The candidate gNB-DU sends a DU-CU TA INFORMATION TRANSFER message to the gNB-CU, which includes the TA values, and the associated PRACH resource information.

15.	The gNB-CU forwards the TA value, and the associated PRACH resource information to the source gNB-DU in the CU-DU TA INFORMATION TRANSFER message.

16.	The source gNB-DU sends to the UE the TA values in the MAC-CE.

17.	The UE evaluate the LTM execution condition, if the execution condition(s) are fulfilled, the UE access to the target cell

21.	The target gNB-DU detects the UE access as specified in TS 38.300 [2].

22.	The target gNB-DU sends the ACCESS SUCCESS message to the gNB-CU with the target cell ID.

23.	The UE sends an RRCReconfigurationComplete message to the target gNB-DU.

24.	The target gNB-DU forwards the RRCReconfigurationComplete message to the gNB-CU via an UL RRC MESSAGE TRANSFER message.

25.	The gNB-CU may send the UE CONTEXT RELEASE COMMAND message to the source gNB-DU to release the resources of prepared cells.

26.	The source gNB-DU responds with a UE CONTEXT RELEASE COMPLETE message.

2.3	Possible Stage 3 (F1AP change for support for Conditional LTM

The following LTM related IEs for Rel-18 intra-CU LTM in F1AP UE CONTEXT SETUP REQUEST message:

| LTM InformationSetup |  | 0..1 |  |  | YES | reject |
| --- | --- | --- | --- | --- | --- | --- |
| >LTM Indicator | M |  | ENUMERATED (true, …) |  | - |  |
| >Reference Configuration | O |  | 9.3.1.292 |  | - |  |
| >CSI Resource Configuration | O |  | 9.3.1.330 |  | - |  |
| >L1 Execution Condition Request | O |  | 9.3.1.xx1 |  | - |  |
| LTM Configuration ID Mapping List | O |  | 9.3.1.294 |  | YES | reject |
| Early Sync Information Request |  | 0..1 |  |  | YES | ignore |
| >Request for RACH Configuration | M |  | ENUMERATED (true, …) |  | - |  |
| >LTM gNB-DUs List |  | 1 |  | This IE contains the IDs of the source gNB-DU and candidate gNB-DU(s). | YES | reject |
| >>LTM gNB-DUs Item IEs |  | 1..< maxnoofLTMgNBDUs> |  |  | - |  |
| >>>LTM gNB-DU ID | M |  | gNB-DU ID 9.3.1.9 |  | - |  |

The following LTM related IEs for Rel-18 intra-CU LTM in F1AP UE CONTEXT SETUP RESPONSE message:

| Early Sync Information |  | 0..1 |  |  | YES | ignore |
| --- | --- | --- | --- | --- | --- | --- |
| >TCI States Configurations List | M |  | OCTET STRING | Includes the LTM-TCI-Info IE, as defined in TS 38.331 [8]. | - |  |
| >Early UL Sync Configuration | O |  | 9.3.1.328 |  | - |  |
| >Early UL Sync Configuration for SUL | O |  | Early UL Sync Configuration 9.3.1.328 | This IE applies for SUL carrier. | - |  |
| LTM Configuration |  | 0..1 |  |  | YES | ignore |
| >SSB Information | M |  | 9.3.1.202 | Includes the SSB Information for the requested target cell. | - |  |
| >Reference Configuration Information | O |  | OCTET STRING | Includes the CellGroupConfig IE, as defined in TS 38.331 [8]. | - |  |
| >Complete Configuration Indicator | O |  | ENUMERATED (complete, ...) |  | - |  |
| >LTM CFRA Resource Configuration | O |  | OCTET STRING | Includes the RACH-ConfigDedicated IE, as defined in TS 38.331 [8]. | - |  |
| >LTM CFRA Resource Configuration for SUL | O |  | OCTET STRING | Includes the RACH-ConfigDedicated IE, as defined in TS 38.331 [8]. This IE applies for SUL carrier. | - |  |
| >L1 Execution Condition Information | O |  | 9.3.1.xx2 |  | - |  |

The following LTM related IEs for Rel-18 intra-CU LTM in F1AP UE CONTEXT MODIFICATION REQUEST message:

| LTM Information Modify |  | 0..1 |  |  | YES | reject |
| --- | --- | --- | --- | --- | --- | --- |
| >LTM Indicator | M |  | ENUMERATED (true, …) |  | - |  |
| >Reference Configuration | O |  | 9.3.1.292 |  | - |  |
| >CSI Resource Configuration | O |  | 9.3.1.330 |  | - |  |
| >L1 Execution Condition Request | O |  | 9.3.1.xx1 |  | - |  |
| LTM CFRA Resource Config List |  | 0..1 |  |  | YES | ignore |
| >LTM CFRA Resource Config Item IEs |  | 1 .. <maxnoofLTMCells> |  |  | EACH | ignore |
| >>Cell ID | M |  | NR CGI 9.3.1.12 |  | - |  |
| >>LTM CFRA Resource Configuration | O |  | OCTET STRING | Includes the RACH-ConfigDedicated IE, as defined in TS 38.331 [8]. | - |  |
| >>LTM CFRA Resource Configuration for SUL | O |  | OCTET STRING | Includes the RACH-ConfigDedicated IE, as defined in TS 38.331 [8]. This IE applies for SUL carrier. | - |  |
| LTM Configuration ID Mapping List | O |  | 9.3.1.294 |  | YES | reject |
| Early Sync Information Request |  | 0..1 |  |  | YES | ignore |
| >Request for RACH Configuration | M |  | ENUMERATED (true, …) |  | - |  |
| >LTM gNB-DUs ID List |  | 1 |  | This IE contains the IDs of the source gNB-DU and candidate gNB-DU(s). | YES | reject |
| >>LTM gNB-DUs Item IEs |  | 1..< maxnoofLTMgNBDUs> |  |  | - |  |
| >>>LTM gNB-DU ID | M |  | gNB-DU ID 9.3.1.9 |  | - |  |
| Early Sync Candidate Cell Information List |  | 0..1 |  |  | YES | ignore |
| >Early Sync Candidate Cell Information Item IEs |  | 1 .. <maxnoofLTMCells> |  |  | EACH | ignore |
| >>Cell ID | M |  | NR CGI 9.3.1.12 |  | - |  |
| >>TCI States Configurations List | O |  | OCTET STRING | Includes the LTM-TCI-Info IE, as defined in TS 38.331 [8]. | - |  |
| >>Early UL Sync Configuration | O |  | 9.3.1.328 |  | - |  |
| >>Early UL Sync Configuration for SUL | O |  | Early UL Sync Configuration 9.3.1.328 | This IE applies for SUL carrier. | - |  |
| >>TA Assistance Information | O |  | ENUMERATED (zero, …) | The value "zero" corresponds to TA value of the cell being equal to zero. | - |  |
| >>UE Based TA Measurement Configuration | O |  | OCTET STRING | Includes the ltm-UE-MeasuredTA-ID contained in the LTM-Candidate IE, as defined in TS 38.331 [8], for the LTM candidate cell identified by the Cell ID IE. | - |  |
| Early Sync Serving Cell Information |  | 0..1 |  |  | YES | ignore |
| >UE Based TA Measurement Configuration | O |  | OCTET STRING | Includes the ltm-ServingCellUE-MeasuredTA-ID contained in the LTM-Config IE, as defined in TS 38.331 [8], for the current serving cell. | - |  |
| LTM Cells To Be Released List | O |  | 9.3.1.291 |  | YES | reject |

The following LTM related IEs for Rel-18 intra-CU LTM in F1AP UE CONTEXT MODIFICATION RESPONSE message:

| Early Sync Information |  | 0..1 |  |  | YES | ignore |
| --- | --- | --- | --- | --- | --- | --- |
| >TCI States Configurations List | M |  | OCTET STRING | Includes the LTM-TCI-Info IE, as defined in TS 38.331 [8]. | - |  |
| >Early UL Sync Configuration | O |  | 9.3.1.328 |  | - |  |
| >Early UL Sync Configuration for SUL | O |  | Early UL Sync Configuration 9.3.1.328 | This IE applies for SUL carrier. | - |  |
| LTM Configuration |  | 0..1 |  |  | YES | ignore |
| >SSB Information | M |  | 9.3.1.202 | Includes the SSB Information for the requested target cell | - |  |
| >Reference Configuration Information | O |  | OCTET STRING | Includes the CellGroupConfig IE, as defined in TS 38.331 [8]. | - |  |
| >Complete Candidate Configuration Indicator | O |  | ENUMERATED (complete, ...) |  | - |  |
| >LTM CFRA Resource Configuration | O |  | OCTET STRING | Includes the RACH-ConfigDedicated IE, as defined in TS 38.331 [8]. | - |  |
| >LTM CFRA Resource Configuration for SUL | O |  | OCTET STRING | Includes the RACH-ConfigDedicated IE, as defined in TS 38.331 [8]. This IE applies for SUL carrier. | - |  |
| >L1 Execution Condition Information | O |  | 9.3.1.xx2 |  | - |  |

9.3.1.xx1	L1 Execution Condition Request

This IE is used to indicate the request from the CU to the DU to generate the L1 Execution condition .

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| L1 Execution Condition Request | M |  | ENUMERATED (True, ...) |  |

9.3.1.xx2	L1 Execution Condition Information

This IE indicates the L1 execution condition information.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| L1-Execution-Condition-Information Item IEs |  | 1..< maxnoofLTMCells> |  |  | - |  |
| >LTM-CSI-Report-Config-ID | M |  | OCTET STRING | Includes the LTM-CSI-ReportConfigId-r18 IE as defined in subclause 6.3.2 in TS 38.331 [8]. | - |  |

| Range bound | Explanation |
| --- | --- |
| maxnoofLTMCells | Maximum no. of Cells configured LTM allowed towards one UE, the maximum value is 8. |

3. Conclusion and proposals

Proposal 1: In F1 interface add an L1 Execution Condition Request IE is added in the UE CONTEXT MODIFICATION REQUEST message for the CU to ask source DU to provide L1 execution condition information.

Proposal 2: In F1 interface an L1 Execution Condition Information IE is added in the UE CONTEXT MODIFICATION RESPONSE message for the source DU to provide L1 execution condition information to the CU.

Proposal 3: In F1 interface an L1 Execution Condition Request IE is added in the UE CONTEXT SETUP REQUEST message for the CU to request candidate DU to provide L1 execution condition information.

Proposal 4: In F1 interface an L1 Execution Condition Information IE is added in the UE CONTEXT SETUP RESPONSE message for the candidate DUs to provide L1 execution condition information to the CU.

Proposal 5: In F1 interface the L1 Execution Condition Information provided by the DU to CU, following RAN2 agreement, has a list of cell and is to be each ID (LTM-CSI-ReportConfigId-r18) correspond to each cell.

Proposal 6: In F1 interface for the conditional LTM, the TA Information transferring can be done by the existing DU-CU/CU-DU TA Information Transfer procedure.

# References

[1] RP-242356 Revised WID: NR mobility enhancements Phase 4 (WID [NR_Mob_Ph4-Core])

[2] R3-247824, Summary of offline discussion on MobilityEnh

[3] R3-250008== R2-2411134, RAN2 agreements on Inter-CU LTM and Conditional LTM

[4] R3-250050 == R3-247932 (BL CR to 38.300) Inter-CU LTM

[5] R3-250051 == R3-247933 (BL CR to 38.423) Xn Support for Inter-CU LTM

[6] R3-250048 == R3-247590 (BL CR to 37.483) Introducing Rel-19 Mobility Enhancement

[7] R3-250049 == R3-247514 (BL CR to 38.470) Introducing Rel-19 Mobility Enhancement