3GPP TSG-RAN WG3 Meeting #127	R3-250172

Athens, Greece, 17th – 21st February 2025

Agenda Item:		11.4

Source:		NEC

Title:	(TP to BLCR TS38.423) NR-DC mobility optimization

Document for:		Discussion and decision

# Introduction

For NR-DC case, RAN3 agreed to use the Data Collection Reporting Initiation procedure to configure the collection of UE performance at SN side, and a CR [1] was agreed to introduce the Data Collection ID IE into the SN Addition Request message to identify a Data Collection Reporting context.

In this contribution, we further discuss the UE performance feedback for NR-DC case.

# Discussion

For NR-DC scenario, if a Data Collection ID is included in S-NODE ADDITION REQUEST message from MN to SN, the target SN needs to collect measured UE performance after SN addition and MN-initiated SN change and reports to MN in DATA COLLECTION UPDATE message.

Regarding the granularity of UE performance metrics, there should be no difference from legacy ones. In last meeting, one LS to SA5 [2] was sent out to ask SA5 to provide feedback on whether the granularity of UE performance metrics specified in TS 28.558 is per-UE or per-DRB, so any conclusion made based on the reply LS can also be applied for this use case.

Proposal 1: UE performance feedback for NR-DC case should follow the discussion on legacy metrics.

In TS38.423, it describes the termination conditions for UE performance collection as below:

### 8.4.13	Data Collection Reporting Initiation

<<<<<<<<<<<<<<<<<<<< Unrelated Text Omitted >>>>>>>>>>>>>>>>>>>>

If the UE Performance Collection Configuration IE is present in the DATA COLLECTION REQUEST message, the NG-RAN node2 shall take it into account for the configuration of UE performance collection and reporting. NG-RAN node2 shall terminate the collection when at least one of the following conditions is fulfilled:

-	the time since UE was successfully handed over to NG-RAN node2 is equal to the value of the Collection Time Duration for UE Performance IE;

-	UE moves to RRC_INACTIVE or RRC_IDLE state;

-	UE is handed over to another cell.

However, the conditions that are applicable for NR-DC case is still missing.

Considering the agreed starting point, i.e. SN Addition and MN-initiated SN Change procedures are considered to be enhanced to trigger the collection of UE performance from SN to MN, the termination conditions after SN Addition and MN-initiated SN Change should be considered, and the following ones are suggested to be added.

-	the time since successful SN Addition procedure or MN-initiated SN Change procedure is equal to the value of the Collection Time Duration for UE Performance IE;

-	SN Change procedure or SN release procedure targeting the SN configured in the Data Collection Reporting Initiation procedure is initiated;

Proposal 2: To agree the above two termination conditions for UE performance collection in NR-DC case.

TP to TS38.423, mirroring proposal 2, is available in Annex.

Proposal 3: To agree the TP to TS38.423 in Annex.

# Proposals

In this contribution, we provided our consideration on UE performance feedback for NR-DC case and the following proposals are made.

Proposal 1: UE performance feedback for NR-DC case should follow the discussion on legacy metrics.

Proposal 2: To agree the above two termination conditions for UE performance collection in NR-DC case.

Proposal 3: To agree the TP to TS38.423 in Annex.

# Reference

[1] R3-247928, (BL CR to TS 38.423) Support of enhancements on AI/ML for NG-RAN

[2] R3-247775, LS on per-UE UE performance metrics

# Annex TP to BLCR TS38.423

<Start of change>

### 8.4.13	Data Collection Reporting Initiation

#### 8.4.13.1	General

This procedure is used by an NG-RAN node to request from another NG-RAN node the reporting of information to support, e.g., AI/ML in NG-RAN.

The procedure uses non UE-associated signalling.

#### 8.4.13.2	Successful Operation

Figure 8.4.13.2-1: Data Collection Reporting Initiation, successful operation

NG-RAN node1 initiates the procedure by sending the DATA COLLECTION REQUEST message to NG-RAN node2 to start information reporting or to stop information reporting. Upon receipt, NG-RAN node2:

-	shall initiate the requested information reporting according to the parameters given in the request in case the Registration Request for Data Collection IE is set to "start"; or

-	shall stop all measurements and predictions and terminate the reporting in case the Registration Request for Data Collection IE is set to "stop".

If the Registration Request for Data Collection IE is set to "start" in the DATA COLLECTION REQUEST message and the Report Characteristics for Data Collection IE indicates cell-specific information reporting, the Cell To Report List for Data Collection IE shall be included.

If NG-RAN node2 is capable of providing all of the requested information, it shall initiate the information reporting as requested by NG-RAN node1 and respond with the DATA COLLECTION RESPONSE message.

If NG-RAN node2 is capable of providing some but not all of the requested information, it shall initiate the information reporting for the admitted requested information and include the Node Measurement Initiation Result List IE or the Cell Measurement Initiation Result List IE or both in the DATA COLLECTION RESPONSE message.

If the Reporting Periodicity for Data Collection IE in the DATA COLLECTION REQUEST message is present, this indicates the periodicity for the reporting of configured measurement objects. The NG-RAN node2 shall report only once, unless otherwise requested within the Reporting Periodicity for Data Collection IE.

If the Requested Prediction Time IE in the DATA COLLECTION REQUEST message is present, it indicates the specific point in time to which the prediction of the requested information applies. The NG-RAN node2 shall take it into account when generating the requested predicted information.

If the UE Trajectory Collection Configuration IE is present in the DATA COLLECTION REQUEST message, the NG-RAN node2 shall take it into account for the configuration of UE trajectory collection and reporting. NG-RAN node2 shall report the UE trajectory only once. NG-RAN node2 shall terminate the collection when at least one of the following conditions is fulfilled:

-	the time since UE was successfully handed over to NG-RAN node2 is equal to the value of the Collection Time Duration for UE Trajectory IE;

-	the number of visited cells within NG-RAN node2 is equal to the value of the Number of Visited Cells IE, if included;

-	UE moves to RRC_INACTIVE or RRC_IDLE state;

-	UE is handed over to a cell belonging to an NG-RAN node different from NG-RAN node2.

The result of the UE trajectory collection is reported at the next available DATA COLLECTION UPDATE message.

If the UE Performance Collection Configuration IE is present in the DATA COLLECTION REQUEST message, the NG-RAN node2 shall take it into account for the configuration of UE performance collection and reporting. NG-RAN node2 shall terminate the collection when at least one of the following conditions is fulfilled:

-	the time since UE was successfully handed over to NG-RAN node2 is equal to the value of the Collection Time Duration for UE Performance IE;

-	UE moves to RRC_INACTIVE or RRC_IDLE state;

-	UE is handed over to another cell;

-	the time since successful SN Addition procedure or MN-initiated SN Change procedure is equal to the value of the Collection Time Duration for UE Performance IE;

-	SN Change procedure or SN release procedure targeting the SN configured in the Data Collection Reporting Initiation procedure is initiated.

The result of the UE performance collection is reported at the next available DATA COLLECTION UPDATE message.

<End of change>