3GPP TSG-RAN WG3 Meeting #127	R3-250175

Athens, Greece, 17th – 21st February 2025

Agenda Item:		17.2

Source:		NEC

Title:			Support on-demand SSB operation

Document for:		Discussion and decision

# Introduction

In previous RAN3 meetings, we had some discussions on Xn impact and F1 impact and one agreement was achieved as follows.

For RRC based signalling, the DU can decide activation/deactivation state of OD-SSB configuration per SCell and convey it to the CU included in the RRC container. There is no RAN3 impact

While the following issues are still open due to e.g. lack of relevant agreements in other WGs:

No Xn impact?

- Normal cell to request NES cell to provide on-demand SSB over Xn for layer3 measurement?

F1 impact?

- NES gNB-CU can indicate NES gNB-DU the allowed cell list for on-demand SSB?

- On-demand SSB operation is decided by CU or DU?

- The gNB-CU indicates the on-demand SSB mode to gNB-DU?

- The information provided from DU to CU, e.g., on-demand SSB state?

In this contribution, we further discuss the above issues and analyse potential RAN3 impacts.

# Discussion

## RAN1 agreements

- Case #1: No always-on SSB on the cell

- Case #2: Always-on SSB is periodically transmitted on the cell

Below are the previous agreements made in RAN1 that may influence RAN3 discussion.

Agreement

- Update the previous RAN1 agreement as follows.

  - At least support L1 measurement based on on-demand SSB

    - For L1 measurement based on on-demand SSB, periodic, semi-persistent, [and aperiodic] L1 measurement reports based on existing CSI framework are supported.

      - FFS on potential enhancements of CSI report configuration and/or triggering/activation mechanisms for L1 measurement based on on-demand SSB

      - The support of LTM is a separate discussion point

Agreement

For a cell supporting on-demand SSB SCell operation,

- Support RRC based signaling to indicate on-demand SSB transmission on the cell at least for the case where this RRC also configures the SCell, activates the SCell, and provides on-demand SSB configuration.

  - FFS: Whether to support RRC based signaling for other cases.

- Support MAC CE based signaling to indicate on-demand SSB transmission on the cell for Scenarios #2 and #2A.

Note: Deactivation and adaptation of on-demand SSB transmission can be separately discussed.

Agreement

For a cell supporting on-demand SSB SCell operation, at least for the following parameter(s), multiple candidate values can be configured by RRC and the applicable value can be indicated by MAC CE for on-demand SSB transmission indication for the cell.

- Periodicity of the on-demand SSB

- FFS: Any other relevant parameters

Agreement

For a cell supporting on-demand SSB SCell operation, at least the following is supported

- On-demand SSB on the cell is not located on synchronization raster.

- On-demand SSB on the cell is non-cell-defining SSB

FFS: Additional support of OD-SSB for CD-SSB located on sync-raster

Agreement

Support L3 measurement based on on-demand SSB

- Further work on L3 measurement is up to RAN2/RAN4

Agreement

- For a cell supporting on-demand SSB SCell operation, deactivation of on-demand SSB transmission is supported. In order to deactivate on-demand SSB transmission from a UE perspective, support at least one of the following options.

  - Option 1: Explicit indication of deactivation for on-demand SSB via MAC-CE for on-demand SSB transmission indication

  - Option 1A: Explicit indication of deactivation for on-demand SSB via RRC for on-demand SSB transmission indication

  - Option 2: Configuration/indication of the number N of on-demand SSB bursts to be transmitted after on-demand SSB is indicated

  - Option 3: Configuration/indication of the duration of on-demand SSB transmission window

  - Option 4: On-demand SSB transmission, if any, is deactivated when UE receives SCell deactivation MAC-CE for the activated SCell

  - Option 4A: On-demand SSB transmission, if any, is deactivated when the timer for SCell deactivation is expired

  - Option 5: On-demand SSB transmission, if any, is deactivated when SCell activation is completed

  - Option 6: Explicit indication of deactivation for on-demand SSB via [group-common] DCI

  - FFS: Each option is applicable to which Cases or Scenarios

  - FFS: Details related to each of the above options

Agreement

- For a cell supporting on-demand SSB SCell operation, support to provide at least the following parameters for on-demand SSB configuration by RRC at least for Case #1.

  - Sub-carrier spacing of the on-demand SSB

    - FFS if this can be absent

  - Physical Cell ID of the on-demand SSB

  - FFS: Time domain location of on-demand SSB burst such as SFN offset and half frame index

  - Downlink transmit power of on-demand SSB

  - FFS: The number N of on-demand SSB bursts to be transmitted after on-demand SSB is indicated

  - FFS whether the above parameters are configured by reusing legacy RRC parameters or new RRC parameters

Agreement

- For a cell supporting on-demand SSB SCell operation and for Case #2 (i.e., Always-on SSB is periodically transmitted on the cell), consider only one or both of the following options for UE to perform L1 measurement based on on-demand SSB.

  - Option 1: A CSI report configuration is associated with both of on-demand SSB and always-on SSB

  - Option 2: A CSI report configuration is associated with one of always-on SSB and on-demand SSB

  - FFS: Whether OD-SSB and always on SSB have same beam or not

Agreement

For a cell supporting on-demand SSB SCell operation and for Case #2 (i.e., Always-on SSB is periodically transmitted on the cell), study at least the following Mux-Cases.

- Mux-Case #1: No time-domain overlap between always-on SSB and on-demand SSB

- Mux-Case #2: Always-on SSB and on-demand SSB overlap at least in time or frequency domain

Below are the new agreements made in RAN1#119 meeting.

Agreement

Response to Q1 (What is the relation in terms of periodicity between always-on SSB and OD-SSB?) of Obj.1:

- The periodicity of on-demand SSB is one of 5 ms, 10 ms, 20 ms, 40 ms, 80 ms, or 160 ms. RAN1 is discussing whether to introduce other periodicity value(s) for on-demand SSB.

- The periodicity of on-demand SSB can be configured separately from the periodicity of always-on SSB.

- RAN1 is discussing what is the relation between periodicity of always-on SSB and periodicity of on-demand SSB and it has been identified that the main use case is that the periodicity of on-demand SSB is equal to or smaller than that of always-on SSB.

Further update to be made based on RAN1#119 progress.

Agreement

Response to Q3 (What is the relation in terms of frequency location between the always-on SSB and OD-SSB?) of Obj.1:

- The frequency location of on-demand SSB is the same as the frequency location of always-on SSB at least for the case where always-on SSB is not CD-SSB. RAN1 is discussing the frequency location of OD-SSB for the case where always-on SSB is CD-SSB.

Agreement

Response to Q4 (What is the spatial relation between the always-on SSB and OD-SSB?) of Obj.1:

- SS/PBCH blocks with the same SSB indexes for always-on SSB and on-demand SSB are quasi co-located with respect to Doppler spread, Doppler shift, average gain, average delay, delay spread, and when applicable, spatial RX parameters.

  - Applies at least for the case when the centre frequency locations of always-on SSB and OD-SSB is same

- When a signal/channel is configured to be QCLed with a SSB index, the signal/channel is QCLed with the same SSB index of always-on SSB and on-demand SSB (if transmitted) with the same QCL parameters according to existing specifications

  - Applies at least for the case when the centre frequency locations of always-on SSB and OD-SSB is same

- At least the case where SSB indices within on-demand SSB burst are identical to SSB indices within always-on SSB burst is supported. RAN1 is discussing whether to support the case where SSB indices within on-demand SSB burst can be subset of SSB indices within always-on SSB burst.

Agreement

- For a cell supporting on-demand SSB SCell operation, support to configure time domain location of on-demand SSB per on-demand SSB periodicity by RRC for both Case #1 and Case #2.

  - For Case #1 (i.e., No always-on SSB on the cell),

    - Based on two parameters, where one is to indicate SFN offset from a reference point and the other is to indicate half frame index

      - The reference point is SFN which satisfies (SFN index *10) modulo (OD-SSB periodicity) = 0

      - If SFN offset parameter is NOT configured, UE assumes SFN offset set to 0.

      - If half frame index parameter is NOT configured, UE assumes half frame index set to 0.

      - The value range of SFN offset is 0 to 15 unless longer periodicity for on-demand SSB than 160 ms is introduced.

      - The value range of half frame index is 0 or 1.

  - For Case #2 (i.e., Always-on SSB is periodically transmitted on the cell), down-select one of the following alternatives.

    - Alt A: Same as for Case #1

    - Alt B: Based on a single parameter which is to indicate the time offset between always-on SSB and on-demand SSB (e.g., similar to ssb-TimeOffset)

Agreement

New periodicity value for on-demand SSB other than the legacy values (i.e., 5 ms, 10 ms, 20 ms, 40 ms, 80 ms, or 160 ms) is NOT introduced in Rel-19.

Agreement

Down-select at least one of the following alternatives.

- Alt 1: If always-on SSB is CD-SSB on a synchronization raster, the frequency location of on-demand SSB is different from the frequency location of always-on SSB.

- Alt 2: If always-on SSB is CD-SSB on a synchronization raster, the frequency location of on-demand SSB is the same as the frequency location of always-on SSB

- Alt 3: Do not support the case where always-on SSB is CD-SSB on a synchronization raster.

Down-select at least one of the following alternatives.

- Alt A: If always-on SSB is CD-SSB and not on a synchronization raster, the frequency location of on-demand SSB can be same or different from the frequency location of always-on SSB, subject to its configuration.

- Alt B: If always-on SSB is CD-SSB and not on a synchronization raster, the frequency location of on-demand SSB is the same as the frequency location of always-on SSB

- Alt C: Do not support the case where always-on SSB is CD-SSB and not on a synchronization raster.

Agreement

Response to Q2 (What is the relation in terms of time location between always-on SSB and OD-SSB?) of Obj.1:

- RAN1 understands the time location of OD-SSB in Q2 refers to the time location of possible OD-SSB burst

- RAN1 is still discussing the relation in terms of time location between always-on SSB and OD-SSB

Agreement

For a cell supporting on-demand SSB SCell operation, support at least the following options to deactivate on-demand SSB transmission from a UE perspective.

- Option 1: Explicit indication of deactivation for on-demand SSB via MAC-CE for on-demand SSB transmission indication

  - Deactivation by RRC is up to RAN2

  - FFS: Which scenario Option 1 is used

- Option 2: Configuration/indication of the number N of on-demand SSB bursts to be transmitted after on-demand SSB is indicated

  - FFS: Whether Option 4, 4a is needed in addition to Option 2

  - FFS: Whether the value of N can be implicitly determined using a timer

## RAN2 agreements

Below are the agreements made in RAN2 that may influence RAN3 discussion.

Scenarios on OD-SSB:

- RAN2 start the discussion from Scenario 2/2A and wait for RAN1 conclusion on Scenario 3A/3B.

OD-SSB transmission indication:

- RRC based OD-SSB transmission indication is used to indicate at least the initial activation/deactivation state of OD-SSB configuration. FFS on reconfiguration.

- New MAC-CE for OD-SSB transmission indication is introduced. We will not change legacy SCell activation/deactivation MAC CE. FFS if we need further optimization for scenario 2A.

Measurement on OD-SSB:

- Measurement based on OD-SSB in both case 1 and case 2 will be considered. (case 1 and case 2 defined in RAN1).

- For Case #1, the UE does not expect to measure SSB when on-demand SSB transmission is deactivated. In other words, the UE expects to measure SSB when on-demand SSB transmission is activated.

- RAN2 does not handle the issue raised in R2-2407414 in OD-SSB based measurements.

- RAN2 study how the UE to perform L3 measurement according to OD-SSB L3 RRM configuration.

Agreements on OD-SSB SCell

- No need to restrict the OD-SSB activation/deactivation state indication in RRC to initial configuration. No special specification effort is required.

- Don’t introduce further new MAC CE that combines SCell activation/deactivation and OD-SSB indication for scenario 2A.

- NW should be able to send OD-SSB indication for multiple SCells simultaneously by a MAC CE.

## RAN3 impacts

## Xn impact

Considering the scenario needs to be analyzed is for UEs in connected mode configured with CA, for both intra-/inter-band CA [1], in which a PCell is always associated to a CD-SSB located on the synchronization raster, we do not see the need to use a neighbour gNB to tiger OD-SSB SCell operation and we think this should be an intra-gNB issue.

In last meeting, one issue was brought up about how to address L3 measurement of an OD-SSB Cell, whether NES cell can be triggered by a neighbor cell over Xn for OD-SSB activation? Considering the existing RAN1 agreement, On-demand SSB on the cell is not located on synchronization raster; On-demand SSB on the cell is non-cell-defining SSB. Besides, RAN2 is also working on how the UE to perform L3 measurement according to OD-SSB L3 RRM configuration and their initial assumption is that UE does not expect to measure SSB when OD-SSB transmission is deactivated if no always-on SSB on the cell.

Therefore, from the above two points of view, no Xn impact is foreseen for OD-SSB SCell operation for now. But this issue can be kept FFS in case RAN1 or RAN2 reaching any new agreements that will change the above working assumptions.

Proposal 1: No Xn impact is foreseen for on-demand SSB SCell operation for now, but FFS if different working assumptions agreed in RAN1/RAN2.

## F1 impact

RAN1 agreed to Support on-demand SSB SCell operation triggered by gNB, so in this case, for split architecture, it is an open issue about which node, e.g. gNB-CU or gNB-DU, can decide OD-SSB configuration and trigger OD-SSB transmission.

According to RAN1 agreement, the following parameters need to be configured by RRC for on-demand SSB:

- Sub-carrier spacing of the on-demand SSB

- Physical Cell ID of the on-demand SSB

- Downlink transmit power of on-demand SSB

Same as in legacy, gNB-DU decides the above parameters for the SSB configuration, therefore, it is reasonable that OD-SSB configuration is still decided by gNB-DU.

Proposal 2: gNB-DU decides OD-SSB configuration.

In last meeting, RAN3 agreed that, for RRC based signalling, gNB-DU can decide activation/deactivation state of OD-SSB configuration per SCell. Considering, after CA is set up, gNB uses MAC CE (SCell Activation/Deactivation) to activate or deactivate data transmission for SCell and RAN2 agreed that this MAC CE can also be used for OD-SSB indication for scenario 2A, therefore it is straightforward to further agree that, for MAC CE based signalling, also gNB-DU can decide activation/deactivation state of OD-SSB configuration per SCell.

Proposal 3: gNB-DU decide activation/deactivation state of OD-SSB configuration per SCell.

Considering gNB-CU is more knowledgeable on the SSB operation status (e.g. no always-on SSB, always-on SSB with a long periodicity or always-on SSB as in legacy) of the cells with all DUs connected to it and neighbour cells, gNB-CU can provide some guidance to gNB-DU for OD-SSB SCell operation, taking both energy saving and UE performance into account. For example, gNB-CU can suggest to gNB-DU e.g. OD-SSB allowed cells and, for each SCell, SSB operation mode, OD-SSB transmission periodicity, OD-SSB transmission duration, etc.

Proposal 4: gNB-CU can provide assistance information to gNB-DU to assist the OD-SSB operation.

In Rel-18 NES, a Cells Allowed to be Deactivated List IE is added in GNB-CU CONFIGURATION UPDATE message, and “If Cells Allowed to be Deactivated List IE is contained in the GNB-CU CONFIGURATION UPDATE message, the gNB-DU shall, if supported, consider that it is allowed to deactivate the SSB beams within the indicated cells for network energy optimization purpose”[1]. However, this IE is added as an assistance information for beam deactivation in a cell owned by gNB-DU, which is used for a different purpose than OD-SSB operation. In order not to impact the Rel-18 beam deactivation operation, we prefer to adopt a new IE for the assistance information for OD-SSB operation, and it can be added in the existing GNB-CU CONFIGURATION UPDATE message.

Proposal 5: To add a new IE in GNB-CU CONFIGURATION UPDATE message for the assistance information for OD-SSB operation.

# Proposals

In this contribution, we summarized the latest RAN1 and RAN2 progress on OD-SSB and analysed the potential RAN3 impacts, and the following proposals were drawn from the analysis.

Proposal 1: No Xn impact is foreseen for on-demand SSB SCell operation for now, but FFS if different working assumptions agreed in RAN1/RAN2.

Proposal 2: gNB-DU decides OD-SSB configuration.

Proposal 3: gNB-DU decide activation/deactivation state of OD-SSB configuration per SCell.

Proposal 4: gNB-CU can provide assistance information to gNB-DU to assist the OD-SSB operation.

Proposal 5: To add a new IE in GNB-CU CONFIGURATION UPDATE message for the assistance information for OD-SSB operation.

# Reference

[1] TS38.473, NG-RAN; F1 application protocol (F1AP)