3GPP TSG-RAN WG3 #127	R3-250181

Greece, Athens, 17th – 21st February 2025

Agenda item:		14.2

Title:		(TP for TS 38.300 and TS 38.413) Discussion on NR NTN supporting MBS broadcast service

Source:		NEC

Document for:		Discussion and Decision

# 1. Introduction

In the previous meetings, RAN3 made the following progress on NR NTN MBS broadcast service enhancement:

RAN3#126: The current MBS broadcast procedures can be reused for NR NTN without any enhancement. Check RAN2 progress on the geographical area information.

The NR NTN support MBS broadcast will be discussed continuously in this document.

# 2. Discussion

2.1	Intended service area discussion

In terms of the MBS intended service area for NR NTN, RAN3 agreed to wait for RAN2’s progress, and RAN2 had the following agreements included in the LS R3-250011:

1. The encoding of TN coverage introduced in Rel-18 in TS38.331, including tn-ReferenceLocation-r18 and tn-DistanceRadius-r18, is reused for the geographical area of the circle. 2. The encoding of Polygon in TS37.355 is reused for the geographical area of the Polygon. 3. The IntendedServiceAreaInfo is considered as the IE name of the geographical area (we can still update the name in the CR implementation if needed)

TS 38.331 contains the details of the tn-ReferenceLocation-r18 and tn-DistanceRadius-r18 IEs.:

CoverageAreaInfo-r18 ::= SEQUENCE { tn-AreaId-r18 TN-AreaId-r18, tn-ReferenceLocation-r18 ReferenceLocation-r17, tn-DistanceRadius-r18 INTEGER(0..65535) }

Where the definition of the Polygon is defined in 37.355:

#### – Polygon The IE Polygon is used to describe a geographic shape as defined in TS 23.032 [15]. -- ASN1START Polygon ::= SEQUENCE (SIZE (3..15)) OF PolygonPoints PolygonPoints ::= SEQUENCE { latitudeSign ENUMERATED {north, south}, degreesLatitude INTEGER (0..8388607), -- 23 bit field degreesLongitude INTEGER (-8388608..8388607) -- 24 bit field } -- ASN1STOP

In addition, RAN3 had an agreement that “RAN3 takes the assumption that geographical service area information can be transmitted from CN with the same format as defined in Uu” in the RAN3#125bis meeting. Consequently, the NGAP interface should carry the tn-ReferenceLocation-r18, tn-DistanceRadius-r18, and Polygon data.

RAN3 agreed to the MBS Service Area Small Area List IE in the last meeting, and each item in the list is Small Area IE which is defined as an OCTET STRING. So, this OCTET STRING should include the tn-ReferenceLocation-r18, tn-DistanceRadius-r18, and Polygon IEs over NGAP. And the semantics description of this OCTET STRING is required. We also provide a TP about it.

Proposal 1: The intended service area for NR NTN MBS service over NGAP should be described by the tn-ReferenceLocation-r18 and tn-DistanceRadius-r18 IEs as defined in TS 38.331 and the Polygon IE as defined in 37.355 should be included in the over NGAP. And the corresponding TP is provided.

Furthermore, in BLCR R3-247035, the intended service area is named as “MBS Service Area Small Area List” and “Small Area”, which implies that the MBS intended service area is inside and smaller than the area covered by an NTN cell. However, RAN2 agreed in the last meeting that “A signalled intended service area for an MBS BC service may include geographic areas across the current serving cell and overlapping neighbour cell(s).” So, we think the “Small Area” in the name of the two IEs is not appropriate because some intending service area is bigger than an NTN cell or some intending service area is located on the cross area of NTN cells. And we suggest changing the IEs’ name to be “MBS NTN Intended Service Area List” and “Intended Service Area”, and TPs are provided in Annex B.

Proposal 2: We suggest changing the IEs’ names of “MBS Service Area Small Area List” and “Small Area” to be “MBS NTN Intended Service Area List” and “Intended Service Area”.

2.2	MBS broadcast session management enhancement

The broadcast session management procedures (i.e., Broadcast Session Setup/Broadcast Session Modification/Broadcast Session Release/Broadcast Session Release Required/Broadcast Session Transport) should be reused for NTN MBS broadcast service as we agreed in the last meeting. Considering the movement of NTN payload, some cases need to be considered:

- Case 1: the Broadcast Session Setup may be initiated when the NTN footprint moves to the intended service area (T1 in Figure 1).

- Case 2: the Broadcast Session Release may be initiated when the footprint leaves an intended service area. The release cause value should be NTN coverage leaves the indicated broadcast service area (T3 in Figure 1).

- Case 3: the Broadcast Session Release Required may be initiated when the footprint leaves an intended service area. The release cause value should be NTN coverage leaves the indicated broadcast service area (T3 in Figure 1).

- Case 4: the Broadcast Session Modification may be initiated when the NTN footprint covers the intended service area (T1-T3 in Figure 1).

Figure 1: Example of NTN payload moves over MBS broadcast service area

The above cases caused by NTN payload movement should be discussed in RAN3. A stage 2 description is beneficial to clarify that the NTN payload movement may trigger some broadcast management procedures. Additionally, for the Broadcast Session Release/Broadcast Session Release Required messages, there is no specific cause value that explains why the broadcast session needs to be released due to the NTN payload movement. This means that a new cause value needs to be defined.

Proposal 3: RAN3 is suggested to discuss the broadcast session management procedures caused by the movement of the NTN payload and agree on the TP 38.300 in Annex A and the TP 38.413 in Annex B.

# 3. Conclusion

Proposal 1: The intended service area for NR NTN MBS service over NGAP should be described by the tn-ReferenceLocation-r18 and tn-DistanceRadius-r18 IEs as defined in TS 38.331 and the Polygon IE as defined in 37.355 should be included in the over NGAP. And the corresponding TP is provided.

Proposal 2: We suggest changing the IEs’ names of “MBS Service Area Small Area List” and “Small Area” to be “MBS NTN Intended Service Area List” and “Intended Service Area”.

Proposal 3: RAN3 is suggested to discuss the broadcast session management procedures caused by the movement of the NTN payload and agree on the TP 38.300 in Annex A and the TP 38.413 in Annex B.

# References

# Annex A: TP for TS 38.413 (on top of R3-247899)

--------------------------------------------------------------Start of the change-----------------------------------------------------------

#### 9.3.1.209	MBS Service Area Information

This IE contains MBS service area information.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| MBS Service Area Cell List |  | 0..<maxnoofCellsforMBS> |  |  |
| >NR CGI | M |  | 9.3.1.7 |  |
| MBS Service Area TAI List |  | 0..<maxnoofTAIforMBS> |  |  |
| >TAI | M |  | 9.3.3.11 |  |
| MBS NTN Intended Service Area List |  | 0..<maxnoofIntendedServiceAreasforMBS> |  |  |
| > Intended Service Area | M |  | 9.3.1.x |  |

Editor’s Note: The IE name and encoding are pending RAN2 discussion.

| Range bound | Explanation |
| --- | --- |
| maxnoofCellsforMBS | Maximum no. of cells allowed within one MBS Service Area. Value is 8192. |
| maxnoofTAIforMBS | Maximum no. of TAs allowed within one MBS Service Area. Value is 1024. |
| maxnoofIntendedServiceAreasIforMBS | Maximum no. of intended service areas allowed within one MBS Service Area. Value is 65536. |

--------------------------------------------------------------Next change-----------------------------------------------------------

#### 9.3.1.x	Intended Service Area

This IE provides information on the Intended Service Area for MBS Broadcast Service.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| Intended Service Area | M |  | OCTET STRING | Containing the tn-ReferenceLocation-r18 and tn-DistanceRadius-r18 IEs as defined in TS 38.331 or the Polygon IE as defined in 37.355 |

--------------------------------------------------------------Next change-----------------------------------------------------------

### 9.4.5	Information Element Definitions

-- ASN1START

-- **************************************************************

--

-- Information Element Definitions

--

-- **************************************************************

NGAP-IEs {

itu-t (0) identified-organization (4) etsi (0) mobileDomain (0)

ngran-Access (22) modules (3) ngap (1) version1 (1) ngap-IEs (2) }

DEFINITIONS AUTOMATIC TAGS ::=

BEGIN

MBS-ServiceAreaInformation ::= SEQUENCE {

mBS-ServiceAreaCellList		MBS-ServiceAreaCellList																OPTIONAL,

mBS-ServiceAreaTAIList		MBS-ServiceAreaTAIList																OPTIONAL,

iE-Extensions					ProtocolExtensionContainer { {MBS-ServiceAreaInformation-ExtIEs} }	OPTIONAL,

...

}

MBS-ServiceAreaInformation-ExtIEs NGAP-PROTOCOL-EXTENSION ::= {

{ ID id-MBS-NTNIntendedServiceAreaList		CRITICALITY ignore	EXTENSION MBS-NTNIntendedServiceAreaList	PRESENCE optional },

...

}

MBS-ServiceAreaCellList ::= SEQUENCE (SIZE(1.. maxnoofCellsforMBS)) OF NR-CGI

MBS-ServiceAreaTAIList ::= SEQUENCE (SIZE(1.. maxnoofTAIforMBS)) OF TAI

MBS-NTNIntendedServiceAreaList ::= SEQUENCE (SIZE(1..maxnoofIntendedServiceAreasforMBS)) OF IntendedServiceArea

IntendedServiceArea ::= OCTET STRING

MBS-SessionID ::= SEQUENCE {

tMGI								TMGI,

nID								NID																			OPTIONAL,

iE-Extensions					ProtocolExtensionContainer { {MBS-SessionID-ExtIEs} } 		OPTIONAL,

...

}

MBS-SessionID-ExtIEs NGAP-PROTOCOL-EXTENSION ::= {

...

}

--------------------------------------------------------------Next change-----------------------------------------------------------

### 9.4.7	Constant Definitions

-- Constant definitions

--

-- **************************************************************

NGAP-Constants {

itu-t (0) identified-organization (4) etsi (0) mobileDomain (0)

ngran-Access (22) modules (3) ngap (1) version1 (1) ngap-Constants (4) }

DEFINITIONS AUTOMATIC TAGS ::=

BEGIN

-- **************************************************************

--

#### -- IE parameter types from other modules.

--

-- **************************************************************

/******* Unchanged part skipped ******/

maxnoofSNSSAIforQMC							INTEGER ::= 16

maxnoofTAforQMC								INTEGER ::= 8

maxnoofThresholdsForExcessPacketDelay		INTEGER ::= 255

maxnoofESNPNs								INTEGER ::= 15

maxnoofCandidateRelayUEs					INTEGER ::= 32

maxnoofSuccessfulPSCellChangeReports       	INTEGER ::= 64

maxnoofCellsTSS								INTEGER ::= 16384

maxnoofPeriodicities						INTEGER ::= 8

maxnoofCAGforMDT							INTEGER ::= 256

maxnoofMDTSNPNs								INTEGER ::= 16

maxnoofPartiallyAllowedS-NSSAIs				INTEGER ::= 8

maxnoofRSPPQoSFlows							INTEGER ::= 2048

maxnoofIntendedServiceAreasforMBS						INTEGER ::= 65536

--------------------------------------------------------------End of the change-----------------------------------------------------------

# Annex A: TP to BLCR for TS 38.300

--------------------------------------------------------------Start of the change-----------------------------------------------------------

### 16.14.5	NG-RAN signalling

### 16.14.5	NG-RAN signalling

The Cell Identity, as defined in TS 38.413 [26] and TS 38.423 [50], used in following cases corresponds to a Mapped Cell ID, irrespective of the orbit of the NTN payload or the types of service links supported:

-	The Cell Identity indicated by the gNB to the Core Network as part of the User Location Information;

-	The Cell Identity used for Paging Optimization in NG interface;

-	The Cell Identity used for Area of Interest;

-	The Cell Identity used for PWS.

The Cell Identity included within the target identification of the handover messages allows identifying the correct target cell. The cell identity used in the NG and Xn handover messages, Xn Setup and Xn NG-RAN Node Configuration Update procedures is expected to be Uu Cell ID.

The Cell Identities used in the RAN Paging Area during Xn RAN paging allow the identification of the correct target cells for RAN paging.

NOTE 1:	The Cell Identity used for RAN Paging is assumed to typically represent a Uu Cell ID.

The mapping between Mapped Cell IDs and geographical areas is configured in the RAN and Core Network.

NOTE 2:	A specific geographical location may be mapped to multiple Mapped Cell ID(s), and such Mapped Cell IDs may be configured to indicate differerent geographical areas (e.g. overlapping and/or with different dimensions).

The gNB is responsible for constructing the Mapped Cell ID based on the UE location information received from the UE, if available. The mapping may be pre-configured (e.g., up to operator's policy) or up to implementation.

NOTE 3:	As described in TS 23.501 [3], the User Location Information may enable the AMF to determine whether the UE is allowed to operate at its present location. Special Mapped Cell IDs or TACs may be used to indicate areas outside the serving PLMN's country.

The gNB reports the broadcasted TAC(s) of the selected PLMN to the AMF as part of ULI. In case the gNB knows the UE's location information, the gNB may determine the TAI the UE is currently located in and provide that TAI to the AMF as part of ULI.

The broadcast session management procedures (e.g. Setup, Modification Release, Release Required) may be initiated when the NTN payload moves to or leaves the intended broadcast service area.

--------------------------------------------------------------End of the change-----------------------------------------------------------

# Annex B: TP to BLCR for TS 38.413

--------------------------------------------------------------Start of the change-----------------------------------------------------------

#### 9.3.1.2	Cause

The purpose of the Cause IE is to indicate the reason for a particular event for the NGAP protocol.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| CHOICE Cause Group | M |  |  |  |
| >Radio Network Layer |  |  |  |  |
| >>Radio Network Layer Cause | M |  | ENUMERATED (Unspecified, TXnRELOCOverall expiry, Successful handover, Release due to NG-RAN generated reason, Release due to 5GC generated reason, Handover cancelled, Partial handover, Handover failure in target 5GC/NG-RAN node or target system, Handover target not allowed, TNGRELOCoverall expiry, TNGRELOCprep expiry, Cell not available, Unknown target ID, No radio resources available in target cell, Unknown local UE NGAP ID, Inconsistent remote UE NGAP ID, Handover desirable for radio reasons, Time critical handover, Resource optimisation handover, Reduce load in serving cell, User inactivity, Radio connection with UE lost, Radio resources not available, Invalid QoS combination, Failure in the radio interface procedure, Interaction with other procedure, Unknown PDU Session ID, Unknown QoS Flow ID, Multiple PDU Session ID Instances, Multiple QoS Flow ID Instances, Encryption and/or integrity protection algorithms not supported, NG intra-system handover triggered, NG inter-system handover triggered, Xn handover triggered, Not supported 5QI value, UE context transfer, IMS voice EPS fallback or RAT fallback triggered, UP integrity protection not possible, UP confidentiality protection not possible, Slice(s) not supported, UE in RRC_INACTIVE state not reachable, Redirection, Resources not available for the slice(s), UE maximum integrity protected data rate reason, Release due to CN-detected mobility, …, N26 interface not available, Release due to pre-emption, Multiple Location Reporting Reference ID Instances, RSN not available for the UP, NPN access denied, CAG only access denied, Insufficient UE Capabilities, RedCap UE not supported, Unknown MBS Session ID, Indicated MBS Session Area Information not served by the gNB, Inconsistent slice info for the session, Misaligned association for the multicast and unicast sessions or flows, eRedCap UE not supported, 2Rx XR UE not supported, NTN coverage leaves the indicated broadcast service area) |  |

/******* Unchanged part skipped ******/

The meaning of the different cause values is described in the following tables. In general, "not supported" cause values indicate that the related capability is missing. On the other hand, "not available" cause values indicate that the related capability is present, but insufficient resources were available to perform the requested action.

| Radio Network Layer cause | Meaning |
| --- | --- |
| Unspecified | Sent for radio network layer cause when none of the specified cause values applies. |
| TXnRELOCOverall expiry | The timer guarding the handover that takes place over Xn has abnormally expired. |
| Successful handover | Successful handover. |
| Release due to NG-RAN generated reason | Release is initiated due to NG-RAN generated reason. |
| Release due to 5GC generated reason | Release is initiated due to 5GC generated reason. |
| Handover cancelled | The reason for the action is cancellation of Handover. |
| Partial handover | Provides a reason for the handover cancellation. The HANDOVER COMMAND message from AMF contained PDU Session Resource to Release List IE or QoS flow to Release List and the source NG-RAN node estimated service continuity for the UE would be better by not proceeding with handover towards this particular target NG-RAN node. |
| Handover failure in target 5GC/ NG-RAN node or target system | The handover failed due to a failure in target 5GC/NG-RAN node or target system. |
| Handover target not allowed | Handover to the indicated target cell is not allowed for the UE in question. |
| TNGRELOCoverall expiry | The reason for the action is expiry of timer TNGRELOCoverall. |
| TNGRELOCprep expiry | Handover Preparation procedure is cancelled when timer TNGRELOCprep expires. |
| Cell not available | The concerned cell is not available. |
| Unknown target ID | Handover rejected because the target ID is not known to the AMF. |
| No radio resources available in target cell | Load on target cell is too high. |
| Unknown local UE NGAP ID | The action failed because the receiving node does not recognise the local UE NGAP ID. |
| Inconsistent remote UE NGAP ID | The action failed because the receiving node considers that the received remote UE NGAP ID is inconsistent. |
| Handover desirable for radio reasons | The reason for requesting handover is radio related. |
| Time critical handover | Handover is requested for time critical reason i.e., this cause value is reserved to represent all critical cases where the connection is likely to be dropped if handover is not performed. |
| Resource optimisation handover | The reason for requesting handover is to improve the load distribution with the neighbour cells. |
| Reduce load in serving cell | Load on serving cell needs to be reduced. When applied to handover preparation, it indicates the handover is triggered due to load balancing. |
| User inactivity | The action is requested due to inactivity on all user data radio bearers (i.e., DRBs and, if applicable, MRBs as per section 16.10.5.2 in TS 38.300 [8]), e.g., NG is requested to be released in order to optimise the radio resources. For L2 U2N Relay UE, this action is requested due to user inactivity on all PDU sessions of L2 U2N Relay UE and its served remote UE(s). |
| Radio connection with UE lost | The action is requested due to losing the radio connection to the UE. |
| Radio resources not available | No requested radio resources are available. |
| Invalid QoS combination | The action was failed because of invalid QoS combination. |
| Failure in the radio interface procedure | Radio interface procedure has failed. |
| Interaction with other procedure | The action is due to an ongoing interaction with another procedure. |
| Unknown PDU Session ID | The action failed because the PDU Session ID is unknown in the NG-RAN node. |
| Unknown QoS Flow ID | The action failed because the QoS Flow ID is unknown in the NG-RAN node. |
| Multiple PDU Session ID instances | The action failed because multiple instances of the same PDU Session had been provided to/from the NG-RAN node. |
| Multiple QoS Flow ID instances | The action failed because multiple instances of the same QoS flow had been provided to the NG-RAN node. |
| Encryption and/or integrity protection algorithms not supported | The NG-RAN node is unable to support any of the encryption and/or integrity protection algorithms supported by the UE. |
| NG intra-system handover triggered | The action is due to a NG intra-system handover that has been triggered. |
| NG inter-system handover triggered | The action is due to a NG inter-system handover that has been triggered. |
| Xn handover triggered | The action is due to an Xn handover that has been triggered. |
| Not supported 5QI value | The QoS flow setup failed because the requested 5QI is not supported. |
| UE context transfer | The action is due to a UE resumes from the NG-RAN node different from the one which sent the UE into RRC_INACTIVE state. |
| IMS voice EPS fallback or RAT fallback triggered | The setup of QoS flow is failed due to EPS fallback or RAT fallback for IMS voice using handover or redirection. |
| UP integrity protection not possible | The PDU session cannot be accepted according to the required user plane integrity protection policy. |
| UP confidentiality protection not possible | The PDU session cannot be accepted according to the required user plane confidentiality protection policy. |
| Slice(s) not supported | Slice(s) not supported. |
| UE in RRC_INACTIVE state not reachable | The action is requested due to RAN paging failure. |
| Redirection | The release is requested due to inter-system redirection or intra-system redirection. |
| Resources not available for the slice(s) | The requested resources are not available for the slice(s). |
| UE maximum integrity protected data rate reason | The request is not accepted in order to comply with the maximum data rate for integrity protection supported by the UE. |
| Release due to CN-detected mobility | The context release is requested by the AMF because the UE is already served by another CN node (same or different system), or another NG interface of the same CN node. |
| N26 interface not available | The action failed due to a temporary failure of the N26 interface. |
| Release due to pre-emption | Release is initiated due to pre-emption. |
| Multiple Location Reporting Reference ID Instances | The action failed because multiple areas of interest are set with the same Location Reporting Reference ID. |
| RSN not available for the UP | The redundant user plane resources indicated by RSN are not available. |
| NPN access denied | Access was denied, or release is requested, for NPN reasons. |
| CAG only access denied | Access was denied because the cell is a non-CAG cell and UE is only allowed to access CAG cells. |
| Insufficient UE Capabilities | The procedure can’t proceed due to insufficient UE capabilities. |
| RedCap UE not supported | The action failed because target NG-RAN node does not support RedCap UE. |
| Unknown MBS Session ID | The action failed because the MBS Session ID is unknown. |
| Indicated MBS Service Area Information not served by the gNB | The action failed because none of the cells in the indicated MBS Service Area Information are served by the NG-RAN node. |
| Inconsistent slice info for the session | The action failed because the slice info of the multicast session is inconsistent. |
| Misaligned association for the multicast and unicast sessions or flows | The action failed because the Associated Unicast QoS Flow ID has already been used, or the Associated Unicast QoS Flow ID is not defined, or the Associated Unicast QoS Flow ID is not released, or multiple MBS QoS flows associated to the same unicast QoS flow, or same multicast session associated to multiple PDU Sessions. |
| eRedCap UE not supported | The action failed because target NG-RAN node does not support eRedCap UE. |
| 2Rx XR UE not supported | The action failed because target NG-RAN node does not support 2Rx XR UE. |
| NTN coverage leaves the indicated broadcast service area | Release is initiated due to the NTN coverage leaves the indicated broadcast service area. |

/******* Unchanged part skipped ******/

CauseRadioNetwork ::= ENUMERATED {

unspecified,

txnrelocoverall-expiry,

successful-handover,

release-due-to-ngran-generated-reason,

release-due-to-5gc-generated-reason,

handover-cancelled,

partial-handover,

ho-failure-in-target-5GC-ngran-node-or-target-system,

ho-target-not-allowed,

tngrelocoverall-expiry,

tngrelocprep-expiry,

cell-not-available,

unknown-targetID,

no-radio-resources-available-in-target-cell,

unknown-local-UE-NGAP-ID,

inconsistent-remote-UE-NGAP-ID,

handover-desirable-for-radio-reason,

time-critical-handover,

resource-optimisation-handover,

reduce-load-in-serving-cell,

user-inactivity,

radio-connection-with-ue-lost,

radio-resources-not-available,

invalid-qos-combination,

failure-in-radio-interface-procedure,

interaction-with-other-procedure,

unknown-PDU-session-ID,

unkown-qos-flow-ID,

multiple-PDU-session-ID-instances,

multiple-qos-flow-ID-instances,

encryption-and-or-integrity-protection-algorithms-not-supported,

ng-intra-system-handover-triggered,

ng-inter-system-handover-triggered,

xn-handover-triggered,

not-supported-5QI-value,

ue-context-transfer,

ims-voice-eps-fallback-or-rat-fallback-triggered,

up-integrity-protection-not-possible,

up-confidentiality-protection-not-possible,

slice-not-supported,

ue-in-rrc-inactive-state-not-reachable,

redirection,

resources-not-available-for-the-slice,

ue-max-integrity-protected-data-rate-reason,

release-due-to-cn-detected-mobility,

...,

n26-interface-not-available,

release-due-to-pre-emption,

multiple-location-reporting-reference-ID-instances,

rsn-not-available-for-the-up,

npn-access-denied,

cag-only-access-denied,

insufficient-ue-capabilities,

redcap-ue-not-supported,

unknown-MBS-Session-ID,

indicated-MBS-session-area-information-not-served-by-the-gNB,

inconsistent-slice-info-for-the-session,

misaligned-association-for-multicast-unicast,

eredcap-ue-not-supported

two-rx-xr-ue-not-supported,

NTN-coverage-leaves-the-indicated-broadcase-service-area

}

--------------------------------------------------------------End of the change-----------------------------------------------------------