3GPP TSG-RAN WG3 Meeting #127	R3-250191

Athens, Greece, Feb 17th - Feb 21st, 2025

Agenda item:	13.3

Source:	Qualcomm Incorporated

Title:	Signaling enhancements for Conditional Intra-gNB/Intra-CU LTM

WID/SID:	NR_Mob_Ph4-Core – Release 19

Document for:	Discussion and decision

# Introduction

In the last two RAN2 meetings (RAN2 #127-bis [1], RAN2 #128 [2]), progress was achieved on the topic of intra-CU conditional LTM, and RAN2 made the following set of basic agreements.

RAN2 #127-bis agreements:

1. Source cell sends the conditional LTM configuration via RRCReconfiguration to UE, which includes the LTM candidate configurations, and the corresponding execution conditions.

2. Event LTM3-like and LTM5-like are used as the conditional LTM execution condition. FFS on reuse of CHO conditions.

3. Source cell and each candidate cell provides its own execution condition for conditional LTM.

4. It is DU to generate the L1 execution condition. FFS on a case that L3 measurement is used.

5. RACH-less Conditional intra-CU LTM is supported.

6. RACH based conditional intra-CU LTM is supported.

7. UE based TA measurement mechanism is supported for conditional intra-CU LTM.

8. PDCCH ordered early TA acquisition is supported for conditional LTM.

9. Rel-18 Early candidate TCI State activation/deactivation is supported for conditional intra-CU LTM.

10. For RACH-less conditional LTM, CG-based first UL transmission on target cell is supported. FFS on DG-based approach.

11. The LTM completion defined for Rel-18 intra-CU LTM is reused for conditional LTM.

RAN2 #128 agreements:

1. The triggering condition of conditional LTM can be based on L3 measurement.

2. CondEventA3 and CondEventA5 conditions can be baseline for the conditional LTM execution.

3. The L1 execution condition of a candidate cell is associated to only one triggering event.

4. For L3 execution condition, it may consist of one or two triggering condition(s). If there are two triggering conditions associated with the same candidate cell, the UE shall consider the execution condition is fulfilled only when both triggering conditions are met. Only single RS type is supported and at most two different trigger quantities can be configured simultaneously for the evaluation of execution condition of a single candidate cell.

5. To support initial and subsequent conditional LTM, the following items can be considered for the configuration of execution condition:

- The CLTM configuration of each candidate cell shall include the execution condition for initial conditional LTM, which is generated by the initial source cell to trigger the CLTM for the candidate cell.

- The CLTM configuration of each candidate cell may include execution conditions for subsequent conditional LTM, which is generated by the candidate cell to trigger the CLTM for other candidate cells when the candidate cell becomes a serving cell.

6. The network can configure measurement reports e.g., L1 periodic, semi-persistent, aperiodic and event triggered report, or L3 measurement reports for conditional LTM, e.g., to trigger PDCCH ordered early RACH.

7. For CLTM, the Candidate Cell TCI States Activation/Deactivation MAC CE is re-used for the early activation/deactivation of TCI state(s) of a CLTM candidate configuration.

8. The Early TA is signalled to the UE from the source cell (i.e., not from the candidate cell directly to the UE). This agreement will be included in the LS to RAN1/3/4.

9. The network can inform the candidate cell’s TA information to UE via new MAC CE, which is the TA value when UE switches to that candidate cell during CLTM.

10. Candidate cell TA is maintained by a new timer.

11. For L1-based conditional LTM the condition evaluation is at MAC level and for L3-based conditional LTM the condition evaluation is at RRC level.

In this contribution, we discuss some of the basic RAN3 signalling aspects of the LTM preparation phase of intra-CU conditional LTM (C-LTM).

# 2 Signalling aspects of the LTM preparation phase of intra-CU conditional LTM (C-LTM)

The distinguishing feature of C-LTM compared to Rel-18 intra-CU LTM is that the UE is configured by the network with L1 execution conditions and L3 execution conditions for LTM cell switch. RAN2 made some agreements on C-LTM execution conditions that are listed in the introduction section (Section 1) above.

Figure 1 below shows an example signalling flow of intra-CU C-LTM. Based on the RRC measurement reports from the UE, the gNB-CU initiates the preparation of one or more candidate cells for intra-CU C-LTM. The preparation of the candidate cells can take place in parallel, as in Rel-18 intra-CU LTM.

Proposal 1. The gNB-CU initiates intra-CU C-LTM preparation and may transmit to an underlying candidate gNB-DU the UE Context Setup Request message including a suggested candidate cell for LTM preparation. The UE Context Setup Request message includes:

- Suggested candidate cell ID,

- Intra-CU C-LTM indication.

Proposal 2: If an LTM candidate cell belongs to the source gNB-DU, the gNB-CU also transmits to the source gNB-DU the UE Context Modification Request message including a suggested candidate cell for LTM preparation. The UE Context Modification Request message includes:

- Suggested candidate cell ID,

- Intra-CU C-LTM indication.

Figure 1: Signalling flow of intra-CU C-LTM

Proposal 3. The preparation of candidate cells in intra-CU C-LTM can take place in parallel, similar as in Rel-18 intra-CU LTM.

If the candidate gNB-DU accepts the LTM configuration request, it generates and transmits the lower layer configuration of the prepared LTM candidate cell in its response to the gNB-CU.

Proposal 4. If the candidate gNB-DU accepts the LTM configuration request, it generates and transmits the lower layer configuration of the prepared LTM candidate cell in its response to the gNB-CU in UE Context Setup Response.

Similarly, if the source gNB-DU accepts the LTM configuration request, it generates and transmits the lower layer configuration of the prepared LTM candidate cell in its response to the gNB-CU in UE Context Modification Response.

When the parallel preparation of the LTM candidate cells is completed, i.e., upon completion of Steps 4a and 4b in Figure 1, the gNB-CU has information regarding the prepared candidate cells in the candidate gNB-DUs and the source gNB-DU. The gNB-CU can then generate the L3 execution conditions for conditional LTM based on this information and the UE RRC measurement reports.

Proposal 5. Upon receiving information regarding the prepared LTM candidate cells from its underlying candidate gNB-DUs and the source gNB-DU, the gNB-CU generates the L3 execution conditions for C-LTM:

- The gNB-CU generates the L3 execution conditions for cell switch from the source cell to other LTM candidate cells, as well as the execution conditions for cell switch from an LTM candidate cell to other candidate cells, i.e., for subsequent LTM.

Furthermore, the gNB-CU forwards information regarding the prepared LTM candidate cells to each of the candidate gNB-DUs and the source gNB-DU using the UE Context Modification procedure. Thus, each candidate gNB-DU and the source gNB-DU then has information regarding the prepared LTM candidate cells in the other candidate gNB-DUs. This is needed to support subsequent Conditional LTM.

Proposal 6. Based on the information regarding all the prepared LTM candidate cells:

- The source gNB-DU generates the L1 execution conditions for the initial LTM cell switch and may also generate the L1 execution conditions for a subsequent LTM cell switch.

- A candidate gNB-DU generates the L1 execution conditions for subsequent LTM cell switch.

Proposal 7. Each candidate gNB-DU and the source gNB-DU transmit the L1 execution conditions for the initial LTM cell switch and subsequent LTM cell switch to the gNB-CU in the UE Context Modification Response messages.

The gNB-CU transmits the L3 execution conditions and/or the L1 execution conditions to the UE in the RRC reconfiguration message containing the LTM configuration for intra-CU C-LTM.

5. RACH-less Conditional intra-CU LTM is supported.

7. UE based TA measurement mechanism is supported for conditional intra-CU LTM.

8. PDCCH ordered early TA acquisition is supported for conditional LTM.

8. The Early TA is signalled to the UE from the source cell (i.e., not from the candidate cell directly to the UE). This agreement will be included in the LS to RAN1/3/4.

9. The network can inform the candidate cell’s TA information to UE via new MAC CE, which is the TA value when UE switches to that candidate cell during CLTM.

10. Candidate cell TA is maintained by a new timer.

RAN2 made the following agreements regarding Early TA acquisition for intra-CU C-LTM.

Based on the agreements above, we think that for the network-triggered Early TA acquisition procedure, the related Rel-18 intra-CU LTM procedures can be simply reused.

Proposal 8. The Rel-18 intra-CU LTM Early TA RACH configuration and TA Information Transfer procedures can be reused for network-triggered Early TA acquisition for intra-CU conditional LTM.

# 3 Conclusion

Based on the above discussion, we recommend that RAN3 discuss the following observations and proposals.

Proposal 1. The gNB-CU initiates intra-CU C-LTM preparation and may transmit to an underlying candidate gNB-DU the UE Context Setup Request message including a suggested candidate cell for LTM preparation. The UE Context Setup Request message includes:

- Suggested candidate cell ID,

- Intra-CU C-LTM indication.

Proposal 2: If an LTM candidate cell belongs to the source gNB-DU, the gNB-CU also transmits to the source gNB-DU the UE Context Modification Request message including a suggested candidate cell for LTM preparation. The UE Context Modification Request message includes:

- Suggested candidate cell ID,

- Intra-CU C-LTM indication.

Proposal 3. The preparation of candidate cells in intra-CU C-LTM can take place in parallel, similar as in Rel-18 intra-CU LTM.

Proposal 4. If the candidate gNB-DU accepts the LTM configuration request, it generates and transmits the lower layer configuration of the prepared LTM candidate cell in its response to the gNB-CU in UE Context Setup Response.

Similarly, if the source gNB-DU accepts the LTM configuration request, it generates and transmits the lower layer configuration of the prepared LTM candidate cell in its response to the gNB-CU in UE Context Modification Response.

Proposal 5. Upon receiving information regarding the prepared LTM candidate cells from its underlying candidate gNB-DUs and the source gNB-DU, the gNB-CU generates the L3 execution conditions for C-LTM:

- The gNB-CU generates the L3 execution conditions for cell switch from the source cell to other LTM candidate cells, as well as the execution conditions for cell switch from an LTM candidate cell to other candidate cells, i.e., for subsequent LTM.

Proposal 6. Based on the information regarding all the prepared LTM candidate cells:

- The source gNB-DU generates the L1 execution conditions for the initial LTM cell switch and may also generate the L1 execution conditions for a subsequent LTM cell switch.

- A candidate gNB-DU generates the L1 execution conditions for subsequent LTM cell switch.

Proposal 7. Each candidate gNB-DU and the source gNB-DU transmit the L1 execution conditions for the initial LTM cell switch and subsequent LTM cell switch to the gNB-CU in the UE Context Modification Response messages.

Proposal 8. The Rel-18 intra-CU LTM Early TA RACH configuration and TA Information Transfer procedures can be reused for network-triggered Early TA acquisition for intra-CU conditional LTM.

# References

[1] Chair notes, RAN2 #127-bis meeting.

[2] Chair notes, RAN2 #128 meeting.