3GPP TSG-RAN WG3 Meeting #127	R3-250195

Athens, Greece, 17-21 Feb, 2025

Title:	Consideration on SA2 Ambient IoT TR conclusion update

Agenda Item:	8.1

Source:	Huawei, China Unicom

Document for:	Discussion

# Introduction

In SA2#166 adhoc e-meeting, SA2 sent an LS to RAN3 in [1], informed us their latest progress and asked a question to us, in this contribution, we discuss the question from SA2 and provide our consideration on the reply.

# Discussion

The question asked by SA2 is:

[Question to RAN3] SA2 has concluded for topology 1, the AIOTF selects the AIOT RAN node(s) and optionally provides a list of the BS readers to AIOT RAN. SA2 assumes that OAM can configure the AIOTF with needed information for this selection. SA2 have further discussed whether the AIOT RAN may provide the AIOT RAN information e.g., supported AIOT RAN serving area, supported BS reader ID list and corresponding BS reader serving area to the core network. SA2 kindly asks RAN3 to clarify whether RAN3 plan to support the above AIOT RAN information exchange with AIOTF. If so, SA2 kindly ask RAN3 to provide the definition of the appropriate parameters.

In the study item discussion, the following was concluded by RAN3:

Reader selection may need coordination between A-IoT RAN node and A-IoT CN. NOTE 5: How to perform reader selection needs further discussions. NOTE 6: In Topology 1, whether the AIoTF needs to be aware of the readers served by the A-IoT RAN node and their location and how this information is provided to the AIoTF needs further discussion.

The Inventory Request from the A-IoT CN to the A-IoT RAN, may include the following: … (2) Scope of inventory request (e.g., a certain area in which the inventory is to be triggered)

### 6.9.1 General A-IoT device location information may be used for the following purposes: (a) improving the A-IoT operation itself, e.g., by A-IoT CN sending a Command to one or more readers (e.g., the last reader(s)) associated to the device rather than sending it blindly. …

### 6.9.2 Topology 1 For support of A-IoT device location, the A-IoT RAN node may report the A-IoT RAN node ID to the A-IoT CN in the Inventory Report or Command Response messages. The A-IoT RAN node may additionally report a Reader ID to the A-IoT CN in the Inventory Report and/or the Command Response messages, which corresponds to the reader to which the A-IoT device responded. NOTE 1: Whether multiple Reader ID(s) can be included by the A-IoT RAN node in the Inventory Report/Command Response message needs further discussion. NOTE 2: Definition and format of the A-IoT RAN node ID and Reader ID needs further discussion. NOTE 3: Whether and how to have awareness of Reader location in A-IoT CN needs further discussion.

Observation 1: It is needed to support inventory triggered by AIOTF towards dedicated reader(s).

Observation 2: It is needed to support inventory triggered by AIOTF towards a certain area.

To support inventory towards dedicated reader(s), the AIOTF needs to aware of readers served by the gNB. To support inventory towards a certain area, the AIOTF needs to aware of inventory areas supported by the gNB. Such information should be reported by the gNB in NG Setup Request, and/or OAM configuration.

From our view, in order to support reader id granularity device locating, the AIOTF has to be aware of the location of the readers, comparing with reporting reader location repeatly at every inventory report, it is much more efficient to provide the location of readers to the AIOTF also in NG Setup Request, and/or OAM configuration.

Proposal: Reply to SA2 that RAN3 concluded that the AIOTF is able to get the mentioned information, via NG signalling, and/or OAM configuration, details subject to work item discussion.

# Conclusion

In this contribution, we discussed the question asked by SA2 by taking all previous discussion into account, and get the following proposal:

Proposal: Reply to SA2 that RAN3 concluded that the AIOTF is able to get the mentioned information, via NG signalling, and/or OAM configuration, details subject to work item discussion.

The draft reply LS is provided in [3].

# Reference

[1] S2-2501343 LS on SA2 Ambient IoT TR conclusion update, Source: SA2, To: RAN3

[2] TR 38.769 Study on solutions for Ambient IoT (Internet of Things) in NR

[3] R3-250196 [Draft] Reply LS on SA2 Ambient IoT TR conclusion update, Source: Huawei [to be RAN3], To: SA2