3GPP TSG-RAN WG3 #127	R3-250203

Athens, GR, 17 – 21 Feb. 2025

Agenda Item:	31.2. Enhancements

Source:	NTT DOCOMO INC., Nokia

Title:	NR to LTE mobility restriction

Document for:	Discussion, approval

Introduction

In TS 38.413, Mobility Restriction List IE is specified, and this IE can be included in some messages (e.g. HANDOVER REQUEST) to indicate restriction information for the UE from AMF to gNB. This IE includes Forbidden TACs within Forbidden Area Information to which the UE is prohibited to access. However, this IE refers TAC for NR defined as 3 octets, hence current NGAP specification does not support to configure 2 octets TAC for E-UTRAN as a forbidden TAC.

TS 23.501 specifies the high level principle of Mobility Restriction. This function refers to the forbiddenAreas defined in TS 29.503, and this arrey(Area) refers to the Tac defined in TS 29.571, which is defined as 2 or 3 octets.

This contribution discusses the enhancement to NGAP specification and LS to SA2 to realize above enhancement.

Discussion

First, in the Mobility Restriction function of NGAP, CR is attached as an enhancement to configure the forbidden TAC for EPS TAC. This CR introduces a new Forbidden Area Information EPS to realize both backward compatibility and configuration of forbidden EPS TAC.

Next, we discuss the current understanding of 5GC specifications. In TS 29.503 sec. 6.1.6.2.4, which specifies the forbiddenAreas exchanged via SBI in 5GC, describes this IE as "List of forbidden areas in 5GS," and the array(Area) referred to by this IE is defined as 2 or 3 octets in TS 29.571 sec. 5.4.2. Our understanding is that NR TAC is defined as 3 octets in TS 23.003, so it can be implied as EPS TAC when forbiddenAreas is 2 octets and NR TAC when it is 3 octets. On the other hand, these specifications do not explicitly specify that EPS TAC can be configured for forbiddenAreas. If RAN3 companies have concerns about this ambiguity, we can send LS to SA2 and request clarification on the definition of forbiddenAreas.

Proposal:	RAN3 to agree to send attached LS.

After receiving feedback from SA2, RAN3 can specify this enhancement in NGAP specification.

Conclusions and proposals

Our proposals are summarized below.

Proposal:	RAN3 to agree to send attached LS.

Annex A: draft CR for TS 38.413

3GPP TSG-RAN WG3 Meeting # 127	R3-25xxxx

Athens, GR,  17th – 21st February, 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.413 | CR | XXXX | rev | - | Current version: | 19.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | x | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Enhancement on mobility restriction |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | NTTDOCOMO, INC. |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R3 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_newRAT-Core |  |  |  |  |  | Date: |  |  | 2025-2-7 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | In TS 38.413, Mobility Restriction List IE is specified, and this IE can be included in some messages (e.g. HANDOVER REQUEST) to indicate restriction information for the UE from AMF to gNB. This IE includes Forbidden TACs within Forbidden Area Information to which the UE is prohibited to access. However, this IE refers TAC for NR defined as 3 octets, hence current NGAP specification does not support to configure 2 octets TAC for E-UTRAN as a forbidden TAC. On the other hand, in TS 23.501 specifies mobility restriction function referring 2 or 3 octets TAC in TS 29.571. Hence AMF supports to configure forbidden TAC both as 2 octets or 3 octets. This CR enables to configure 2 octets TAC as a forbidden TAC in NGAP specification. Impact Analysis: Impact assessment towards the previous version of the specification (same release): This CR has isolated impact with the previous version of the specification (same release) on Mobility Restriction functionality. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Add new Forbidden Area Information EPS IE within Mobility Restriction List IE to indicate 2 octets forbidden TAC(s) reffering 9.3.3.16 EPS TAC. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Configuration of forbidden area with 2 octets TAC(s) cannot be supported in TS 38.413. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 9.3.1.85 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

Start of Change

9.3.1.85	Mobility Restriction List

This IE defines roaming or access restrictions for subsequent mobility action for which the NG-RAN provides information about the target of the mobility action towards the UE, e.g., handover, or for SCG selection during dual connectivity operation or for assigning proper RNAs. NG-RAN behaviour upon receiving this IE is specified in TS 23.501 [9].

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Serving PLMN | M |  | PLMN Identity 9.3.3.5 |  | - |  |
| Equivalent PLMNs |  | 0..<maxnoofEPLMNs> |  | Allowed PLMNs in addition to Serving PLMN. This list corresponds to the list of "equivalent PLMNs" as defined in TS 24.501 [26]. This list is part of the roaming restriction information. Roaming restrictions apply to PLMNs other than the Serving PLMN and Equivalent PLMNs. | - |  |
| >PLMN Identity | M |  | 9.3.3.5 |  | - |  |
| RAT Restrictions |  | 0..<maxnoofEPLMNsPlusOne> |  | This IE contains RAT restriction related information as specified in TS 23.501 [9]. | - |  |
| >PLMN Identity | M |  | 9.3.3.5 |  | - |  |
| >RAT Restriction Information | M |  | BIT STRING { e-UTRA (0), nR (1), nR-unlicensed (2), nR-LEO (3), nR-MEO (4), nR-GEO (5), nR-OTHERSAT (6)} (SIZE(8, …)) | Each position in the bitmap represents a RAT. If a bit is set to "1", the respective RAT is restricted for the UE. If a bit is set to "0", the respective RAT is not restricted for the UE. Bit 7 reserved for future use. | - |  |
| >Extended RAT Restriction Information | O |  | 9.3.1.126 | If this IE is included, the RAT Restriction Information IE is ignored. | YES | ignore |
| Forbidden Area Information |  | 0..<maxnoofEPLMNsPlusOne> |  | This IE contains Forbidden Area information as specified in TS 23.501 [9]. | - |  |
| >PLMN Identity | M |  | 9.3.3.5 |  | - |  |
| >Forbidden TACs |  | 1..<maxnoofForbTACs> |  |  | - |  |
| >>TAC | M |  | 9.3.3.10 | The TAC of the forbidden TAI. | - |  |
| Forbidden Area Information EPS |  | 0..<maxnoofEPLMNsPlusOne> |  | This IE contains Forbidden Area information as specified in TS 23.501 [9]. | - |  |
| >PLMN Identity | M |  | 9.3.3.5 |  | - |  |
| >Forbidden EPS TACs |  | 1..<maxnoofForbEPSTACs> |  |  | - |  |
| >>EPS TAC | M |  | 9.3.3.16 | The EPS TAC of the forbidden EPS TAI. | - |  |
| Service Area Information |  | 0..<maxnoofEPLMNsPlusOne> |  | This IE contains Service Area Restriction information as specified in TS 23.501 [9]. | - |  |
| >PLMN Identity | M |  | 9.3.3.5 |  | - |  |
| >Allowed TACs |  | 0..<maxnoofAllowedAreas> |  |  | - |  |
| >>TAC | M |  | 9.3.3.10 | The TAC of the allowed TAI. | - |  |
| >Not Allowed TACs |  | 0..<maxnoofAllowedAreas> |  |  | - |  |
| >>TAC | M |  | 9.3.3.10 | The TAC of the not-allowed TAI. | - |  |
| Last E-UTRAN PLMN Identity | O |  | PLMN Identity 9.3.3.5 | Indicates the E-UTRAN PLMN ID from where the UE formerly handed over to 5GS and which is preferred in case of subsequent mobility to EPS. | YES | ignore |
| Core Network Type Restriction for Serving PLMN | O |  | ENUMERATED (EPCForbidden,…) | Indicates whether the UE is restricted to connect to EPC for the Serving PLMN as specified in TS 23.501 [9]. | YES | ignore |
| Core Network Type Restriction for Equivalent PLMNs |  | 0..<maxnoofEPLMNs> |  |  | YES | ignore |
| >PLMN Identity | M |  | 9.3.3.5 | Includes any of the Equivalent PLMNs listed in the Mobility Restriction List IE for which CN Type restriction applies as specified in TS 23.501 [9]. | - |  |
| >Core Network Type Restriction | M |  | ENUMERATED (EPCForbidden, 5GCForbidden,…) | Indicates whether the UE is restricted to connect to EPC or to 5GC for this PLMN. | - |  |
| NPN Mobility Information | O |  | 9.3.1.184 |  | YES | reject |

| Range bound | Explanation |
| --- | --- |
| maxnoofEPLMNs | Maximum no. of equivalent PLMNs. Value is 15. |
| maxnoofEPLMNsPlusOne | Maximum no. of allowed PLMNs. Value is 16. |
| maxnoofForbTACs | Maximum no. of forbidden Tracking Area Codes. Value is 4096. |
| maxnoofForbEPSTACs | Maximum no. of forbidden Evolved Packet System Tracking Area Codes. Value is 4096. |
| maxnoofAllowedAreas | Maximum no. of allowed or not allowed Tracking Areas. Value is 16. |

End of Change

Annex B: LS to SA2, CC to CT4

3GPP TSG-RAN WG3 #127	R3-25xxxx

Athens, GR, 17 – 21 Feb. 2025

Title:	[Draft] LS on NR to LTE mobility restriction

Response to:	-

Release:	Rel-19

Work Item:	NR_newRAT-Core

Source:	NTT DOCOMO (to be RAN3)

To:	SA2

Cc:	CT4

Contact Person: 	Mio Nakamura

mio.nakamura.ue@nttdoomo.com

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

Attachments:	None

- Overall Description:

In the current NGAP specification, only NR TAC can be configured as a forbidden area for mobility restriction. RAN3 discussed enhancement on the mobility restriction function to enable configuring EPS TAC as a forbidden area. On the other hand, we could not conclude whether this enhancement affects the core network related specifications because there is some ambiguity as to whether forbiddenAreas specified in 29.503 support specifying EPS TAC.

- Actions:

RAN3 kindly asks SA2 to provide feedback on whether the Mobility Restriction function specified in TS 23.501 and TS 29.503 allows to configure EPS TAC defined by 2 octet as forbidden area defined in TS 29.571. Also, if necessary, please provide references to other appropriate specifications for this function or introduce necessary clarification on those specifications.

- Date of Next TSG-RAN3 Meetings:

TSG-RAN3 Meeting #127bis	7th – 11th April 2025		China

TSG-RAN3 Meeting #128	19th – 23th May 2025		Malta