3GPP TSG-RAN WG3 Meeting #127	R3-250208

Athens, Greece, 17 – 21 February 2025

Agenda Item:	22.2

Source: 				LG Electronics Inc.

Title:			Discussion on service continuity in Multi-hop relay

Document for:		Discussion and Decision

1.	Introduction

New Rel-19 WID on NR sidelink multi-hop relay was agreed at RAN #104 [1]. One of the objectives in this WID is to specify intra-gNB service continuity scenarios for Multi-hop U2N Relay as follows:

… The objective of this work item is to specify solutions that are needed to support multi-hop Layer-2 UE-to-Network relay for a single indirect path via SL relay UEs based on Rel-17/18 SL relay functionalities [RAN2, RAN3] - Specify mechanisms to support up to two additional hops relays on top of Rel-17 U2N relay. The work starts with one additional hop relay (i.e., remote UE -> first relay UE -> last relay UE -> gNB) until RAN#107 and further check will be made in RAN#107 if it can be easily extended to two additional hops relays (i.e., remote UE -> first relay UE -> second relay UE -> last relay UE -> gNB). A necessary criterion for the specified mechanisms is easy extensibility to support two additional hop relays for the work done until RAN#107 and to be forward compatible for future extensions for additional relays. - Relay discovery and (re)selection [RAN2] - Signalling support for relay UEs and remote UE authorization if SA2 concludes it is needed [RAN3] - Impact on SRAP and QoS handling for multi-hop [RAN2] - Control plane procedures [RAN2, RAN3] - Specify the following intra-gNB service continuity scenarios for multi-hop U2N relay based on Rel-17/18 procedures (for remote UE): First Priority: - Intra-gNB multi-hop indirect to direct path switching using existing framework - Intra-gNB multi-hop indirect to single-hop indirect path switching using existing framework Second Priority in order of importance: - Intra-gNB direct to multi-hop indirect path switching - Intra-gNB single-hop indirect to multi-hop indirect path switching The scenarios C and D are limited to path switching to a target indirect path consisting of the last relay UE in “direct” RRC Connected mode and all the other intermediate relay(s) in “indirect” RRC Connected mode to the same cell. NOTE: The current existing measurement framework and existing data forwarding mechanism should be reused. …

In this contribution, we will discuss the RAN3 impacts on intra-gNB service continuity for Multi-hop U2N Relay, and provide our view on it.

2.	Discussion

In RAN2#128 meeting, the following was agreed in [2]:

Agreement in RAN2#128:

For scenario B, RAN2 supports the case that the target U2N relay UE is a new relay UE which is not on the source relay path, and existing Rel-18 indirect path to indirect path switching can be reused.

For scenario B, RAN2 will support the case that the target single-hop relay UE is the last relay UE on the source path using the existing Rel-18 i2i path switching.  Spec impact will be minimised.

For scenario B, RAN2 does not specify anything to support the case that that the target U2N relay UE is an intermediate relay UE which is on the source relay path using a single procedure.  This case can be handled in the baseline control plane model, if necessary and subject to network implementation, by sequential path switches for the remote UE and the target intermediate relay UE.

Agreements for baseline control plane model:

The intra-gNB switching from indirect to direct path procedure defined in TS 38.300 for single hop case can be used as the baseline for multi-hop indirect to direct path switching.

Event X1 /X2 can be reused in multi-hop indirect to direct path switching with the understanding that the “first relay UE” in multi-hop relay link is “serving L2 U2N Relay UE” to be reported.

The intra-gNB switching from indirect path to indirect path procedure defined in TS 38.300 in single hop case can be used as baseline for multi-hop indirect to single hop indirect path switching procedure.

The following measurement events can be reused in multi-hop indirect to single-hop indirect path switching:

-	Event Y2

-	Event Z1 with the understanding that the “first relay UE” in multi-hop relay link is “serving L2 U2N Relay UE” to be reported.

Agreements:

Figure 1 of R2-2410589 can be used as a baseline for intra-gNB multi-hop indirect to direct path switching procedure to be captured in TS 38.300.

Figure 2 of R2-2410589 can be used as a baseline for intra-gNB multi-hop indirect to single-hop indirect path switching to be captured in TS 38.300.

According to RAN2 agreements, the Rel-17 intra-gNB switching from single-hop indirect to direct path procedure in clause 16.12.6.1 of TS 38.300 can be used as the baseline for Scenario A (i.e., Multi-hop indirect to direct path switching). Also, it was agreed in RAN2 that Figure 1 of R2-2410589 can be used as a baseline for Scenario A as follows:

R2-2410589 [3]: Figure 1: Intra-gNB multi-hop indirect to direct path switching procedure Step 1. Remote UE triggers the measurements report when the sidelink measurement quantity between First Relay UE and Remote UE indicates deteriorated link quality. Step 2. The gNB decides to switch the L2 U2N Remote UE onto direct Uu path. Step 3. The gNB sends the RRCReconfiguration message to the L2 U2N Remote UE. The L2 U2N Remote UE stops User Plane and Control Plane transmission via the First Relay UE after reception of the RRCReconfiguration message with the path switch configuration. Step 4. The Remote UE performs Random Access procedure to gNB. Step 5. The UE (i.e., L2 U2N Remote UE in previous steps) sends the RRCReconfigurationComplete message to the gNB via the direct path, using the configuration provided in the RRCReconfiguration message. From this step, the UE (i.e., L2 U2N Remote UE in previous steps) uses the RRC connection via the direct path to the gNB . Step 6. The gNB sends the RRCReconfiguration message to the Relay UE(s) (last relay, intermediate relay and the first relay UE) to reconfigure the connection between the Relay UEs and the gNB. The RRCReconfiguration message to the L2 U2N Multi-hop Relay UEs can be sent any time after step 3 based on gNB implementation (e.g., to release Uu Relay RLC Channel and PC5 Relay RLC channel configuration for relaying, and bearer mapping configuration related to the L2 U2N Remote UE ). Step 7. Either L2 U2N First Relay UE or L2 U2N Remote UE's AS layer indicates upper layers to release PC5 unicast link after receiving the RRCReconfiguration message from the gNB. The timing to execute link release is up to UE implementation . Step 8. The data path is switched from multi-hop indirect path to direct path between the UE (i.e., previous L2 U2N Remote UE) and the gNB. The PDCP re-establishment or PDCP data recovery in uplink is performed by the UE (i.e., previous L2 U2N Remote UE) for lossless delivery during path switch if gNB configures it . NOTE 1: Step 8 can be executed any time after step 4. Step 8 is independent of step 6 (includes 6-1, 6-2 and 6-3) and step 7.

From RAN3’s point of view, when the gNB-DU serving the direct path and the gNB-DU serving the multi-hop relays are different, the existing Inter-gNB-DU mobility procedure in Figure 8.2.1.1-1 of TS 38.401 can be used as a baseline to support Scenario A. For this case, the gNB-CU can send the UE CONTEXT SETUP REQUEST message for the Remote UE to the target gNB-DU, which contains the HandoverPreparationInformation IE and the serving cell ID on the direct path. Hence, there is no need to enhance the UE Context Setup procedure.

Proposal 1: For the inter-gNB-DU switch from multi-hop indirect to direct path, the existing Inter-gNB-DU mobility procedure in Figure 8.2.1.1-1 of TS 38.401 can be used as a baseline.

Proposal 1a: Without any enhancements, the existing UE Context Setup procedure is reused to support the inter-gNB-DU switch from multi-hop indirect to direct path.

For the case where the gNB-DU serving the direct path and the gNB-DU serving the multi-hop relays are same, the existing intra-gNB-DU handover procedure in Clause 8.2.1.2 of TS 38.401 can be used as a baseline. Similar to the inter-gNB-DU case, the UE CONTEXT MODIFICATION REQUEST message for the Remote UE is used to support the intra-gNB-DU case in Scenario A. Also, this message already contains the HandoverPreparationInformation IE and the serving cell ID on the direct path.

Proposal 2: For the intra-gNB-DU switch from multi-hop indirect to direct path, the existing intra-gNB-DU handover procedure in Clause 8.2.1.2 of TS 38.401 can be used as a baseline.

Proposal 2a: Without any enhancements, the existing UE Context Modification procedure is reused to support the intra-gNB-DU switch from multi-hop indirect to direct path.

For example, the inter-gNB-DU switch from multi-hop indirect to direct path can be illustrated in Figure 2.

Figure 2: Overall procedure for inter-gNB-DU switch from multi-hop indirect to direct path

From F1 signaling point of view, there is no big difference between the existing Inter-gNB-DU mobility procedure and the inter-gNB-DU switch from multi-hop indirect to direct path. The main difference is that in inter-gNB-DU switch from multi-hop indirect to direct path, the gNB sends the RRCReconfiguration message to the Relay UE(s) to reconfigure the connection for source relay path. However, this point can be covered by the RAN2 baseline procedure. Hence, there is no need for the RAN3 to capture inter-gNB-DU switch from multi-hop indirect to direct path into TS 38.401. Also, in Rel-17 U2N Relay, it was already agreed in RAN3 that due to a similar reason, the overall procedure for path switch from single-hop indirect to direct path is not introduced in TS 38.401.

Proposal 3: No need to capture the procedure for Scenario A into TS 38.401.

RAN2 already agreed that Rel-18 existing indirect path to indirect path switching can be used as a basis for Scenario B. Also, it was agreed in RAN2 that Figure 2 of R2-2410589 can be used as a baseline for Scenario B as follows:

R2-2410589 [3]: Figure 3. Intra-gNB multi-hop indirect to single-hop indirect path switching procedure Step 1. Remote UE reports one or multiple candidate Relay UE(s), when the sidelink measurement quantity between First Relay UE and Remote UE or Relay UE and next/last-hop Relay UE shows deteriorated link quality. Step 2. The gNB decides to switch the L2 U2N Remote UE to a target L2 U2N Relay UE under the same gNB. Step 3. The gNB sends an RRCReconfiguration message to the target L2 U2N Relay UE, which includes at least the L2 U2N Remote UE's local ID and L2 ID, Uu and PC5 Relay RLC channel configuration for relaying, and bearer mapping configuration. Step 4. The gNB sends the RRCReconfiguration message to the L2 U2N Remote UE. The RRCReconfiguration message includes at least the target L2 U2N Relay UE ID, Remote UE's local ID, PC5 Relay RLC channel configuration for relay traffic, and the associated end-to-end radio bearer(s). The L2 U2N Remote UE stops UP and CP transmission over the multi-hop indirect path via the source L2 U2N multi-hop relay after the reception of the RRCReconfiguration message from the gNB. Step 5. The L2 U2N Remote UE establishes PC5-RRC connection with the target L2 U2N Relay UE. Step 6. The L2 U2N Remote UE sends the RRCReconfigurationComplete message to the gNB via the target L2 U2N Relay UE, using the configuration provided in the RRCReconfiguration message. From this step, the L2 U2N Remote UE uses the RRC connection via the target L2 U2N Relay UE to the gNB . Step 7. The gNB sends the RRCReconfiguration message to the source L2 U2N multi-hop relay to reconfigure the connection between the source L2 U2N multi-hop relay UEs and the gNB. The RRCReconfiguration message to the source L2 U2N multi-hop relay UEs can be sent any time after step 4 based on gNB implementation (e.g., to release Uu and PC5 Relay RLC channel configuration for relaying, and bearer mapping configuration related to the L2 U2N Remote UE). Step 8. Either source First L2 U2N Relay UE's AS layer or L2 U2N Remote UE's AS layer indicates upper layers to release PC5 unicast link after receiving the RRCReconfiguration message from the gNB. The timing to execute link release is up to UE implementation after step 4 or step7. Step 9. The data path is switched from the source L2 U2N Relay UE to the target L2 U2N Relay UE between the L2 U2N Remote UE and the gNB. This step can be any time after step 7.

In Rel-17 intra-gNB switch from direct to indirect path and Rel-18 intra-/inter-gNB switch from indirect to indirect path, the gNB-CU sends the Path Switch Configuration IE to the gNB-DU. We think that in Rel-19 Multi-hop Relay, this IE can be reused to support Scenario B (i.e., path switch from multi-hop indirect to single-hop indirect). However, this is pending to RAN2 decision.

Proposal 4: Whether to reuse the existing Path Switch Configuration IE for support of Scenario B is pending to RAN2 progress.

RAN2 decides not to specify the single procedure to support the path switch case that that the Target relay UE is the intermediate relay UE belonging to the source relay path. RAN2 considers that this case can be supported by sequential path switches for the Remote UE and the Target Relay UE. Therefore, the single procedure to support this case does not need to be considered in RAN3 as well.

Observation 1: RAN2 decides not to specify the single procedure to support the path switch case that that the Target relay UE is the intermediate relay UE belonging to the source relay path.

However, RAN2 supports the case that the target single-hop Relay UE is the last Relay UE on the source path. From RAN3’s perspective, this means that the gNB-DU serving the Remote UE is not changed during the path switch. In this case, the gNB-CU initiates the UE Context Modification procedure to the gNB-DU to inform the path switch from the multi-hop indirect to single-hop indirect path. As discussed in Proposal 4, we think that the Path Switch Configuration IE can be included in the UE CONTEXT MODIFICATION REQUEST message. Also, the gNB-CU can request the configuration for the PC5 Relay RLC channel between Remote UE and Target Relay UE by using this F1AP message. Therefore, it seems that no enhancement is needed for the intra-gNB-DU case for Scenario B.

Regarding the case that Target Relay UE is a new Relay UE which is not on the source relay path, there may be two sub-cases. When the gNB-DU serving the source relay path and the gNB-DU serving the Target Relay UE are same, the Remote UE moves from one cell serving the source relay path to another cell serving the Target Relay UE within the same gNB-DU. As mentioned above, this can be supported by the existing procedure without no spec impact.

For the case where the gNB-DU serving the source relay path and the gNB-DU serving the Target Relay UE are different, the inter-gNB-DU switch from multi-hop indirect to single-hop indirect path can be illustrated in Figure 4.

Figure 4: Overall procedure for inter-gNB-DU switch from multi-hop indirect to single-hop direct path

In Step 4, the gNB-CU sends to the target gNB-DU the UE CONTEXT SETUP REQUEST message containing the existing Path Switch Configuration IE. Upon reception of this IE, the gNB-DU becomes aware of the inter-gNB-DU switch from multi-hop indirect to single-hop indirect path, and then generates the ReconfigurationWithSync IE including the sl-PathSwitchConfig-r17 IE. Without any enhancements, the F1 signaling can support the inter-gNB-DU switch from multi-hop indirect to single-hop indirect path.

Proposal 5: Without any enhancements, the existing UE Context Setup procedure is reused to support the inter-gNB-DU switch from multi-hop indirect to single-hop indirect path.

Proposal 6: Without any enhancements, the existing UE Context Modification procedure is reused to support the intra-gNB-DU switch from multi-hop indirect to single-hop indirect path.

As discussed in Proposal 3, from F1 signaling point of view, there is no big difference between the procedure for the inter-gNB-DU switch from single-hop indirect to single-hop indirect path and the inter-gNB-DU switch from multi-hop indirect to single-hop indirect path. Also, in Rel-18 U2N Relay enhancement, it was already agreed in RAN3 that the overall procedure for path switch from single-hop indirect to single-hop indirect path is not introduced in TS 38.401. Hence, there is no need for the RAN3 to capture inter-gNB-DU switch from multi-hop indirect to single-hop indirect path into TS 38.401

Proposal 7: No need to capture the overall procedure for Scenario B into TS 38.401.

3.	Conclusion

In this contribution, we focused on the RAN3 impacts on intra-gNB service continuity for Multi-hop U2N Relay, and then provided our view on it. The following proposals are kindly suggested to RAN3:

Proposal 1: For the inter-gNB-DU switch from multi-hop indirect to direct path, the existing Inter-gNB-DU mobility procedure in Figure 8.2.1.1-1 of TS 38.401 can be used as a baseline.

Proposal 1a: Without any enhancements, the existing UE Context Setup procedure is reused to support the inter-gNB-DU switch from multi-hop indirect to direct path.

Proposal 2: For the intra-gNB-DU switch from multi-hop indirect to direct path, the existing intra-gNB-DU handover procedure in Clause 8.2.1.2 of TS 38.401 can be used as a baseline.

Proposal 2a: Without any enhancements, the existing UE Context Modification procedure is reused to support the intra-gNB-DU switch from multi-hop indirect to direct path.

Proposal 3: No need to capture the procedure for Scenario A into TS 38.401.

Proposal 4: Whether to reuse the existing Path Switch Configuration IE for support of Scenario B is pending to RAN2 progress.

Proposal 5: Without any enhancements, the existing UE Context Setup procedure is reused to support the inter-gNB-DU switch from multi-hop indirect to single-hop indirect path.

Proposal 6: Without any enhancements, the existing UE Context Modification procedure is reused to support the intra-gNB-DU switch from multi-hop indirect to single-hop indirect path.

Proposal 7: No need to capture the overall procedure for Scenario B into TS 38.401.

4.	Reference

- RP-242349, “Revised WID on NR sidelink multi-hop relay,” LG Electronics, InterDigital, FirstNet, September, 2024.

- R2-24xxxxx, “Report from session on positioning and sidelink relay,” Session Chair (MediaTek), November, 2024.

- R2-2410589, “Discussion on service continuity for Multi-hop Relay,” Huawei, HiSilicon, November, 2024.