3GPP TSG-RAN WG3 Meeting #127															R3-250213

Athens, Greece, 17th – 21st Feb 2025

Agenda item:	8.1

Source:	Nokia, Nokia Shanghai Bell

Title:	Discussion on the LS related to XR

Document for:	Discussion and Decision

# 1	Introduction

RAN3 received a LS ([1]) from RAN2 on XR UL bit rate control.

RAN3 also received a reply LS ([2]) from SA2 on available data rate reporting.

RAN3 also received LS from SA4 and SA on multi-modality awareness.

This contribution analyses these LS.

# 2	Discussion on RAN2 LS on XR UL bit rate control

RAN2 LS ([1]) describes following:

1. Overall Description: RAN2 has made the following agreement regarding a solution on UL bit rate control - Rate indication from gNB to the UE on a per QoS flow level is supported. FFS the details, e.g. if: 1) flows are indicated by MAC CE or 2) by RRC while MAC CE is per DRB. 2. Actions: To SA4, RAN3: ACTION: RAN2 kindly asks SA4 and RAN3 to take above into consideration and provide feedback on the agreement provided, if any.

Regarding whether the XR related control is per DRB or per QoS flow, RAN3 have discussed similar issues in Rel-18, and agreed the XR related control is performed per DRB. For example,

- TS 38.473:

If the ECN Marking or Congestion Information Reporting Request IE is included in the UE CONTEXT SETUP REQUEST message, the gNB-DU shall, if supported, use it accordingly for the specific DRB.

If the PSI based SDU Discard UL IE is included in the DRB To Be Setup List IE, the gNB-DU shall, if supported, take it into account to perform UL PSI based SDU discarding activation or deactivation for the indicated DRB as defined in TS 38.321 [16].

- TS 38.425:

5.5.3.61 UL Congestion Information

Description: For the cases of ECN marking Request, this field indicates the percentage of UL IP packets up to two decimal points that should be ECN marked for a DRB.

For the case of Congestion Information Request, this field should be interpreted as a percentage of congestion level in UL up to two decimal points for a DRB.

5.5.3.62 DL Congestion Information

Description: For the cases of ECN marking Request, this field indicates the percentage of DL IP packets up to two decimal points that should be ECN marked for a DRB.

For the case of Congestion Information Request, this field should be interpreted as a percentage of congestion level in DL up to two decimal points for a DRB.

The RAN2 agreement about per QoS flow Rate indication requires a different handling than current XR related control in F1 and gNB-DU.

Observation: in current F1, the control of ECN marking or congestion information reporting, and PSI based SDU discard UL is performed per DRB.

To support the per QoS flow Rate indication, it requires further enhancement to gNB-DU, and the F1AP interface. For example,

- The gNB-DU need to know which QoS flow requires rate indication. So the gNB-CU need to provide this indication to the gNB-DU. The new indication needs to be added in QoS flow level, instead of the DRB level indication used for ECN marking or congestion information reporting or PSI based discard UL.

- The gNB-DU performs rate control per QoS flow.

The other question is how the gNB know the QoS Flow requiring rate indication. This question may need to be considered together with other XR related control information that gNB received from the CN, e.g. ECN marking or congestion information reporting, MMSID, available data reporting, etc. It may be straightforward that this information is also received from CN. So RAN3 can assume CN provides gNB about which QoS flow requires rate control, unless RAN2 has concern.  This question can be included in the reply LS to RAN2.

Proposal 1: RAN3 reply to RAN2 based on following:

- current XR related control (i.e. the control of ECN marking or congestion information reporting, and PSI based SDU discard UL) in F1 interface is per DRB.

- If need to support the per QoS flow Rate indication, F1AP enhancement is needed.

- whether RAN2 has concern on CN providing gNB about which QoS flow requires UL rate control.

The draft Reply LS can be found in [5].

# 3	Discussion on SA2 reply LS on available data rate reporting

The SA2 reply LS ([2]) describes following:

1. Overall Description: SA2 thanks RAN3 for the LS on FS_XRM Ph2 (R3-244844/S2-2411282) and kindly asks RAN3 to consider the following SA2 feedback. - From RAN3 perspective, the control plane based solution providing periodicity per QoS flow as part of the TSCAI is sufficient for semi-static periodicity case. Other potential cases (e.g. dynamic case) need to be clarified by SA2, if any. Feedback from SA2: SA2 has agreed to exclude the in-band signalling of the periodicity in Rel-19. - For semi-static periodicity traffic, it is feasible for NG-RAN node to provide the available data rate to CN via UP. For dynamic traffic, some companies have concern that the reporting of available data rate may not be accurate. RAN3 would like SA2 to further clarify the definition of “available data rate”, e.g., it is only below the GFBR, or between GFBR and MFBR, or even above MBFR; whether it should be periodic reporting, on demand reporting, or threshold(s) defined reporting. Feedback from SA2: From SA2 perspective, it is expected that the available data rate is properly reflecting the resources that are currently provided by the NG-RAN for the QoS Flow. While it is up to the application layer how to make use of it, we can however assume that the application will typically exchange data with a somewhat lower rate than what is reserved or measured at NG-RAN. Regarding the definition of “available data rate”, it refers to the data rate that NG-RAN is currently able to provide for a QoS Flow with GFBR. The “available data rate” can be between the GFBR and the MFBR for a QoS Flow with MFBR>GFBR. The “available data rate” can also be below the GFBR for a QoS Flow with QoS Notification Control enabled without AQP. The “available data rate” should however not be above the MFBR (as the NG-RAN is not required to provide more resources that what is required for the MFBR). Periodic reporting should not be useful for the application layer (as the “available data rate” should often be identical to the GFBR). Threshold based reporting is considered as useful with one or more threshold values provided by the application. When the data rate that NG-RAN is currently able to provide for a QoS Flow crosses one of the thresholds, the NG-RAN is expected to provide the reference to the respective threshold in the UL user plane packets and the application can then adjust its data rate accordingly. The usage of the first packets after a change in the “available data rate” should be sufficient to transfer the information to the UPF. Similar to the logic described in 23.501 clause 5.7.2.4.1a, the NG-RAN is expected to immediately report a change to a lower threshold value for the “available data rate” and a change to a higher threshold value with some hysteresis. 2. Actions: To RAN3 group: ACTION: SA2 kindly asks RAN3 group to take the above into account and to provide feedback if necessary.

For the 1st topic, SA2 has agreed to exclude the in-band signalling of the periodicity in Rel-19. Current RAN3 specs already support the control plane based solution providing periodicity per QoS flow as part of the TSCAI. No further action is required in RAN3.

For the 2nd topic, the SA2 reply is clear that the available data reporting is only for GBR. The gNB can report the available data rate based on the estimation. This requires the enhancements to both control plane (NGAP/F1AP/XnAP), and user plane (NG-U/F1-U).

- For the control plane, the 5GC need to request the gNB about which DRB QoS flow requires the available data reporting. Then the gNB-CU need to request the gNB-DU.  During the mobility, target gNB also need to know it either from source gNB over XnAP, or from AMF over NGAP.

- For the user plane, the gNB-DU need to report the available data report to gNB-CU, which is then further reported to the gNB-CU.

The detailed Stage-2/3 TP can be discussed when the TU is re-assigned to XR in RAN3.

Proposal 2: To support available data reporting for GBR QoS flow, enhancement is needed for Control plane (NGAP/F1AP/XnAP), and user plane (NG-U/F1-U).

# 4	Discussion on SA4 reply LS on multi-modality awareness

For the SA4 reply LS ([3]), it states:

1. Overall Description: SA4 thanks RAN2 LS in S4-241802/R2-2409272 on multi-modality awareness, and would like to provide the feedback as following. - RAN2 considers that based on multi-modal information: - The gNB may perform joint admission control. Details can be left up to RAN3 in potential WI phase. - The gNB may consider this information during QoS flow to DRB mapping (up to gNB implementation). Answer: From an XR service perspective, it is generally desirable that the gNB should try to grant all associated QoS Flows for the XR service whenever possible. However, not all applications require all QoS flows to be granted. If the gNB cannot grant all associated QoS Flows, only the application layer can determine how to handle the case when the QoS flow for one modality cannot be established e.g. due to RAN resource limitations. For instance, it may be acceptable to an application if the QoS flow for video cannot be established with the requested QoS requirements but it may not be acceptable if the QoS flow for audio cannot be established. The conditions under which particular QoS flows are considered essential for an application to continue are application specific and cannot be determined by 5GS. - For DL, whether traffic synchronization (on a per packet basis) can be achieved depends on whether packet level synchronization information can be provided from CN to RAN. - RAN2 thinks PDU Set discard across QoS flows of the same multi-modal service based on the dependency information between the mutli-modal flows can only be achieved in case the synchronization information can be available at the UE which is up to SA2/SA4.. Answer: For DL, from the application perspective, there are well-defined mechanisms to support synchronized playout among different media flows, e.g. RTP timestamps, jitter buffering mechanism in the RTP client. These mechanisms guarantee frame level synchronization but do not provide packet level synchronization among different media flows. There is no evidence to suggest 5GS per packet inter-modal synchronization is needed for XR services, as the very low round trip latency requirements will prevent the system from introducing unacceptable inter-modal skew. Regarding PDU Set discard, it is recommended that the 5GS not jointly discard packets across multi-modal flows. Receiving packets for some modalities is preferred than receiving none at all as this enables the application to use the received modalities to better manage the user experience (e.g., pause the video while rendering received audio packets, or render received video packets while concealing an audio packet drop).

SA4 LS indicates the gNB should not perform joint admission control, since it is acceptable that one or some QoS flow(s) are established in case the gNB cannot grant all associated QoS flows.

SA4 LS also indicates the gNB should not perform join discard.

In a summary, the usage of MMSID is up to gNB’s implementation.

Proposal 3: the gNB should not perform the joint admission control and joint discarding based on multi-modal information. The usage of MMSID is up to gNB’s implementation.

# 5	Conclusion

In this contribution, we have analyzed the LS from RAN2, SA2 and SA4. Our proposals are:

Proposal 1: RAN3 reply to RAN2 based on following:

- current XR related control (i.e. the control of ECN marking or congestion information reporting, and PSI based SDU discard UL) in F1 interface is per DRB.

- If need to support the per QoS flow Rate indication, F1AP enhancement is needed.

- Whether RAN2 has concern on CN providing gNB about which QoS flow requires UL rate control.

The draft Reply LS can be found in [5].

Proposal 2: To support available data reporting for GBR QoS flow, enhancement is needed for Control plane (NGAP/F1AP/XnAP), and user plane (NG-U/F1-U).

Proposal 3: the gNB should not perform the joint admission control and joint discarding based on multi-modal information. The usage of MMSID is up to gNB’s implementation.

# References

- R3-250012	LS on XR UL bit rate control	RAN2(Meta)

- R3-250018	LS Reply on FS_XRM Ph2	SA2(Huawei)

- R3-250027	LS Reply on multi-modality awareness	SA4(Huawei)

- R3-250032	LS  on multi-modality awareness	TSG SA(China mobile)

- R3-250214  [DRAFT] Reply LS on XR UL bit rate control