3GPP TSG-RAN WG3 Meeting #127															R3-250220

Athens, Greece, 17th – 21st Feb 2025

Agenda item:	12.2

Source:	Nokia, Nokia Shanghai Bell

Title:	(TP for TS 38.423 and TS 38.413) Discussion on WAB mobility

Document for:	Discussion and Decision

# 1	Introduction

RAN#105 agreed WI on topological enhancements for NR [1]. The objectives related to WAB mobility are following, [1]:

The objectives of the Wireless Access Backhaul (WAB) work are as follows: - Specifications for the support of WAB including [RAN3]: - … - Handling of WAB-gNB’s traffic (including Xn, NG and OAM traffic) during WAB-node mobility, including the case where the WAB-MT’s BH PDU session changes. - Support the UE’s AMF change for UEs connected to, or camped on, a WAB-gNB. - … NOTE 6: Mobility procedures to be used for the UEs served by a WAB-gNB are legacy UE mobility procedures. Mobility of the WAB-MTs is based on legacy UE mobility procedures.

This contribution analyses the issues related to WAB-node mobility, and the support for UEs sent to RRC_INACTIVE via a WAB-gNB.

# 2	WAB-gNB mobility without change of UE’s AMF(s)

In last RAN3 meeting, the following was agreed:

##### --- Start of TR38.799 text ---

##### 4.3.4.2.1 WAB-gNB mobility without change of UE’s AMF(s)

During WAB-node movement, radio configuration parameters of the WAB-gNB may be changed (e.g., cell ID, PCI and/or TAC) without the change of the UE’s AMF(s). This change of radio configuration parameters may require the UE handling by means of, e.g., intra-gNB handover and/or Mobility Registration Update as defined in TS 23.502 [7].

##### --- End of TR38.799 text ---

There is no Stage-3 impact to RAN3. Stage-2 text may be needed.

Proposal 1: Update Stage-2 to capture the WAB-node mobility without change of UE’s AMF.

The TP can be found in ([3])

# 3	WAB-gNB mobility with change of UE’s AMF(s)

The SI concluded with the approval that two logical WAB-gNB based approach is feasible, and proposing to study three options with single WAB-gNB approach further. SA2 has discussed the issue, and reply RAN3 LS in ([5]). The SA2 LS states:

SA2 Answer on the two logical gNB solution:

SA is happy to endorse the two logical gNB solutions as there is no impact to existing behaviour in the CN and systemwide.

SA2 Answer on the single gNB solutions:

There is no consensus on the support of a single logical gNB solution in SA2.

Based on the SA2 reply LS, it is proposed to update the RAN3 Stage-2 spec to capture the two logical gNB solution, and stop the discussion on single gNB solution.

Proposal 2-1: agree the proposed TP to support WAB-gNB mobility with change of UE’s AMF in ([3])

PCI (re-)configuration for WAB

When the mobile WAB moves (with or without change of UE’s AMF), there may be a potential PCI collision between the cells from different WAB, or between WAB’s cell and a stationary gNB-DU’s cell. The WAB may need to change PCI for its cell. The connected UE will see the different cell before and after the PCI change. HO procedure can be used to move the UE from the “old” WAB cell using an old PCI and the “new” WAB cell using the new PCI. It can be considered as inter-cell intra-gNB HO. To support this HO, the WAB may need to have both “old” cell and “new” cell activate simultaneously.  This is similar to Rel-18 mobile IAB-DU migration that the UE is HO from the source logical IAB-DU’s cell to the co-located target logical IAB-DU’s cell.  After all connected UEs are HO to the new cell, the WAB-gNB may deactivate the old cell.

Observation 2-1: To support for PCI change of a WAB’s cell, the WAB need to have two logical cells active simultaneously. The connected UE can be HO from the old cell to the new cell.

Proposal 2-2: To support the mobility of WAB-node with the change of UE’s AMF, and the support for PCI change, the WAB-gNB need to have two logical cells active simultaneously.

# 4	Change of backhaul resources during mobility

WAB-gNB resources/interfaces, e.g. NG, OAM, Xn, and on-board UE sessions may be mapped to different backhaul PDU sessions and network slices. During WAB-node mobility, the availability of different network slices and/or QoS resources from different BH-gNBs could change, which could impact the backhaul quality of the WAB-node and hence the resources of the WAB-gNB and the UEs connected to it (note that this problem could even happen when WAB is not moving, e.g. if the serving BH-gNB is experiencing congestion and needs to scale down BH resources).

Depending on which slices/resources are impacted the problem can be more severe. For example, if a slice or PDU session associated with the WAB-gNB’s NG interface can no longer be supported, there would be loss of connection to the UE’s 5GC, in which case both the WAB-gNB and the on-board UEs would be disconnected suddenly.

Observation 3-1: During WAB-node mobility, WAB-gNB resources can be degraded due different QoS resources and/or network slices that can be supported at BH-gNBs.

Therefore, it would be beneficial for the BH-gNB to be able to indicate to the WAB-gNB when backhaul service requirements can no longer be maintained so that the WAB-gNB can react accordingly, e.g. free up resources by releasing Xn with neighbouring nodes, handing over UEs to surrounding gNBs, remapping not supported slices/resources to supported slices/resources, etc.

Proposal 3-1: BH-gNB can indicate to the WAB-gNB that specific BH resources cannot be maintained.

In order to make this determination, it would be useful for the BH-gNB to know which slices/PDU sessions are associated with specific resources at the WAB-gNB (or to at least have some awareness of which slices/sessions should be prioritized). During handover preparation, such information could be used by the source BH-gNB to determine whether a candidate target BH-gNB can maintain BH service and inform the WAB-gNB if QoS is expected to be degraded.

Proposal 3-2: BH-gNB is aware which slices/PDU sessions are associated with specific resources at the WAB-gNB and may use this information to determine whether a target BH-gNB is able to maintain BH resources for the WAB-gNB.

# 5	PCI collision avoidance

For PCI collision detection and avoidance with WAB deployment, there are similar options as in legacy systems as well as with mobile IAB. These can be the served cell and neighbour cell information exchange over Xn which can indicate PCIs in the surrounding cells. Additionally, UE ANR measurements using the legacy procedures can be used to detect unknown cells. The ANR procedure can be initiated by the WAB-gNB but also the WAB-MT itself can scan the cells to detect new cells when moving. All, these options are supported by existing specifications.

Furthermore, like considered for mIAB, there could be a split of PCI value ranges for fixed and mobile cells which prevents the collisions between fixed and mobile cells. The remaining problem would be the potential PCI collision between the WAB cells. However, also for such case the legacy features can be used to prevent the collisions.

Observation 4-1: Legacy procedures are applicable for PCI collision detection for WAB deployments.

If a collision is detected, or there is a risk of PCI collision, WAB-gNB can be configured with a new PCI by OAM. The PCI change would be a cell change for the UEs despite connecting to the same node. The WAB-gNB can instantiate another logical cell with the new PCI and UEs connected to the WAB-gNB can be handed over from the old cell to the new cell. The procedure in this case can be identical to the mobility scenarios e.g. when the AMF changes and  a new cell (or logical gNB) is configured to hand over UEs to the new cell/gNB.

Observation 4-2: The PCI change can apply the WAB mobility procedure where new (logical) cell is activated and UEs are handed over between old and new cell.

Proposal 4: No WAB specific enhancements are needed for PCI collision avoidance.

# 6	Multi-hop avoidance

Last meeting agreed to further discuss whether Solution 1 or Solution 3 need to be selected.

To be further discussed whether Solution 1 or Solution 3 (or both) need to be selected

1. For initial access, WAB-gNB may use dedicated frequencies and/or PCIs and potential other legacy OTA parameters (e.g. NR CGI), to ensure that the WAB-MTs of other WAB-nodes avoid (re)selecting the WAB-gNB cells.

3. For initial access, WAB-gNB-cells broadcast a new indicator in SIB to bar WAB-MT, and the WAB-MT avoids (re)selection of cells broadcasting this indicator.

For Solution 1: It is a deployment choice to use dedicated frequency and/or PCI. Rel-18 agreed “The PCI space can be partitioned between mobile IAB cells and stationary cells by implementation.” in TS 38.401. It can be similar for WAB that PCI space is separate for WAB cells and stationary cells by implementation.

In addition, there are other existing methods can be used, for example, operator can use existing UAC to prevent a WAB-MT access a WAB-gNB. Also, at the initial deployment, there are limited number of WABs deployed. So it is also possible to configure the WAB node with a forbidden list, e.g. the cell IDs of WAB-gNBs, thus the WAB-MT can avoid connect with the WAB cells.  There is no showstopper for Solution 1, even it may add some restriction.

For Solution 3: a WAB-gNB cell broadcast a new indication in SIB. By this indication, WAB-MT does not connect with the WAB-gNB. This option is similar to legacy IAB. However, this Option requires AS changes, and should be discussed in RAN2. It further requires mechanisms to disallow handover of WAB-MT to a WAB-gNB.

In a summary, our preference is Solution 1.

Proposal 5: For multi-hop avoidance, RAN3 adopt Solution 1.

# 7	Additional ULI

RAN3 received a reply LS ([4]) from SA2 on Additional ULI. The agreed SA2 23.501 CR 5968 states:

The MWAB-gNB shall also provide the Additional User Location Information (Additional ULI) that reflects the location of the MWAB-UE to the AMF when it sends the User Location Information (ULI) over N2 messages. In the Additional ULI, the MWAB-gNB provides the TAI/NR CGI of the MWAB Broadcasted PLMN/SNPN with which the UE is registered as specified in TS 38.401 [42].

NOTE:	For the case of MWAB broadcasted PLMN/SNPN being different from the BH PLMN, MWAB-gNB formulates the Additional ULI as specified in TS 38.401 [42] based on MWABs geo-location. The MWAB can determine its geo-location using GNSS or via a MWAB-UE initiated 5GC-MO-LR procedures as specified in TS 23.273 [87]. If the MWAB broadcasted PLMN/SNPN is the same as the BH PLMN, then the TAI/NR CGI of the cell serving the MWAB-UE is included in the Additional ULI as specified in TS 38.401 [42].

Stage-2 TP is needed to clarify how the WAB-gNB determines the Additional ULI. TS 38.401 TP can be found in ([3]).

TS 38.413 TP can be found in Annex B – Text proposal for TS 38.413

Proposal 6: Agree the Stage-2 TP in ([4]) and Stage-3 TP in Annex B TP to support Additional ULI.

# 8	Radio-resource coordination between access and backhaul links

As noted earlier, resource coordination between access and BH links are relevant only for in-band operation of WAB-MT and WAB-gNB. Out-of-band deployment inherently provides isolation between access and BH and both links can be operated independently without need for coordination.

Observation 7-1: WAB resource coordination is not needed for out-of-band deployments.

WAB in-band deployment corresponds to that of IAB and it is acknowledged that IAB capabilities can be the basis for WAB as well. Legacy IAB releases provide signalling options for variety of resource usage scenarios.

WAB, however, may have following specific assumptions that could differ from IAB. E.g. it could be desirable that the BH network would require no or limited enhancements implemented specifically for WAB support. This is different from IAB where the donor nodes must be IAB capable e,g, for routing (BAP) support. IAB donor is therefore always a specific implementation even based on an existing gNB. BH network for WAB, on the other hand, can consider IAB type features optional as the BH connection itself is a legacy PDU session without additional requirements for the radio interface.

For radio resource coordination, (IAB) Xn signalling supports the exchange of cell resource configurations (in gNB-DU Cell Resource Configuration) including the DL/UL/F configuration (TDD pattern) as well as H/S/NA configuration for the DU (access) resources. This signalling can be re-used also for WAB deployments when Xn is established between the BH-gNB and the WAB-gNB. The requirement is that the BH-gNB knows the co-location of WAB-MT and WAB-gNB, see the discussion below.

Observation 7-2: Xn signaling to exchange cell resource configurations, with DUF and HSNA configurations, can be reused for WAB deployments.

As a simple example for resource configurations, the BH-gNB could have a split of HARD and NA configuration for DL and UL resources while the WAB-gNB has the opposite HARD/NA configuration. Such configurations can come from OAM or they could be fixed for certain WAB deployment. The lower layer control (e.g. DCI for explicit indication of resource availability) as specified for IAB can be left up to the degree how much WAB specific features are implemented in the BH-gNB which also the WAB implementation supports. As there are no RAN1 involvement, no WAB related features can be specified. Lower layer control is also beyond RAN3 scope. Also F1 is out of the WI scope.

Proposal 7-1: Xn signalling to exchange cell resource configurations specified for IAB can be made applicable to WAB. F1 enhancements and lower layer control is out of the WI scope.

Detection of co-location of WAB-MT and WAB-gNB

For BH-gNB serving WAB-MT, it is beneficial in some scenarios to detect the co-location of the WAB-MT and WAB-gNB. For example, the BH-gNB may use this information, e.g. for resource multiplexing handling. In IAB case, the Donor detects the co-location based on the IAB-MT’s BAP address received in the F1 SETUP REQUEST message. In WAB, the co-location detection may be performed via the Xn interface. For example, when the WAB-gNB initiates the Xn Setup with the gNB serving WAB-MT, the WAB-gNB may include an indication in the XN SETUP REQUEST message. The indication may be a UE ID (e.g. C-RNTI) received from the BH-gNB during the RRC establishment procedure. The detail of the UE ID for WAB-MT can be further discussed.

### Note that RAN3 TR also includes the following:

### For resource coordination between the access link and backhaul link, the BH-gNB may need to discover co-location of the WAB-MT and the WAB-gNB.

Proposal 7-2: The BH-gNB detects the co-location of the WAB-gNB and WAB-MT during the XnAP Setup procedure initiated by the WAB-gNB, via the WAB-MT’s ID (e.g. C-RNTI) received from the WAB-gNB in XnAP SETUP REQUEST message or XnAP SETUP RESPONSE message.

The co-location detection is also needed when the WAB-MT is handover a neighboring BH-gNB that already have Xn established with the WAB-gNB. WAB-gNB can inform the neighboring BH-gNBs about the WAB-MT co-location via the XnAP NG-RAN node Configuration Update procedure.

Proposal 7-3: The BH-gNB can also detect the co-location of the WAB-gNB and WAB-MT during the XnAP NG-RAN node Configuration Update procedure initiated by the WAB-gNB, via the WAB-MT’s ID (e.g. C-RNTI) received from the WAB-gNB in XnAP NG-RAN NODE CONFIGURATION UPDATE message.

TP for TS 38.423 can be found in Annex A – Text proposal for TS 38.423

# 9	Conclusion

In this contribution we have analyzed WAB mobility from RAN3 perspective.

Proposal 1: Update Stage-2 to capture the WAB-node mobility without change of UE’s AMF.

The TP can be found in ([3])

Proposal 2-1: agree the proposed TP to support WAB-gNB mobility with change of UE’s AMF in ([3])

Observation 2-1: To support for PCI change of a WAB’s cell, the WAB need to have two logical cells active simultaneously. The connected UE can be HO from the old cell to the new cell.

Proposal 2-2: To support the mobility of WAB-node with the change of UE’s AMF, and the support for PCI change, the WAB-gNB need to have two logical cells active simultaneously.

Observation 3-1: During WAB-node mobility, WAB-gNB resources can be degraded due different QoS resources and/or network slices that can be supported at BH-gNBs.

Proposal 3-1: BH-gNB can indicate to the WAB-gNB that specific BH resources cannot be maintained.

Proposal 3-2: BH-gNB is aware which slices/PDU sessions are associated with specific resources at the WAB-gNB and may use this information to determine whether a target BH-gNB is able to maintain BH resources for the WAB-gNB.

Observation 4-1: Legacy procedures are applicable for PCI collision detection for WAB deployments.

Observation 4-2: The PCI change can apply the WAB mobility procedure where new (logical) cell is activated and UEs are handed over between old and new cell.

Proposal 4: No WAB specific enhancements are needed for PCI collision avoidance.

Proposal 5: For multi-hop avoidance, RAN3 adopt Solution 1.

Proposal 6: Agree the Stage-2 TP in ([4]) and Stage-3 TP in Annex B TP to support Additional ULI.

Proposal 7-1: Xn signalling to exchange cell resource configurations specified for IAB can be made applicable to WAB. F1 enhancements and lower layer control is out of the WI scope.

Proposal 7-2: The BH-gNB detects the co-location of the WAB-gNB and WAB-MT during the XnAP Setup procedure initiated by the WAB-gNB, via the WAB-MT’s ID (e.g. C-RNTI) received from the WAB-gNB in XnAP SETUP REQUEST message or XnAP SETUP RESPONSE message.

Proposal 7-3: The BH-gNB can also detect the co-location of the WAB-gNB and WAB-MT during the XnAP NG-RAN node Configuration Update procedure initiated by the WAB-gNB, via the WAB-MT’s ID (e.g. C-RNTI) received from the WAB-gNB in XnAP NG-RAN NODE CONFIGURATION UPDATE message.

TP for TS 38.423 can be found in Annex A – Text proposal for TS 38.423

# References

- RP-242395, New WID on additional topological enhancements for NR

- TR 38.799 v2.0.0

- R3-250220 (TP for TS 38.401) Discussion on NG management and Xn management for WAB

# Annex A – Text proposal for TS 38.423

### 8.4.1	Xn Setup

#### 8.4.1.1	General

The purpose of the Xn Setup procedure is to exchange application level configuration data needed for two NG-RAN nodes to interoperate correctly over the Xn-C interface.

NOTE 1:	If Xn-C signalling transport is shared among multiple Xn-C interface instances, one Xn Setup procedure is issued per Xn-C interface instance to be setup, i.e. several Xn Setup procedures may be issued via the same TNL association after that TNL association has become operational.

NOTE 2:	Exchange of application level configuration data also applies between two NG-RAN nodes in case the SN (i.e. the gNB) does not broadcast system information other than for radio frame timing and SFN, as specified in the TS 37.340 [8]. How to use this information when this option is used is not explicitly specified.

The procedure uses non UE-associated signalling.

#### 8.4.1.2	Successful Operation

Figure 8.4.1.2: Xn Setup, successful operation

The NG-RAN node1 initiates the procedure by sending the XN SETUP REQUEST message to the candidate NG-RAN node2. The candidate NG-RAN node2 replies with the XN SETUP RESPONSE message.

----- Unmodified part skipped -----

If the Barring Exemption for Emergency Call Information IE is included in the Served Cell Information NR IE in the XN SETUP REQUEST message or the XN SETUP RESPONSE message, the receiving NG-RAN node may use this information to determine a suitable target in case of subsequent outgoing mobility during emergency call.

If the Identity of WAB-MT IE is included in the XN SETUP REQUEST message or in the XN SETUP RESPONSE message, the receiving NG-RAN node shall, if supported, consider the information therein for discovering the co-location of a WAB-gNB and a WAB-MT.

Interactions with other procedures:

If the NG-RAN node1 receives a XN SETUP RESPONSE message containing a Local NG-RAN Node Identifier identical to the Local NG-RAN Node Identifier included in the corresponding XN SETUP REQUEST message, the NG-RAN node1 may initiate the NG-RAN node Configuration Update procedure including in the NG-RAN NODE CONFIGURATION UPDATE message a new Local NG-RAN Node Identifier, different from the Local NG-RAN Node Identifier of each of its neighbour NG-RAN Nodes.

----- Next Change -----

### 8.4.2	NG-RAN node Configuration Update

#### 8.4.2.1	General

The purpose of the NG-RAN node Configuration Update procedure is to update application level configuration data needed for two NG-RAN nodes to interoperate correctly over the Xn-C interface.

NOTE:	Update of application level configuration data also applies between two NG-RAN nodes in case the SN (i.e. the gNB) does not broadcast system information other than for radio frame timing and SFN, as specified in the TS 37.340 [8]. How to use this information when this option is used is not explicitly specified.

The procedure uses non UE-associated signalling.

#### 8.4.2.2	Successful Operation

Figure 8.4.2.2-1: NG-RAN node Configuration Update, successful operation

The NG-RAN node1 initiates the procedure by sending the NG-RAN NODE CONFIGURATION UPDATE message to a peer NG-RAN node2.

----- Unmodified part skipped -----

If the TAI NSAG Support List IE is contained in the NG-RAN NODE CONFIGURATION UPDATE message, the NG-RAN node shall, if supported, take this IE into account for slice aware cell reselection.

If the TAI Slice Unavailable Cell List IE is contained in the NG-RAN NODE CONFIGURATION UPDATE message, the NG-RAN node2 shall, if supported, take this IE into account to deduce slice resource allocation.

If the Identity of WAB-MT IE is included in the NG-RAN NODE CONFIGURATION UPDATE message, the receiving NG-RAN node shall, if supported, consider the information therein for discovering the co-location of a WAB-gNB and a WAB-MT.

Update of Served Cell Information NR:

-	If Served Cells NR To Add IE is contained in the NG-RAN NODE CONFIGURATION UPDATE message, NG-RAN node2 shall add cell information according to the information in the Served Cell Information NR IE.

-	If Served Cells NR To Modify IE is contained in the NG-RAN NODE CONFIGURATION UPDATE message, NG-RAN node2 shall modify information of cell indicated by Old NR-CGI IE according to the information in the Served Cell Information NR IE.

----- Next Change -----

#### 9.1.3.1	XN SETUP REQUEST

This message is sent by a NG-RAN node to a neighbouring NG-RAN node to transfer application data for an Xn-C interface instance.

Direction: NG-RAN node1 à NG-RAN node2.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.2.3.1 |  | YES | reject |
| Global NG-RAN Node ID | M |  | 9.2.2.3 |  | YES | reject |
| TAI Support List | M |  | 9.2.3.20 | List of supported TAs and associated characteristics. | YES | reject |
| AMF Region Information | M |  | 9.2.3.83 | Contains a list of all the AMF Regions to which the NG-RAN node belongs. | YES | reject |
| List of Served Cells NR |  | 0 .. <maxnoofCellsinNG-RAN node> |  | Contains a list of cells served by the gNB. If a partial list of cells is signalled, it contains at least one cell per carrier configured at the gNB | YES | reject |
| >Served Cell Information NR | M |  | 9.2.2.11 |  | – |  |
| >Neighbour Information NR | O |  | 9.2.2.13 |  | – |  |
| >Neighbour Information E-UTRA | O |  | 9.2.2.14 |  | – |  |
| >Served Cell Specific Info Request | O |  | 9.2.2.102 |  | YES | ignore |
| List of Served Cells E-UTRA |  | 0 .. <maxnoofCellsinNG-RAN node> |  | Contains a list of cells served by the ng-eNB. If a partial list of cells is signalled, it contains at least one cell per carrier configured at the ng-eNB | YES | reject |
| >Served Cell Information E-UTRA | M |  | 9.2.2.12 |  | – |  |
| >Neighbour Information NR | O |  | 9.2.2.13 |  | – |  |
| >Neighbour Information E-UTRA | O |  | 9.2.2.14 |  | – |  |
| >SFN Offset | O |  | 9.2.2.75 | Associated with the ECGI IE in the Served Cell Information E-UTRA IE | YES | ignore |
| Interface Instance Indication | O |  | 9.2.2.39 |  | YES | reject |
| TNL Configuration Info | O |  | 9.2.3.96 |  | YES | ignore |
| Partial List Indicator NR | O |  | Partial List Indicator 9.2.2.46 | Value “partial” indicates that a partial list of cells is included in the List of Served Cells NR IE. | YES | ignore |
| Cell and Capacity Assistance Information NR | O |  | 9.2.2.41 | Contains NR cell related assistance information. | YES | ignore |
| Partial List Indicator E-UTRA | O |  | Partial List Indicator 9.2.2.46 | Value “partial” indicates that a partial list of cells is included in the List of Served Cells E-UTRA. | YES | ignore |
| Cell and Capacity Assistance Information E-UTRA | O |  | 9.2.2.42 | Contains E-UTRA cell related assistance information. | YES | ignore |
| Local NG-RAN Node Identifier | O |  | 9.2.2.101 |  | YES | ignore |
| Neighbour NG-RAN Node List |  | 0..<maxnoofNeighbourNG-RAN nodes> |  |  | YES | ignore |
| >Global NG-RAN Node ID | M |  | 9.2.2.3 |  | – |  |
| >Local NG-RAN Node Identifier | M |  | 9.2.2.101 |  | – |  |
| Identity of WAB-MT | O |  | x | Contains the identity of the WAB-MT assigned by the BH-gNB | YES | ignore |

| Range bound | Explanation |
| --- | --- |
| maxnoofCellsinNG-RAN node | Maximum no. cells that can be served by a NG-RAN node. Value is 16384. |
| maxnoofNeighbourNG-RAN nodes | Maximum no. of neighbour NG-RAN nodes. Value is 256. |

#### 9.1.3.2	XN SETUP RESPONSE

This message is sent by a NG-RAN node to a neighbouring NG-RAN node to transfer application data for an Xn-C interface instance.

Direction: NG-RAN node2 à NG-RAN node1.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.2.3.1 |  | YES | reject |
| Global NG-RAN Node ID | M |  | 9.2.2.3 |  | YES | reject |
| TAI Support List | M |  | 9.2.3.20 | List of supported TAs and associated characteristics. | YES | reject |
| List of Served Cells NR |  | 0 .. <maxnoofCellsinNG-RAN node> |  | Contains a list of cells served by the gNB. If a partial list of cells is signalled, it contains at least one cell per carrier configured at the gNB | YES | reject |
| >Served Cell Information NR | M |  | 9.2.2.11 |  | – |  |
| >Neighbour Information NR | O |  | 9.2.2.13 |  | – |  |
| >Neighbour Information E-UTRA | O |  | 9.2.2.14 |  | – |  |
| >Served Cell Specific Info Request | O |  | 9.2.2.102 | This IE is not used in this version of the specification. | YES | ignore |
| List of Served Cells E-UTRA |  | 0 .. <maxnoofCellsinNG-RAN node> |  | Contains a list of cells served by the ng-eNB. If a partial list of cells is signalled, it contains at least one cell per carrier configured at the gNB | YES | reject |
| >Served Cell Information E-UTRA | M |  | 9.2.2.12 |  | – |  |
| >Neighbour Information NR | O |  | 9.2.2.13 |  | – |  |
| >Neighbour Information E-UTRA | O |  | 9.2.2.14 |  | – |  |
| >SFN Offset | O |  | 9.2.2.75 | Associated with the ECGI IE in the Served Cell Information E-UTRA IE | YES | ignore |
| Criticality Diagnostics | O |  | 9.2.3.3 |  | YES | ignore |
| AMF Region Information | O |  | 9.2.3.83 | Contains a list of all the AMF Regions to which the NG-RAN node belongs. | YES | reject |
| Interface Instance Indication | O |  | 9.2.2.39 |  | YES | reject |
| TNL Configuration Info | O |  | 9.2.3.96 |  | YES | ignore |
| Partial List Indicator NR | O |  | Partial List Indicator 9.2.2.46 | Value “partial” indicates that a partial list of cells is included in the List of Served Cells NR IE. | YES | ignore |
| Cell and Capacity Assistance Information NR | O |  | 9.2.2.41 | Contains NR cell related assistance information. | YES | ignore |
| Partial List Indicator E-UTRA | O |  | Partial List Indicator 9.2.2.46 | Value “partial” indicates that a partial list of cells is included in the List of Served Cells E-UTRA. | YES | ignore |
| Cell and Capacity Assistance Information E-UTRA | O |  | 9.2.2.42 | Contains E-UTRA cell related assistance information. | YES | ignore |
| Local NG-RAN Node Identifier | O |  | 9.2.2.101 |  | YES | ignore |
| Neighbour NG-RAN Node List |  | 0..<maxnoofNeighbourNG-RAN nodes> |  |  | YES | ignore |
| >Global NG-RAN Node ID | M |  | 9.2.2.3 |  | – |  |
| >Local NG-RAN Node Identifier | M |  | 9.2.2.101 |  | – |  |
| Identity of WAB-MT | O |  | x | Contains the identity of the WAB-MT assigned by the BH-gNB | YES | ignore |

| Range bound | Explanation |
| --- | --- |
| maxnoofCellsinNG-RAN node | Maximum no. cells that can be served by a NG-RAN node. Value is 16384. |
| maxnoofNeighbourNG-RAN nodes | Maximum no. of neighbour NG-RAN nodes. Value is 256. |

----- Next Change -----

#### 9.1.3.4	NG-RAN NODE CONFIGURATION UPDATE

This message is sent by a NG-RAN node to a neighbouring NG-RAN node to transfer updated information for an Xn-C interface instance.

Direction: NG-RAN node1 à NG-RAN node2.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.2.3.1 |  | YES | reject |
| TAI Support List | O |  | 9.2.3.20 | List of supported TAs and associated characteristics. | GLOBAL | reject |
| CHOICE Initiating NodeType | M |  |  |  | YES | ignore |
| >gNB |  |  |  |  |  |  |
| >>Served Cells To Update NR | O |  | 9.2.2.15 |  | YES | ignore |
| >>Cell Assistance Information NR | O |  | 9.2.2.17 |  | YES | ignore |
| >>Cell Assistance Information E-UTRA | O |  | 9.2.2.43 |  | YES | ignore |
| >>Served Cell Specific Info Request | O |  | 9.2.2.102 |  | YES | ignore |
| >ng-eNB |  |  |  |  |  |  |
| >>Served Cells to Update E-UTRA | O |  | 9.2.2.16 |  | YES | ignore |
| >>Cell Assistance Information NR | O |  | 9.2.2.17 |  | YES | ignore |
| >>Cell Assistance Information E-UTRA | O |  | 9.2.2.43 |  | YES | ignore |
| TNLA To Add List |  | 0..1 |  |  | YES | ignore |
| >TNLA To Add Item |  | 1..<maxnoofTNLAssociations> |  |  | – |  |
| >>TNLA Transport Layer Information | M |  | CP Transport Layer Information 9.2.3.31 | CP Transport Layer Information of NG-RAN node1 | – |  |
| >>TNL Association Usage | M |  | 9.2.3.84 |  | – |  |
| TNLA To Update List |  | 0..1 |  |  | YES | ignore |
| >TNLA To Update Item |  | 1..<maxnoofTNLAssociations> |  |  | – |  |
| >>TNLA Transport Layer Information | M |  | CP Transport Layer Information 9.2.3.31 | CP Transport Layer Information of NG-RAN node1 | – |  |
| >>TNL Association Usage | O |  | 9.2.3.84 |  | – |  |
| TNLA To Remove List |  | 0..1 |  |  | YES | ignore |
| >TNLA To Remove Item |  | 1..<maxnoofTNLAssociations> |  |  | – |  |
| >>TNLA Transport Layer Information | M |  | CP Transport Layer Information 9.2.3.31 | CP Transport Layer Information of NG-RAN node1 | – |  |
| Global NG-RAN Node ID | O |  | 9.2.2.3 |  | YES | reject |
| AMF Region Information To Add | O |  | AMF Region Information 9.2.3.83 | List of all added AMF Regions to which the NG-RAN node belongs. | YES | reject |
| AMF Region Information To Delete | O |  | AMF Region Information 9.2.3.83 | List of all deleted AMF Regions to which the NG-RAN node belongs. | YES | reject |
| Interface Instance Indication | O |  | 9.2.2.39 |  | YES | reject |
| TNL Configuration Info | O |  | 9.2.3.96 |  | YES | ignore |
| Coverage Modification List |  | 0 .. 1 |  | List of cells with modified coverage. | GLOBAL | reject |
| >Coverage Modification Item |  | 0 .. <maxnoofCellsinNG-RAN node> |  |  | – |  |
| >>Global NG-RAN Cell Identity | M |  | Global Cell Identity 9.2.2.73 | Global Cell Identity of the cell to be modified. In this version of the specification, only a NG-RAN cell identifier can be included. | – |  |
| >>Cell Coverage State | M |  | INTEGER (0..63, …) | Value ‘0’ indicates that the cell is inactive. Other values Indicates that the cell is active and also indicates the coverage configuration of the concerned cell. | – |  |
| >>Cell Deployment Status Indicator | O |  | ENUMERATED(pre-change-notification, ...) | Indicates the Cell Coverage State is planned to be used at the next reconfiguration. | – |  |
| >>Cell Replacing Info | C-ifCellDeploymentStatusIndicatorPresent |  |  |  | – |  |
| >>>Replacing Cells |  | 0 .. <maxnoofCellsinNG-RAN node> |  |  | – |  |
| >>>>Global NG-RAN Cell Identity |  |  | Global Cell Identity 9.2.2.73 | Global Cell Identity of a cell that may replace all or part of the coverage of the cell to be modified. In this version of the specification, only a NG-RAN cell identifier can be included. | – |  |
| >>SSB Coverage Modification List |  | 0..1 |  | List of SSB beams with modified coverage. | – |  |
| >>>SSB Coverage Modification Item |  | 0..<maxnoofSSBAreas> |  |  | – |  |
| >>>>SSB Index | M |  | INTEGER (0..63) | Identifier of the SSB beam to be modified. | – |  |
| >>>>SSB Coverage State | M |  | INTEGER (0..15, …) | Value ‘0’ indicates that the SSB beam is inactive. Other values Indicates that the SSB beam is active and also indicates the coverage configuration of the concerned SSB beam. | – |  |
| >>Coverage Modification Cause | O |  | ENUMERATED (coverage, cell edge capacity, ..., network energy saving) | Indicates the reason for the coverage modification in NG-RAN node1. | YES | ignore |
| Local NG-RAN Node Identifier | O |  | 9.2.2.101 |  | YES | ignore |
| Neighbour NG-RAN Node List |  | 0..<maxnoofNeighbourNG-RAN nodes> |  |  | YES | ignore |
| >Global NG-RAN Node ID | M |  | 9.2.2.3 |  | – |  |
| >Local NG-RAN Node Identifier | M |  | 9.2.2.101 |  | – |  |
| Local NG-RAN Node Identifier Removal | O |  | Local NG-RAN Node Identifier 9.2.2.101 |  | YES | ignore |
| Identity of WAB-MT | O |  | x | Contains the identity of the WAB-MT assigned by the BH-gNB | YES | ignore |

| Range bound | Explanation |
| --- | --- |
| maxnoofTNLAssociations | Maximum numbers of TNL Associations between the NG RAN nodes. Value is 32. |
| maxnoofCellsinNG-RAN node | Maximum no. cells that can be served by a NG-RAN node. Value is 16384. |
| maxnoofSSBAreas | Maximum no. SSB Areas that can be served by a cell. Value is 64. |
| maxnoofNeighbourNG-RAN nodes | Maximum no. of neighbour NG-RAN nodes. Value is 256. |

| Condition | Explanation |
| --- | --- |
| ifCellDeploymentStatusIndicatorPresent | This IE shall be present if the Cell Deployment Status Indicator IE is present. |

# Annex B – Text proposal for TS 38.413

The following TP proposed the Additional ULI for WAB.

----- Start of Change -----

#### 9.3.1.16	User Location Information

This IE is used to provide location information of the UE.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| CHOICE User Location Information | M |  |  |  | - |  |
| >E-UTRA user location information |  |  |  |  |  |  |
| >>E-UTRA CGI | M |  | 9.3.1.9 |  | - |  |
| >>TAI | M |  | 9.3.3.11 |  | - |  |
| >>Age of Location | O |  | Time Stamp 9.3.1.75 | Indicates the UTC time when the location information was generated. | - |  |
| >>PSCell Information | O |  | NG-RAN CGI 9.3.1.73 |  | YES | ignore |
| >NR user location information |  |  |  |  |  |  |
| >>NR CGI | M |  | 9.3.1.7 |  | - |  |
| >>TAI | M |  | 9.3.3.11 | This IE is ignored if the NR NTN TAI Information IE is present. | - |  |
| >>Age of Location | O |  | Time Stamp 9.3.1.75 | Indicates the UTC time when the location information was generated. | - |  |
| >>PSCell Information | O |  | NG-RAN CGI 9.3.1.73 |  | YES | ignore |
| >>NID | O |  | 9.3.3.42 |  | YES | reject |
| >>NR NTN TAI Information | O |  | 9.3.3.53 |  | YES | ignore |
| >>Mobile IAB-MT User Location Information | O |  | 9.3.1.260 | Indicates the user location information of a mobile IAB-MT, which is co-located with the mobile IAB-DU which serves the UE. | YES | ignore |
| >>WAB-MT User Location Information | O |  | 9.3.1.x | Indicates the user location information of a WAB-MT, which is co-located with the WAB-gNB which serves the UE. | YES | ignore |
| >N3IWF user location information with port number |  |  |  |  |  |  |
| >>IP Address | M |  | Transport Layer Address 9.3.2.4 | UE's local IP address used to reach the N3IWF | - |  |
| >>Port Number | M |  | OCTET STRING (SIZE(2)) | UDP source port number if NAT is detected. | - |  |
| >>TAI | O |  | 9.3.3.11 |  | YES | ignore |
| >TNGF user location information |  |  |  |  | YES | ignore |
| >>TNAP ID | M |  | OCTET STRING | TNAP Identifier used to identify the TNAP. Details in TS 29.571 [35]. | - |  |
| >>IP Address | M |  | Transport Layer Address 9.3.2.4 | UE's local IP address used to reach the TNGF. | - |  |
| >>Port Number | O |  | OCTET STRING (SIZE(2)) | UDP source port number if NAT is detected. | - |  |
| >>TAI | O |  | 9.3.3.11 |  | YES | ignore |
| >TWIF user location information |  |  |  |  | YES | ignore |
| >>TWAP ID | M |  | OCTET STRING | TWAP Identifier used to identify the TWAP. Details in TS 29.571 [35]. | - |  |
| >>IP Address | M |  | Transport Layer Address 9.3.2.4 | Non-5G-Capable over WLAN device's local IP address used to reach the TWIF. | - |  |
| >>Port Number | O |  | OCTET STRING (SIZE(2)) | UDP source port number if NAT is detected. | - |  |
| >>TAI | O |  | 9.3.3.11 |  | YES | ignore |
| >W-AGF user location information |  |  |  | Indicates the location information via wireline access as specified in TS 23.316 [34]. | YES | ignore |
| >>W-AGF user location information | M |  | 9.3.1.164 |  | - |  |
| >N3IWF user location information without port number |  |  |  |  | YES | ignore |
| >>IP Address | M |  | Transport Layer Address 9.3.2.4 | UE's local IP address used to reach the N3IWF. | - |  |
| >>TAI | O |  | 9.3.3.11 |  | - |  |

----- Next Change -----

#### 9.3.1.x	WAB-MT User Location Information

This IE contains the NR CGI and the TAI reflecting the location of the WAB  which serves the UE.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| NR CGI | M |  | 9.3.1.7 |  |
| TAI | M |  | 9.3.3.11 |  |

----- Next Change -----

### 9.4.5	Information Element Definitions

-- ASN1START

-- **************************************************************

--

-- Information Element Definitions

--

-- **************************************************************

NGAP-IEs {

itu-t (0) identified-organization (4) etsi (0) mobileDomain (0)

ngran-Access (22) modules (3) ngap (1) version1 (1) ngap-IEs (2) }

DEFINITIONS AUTOMATIC TAGS ::=

<<<<<<<<<<<<<<<<<<<< Unaffected part is skipped >>>>>>>>>>>>>>>>>>>>

id-QoERVQoEReportingPaths,

id-UserLocationInformationN3IWF-without-PortNumber,

id-WAB-MTUserLocationInformation,

maxnoofAllowedAreas,

maxnoofAllowedCAGsperPLMN,

<<<<<<<<<<<<<<<<<<<< Unaffected part is skipped >>>>>>>>>>>>>>>>>>>>

UserLocationInformationNR ::= SEQUENCE {

nR-CGI				NR-CGI,

tAI					TAI,

timeStamp			TimeStamp															OPTIONAL,

iE-Extensions		ProtocolExtensionContainer { {UserLocationInformationNR-ExtIEs} }	OPTIONAL,

...

}

UserLocationInformationNR-ExtIEs NGAP-PROTOCOL-EXTENSION ::= {

{ ID id-PSCellInformation						CRITICALITY ignore	EXTENSION NGRAN-CGI											PRESENCE optional	}|

{ ID id-NID										CRITICALITY reject	EXTENSION NID												PRESENCE optional	}|

{ ID id-NRNTNTAIInformation						CRITICALITY ignore	EXTENSION NRNTNTAIInformation 							PRESENCE optional	}|

{ ID id-MobileIAB-MTUserLocationInformation		CRITICALITY ignore	EXTENSION MobileIAB-MTUserLocationInformation 		PRESENCE optional	}|

{ ID id-WAB-MTUserLocationInformation			CRITICALITY ignore	EXTENSION WAB-MTUserLocationInformation 				PRESENCE optional	},

...

}

<<<<<<<<<<<<<<<<<<<< Unaffected part is skipped >>>>>>>>>>>>>>>>>>>>

-- W

W-AGF-ID ::= CHOICE {

w-AGF-ID				BIT STRING (SIZE(16, ...)),

choice-Extensions		ProtocolIE-SingleContainer { {W-AGF-ID-ExtIEs} }

}

W-AGF-ID-ExtIEs NGAP-PROTOCOL-IES ::= {

...

}

WarningAreaCoordinates ::= OCTET STRING (SIZE(1..1024))

WAB-MTUserLocationInformation ::= SEQUENCE {

nRCGI							NR-CGI,

tAI								TAI,

iE-Extensions					ProtocolExtensionContainer { { WAB-MTUserLocationInformation-ExtIEs} }	OPTIONAL,

...

}

WAB-MTUserLocationInformation-ExtIEs NGAP-PROTOCOL-EXTENSION ::= {

...

}

WarningAreaCoordinates ::= OCTET STRING (SIZE(1..1024))

----- Next Change -----

### 9.4.7	Constant Definitions

-- ASN1START

-- **************************************************************

--

-- Constant definitions

--

-- **************************************************************

NGAP-Constants {

itu-t (0) identified-organization (4) etsi (0) mobileDomain (0)

ngran-Access (22) modules (3) ngap (1) version1 (1) ngap-Constants (4) }

DEFINITIONS AUTOMATIC TAGS ::=

<<<<<<<<<<<<<<<<<<<< Unaffected part is skipped >>>>>>>>>>>>>>>>>>>>

id-UserLocationInformationN3IWF-without-PortNumber		ProtocolIE-ID ::= 439

id-AUN3DeviceAccessInfo									ProtocolIE-ID ::= 440

id-WAB-MTUserLocationInformation						ProtocolIE-ID ::= a1

END

-- ASN1STOP

----- End of Change -----